3GPP TSG SA WG3 (Security) Meeting #86bis	    S3-170808

27-31 March 2017, Busan (Korea)

Source:	NEC

Title:	pCR to TR 33.899: Inter NG (R)AN handover with Xn interface

Document for:	Approval

Agenda Item:	5.1.1

Work Item / Release:	FS_NSA / Rel-14

Abstract: This pCR proposes security procedure of Inter NG (R)AN handover (with Xn interface) for Security Area #1 Security Architecture for NextGen.

#### Introduction

#### This pCR proposes a solution for Inter NG (R)AN handover with Xn interface.

#### Proposal

***** Start of Change *****

#### 5.1.4.x	Solution #1.x Inter NG (R)AN handover with Xn interface

#### 5.1.4.x.1	Introduction

This solution proposes the handover procedure for Inter NG (R)AN (with Xn interface) scenario.

#### 5.1.4.x.2	Solution details

#### Figure 5.1.4.x.2-1 shows the procedure for Inter NG (R)AN handover with Xn interface.

Figure 5.1.4.x.2-1: Inter NG (R)AN handover with Xn interface.

The procedure for Inter NG (R)AN handover (with Xn interface) is as follows.

- If the handover is required, the UE sends the Measurement Report to the source NG (R)AN.

- The source NG (R)AN evaluates the report and makes the handover decision.

- The source NG (R)AN refreshes the AN keys (derives KAN*).

- The source NG (R)AN sends the Handover Request(UE Security Capabilities, Handover Restriction List, NSSAI, KAN*) to the target NG (R)AN via the Xn interface.

- The NG (R)AN checks if the NSSAI can be supported or not.

- If the UE qualifies for accessing the slice via the new NG (R)AN, the target NG (R)AN sends the Handover Request ACK to the source NG (R)AN.

- The source NG (R)AN sends the Handover Command to the UE.

- Using the parameter sent in the Handover Command, the UE derives the KAN* and the subsequent RRC and UP integrity and encryption keys.

- The UE sends the Handover Complete command to the target NG (R)AN.

- The target NG (R)AN sends the Path Switch Request to the AMF.

- The AMF sends the Path Switch Response to the target NG (R)AN.

#### 5.1.4.x.3	Evaluation

#### FFS

***** End of Change *****