3GPP TSG-SA WG3 Meeting #91 	S3-181261

# Belgrade, 16 - 20 April, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fast Re-authentication Process for EAP-AKA’ |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon，China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  |  | Date: |  | 2018-4-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | EAP-AKA’ is one of the primary authentication method supported by TS 33.501. Fast Re-authentication for EAP-AKA' is specified in RFC 5448. And also, in clause 6.3 of TS 33.402, Fast Re-authenticaiton processes for EAP-AKA’ are specified for non-3GPP access. Therefore, in the TS 33.501, fast reauthenticatoin procedure shall be specified also as EAP-AKA’ are supported for both 5G and untrusted non-3GPP access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add in Fast Re-authentication process for EAP-AKA’ |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The load of core networks might be heavy and additional computing resource need to be allocated for authenticaiton. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*************** Start of Change 1 ****************

#### 6.1.3.x	Fast Re-authentication procedure for EAP-AKA’

Fast re-authentication for EAP-AKA' is specified in RFC 5448 [23]. Fast re-authentication re-uses keys derived on the previous full authentication. Fast re-authentication does not involve the ARPF nor the credentials used with EAP-AKA’ (e.g USIM application in case of terminal with 3GPP access capabilities), and does not involve the handling of AKA authentication vectors, which makes the procedure faster and reduces the load on the APRF

UE and AUSF shall implement fast re-authentication for EAP-AKA'. Its use is optional and depends on operator policy. If fast re-authentication for EAP-AKA' is used, the AUSF shall indicate this to the UE by means of sending the re-authentication identity to the UE as in step 3 of subclause 6.13.1.

The security level of fast re-authentication for EAP-AKA' is lower as it does not prove the presence of the credentials used with EAP-AKA’ (e.g. presence of USIM application in case of terminal with 3GPP access capabilities) on the user side. The operator should take this into account when defining the policy on fast re-authentication.

Fast re-authentications for EAP-AKA' generates new keys MSK, which may be used for renewing session key used for protection in the access network.

The access network identity shall not change when going from the full to the fast re-authentication process. If this happens, the re-authentication process shall be terminated as defined in RFC 5448 [23].

In this section it is described how the process works for 5G access to 5GC.

Figure 6.1.3.1-1: Fast Re-uthentication procedure for EAP-AKA'

When UE has a valid key for re-authentication derived from the authentication previously but other keys such as KAMF have been lost, UE can re-establish the security context with network quickly with following procedures:

1) UE sends an EAP Response/Identity containing a re-authentication identity (this identity was previously delivered by AUSF in a full authentication procedure).

2)	The SEAF forwards the EAP Response/Identity to the AUSF.

3)	The AUSF initiates the Counter (which was initialized to one in the full authentication process) and sends it in the EAP Request message, together with the NONCE, the MAC (calculated over the NONCE) and a protected re-authentication ID for a next fast re-authentication. If the AUSF is not able to deliver a re-authentication identity, next time the UE shall force a full-authentication (to avoid the use of the re-authentication identity more than once).

Note 1: The AUSF may send a result indication to the UE, in order to indicate that the success result message at the end of the process shall be protected (if the outcome is successful). The protection of result messages depends on home operator's policies.

Note 2: The AUSF may fail to recognize the identity. In this case, the AUSF may, as in the case of a full authentication, instead perform an EAP AKA' method specific identity request; i.e. "EAP-Request/AKA' identity [Any identity]" in order to obtain a more reliable identity. This should however only be used in case the server fails to recognize the identity, as otherwise the purpose of fast re-authentication is defeated.

4)	The SEAF forwards the EAP Request message to the UE.

5)	The UE verifies that the Counter value is fresh and the MAC is correct, and it sends the EAP Response message with the same Counter value (it is up to the AUSF server to step it up) and a calculated MAC.

Note 3:	The UE shall include in this message the result indication if it received the same indication from the AUSF. Otherwise, the UE shall omit this indication.

6)	The SEAF forwards the response toward the AUSF.

7)	The AUSF verifies that the Counter value is the same as it sent, and the MAC is correct, and sends the message EAP Request/AKA'-Notification, previous to the EAP Success message, if the 3GPP AAA Server requested previously to use protected success result indications. The message EAP Request/AKA'-Notification is MAC protected, and includes an encrypted copy the Counter used in the present re-authentication process.

8)	The SEAF forwards the EAP Request/AKA'-Notification message to the UE.

9)	The UE sends the EAP Response/AKA'-Notification.

10)	The SEAF forwards the EAP Response/AKA'-Notification message toward the AUSF. The AUSF shall ignore the contents of this message.

Note 4: step 7 – 10 are optional.

11)	The AUSF sends an EAP Success message. If some extra keying material was generated for Access Network specific confidentiality and/or integrity protection, then the 3GPP AAA Server includes this derived keying material in the underlying AAA protocol message. (i.e., not at EAP level). The SEAF stores the keying material which may be used in communication with the authenticated UE.

12)	The EAP Success message is forwarded to the UE.

The re-authentication process may fail at any moment, for example because of unsuccessful checking of MACs or no response from the UE after a network request. In that case, the EAP AKA' process will be terminated as specified in RFC 5448 [23] and an indication shall be sent to UDM.

*************** End of Change 1 ****************