# 3GPP TSG SA WG3 (Security) Meeting #94	S3-190238

# Kochi (India), 28 January – 1 February 2019

# Source:	Huawei, HiSilicon

# Title:	Dynamic UP security policy control solution for URLLC

# Document for:	Approval

Agenda Item:	8.13

# 1	Decision/action requested

This contribution proposes a pCR for a solution to TR 33.825.

# 2	References

[1]	3GPP TS 23.725 Study on enhancement of Ultra-Reliable Low-Latency Communcation (URLLC) v0.4.0

[2]            3GPP TS 23.501 System Architecture for the 5G System v15.4.0

[3]            3GPP TS 23.502 Procedures for the 5G System v15.4.0

[4]            3GPP TS 23.503 Policy and Charging Control Framework for the 5G System v15.2.0

# 3	Rationale

According to TS 23.501 [2] and TS 23.502 [3], the SMF should provide User Plane Security Enforcement information based on the UP security policy for a PDU session to the NG-RAN during the PDU session establishment procedure. The UP security policy is stored in the UDM as subscriber profile, which means the network does not consider the requirement from the applications while enforcing UP Security Policy.

According to TS 23.503 [4], the PCF will authorize the PCC rule and PDU Session related information for dynamic session management policy control considering the QoS requirement of the application. Hence, PCF could know the QoS requirement from the application and the RAN capability from the SMF.

The RAN may enforce UP Security Policy in low speed (e.g. while enforcing user plane integrity protection). The RAN serving the UE may be changed during UE mobility. Different RAN may have different capabilities to enforce UP Security Policy. So, the RAN capability should also be considered.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.825.

***	BEGIN CHANGES	***

## 6.X	Solution #X: Dynamic UP security policy control for URLLC service

### 6.X.1	Introduction

This solution addresses Key Issue #4: Security policy for URLLC service.

According to TS 23.501 [xx] and TS 23.502 [xy], the SMF shall provide the UP security policy to the NG-RAN during the PDU session establishment procedure.

According to TS 23.503 [xz], the PCF will authorize the PCC rule and PDU Session related information for dynamic session management policy control considering the QoS requirement of the application.

### 6.X.2	Solution details

The User Plane Secuirty Policy should be authorized by PCF considering RAN Capability for UP Security Policy enforcement and QoS requirement from applications. The PCF may further optimized the UP Security Policy, e.g. chaning “Required” to “Not Needed” or changing “preferred” to “Not needed” The PCF will send the optimizedUP Security Policy to the SMF during PDU Session Establishment.

6.1.3	Evaluation

***	END OF CHANGES	***