3GPP TSG-SA WG3 Meeting #95Bis 	S3-191952

# Sapporo(Japan), 24-28 June 2019																		revision of S3-19xabc

# Source:	Intel

# Title:	Evaluation to Security solution 4 for UL small data transfer in RRC Suspend and Resume with early data transmission (EDT)

# Document for:	Approval

Agenda Item:	8.6

# 1	Decision/action requested

It is requested to approve the evaluation for EDT Small Data transfer solution 4.

# 2	References

[1]	S3-191259, Solution for integrity protection of UL EDT data

# 3	Rationale

In SA3 #95, Intel presented a detailed discussion paper [1]on why the existing solution #4 does not work with the ordering of PDCP PDU. pCR adds evaluation from the discussion paper

# 4	Detailed proposal

********************* Start of Changes**********************

## 6.4	Solution #4: Security solution for UL small data transfer in RRC Suspend and Resume with early data transmission (EDT)

### 6.4.1	Introduction

This solution addresses key issue #2: Integrity protection of small data, key issue #3: Encryption of small data and key issue #8: Key and mac size for protection of small data in this TR.

This security solution address solution 19: 5GS UP Optimization for Infrequent Small Data Transmission in SA2 TR 23.724 [2]. This solution is to optimize small data transmission for the UEs from CM-IDLE mode. It enables transfer of user plane data without need for the Service Request procedure. According to solution 19 in SA2 TR 23.724 [2], it is proposed to keep AS context in the NG-RAN including information the UPF connection and relevant QoS flow(s) to be used for the connection, even when UE moves into CM-IDLE. When UE resumes the connection, the NG-RAN uses the information to activate DRB, AS security and User Plane connectivity over N3 for the small data packet delivery to the UPF.

In EPS/LTE Rel-15, early data transmission (EDT) feature was added to the RRC Connection Resume/Suspend procedures in eNB as specified in TS 33.401 [7]. Uplink user plane data is included as early data transmission with the RRC Connection Resume Request (msg 3). As described in TS 33.401 [7], the UE and the target eNB uses the newly derived KUPenc for ciphering/deciphering of the UL EDT data in PDCP layer in the RRC Connection Resume Request message, and user DL data (if included) in PDCP layer in the RRC Connection Suspend or RRC Connection Resume message.

Note:	The current solution in TS33.401 does not provide integrity protection of the EDT UL UP data included during the RRC Resume procedure.

This solution proposes to re-use the security solution for RRC Suspend and Resume procedures with EDT as defined in EPS/LTE Rel-15 in clause 7.2.11.3 in TS 33.401 [7] for E-UTRA connected to 5GC, with the following enhancements:

- . The UL UP data is integrity protected while being transmitted over SRB0 during the RRC Resume procedure as detailed in this solution.

### 6.4.2	Solution details

#### 6.4.2.1	UL data transmission from CM-IDLE (SUSPEND) with Early Data Transmission to old eNB

The following figure shows a single MO small data delivered from the UE using Early Data Transmission.

Figure 6.4.2.2-1: Single MO data Transmission using  and Early Data Transmission (EDT) with UL UP IP

Figure 6.4.2.2-1 shows the following steps:

- The UE calculate the hash of UL UP data as follows:

  - Over each PDCP PDU, HASHUE-PDU(i) of the UL UP data.

  - When the UE calculate the ShortResumeMAC-I, the UE uses a predefined order based on the increasing DRB-ID and PDCP SN of the PDCP PDUs when using the Hashes of the PDCP PDUs as input parameters to the ShortResumeMAC-Input; thus the HASHUE-PDUs are protected by the ShortResumeMAC-I token, e.g., ShortResumeMAC-Input “Source (C-RNTI+PCI) + Target Cell ID + HASHUE-PDU(1) + HASHUE-PDU(2) ”

2.	The UE sends the UL small data included as UL EDT (Early Data Transmission) with the RRC Connection Resume Request (msg 3) on SRB0 and hence it is integrity protected. The UE includes information to be used for context identification and re-establishment in the RRC Connection Resume Request message: The Resume ID and a ShortResumeMAC-I.

For protection of the UL EDT in the RRC Connection Resume Request message the description in clause TS 33.401 [7] for protection of EDT is re-used with the following enhancements:

- the ShortResumeMAC-I is calculated over the input parameters as source C-RNTI, source PCI, resume constant, target Cell-ID, and pre-ordered HASHUE-PDU(s) and HASHUE-data  using the stored old KRRCint used with the source eNB where the UE was suspended, for both calculation and verification.

- 1b.	The RAN immediately suspends the RRC connection (to minimize UE power consumption) as described in TS 23.724 [2].

3.	The RAN forwards the UL data received from the UE to the UPF as described in TS 23.724 [2].

4.	UE and RAN remain suspended as described in TS 23.724 [2].

#### 6.4.2.2	UL UP EDT from RRC SUSPEND to new eNB

Since the new eNB does not have the UE context, the new eNB sends a RETRIEVE UE CONTEXT REQUEST message to the old eNB. The new eNB include the UE I-RNTI, the ShortResumeMAC-I, and the UL UP data for the old eNB to calculate the HASHUE-PDU(s) as done by the UE and validate the ShortResumeMAC-I using the old Krrcint. If successful, the old eNB sends a RETRIEVE UE CONTEXT RESPONSE message with the UE context included and the new KeNB* or NH.

After the new eNB receives the RETRIEVE UE CONTEXT RESPONSE message, it decrypt the UL UP data and forward it to the UPF. In addition, the new eNB derive the new Krrcint and Krrcenc to protect the RRCConnectionRelease message which include an indication of suspend.

#### 6.4.2.3	Connection Suspend procedure using the Early Data Transmission (EDT) feature

This procedure is used by the RAN to suspend the connection if 5GS UP Optimization is supported as described in solution 19 in TS 23.724 [2].

### 6.4.3	Evaluation

In LTE including eLTE (LTE UE using NR PDCP), integrity protection of user plane data is not supported. Therefore, any solution requires a change in UE behavior and would not be compatible with legacy eNBs. Today RRC first calculates the shortResumeMAC-I before PDCP is restored and includes it in RRCConnectionResumeRequest message. In this solution, RRC would need to delay the shortResumeMAC-I calculation until PDCP is restored, new AS keys are derived and integrity protection and ciphering in the PDCP layer is resumed. It is because the HASH code needed in the calculation of shortResumeMAC-I is calculated from the PDCP PDU, which is already processed by PDCP (i.e., ciphered).

Integrity protection of EDT UL data is a non-backward compatible change. This also requires a change in the RRC/PDCP/MAC layer modeling. After PDCP PDUs have been processed, the PDCP needs to calculate the HASH code and inform RRC. The RRC can now calculate the new shortResumeMAC-I and include it in the RRC message and forward it to lower layer for transmission, however if EDT is canceled and UE fallbacks to legacy resume procedure if the UE receives legacy UL grant in RAR. In this case, UE cannot send the EDT data and needs to update the Msg3 PDU to send only the RRC message.

However, updating the Msg3 PDU requires updating the RRC message itself because the new shortResumeMAC-I needs to be replaced with the legacy shortResumeMAC-I (calculated with legacy input parameters). Due to timing requirement after receiving the RAR, UE may not have sufficient time to reconstruct a new RRC message.

In this solution, in case of EDT fallback due to legacy UL grant in RAR, the new shortResumeMAC-I needs to be replaced with legacy shortResumeMAC-I and UE may not have sufficient time to calculate the legacy shortResumeMAC-I and construct the new RRCConnectionResumeRequest message.

In addition to RRC/MAC modeling impact, as indicated in RAN3 LS [R3-191116], this solution has significant network impact. When target eNB receives the RRC connection resume request message with UL EDT data, it won’t be able to calculate the HASH ID of the UL EDT data as it would not have the UE’s context available to know the correct RLC/PDCP header size used in the UL data. Therefore, integrity protection cannot be verified, and this solution is currently not feasible.

********************* End of Changes**********************