3GPP TSG-SA WG3 Meeting #95-Bis 	S3-191955

# Sapporo, Japan 24 June-28thth June 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0613 | rev | - | Current version: |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Procedure for steering of UE in VPLMN during mobility registration update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  |  | Date: |  | 2019-06-17 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 3GPP TS 33.501 already clarified how UE shall generate mapped KAMF from KASME during mobility from 4G to 5G. but it is not clear how the UE shall generate the KAUSF in this scenario. Without defining the KAUSF in such scenario the UE will not be able to receive SOR information or UE parameters update transparent container. TS 29.503 defines REAUTHENTICATION_REQUIRED” application error can (only) be sent in response to Nudm_UECM_Registration. And this operation has not been included in the message flow from TS 23.122, annex C. A clarification is needed in TS 33.501 that this issue can be solved by the UDM by responding with the application error “REAUTHENTICATION_REQUIRED” when the UE performs the inter-RAT change from 4G to 5G. |  |  |  |  |  |  |  |  |
| Summary of change: |  | CR add clarification on when UDM can request reauthentication procedure to from HPLMN to UE for SOR information transfer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.14.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 6.14	Steering of roaming security mechanism

### 6.14.1	General

This clause describes the security functions necessary to support steering of the UE in the VPLMN during registration procedure and also after registration as described in TS 23.122 [53] Annex C. The security functions are described in the context of the functions supporting the control plane solution for steering of roaming in 5GS.

If the control plane solution for Steering of Roaming is supported by the HPLMN, the AUSF shall store the KAUSF after the completion of the primary authentication.

The content of Steering Information List as well as the conditions for sending it to the UE are described in TS 23.122 [53] Annex C and are not repeated below. For example, the Steering Information List may include a list of preferred PLMN/access technology combinations or HPLMN indication that 'no change of the "Operator Controlled PLMN Selector with Access Technology" list stored in the UE is needed and thus no list of preferred PLMN/access technology combinations is provided'.

### 6.14.2	Security mechanisms

#### 6.14.2.1	Procedure for steering of UE in VPLMN during registration

The security procedure for the case when the UE registers with VPLMN AMF is described below in figure 6.14.2.1-1:

Figure 6.14.2.1-1: Procedure for providing list of preferred PLMN/access technology combinations

1)	The UE initiates registration by sending Registration Request message to the VPLMN AMF.

2-3)	The VPLMN AMF executes the registration procedure as defined in sub-clause 4.2.2.2.2 of 3GPP TS 23.502 [8]. As part of the registration procedure, the VPLMN AMF executes primary authentication of the UE and then initiates the NAS SMC procedure, after the authentication is successful.

4)	The VPLMN AMF invokes Nudm_SDM_Get service operation message to the UDM to get amongst other information the Access and Mobility Subscription data for the UE (see step 14b in sub-clause 4.2.2.2.2 of 3GPP TS 23.502 [8]).

Note: When the UE performs the inter-RAT change from 4G to 5G, in the absence of KAUSF , UDM may request authentication by application error “Reauthentication Required”. In this case, 5G authentication procedure takes place before proceeding to the next step.

5)	The UDM decides to send the Steering Information, and obtains the list as descirbed in TS 23.122 [53].

6-7)	The UDM shall invoke Nausf_SoRProtection service operation message to the AUSF to get SoR-MAC-IAUSF and CounterSoR as specified in sub-clause 14.1.3 of this document. If the HPLMN decided that the UE is to acknowledge the successful security check of the received Steering Information List, then the UDM shall set the corresponding indication in the SoR header (see TS 24.501 [35]) and include the ACK Indication in the Nausf_SoRProtection service operation message to signal that it also needs the expected SoR-XMAC-IUE, as specified in sub-clause 14.1.3 of this document.

The details of the CounterSoR is specified in sub-clause 6.14.2.3 of this document. In case, the Steering Information List is not available or HPLMN determines that no steering of the UE is required, then the List indication valuein the SoR header shall be set to null and list shall not be included. The inclusion of list of preferred PLMN/access technology combinations (if provided) and the SoR header in the calculation of SoR-MAC-IAUSF allows the UE to verify that the Steering Information List received is not tampered with or removed by the VPLMN and if the UDM requested an acknowledgement. The expected SoR-XMAC-IUE allows the UDM to verify that the UE received the Steering Information List.

8)	The UDM responds to the Nudm_SDM_Get service operation to the VPLMN AMF, which shall include the SoR header, Steering Information List, SoR-MAC-IAUSF and CounterSoR within the Access and Mobility Subscription data. If the UDM requests an acknowledgement, it shall temporarily store the expected SoR-XMAC-IUE.

9)	The VPLMN AMF shall include the Steering Information List, the SoR-MAC-IAUSF, CounterSoR and the SoR header to the UE in the Registration Accept message;

10)	 On receiving the Registration Accept message, if the USIM is configured with the indication that the UE shall receive the Steering Information List, then the UE shall calculate the SoR-MAC-IAUSF in the same way as the AUSF (as specified in Annex A.17) on the received Steering information, the CounterSoR and the SoR header and verifies whether it matches the SoR-MAC-IAUSF value received in the Registration Accept message. Based on the SoR-MAC-IAUSF verification outcome, the behaviour of the UE is specified in TS 23.122 [53].

11) If the UDM has requested an acknowledgement from the UE and the UE verified that the Steering Information List has been provided by the HPLMN in step 9, then the UE shall send the Registration Complete message to the serving AMF. The UE shall generate the SoR-MAC-IUE as specified in Annex A.18 and includes the generated SoR-MAC-IUE in a transparent container in the Registration Complete message.

12)	The AMF sends a Nudm_SDM_Info request message to the UDM. If a transparent container with the SoR-MAC-IUE was received in the Registration Complete message, the AMF shall include the transparent container in the Nudm_SDM_Info request message.

13)	If the HPLMN indicated that the UE is to acknowledge the successful security check of the received Steering Information List in step 8, then the UDM shall compare the received SoR-MAC-IUE with the expected SoR-XMAC-IUE that the UDM stored temporarily in step 8.