3GPP TSG-SA WG3 Meeting #95bis 	S3-192423

# Sapporo (Japan), 24-28 June 2019										        			revision of S3-191899

# Source:	NEC

# Title:	New solution for KI #4

# Document for:	Approval

Agenda Item:	8.17

# 1	Decision/action requested

This pCR fixes some edotirial issues in the latest version of TR 33.853.

# 2	References

[1]	3GPP TR 33.853, key issues and potential solutions for Integrity protection of the User Plane, v0.3.0 (S3-191775)

# 3	Rationale

This contribution proposes a new solution to KI #4, Integrity protection capability imbalance in enodeB connected to 5GC.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in [1].

***	BEGIN CHANGES	***

## 6.x	Solution #x: Improved MR-DC bearer handling

### 6.x.1	Introduction

This solution addresses KI #4, Integrity protection capability imbalance in enodeB connected to 5GC.

This solution introduces enhancement to the bearer establishment and offload behaviour in Rel.15 in MR-DC scenarios. Scenarios where Rel.15 behaviour dictates suboptimal handling are addressed in such a way that the UP IP is activated whenever possible.

This solution affects the bearer establishment and offload decision in MN.

### 6.x.2	Network options affected

This solution is applicable to the following network options:

- Option 4 - 5G core based Dual Connectivity (NR master - eUTRA secondary)

- Option 7 - 5G core based Dual Connectivity (eUTRA master - NR secondary)

### 6.x.3	Solution Description

In Rel-15 MR-DC handling of UP IP, there are certain constraints exist. Specifically:

1. In NGEN-DC scenario and UP security policy is "required", then the MN (eNB) rejects bearer establishment.

2. In NGEN-DC scenario and UP security policy is "preferred", then the MN (eNB) establishes bearer without UP IP.

3. In NE-DC scenario and UP security policy is "preferred", then the MN (gNB) activates the bearer with or without UP IP. When UP IP is activated, then the MN does not offload bearer to the SN (eNB).

In all of the above scenarios, it is possible to introduce improvements to: i) accept bearer establishment instead of rejecting it (case 1), ii) select whether to activate UP IP or not (case 2, 3). The improvement is described in Table 6.x.3-1 below. In all of these cases, these improvements alleviate the existing constraints in Rel.15 and introduces more cases where UP IP is activated in the bearer, enhancing the overall security of the user plane.

Table 6.x.3-1: Improvement of MR-DC handling over Rel.15 behaviour

| Case | Scenario description | Per Rel.15 MR-DC bearer handling | Improvement in this solution |
| --- | --- | --- | --- |
| 1 | NGEN-DC scenario (MN=eNB, SN=gNB) and UP IP CN security policy = "required" | The MeNB rejects bearer establishment. | The MeNB establishes bearer and immediately offloads it to SgNB. The MeNB instructs the SgNB to activate UP IP. The SgNB accepts the bearer with UP IP. |
| 2 | NGEN-DC scenario (MN=eNB, SN=gNB) and UP IP CN security policy = "preferred" | The MeNB establishes bearer without (i.e. deactivating) UP IP. Bearer can be offloaded to the SgNB. In this case, the SgNB accepts bearer offload from MeNB without UP IP. | The MeNB establishes bearer either with or without UP IP. If the MeNB establishes bearer with UP IP, the MeNB immediately offloads it to SgNB. The MeNB instructs the SgNB to activate UP IP. The SgNB accepts the bearer with UP IP. If the MeNB establishes bearer without UP IP, the MeNB can offload it to SgNB. The MeNB instructs the SgNB to activate UP IP. The SgNB accepts the bearer optionally with UP IP. |
| 3 | NE-DC scenario (MN=gNB, SN=eNB) and CN UP IP security policy = "preferred" | If the MgNB activates bearer with UP IP, the MgNB does not offload bearer to the SeNB. If the MgNB activates without (i.e. deactivating) UP IP, the MgNB can offload bearer to the SeNB. The SeNB accepts the bearer offload without UP IP. | If the MgNB activates bearer with UP IP, the MgNB may offload the bearer to the SeNB. The SeNB accepts the bearer without UP IP. If the MgNB activates without (i.e. deactivating) UP IP, the MgNB can offload bearer to the SeNB. The SeNB accepts the bearer offload without UP IP. |

Editor's Note: clarification on termination point of offloaded bearers needs to be added.

Editor's Note: additional detail on bearer establishment and offload needs to be described.

Editor's Note: additional clarification to notes on #3 needs to be added.

### 6.x.4	Solution Evaluation

Editor's Note: TBD.

***	END OF CHANGES	***