3GPP TSG-SA WG3 Meeting #95-Bis 	S3-192215

# Sapporo (Japan), 24-28 June 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | 0023 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction of CAPIF-3e/4e/5e requirements clause. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCAPIF-Sec |  |  |  |  |  |  | Date: |  | 2019-06-17 |
| Category: | D |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Wrong implementation of SA plenary approved CR (0019), which was agreed in SA3#95 meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change font color to black. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency of normative text format. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### ***** Start of Change *****

## 4.6	Security requirements on the CAPIF-3e/4e/5e reference points

The security requirements for CAPIF-3e/4e/5e reference points are:

-	 [CAPIF-SEC-4.6 -a] The transport of messages over the CAPIF-3e/4e/5e reference points shall be integrity protected.

-	 [CAPIF-SEC-4. 6 -b] The transport of messages over the CAPIF-3e/4e/5e reference points shall be confidentiality protected.

-	 [CAPIF-SEC-4. 6 -c] The transport of messages over the CAPIF-3e/4e/5e reference points shall be protected from replay attacks.

-	 [CAPIF-SEC-4. 6 -d] The CAPIF core function shall be able to authenticate the service API publishers to publish and manage the service API information.

-	 [CAPIF-SEC-4. 6 -e] The CAPIF core function shall be able to authorize the service API publishers to publish and manage the service API information.

-	 [CAPIF-SEC-4. 6 -f] The CAPIF core function shall be able to request explicit grant of new API invoker’s onboarding.

-     [CAPIF-SEC-4.6-g] The CAPIF core function shall be able to authenticate the API Management function’s registration request for API Provider domain functions.

-     [CAPIF-SEC-4.6-h] The CAPIF core function shall be able to authenticate the API Management function’s registration update request for API provider domain functions.

#### ***** End of Change *****