3GPP TSG-SA3 Meeting #98Bis-e 	S3-200798

# e-meeting, 14 -17 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | draftCR | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Access token ownership verification by the producer in indirect communication scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2020-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When Token-based authorization is used, the validation of the access token shall include the validation of the access topken ownership. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the case of Indirect communication, i.e., SCP shall forward the authenticated NFc Instance Id to the producer NF. The producer NF matches the NFc Instance ID in the access token with the received NF Instance Id sent by SCP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of access token ownership verification introduces the possibility of compromised NF to impersonate other NFs by using the other NFs access token. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of Change ****************

This document builds on the structure captured in Ericsson’s contribution at SA3#98e S3-200285 and two new Nokia’s contributions S3-20abcd and S3-20abcd

The changes proposed by this draftCR is color coded differently and highlighted.

#### 13.4.1.X	Service access authorization in indirect communication scenarios

##### 13.4.1.X.1	General

Editor's Note: General introduction to be added.

##### 13.4.1.X.2		Authorization for indirect communication without delegated discovery procedure

Figure 13.4.1.X.Y-1: Authorization and service invocation procedure

Discovery of the NF Service Producer:

0.	Optionally, the NF Service Consumer may discover the NF Service Producer before requesting authorization to invoke the services of the NF Service Producer.

NF Service Consumer authorization:

The NF Service Consumer and NRF perform the "Access token request before service access" procedure as described in clause 13.4.1.1. If the NF Service Consumer has already discovered the NF Service Producer (Step 0), it can also perform the "Access token request for a specific NF Producer/NF Producer service instance" procedure as described in clause 13.4.1.1.

Service request:

The NF Service Consumer, SECOP, NRF and NF Service Producer perform the procedure "Indirect Communication without delegated discovery Procedure" described in clause 4.17.11 of TS 23.502 [8]. The following steps describe how the access token received in steps 1 and 2 is used in this procedure.

3.	If no cached data is available, the NF Service Consumer discovers the NF Service Producer via the SECOP.

4.	The NF Service Consumer sends a service request for the specific service to the SECOP. The service request includes the access token for the NF Service Producer providing the service as received in step 5.

5.	The SECOP selects a NF Service Producer instance, performs the API root modifications and forwards the received request to the selected NF Service Producer instance. The request contains the token as received in 6. and valid for the NF Service Consumer.

The SECOP includes in the message the NF Service Consumer’s Instance Id (NfInstanceId) that has been authenticated at the transport layer using NF Service Consumer’s TLS certificate.

6.	To authorize the access the NF Service Producer validates the token by verifying the signature and checking if the requested service is part of the token's scope.

The NF Service Producer checks that the NFc Instance Id in the subject claim matches the NFc Instance Id that was shared by the SECOP that authenticated the NF consumer at the transport layer based on TLS client certificate.

If the checks are ok the NF Service Producer processes the request and provides a response.

7.	The SECOP performs revers API root modifications and forwards the response.

##### 13.4.1.X.3		Authorization for indirect communication with delegated discovery procedure (Model D)

###### 13.4.1.X.3.0	General

Editor’s Note: General clause to be added

###### 13.4.1.X.3.1	NF consumer has a direct connection with the NRF

The procedure in the clause applies to deployment scenarios where the NF consumer has a direct connection with the NRF and uses it for service requests towards NRF, while using indirect communication model for service requests for other NFs.

NF consumer obtains authentication code from NRF

Before sending a service request to the SECOP, the Network Function obtains an authentication code from the NRF over a mutually authenticated TLS connection.

The authentication code is in the form of a JWT according to RFC 7523 [XX], with the following set of claims:

- NF Instance Id (Subject)

- Issued at (time)

- NRF (Issuer)

- Issuer’s signature

The authentication code is intended for consumption by the NRF (authorization server) before a specific expiration time.  The subject of the JWT ("NF Instance Id") is the party on behalf of whom the new access token will be requested by SeCoP.

The SeCoP’s identity may be added in the token by NRF if this information is available at the NRF.

In deployments where the NFs and the SeCoPs are co-located in the same deployment unit and SeCoP is setup as a side-car proxy, all communication passes through the side-car proxy. In this scenario, the authentication code may be obtained via SeCoP.

Access token request during service access

The following procedure describes how the SeCoP connected to the NF consumer obtains an access token before forwarding the service access to the selected NF service producer.

Pre-requisite:

a. The NF Service consumer is registered with the NRF

b. The NF Service consumer has obtained a JWT based authenticaion code digitally signed from the NRF

Figure 13.4.1.X.3.1-1 SeCoP obtaining access token token using NRF-granted authentication code

1. The NFc shall invoke the API requesting specific service towards the SeCoPc.

The NFc includes its instance Id and NF type in the request. The request may include discovery and selection parameters necessary to discover and select a NF service producer instance, and authentication code..

2. The SeCoP performs service discovery and selection of a target NF producer.

3. The SeCoP shall obtain an access token for service access authorization on behalf of the NF consumer. The Access Token Request message includes the following:

-	the NF instance Id of the selected producer

-	the NF instance Id of NF consumer on behalf of which access token is requested

-	expected NF service name(s),

-	NF type of the expected NF producer instance and NF consumer and

-	authentication code, sent by the NFc in step 1.

4. The NRF shall verify the integrity of the authentication code by checking the signature using its public key. If integrity check is successful, it shall verify the claims in the authorization code as follows:

-	authentication code is active – i.e. not expired

-	authentication code is specific to it – i.e. the audience is set to NRF’s FQDN

-	Subject in the authentication code matches the NF Instance Id of the consumer included in the request

Once above checks are successful, the NRF shall authorize the NF consumer. The NRF determines the scope of access based on what the NF consumer is authorized to access.

5. The NRF generates an access token with appropriate claims included. The NRF shall digitally sign the generated access token based on a shared secret or private key as described in RFC 7515 [45].

The claims in the token includes the NF Instance Id of NRF (issuer), NF Instance Id of the NF Service consumer (subject), NF Instance Id of the NF Service producer (audience), expected service name(s) (scope) and expiration time (expiration).

6. The NRF sends the access token along with the expiration time to the SeCoP.

7. The SeCoP forwards the request to the selected NF service producer along with the access token. It may temporarily cache the access token till it receives the service response from the NF service producer.

The SECOP includes in the message the NF Service Consumer’s Instance Id (NfInstanceId) that has been authenticated at the transport layer using NF Service Consumer’s TLS certificate.

Access token verification before service access

The SeCoP connected to the NF producer (SeCoPp) receives the service request along with the access token. The SeCoPp forwards the service request to the NF producer.

The NF producer verifies the integrity of the access token and performs steps as specified in “Service access request based on token verification” in clause 13.4.1.1 with one difference:

The NF producer checks that the NFc Instance Id in the subject claim matches the NFc Instance Id that was shared by the SECOP that authenticated the NF consumer at the transport layer based on TLS client certificate.

.

###### 13.4.1.X.3.2	NF consumer does not have a direct connection with the NRF

The procedure in the clause applies to deployment scenarios where the NF consumer uses indirect communication model for all service requests.

Access Token Request during service access

Figure 13.4.1.X.3.2-1 SeCoP obtaining access token using authentication based on NFc Identity assertion

1. The NFc shall send a service request message to the SCP with the following additional parameter SignedNFParms, which includes NFc Instance Id, Time stamp, SCP ID, NFc’s certificate and NFc’s signature.

2. The SeCoP performs discovery of the target NF producers and selects a target NF producer.

3. The SeCoP sends the access token request message towards the NRF according to clause 13.4.1.1 with the following difference: It includes SignedNFParms obtained from the NFc in Step 1.

4. The NRF validates NFc’s signature based on the public key in the received certificate. and obtains NFc params included by the NFc.

The NRF checks if the SCP that sent the message is allowed to represent the consumer. This is checked by comparing SCP ID sent by the NFc with the SCP ID of the SCP that sent the message.

The time stamp is verified to detect replay attacks.

The NRF retrieves NFc’s identity on behalf of which the SCP is obtaining an access token.

5-6. The NRF authorizes the NFc and generates a digitall signed access token as specified in clause 13.4.1.1.

7. The NRF shall send access token to the SeCoP in the Nnrf_AccessToken_Get response operation.

8 The SeCoP sends the service request to the producer including the access token received in step 7. The SECOP includes in the message the NF Service Consumer’s Instance Id (NfInstanceId) that has been authenticated at the transport layer using NF Service Consumer’s TLS certificate.

Access token verification before service access

The SeCoP connected to the NF producer (SeCoPp) receives the service request along with the access token. The SeCoPp forwards the service request to the NF producer.

The NF producer verifies the integrity of the access token and performs steps as specified in “Service access request based on token verification” in clause 13.4.1.1 with one difference:

The NF producer checks that the NFc Instance Id in the subject claim matches the NFc Instance Id that was shared by the SECOP that authenticated the NF consumer at the transport layer based on TLS client certificate.

*************** End of Change  ****************