3GPP TSG-SA3 Meeting #99e	S3-200959

# e-meeting, 11 -15 May 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0801 | rev | - | Current version: | 16.2.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for distribution of subscriber identities and security data in clause 6.3.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  | Date: |  |  | 2020-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The sentence “Once the subscriber identities and security data have been transmitted from an old to a new network entity the old network entity shall delete the data. ” means that once the old AMF sends the security data to the new network entity, the old AMF will delete the security data in old AMF. However, in the latest version of TS 23.502 clause 4.2.2.2.3 step 8, the target AMF can retrieve UE context from old AMF and receive UE context from initial AMF. It implies that both old AMF and initial AMF hold the UE context. The old AMF shall not delete the UE context until it receive the Namf_Communication_RegistrationStatusUpdate message which indicated the registration in target AMF is completed. Therefore, old network entity must confirm the security data have been transmitted to new network entity successfully. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification for distribution of subscriber identities and security data in clause 6.3.1.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The old network entity may delete the security data in a wrong way. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.3.1.2	Distribution of subscriber identities and security data within one 5G serving network domain

The transmission of the following subscriber identities and security data is permitted between 5G core network entities of the same serving network domain:

-	SUPI in the clear

-	5G security contexts, as described in clause 6.9

A 5G authentication vector shall not be transmitted between SEAFs.

Once the subscriber identities and security data have been successfully transmitted from an old to a new network entity, the old network entity shall delete the datawhen the old network entity receivesaclear indication.

* * * End Change * * * *