3GPP TSG-SA3 Meeting #99e 	S3-201134

# e-meeting, 11 -15 May 2020													Revision of S3-20xxxx

# Source:	China Mobile

# Title:	Adding security requirements and related test cases for GVNP of type 2

# Document for:	Approval

Agenda Item:	5.6

1	Decision/action requested

It is proposed to add the security requirements and test cases for GVNP of type 2 into clause 5.2.5.y.

2	Rationale

This contribution describes the security requirements and test cases for GVNP of type 2 to mitigate the security threats described in clause 5.2.4.3.2.

3	Detailed proposal

****************** Start of the change ******************

#### 5.2.5.y Security functional requirements and related test cases for GVNP of type 2

#### 5.2.5.y.1 Introduction

All texts from clause 5.2.5.5.1 can be basically applied to GVNP of type 2. The proposed security requirements for GVNP of type 2 are described in following sub-clauses.

#### 5.2.5.y.2 Security functional requirements deriving from 3GPP specifications and related test cases

###### 5.2.5.y.2.1 Security functional requirements deriving from 3GPP specifications – general approach

The clause 4.2.2.1 in TS 33.117 [4] also applies to security functional requirements deriving from 3GPP specifications and the corresponding test cases of GVNP type 2.

#### 5.2.5.y.3 Technical baseline

All texts from clause 5.2.5.5.3 apply to GVNP of type 2.

#### 5.2.5.y.4 Operating systems

All text from TS 33.117 [4], clause 4.2.4 also applies to guest operating systems and host operating systems for GVNP of type 2.

#### 5.2.5.y.5 Web servers

All text from TS 33.117 [4], clause 4.2.5 also applies to GVNP of type 2.

#### 5.2.5.y.6 Virtualized Network devices

All text from TS 33.117 [4], clause 4.2.6 also applies to GVNP of type 2.

In addition, a VNF shall be instantiated from a trusted image. The detailed security requirements and related test cases are as following.

###### 5.2.5.y.6.1 Instantiating VNF from trusted image

Requirement Name: Instantiating VNF from trusted image

Requirement Description:

A VNF shall be initiated from a trusted image which has a digital signature signed by an authorized party. The authorized party is trusted by the operators.

Threat Reference: TR 33.926 [3], Clause5.3.4.1, "Software Tampering "; TR 33.848, Clause5.18, “Key Issue 17: Software Catalogue Image Exposure”

Test case:

Test Name: TC_INSTANTIATINGVNFFROMTRUSTEDIMAGE

Purpose:

To test whether a VNF is instantiated from a trusted image.

Procedure and execution steps:

Pre-Condition:

There are VNF and virtualisation layer, and a NFVO, a VNFM, a VIM and an OMA (or simulated NFVO, a VNFM, a VIM, an OAM) on the test environment.

Execution Steps

Execute the following steps:

1. The tester boarded a VNF image with a correct digital signature into the NFVO.

2. The tester initiates to create a VNF from the VNF image with the correct digital signature through the NFVO. The NFVO will collaborate with the VNFM, VIM and the OAM to create the VNF successfully.

3. The tester boarded a VNF image with an incorrect digital signature or without a digital signature into the NFVO.

4. The tester initiates to create a VNF from the VNF image with the incorrect digital signature or without the digital signature through the NFVO. The NFVO will refuse to corret the VNF.

Expected Results:

1. In the step 2, the VNF is created successfully.

2. In the step 4, the VNF is created in failure.

Expected format of evidence:

Screenshots contain the VNF creation result.

****************** End of the change ******************