3GPP TSG-SA3 LI Meeting #7 	Tdoc S3LI02_162

San Diego, California, USA. 14-16 November 2002

| CR-Form-v7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR | CRNum |  rev | - |  Current version: | 5.1.0 |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Miscellaneous Changes to TS 33.108 to Clarify U.S. LI Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source:  | Nortel Networks, AT&T Wireless, Cingular |  |  |  |  |  |  |  |  |  |
| Work item code:  |  |  |  |  |  |  |  | Date:  |  | 11/14/2002 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | REL-5 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) |  |
| Reason for change:  |  | Required changes to clarify U.S. LI requirements. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Clarifications on wording and clarifying additions. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | TS 33.108 may not be clear with respect to U.S. LI requirements. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 3, 4, 6, Annex C, Annex F, new Annex I. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at http://www.3gpp.org/specs/CR.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

## 3.1 Definitions

[Editing information:  Modify the following sentence at the beginning of subclause 3.1 as indicated.]

For the purposes of the present document, the following terms and definitions apply and are not intended to supplant definitions for the same terms under national law.

[Editing information:  Modify the following definition as indicated.]

intercept related information: collection of information or data associated with telecommunication services involving the target identity, including but not necessarily limited to communication associated information or data (e.g. unsuccessful communication attempts), service associated information or data, signalling information used to establish or control the telecommunications service, event time and date, and location information.

## 4.1	Basic principles for the handover interface

[Editing information:  Modify the following sentences in subclause 4.1 as indicated.]

Lawful interception may require functions to be provided in the switching or routing nodes of a telecommunications network.

The specification of the handover interface is subdivided into three logical ports each optimised to the different purposes and types of information being exchanged.

The interface is extensible (i.e., the interface may be modified in the future as necessary).

## 4.3	Functional requirements

[Editing information:  Modify the following sentences in subclause 4.3 as indicated.]

A lawful authorization shall describe the kind of information (Intercept Related Information (IRI) only, or IRI with Content of Communication (CC)) that is required by this LEA, the identifiers for the interception subject, the start and stop time of LI, and the addresses of the LEAs for delivery of CC and/or IRI and further information.

### 4.4.1	Handover interface port 2 (HI2)

[Editing information:  Modify the sentences in subclause 4.4.1 as indicated.]

- The handover interface port 2 shall transport the IRI from the NWO/AP/SvP's IIF to the LEMF.

The delivery shall be performed via data communication methods which are suitable for the network infrastructure and for the kind and volume of data to be transmitted. In the U.S., delivery is subject to the facilities procured by the government.

The delivery can in principle be made via different types of lower communication layers, which should be standard or widely used data communication protocols.

The individual IRI parameters shall be coded using ASN.1 and the basic encoding rules (BER). The format of the parameter’s information content shall be based on existing telecommunication standards, where possible.

The individual IRI parameters have to be sent to the LEMF at least once (if available).

The IRI records shall contain information available from normal network  operating procedures. In addition the IRI records shall include information for identification and control purposes as specifically required by the HI2 port.

### 4.4.2	Handover interface port 3 (HI3)

[Editing information:  Modify the following sentence in subclause 4.4.2 as indicated.]

The port HI3 shall transport the content of the communication (CC) of the intercepted telecommunication service to the LEMF. The content of communication shall be presented, subject to national requirements, as a transparent en-clair copy of the information flow during an established, frequently bi-directional, communication of the interception subject.

## 4.5	HI2: Interface port for intercept related information

[Editing information:  Modify the following sentences in subclause 4.5 as indicated.]

The HI2 interface port shall be used to transport all intercept-related information (IRI). Only information which is part of standard network signalling procedures shall be used within communication related IRI.

Sending of the intercept-related information (IRI) to the LEMF shall in general take place as soon as possible, after the relevant information is available.

In exceptional cases (e.g. data link failure), the intercept related information may or may not be buffered for later transmission for a specified period of time.

### 4.5.1	Data transmission protocols

[Editing information:  Modify the following sentences in subclause 4.5.1 as indicated.]

The protocol used by the “LI application” for the encoding and the sending of data between the MF and the LEMF is based on already standardized data transmission protocols (e.g. ROSE or FTP).

The specified data communication methods provide a general means of data communication between the LEA and the NWO/AP/SvP’s mediation function. They are used for the delivery of:

•	HI2 type of information (IRI records);

•	Certain types of content of communication (e.g., SMS).

### 6.2.2	Quality

[Editing information:  Add the following sentence to subclause 6.2.2 as indicated.]

The quality of service associated with the result of interception should be (at least) equal to the quality of service of the original content of communication. This may be derived from the QoS class used for the original intercepted session [7]. Law enforcement and the service provider must negotiate the appropriate QOS.

### 6.2.3	Reliability

[Editing information:  Add the following sentence to subclause 6.2.3 as indicated.]

The reliability associated with the result of interception should be (at least) equal to the reliability of the original content of communication. This may be derived from the QoS class used for the original  intercepted session [7].

In the U.S. reliability is determined by what law enforcement procures.

## 6.5	IRI for packet domain

[Editing information:  Modify the following sentences in subclause 6.5 as indicated.]

Intercept related information will in principle be available in the following phases of a data transmission:

1.	At connection attempt when the target identity becomes active, at which time packet transmission may or may not occur (i.e., set up of a data context, target may be the originating or terminating party);

2.	At the end of a connection, when the target identity becomes inactive (i.e., removal of a data context);

3.	At certain times when relevant information are available.

[Editing information:  Modify Table 6.2 in subclause 6.5 as indicated.]

Table 6.2: Mapping between Events information and IRI information

| parameter | description | HI2 ASN.1 parameter |
| --- | --- | --- |
| observed MSISDN | Target Identifier with the MSISDN of the target subscriber (monitored subscriber). | partyInformation (party-identiity) |
| observed IMSI | Target Identifier with the IMSI of the target subscriber (monitored subscriber). | partyInformation (party-identity) |
| observed IMEI | Target Identifier with the IMEI of the target subscriber (monitored subscriber) | partyInformation (party-identity) |
| observed PDP address | PDP address used by the target.. | partyInformation (services-data-information) |
| event type | Description which type of event is delivered: PDP Context Activation, PDP Context Deactivation,GPRS Attach, etc. | gPRSevent |
| event date | Date of the event generation in the xGSN | timeStamp |
| event time | Time of the event generation in the xGSN |  |
| access point name | The APN of the access point | partyInformation (services-data-information) |
| PDP type | This field describes the PDP type as defined in TS GSM 09.60, TS GSM 04.08, TS GSM 09.02 | partyInformation (services-data-information) |
| initiator | This field indicates whether the PDP context activation, deactivation, or modification is MS directed or network initiated. | initiator |
| correlation number | Unique number for each PDP context delivered to the LEMF, to help the LEA, to have a correlation between each PDP Context and the IRI. | gPRSCorrelationNumber |
| lawful interception identifier | Unique number for each lawful authorization. | lawfulInterceptionIdentifier |
| location information | When lawfully authorized, this field provides the location information of the target that is present at the SGSN at the time of event record production. | locationOfTheTarget |
| SMS | The SMS content with header which is sent with the SMS-service | sMS |
| failed context activation reason | This field gives information about the reason for a failed context activation of the target subscriber. | gPRSOperationErrorCode |
| failed attach reason | This field gives information about the reason for a failed attach attempt of the target subscriber. | gPRSOperationErrorCode |
| service center address | This field identifies the address of the relevant server within the calling (if server is originating) or called (if server is terminating) party address parameters for SMS-MO or SMS-MT. | serviceCenterAddress |
| umts QOS | This field indicates the Quality of Service associated with the PDP Context procedure. | qOS |
| context deactivation reason | This field gives information about the reason for context deactivation of the target subscriber. | gPRSOperationErrorCode |
| network identifier | Operator ID plus SGSN or GGSN address. | networkIdentifier |
| iP assignment | Observed PDP address is statically or dynamically assigned. | iP-assignment |
| SMS originating address | Identifies the originator of the SMS message. | DataNodeAddress |
| SMS terminating address | Identifies the intended recipient of the SMS message. | DataNodeAddress |
| SMS initiator | Indicates whether the SMS is MO, MT, or Undefined | sms-initiator |
| serving SGSN number | An E.164 number of the serving SGSN. | ServingSGSN-Number |
| Serving SGSN address | An IP address of the serving SGSN. | ServingSGSN-Address |

NOTE:	LIID parameter must be present in each record sent to the LEMF.

[Editing information:  Modify Table 6.9 in subclause 6.5 as indicated.]

Table 6.9: Start Of Interception (with PDP Context Active) BEGIN Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| observed PDP address | C | Provide to identify the: - static address requested by the intercept subject’s MS, and allocated by the Network for a successful PDP context activation. - address allocated dynamically by the network to the intercept subject MS in association with a PDP context activation (i.e., address is sent by the Network in an Activate PDP Context Accept) for a successful PDP context activation procedure when the PDP Context activation request does not contain a static PDP address. - address offered by the network in association with a network-initiated PDP context activation request when the intercept subject’s MS accepts the network-initiated PDP context activation request. |
| event type | C | Provide Start Of Interception With PDP Context Active event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| access point name | C | Provide to identify the: - packet data network to which the intercept subject requested to be connected when the intercept subject’s MS is successful at performing a PDP context activation procedure (MS to Network). - access point of the packet data network that requested to be connected to the MS when the intercept subject’s MS accepts a network-initiated PDP context activation (Network to MS). |
| PDP type | C | Provide to describe the PDP type of the observed PDP address. The PDP Type defines the end user protocol to be used between the external packet data network and the MS. |
| initiator | C | Provide to indicate whether the PDP context activation is network-initiated, intercept-subject-initiated, or not available. |
| network identifier | M | Shall be provided. |
| correlation number | C | Provide to uniquely identify the PDP context delivered to the LEMF and to correlate IRI records with CC. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized and reasonably available, to identify location information for the intercept subject’s MS. |
| QOS | C | Provide to identify the QOS parameters. |

## C.1.2	Definition of ULIC header version 0

[Editing information:  Modify the following sentence in clause C.1.2 as indicated. The change is proposed as this statement does not seem possible.]

The correlation number consist of 8 octets The requirements for this identification are similar to that defined for charging in [12], chapter 5.4. Therefore it is proposed to use the Charging-ID,  defined in [12] , chapter 5.4 as part of correlation number. The Charging-ID is signaled to the new SGSN in case of SGSN-change so the tunnel identifier could be used “seamlessly” for the HI3 interface.

## Annex F (informative)

Profiles for FTP

[Editing information:  Delete Annex F.]

[Editing information:  Create  a new Annex I for normative U.S. requirements. The reason another U.S. Annex is proposed is that its unclear if a single annex can contain both Normaive and Informative clauses. If it is possible for both Normative and Informative material to be in a single annex, one annex for U.S. requires could be created. ]

Annex I (normative):
United States lawful interception

With respect to the handover interfaces they must be capable of delivering intercepted communications and IRI information to the government in a format such that they may be transmitted by means of equipment, facilities, or services procured by the government to a location other than the premises of the carrier.

With respect to location information ‘when authorized’ means the ability to provide location information on a per-surveillance basis.

The delivery methods described in this document are optional methods and no specific method is required in the U.S..

The specification of lawful intercept capabilities in this document does not imply that those services supported by these lawful intercept capabilities are covered by CALEA. Inclusion of a capability in this document does not imply that capability is required by CALEA. This document is intended to satisfy the requirements of section 107 (a) (2) of the Communications Assistance for Law Enforcement Act, Pub. L. 103-414 such that a telecommunications carrier, manufacturer, or support service provider that is in compliance with this document shall have “Safe Harbor”.

In the United States surveillance on the GGSN is not required, but is an option that may be negotiated between the service provider and law enforcement.

A TSP shall not be responsible for decrypting or decompressing, or ensuring the government's ability to decrypt or decompress, any communication encrypted or compressed by a subscriber or customer, unless the encryption or compression was provided by the TSP and the TSP possesses the information necessary to decrypt or decompress the communication. A TSP that provides the government with information about how to decrypt or decompress a communication (e.g., identifying the type of compression software used to compress the communication, directing the government to the appropriate vendor that can provide decryption or decompression equipment, or providing the encryption key used to encrypt the communication) fully satisfies its obligation under the preceding sentence.