3GPP TSG-SA3 Meeting #98-LI	s3i250409

# Florence, Italy, 15th Jul 2025 - 18th Jul 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0776 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to XID allocation and triggering criteria for LI for Home Routed Roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2025-07-15 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The currently defined XID allocation for the N9HR LI solution may create conflicts as it is possible for multiple events to happen simultaneously for which new triggered tasks would need to be created in the BBIFF-U. Additionally, the details on the TargetIdentifiers sent to the BBIFF-U are unclear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifies triggers for home routed roaming. Clarifies and adds target identifiers to allow UPF to correctly isolate flows. Removes redundant provisioning elements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The N9HR LI solution will not function correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.3.1, 7.10.3.2, 7.10.3.2.1, 7.10.3.2.2, 7.10.3.2.3, 7.10.3.3.1, 7.10.3.5, 7.10.3.5.1, 7.10.3.5.1a, 7.10.3.5.2, 7.10.3.5.3, 7.10.3.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 33.128 CR 0775, 0776 and 0754 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Schema changes for this CR can be found on the Forge: Merge Request: !342 Commit Hash: 780eb498a808b726ddd001a6e25838a3572438eb CR 0764 and 0775 are not mirrors, however they incorporate some of the changes from this document into Rel 17 and Rel 18. CR 0754 makes some changes to clauses referenced by this document. If those changes are not made, the references in this document will need updates. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | s3i250386 |  |  |  |  |  |  |  |  |

## **** START OF FIRST CHANGE (MAIN DOCUMENT) ****

##### 6.2.3.3.1	LI_T3 interface specifics

When interception of communication contents is authorised or the delivery of packet header information is authorised and approach 2 described in clause 6.2.3.5 is used, the CC-TF present in the SMF sends a trigger to the CC-POI present in the UPF over the LI_T3 interface.

When the CC-TF in the SMF detects that a PDU session is being established (i.e. when the SMF sends the N4: PFCP Session Establishment Request to the UPF, see TS 29.244 [15] clause 6.3.2) for a target UE, it shall send an activation message to the CC-POI in the UPF over the LI_T3 interface. The activation message shall contain the correlation identifiers that the CC-POI in the UPF shall use with the xCC. This can be achieved by sending an ActivateTask message as defined in ETSI TS 103 221-1 [7] clause 6.2.1 with the following details.

Table 6.2.3.3.1-1: ActivateTask message for triggering the CC-POI in the UPF

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| XID | Allocated by the CC-TF as per ETSI TS 103 221-1 [7]. | M |
| TargetIdentifiers | Packet detection criteria as determined by the CC-TF in the SMF, which enables the UPF to isolate target traffic. The CC-POI in the UPF shall support at least the identifier types given in table 6.2.3.3.1-2. NOTE: This value is the target identifier for the CC-POI in the UPF and may be different from the target identifier specified in the warrant. | M |
| DeliveryType | Set to “X3Only”. | M |
| ListOfDIDs | Delivery endpoints for LI_X3. These delivery endpoints shall be configured by the CC-TF in the SMF using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |
| CorrelationID | Correlation ID to assign to X3 PDUs generated by the CC-POI in the UPF. This field is populated with the same CorrelationID the IRI-POI in the SMF uses for the associated xIRI. | M |
| ProductID | Shall be set to the XID of the Task Object associated with the interception at the CC-TF. This value shall be used by the CC-POI in the UPF to fill the XID of X3 PDUs. | M |

The CC-TF in the SMF shall not send the ListOfServiceTypes parameter of the ActivateTask message to the CC-POI in the UPF.

Table 6.2.3.3.1-2: Target Identifier Types for LI_T3

| Identifier type | Owner | ETSI TS 103 221-1 [7] TargetIdentifier type | Definition |
| --- | --- | --- | --- |
| GTP Tunnel ID | 3GPP | gtpuTunnelId | F-TEID (see XSD schema) |
| UE IP Address | ETSI | IPv4Address or IPv6Address | See ETSI TS 103 221-1 [7] |
| UE TCP/UDP Port | ETSI | TCPPort or UDPPort | See ETSI TS 103 221-1 [7] |
| PFCP Session ID | 3GPP | TargetIdentifierExtension / FSEID | F-SEID (see XSD schema) |
| PDR ID | 3GPP | TargetIdentifierExtension / PDRID | 32 bit unsigned integer (see XSD schema) |
| QER ID | 3GPP | TargetIdentifierExtension / QERID | 32 bit unsigned integer (see XSD schema) |
| Network Instance | 3GPP | TargetIdentifierExtension / NetworkInstance | Octet string (see XSD schema) |
| GTP Tunnel Direction | 3GPP | TargetIdentifierExtension / GTPTunnelDirection | Enumeration (see XSD schema) |
| PDR | 3GPP | TargetIdentifierExtension/PDR | Octet string (see XSD schema). Contains a packet detection rule encoded per TS 29.244 [15] Table 7.5.2.2-1 omitting the first 4 octets. |

When the CC-TF in the SMF detects that a targeted PDU session is changing (i.e. when the SMF sends the N4: PFCP Session Modification Request to the UPF, see TS 29.244 [15] clause 6.3.3) in a way that requires changes to the interception already activated by the CC-POI in the UPF, the CC-TF shall modify the interception at the CC-POI in the UPF over the LI_T3 interface. This is achieved by sending a ModifyTask message as defined in ETSI TS 103 221-1 [7] clause 6.2.2. The ModifyTask message contains the same details as the ActivateTask message with the following fields updated as appropriate.

Table 6.2.3.3.1-3: Parameters that may be changed in a ModifyTask message when updating interception at the CC-POI in the UPF

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| TargetIdentifiers | Updated packet detection criteria as determined by the CC-TF in the SMF. NOTE: See notes on TargetIdentifiers in table 6.2.3.3.1-1. | M |

When the CC-TF in the SMF detects that a targeted PDU session is changing (i.e. when the SMF sends the N4: PFCP Session Modification Request to the UPF) for which the interception had not been previously activated in the CC-POI in the UPF (e.g. in case of previous unsuccessful LI activation at the CC-POI in the UPF by the CC-TF in the SMF), the CC-TF shall send an activation message to the CC-POI in the UPF over the LI_T3 interface. The activation message shall contain the correlation identifiers that the CC-POI in the UPF shall use with the xCC. This can be achieved by sending an ActivateTask message as defined in ETSI TS 103 221-1 [7] clause 6.2.1 with the details provided by table 6.2.3.3.1-1.

When the CC-TF in the SMF detects that the PDU session has been released (i.e. when the SMF sends the N4: PFCP Session Deletion Request to the UPF, see TS 29.244 [15] clause 6.3.4) for a target UE, it shall send a deactivation message to the CC-POI in the UPF over the LI_T3 interface. When using ETSI TS 103 221-1 [7] this is achieved by sending a DeactivateTask message with the XID field set to the XID associated with the interception, as described in ETSI TS 103 221-1 [7] clause 6.2.3.

By default, interception shall occur at the anchor UPF as described in clause 6.2.3.3.3.

When a warrant that includes the service scoping of CC is activated for a target UE with an established PDU session and when the IRI-POI present in the SMF generates the xIRI containing an SMFStartOfInterceptionWithEstablishedPDUSession record (see clause 6.2.3.2.5), the CC-TF present in the SMF shall send an activation message to the CC-POI present in the UPF to generate the xCC.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.10.3.2	Provisioning over LI_X1

##### 7.10.3.2.1	General

For Phase-1 of HR LI, the following LI functions are provisioned over LI_X1 by the LIPF using the X1 protocol defined in ETSI TS 103 221-1 [7] with the LIPF playing the role of ADMF and the following LI functions playing the role of NE as per the reference model depicted in ETSI TS 103 221-1 [7]:

-	BBIFF-C present in the SMF.

-	BBIFF-C present in the SGW-C.

-	BBIFF present in the SGW.

-	LMISF-IRI.

As described in clause 7.10.1, the Phase-1 of HR LI applies to all inbound roaming UEs that use the IMS-based services with home-routed roaming, possibly for a given APN or DNN. The target identities "HR" and "IMSSignaling" are used for Phase-1 of HR LI.

The same XID shall be used for the Phase-1 task at the BBIFF/BBIFF-C and LMISF-IRI.

##### 7.10.3.2.2	Provisioning of BBIFF-C and BBIFF

The BBIFF-C/BBIFF shall be provisioned by the LIPF over LI_X1 using the X1 protocol as described in clause 5.2.2 with the additional details in Table 7.10.3.2.2-1.

The additional details for the LI_X1 ActivationTask message is shown in table 7.10.3.2.2-1.

Table 7.10.3.2.2-1: Additional details for the ActivateTask message for activating BBIFF-C and BBIFF

| ETSI TS 103 221-1 field name | Description | M/C/O |
| --- | --- | --- |
| XID | Shall be set to a value assigned by the LIPF. This shall be same as the XID used for ActivateTask as shown in table 7.10.3.2.3-1 when LMISF-IRI is configured using the ActivateTask. | M |
| TargetIdentifiers | Shall contain Target Identifiers of type "HR" and "IMSSignaling" (see table 7.10.3.2.2-2). It may include an APN or a DNN. | M |
| DeliveryType | Set to "X2andX3". | M |
| ListOfDIDs | Shall give the DID of the LMISF-IRI to which the xIRI and xCC should be delivered respectively. The delivery endpoints are configured using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to the task activation. Separate delivery endpoints for xIRI and xCC should be configured. | M |

Table 7.10.3.2.2-2: Target Identifier Type for enabling HR LI

| Identifier type | ETSI TS 103 221-1 [7] TargetIdentifier type | Definition |
| --- | --- | --- |
| HR | TargetIdentifierExtension /HR | Empty tag (see XSD schema) |
| IMSSignaling | TargetIdentifierExtension/IMSSignaling | Empty tag (see XSD schema) |
| APN | TargetIdentifierExtension/APN | UTF-8 string (see XSD schema) |
| DNN | TargetIdentifierExtension/DNN | UTF-8 string (see XSD schema) |

##### 7.10.3.2.3	Provisioning of LMISF-IRI

The LMISF-IRI listed as the delivery endpoint over LI_X2_LITE for xIRI generated by the BBIFF-C/BBIFF and for the xCC generated by the BBIFF-U/BBIFFshall be provisioned by the LIPF over LI_X1 using the X1 protocol as described in clause 5.2.2 with the additional details in table 7.10.3.2.3-1.

The provisioning of LMISF-IRI for Phase-1 is to enable it to receive the xIRIs and xCC sent from the BBIFF-C (SMF, SGW-C), BBIFF-U (UPF, SGW-U) and BBIFF (SGW). As an alternate deployment option, LMISF-IRI may be presumed to be enabled to receive such xIRI/xCC by default. This clause does not apply to such alternate deployment option.

Table 7.10.3.2.3-1 shows the additional details for the LI_X1 ActivateTask message used for provisioning the LMISF-IRI for Phase-1.

Table 7.10.3.2.3-1: Additional details for the ActivateTask message for activating the LMISF-IRI for Phase-1

| ETSI TS 103 221-1 field name | Description | M/C/O |
| --- | --- | --- |
| XID | Shall be set to a value assigned by the LIPF. This shall be same as the XID used for ActivateTask as shown in table 7.10.3.2.2-1. | M |
| TargetIdentifiers | Shall contain Target Identifiers of type "HR" and "IMSSignaling" (see table 7.10.3.2.2-2). It may include an APN or a DNN (see NOTE 1). | M |
| DeliveryType | Set to "X2andX3". LMISF-IRI shall use this only to enable the receiving of xIRI and xCC from the BBIFF-C/BBIFF. | M |
| ListOfDIDs | Shall be given as an empty list, since DIDs are not required in LMISF-IRI for Phase-1. See NOTE 2. | M |
| NOTE 1: The APN or DNN may be included only for the LMISF-IRI that is provisioned as the destination end point for the BBIFFs with the same APN/DNN. NOTE 2: The usual behaviour of LI functions to reject tasks without a least one valid destination for the DeliveryType provisioned shall be ignored in this case. |  |  |

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

##### 7.10.3.3.1	General

LI_X2_LITE is an interface between the BBIFF-C/BBIFF and the LMISF-IRI used to carry xIRI messages for HR LI Phase-1.

For N9HR LI, the BBIFF-C present in the SMF shall generate the xIRIs as described in clause 7.10.3.3.2. For S8HR LI, the BBIFF-C present in the SGW-C and BBIFF present in the SGW shall generate the xIRIs as defined in clause 7.10.3.3.3.

The xIRIs are generated only when the following prior conditions are met:

-	If the received ActivateTask received by the BBIFF-C includes an APN or a DNN:

-	the APN (for S8HR) or the DNN (for N9HR) used for the IMS-based service matches the APN or the DNN in the ActivateTask.

-	If the ActivateTask doesn’t include an APN or a DNN:

-	the MCC + MNC of the Operator Identifier field of the DNN (for N9HR) or APN (for S8HR) is different from the MCC+MNC configured in the SMF (N9HR) or SGW-C/SGW (S8HR) - see TS 29.502 [16] clause 6.1.6.2.2 and 23.003 [19] clause 9.1.2.

AND

-	the Network Identifier field of DNN (for N9HR) or APN (for S8HR) contains "IMS" (IMS services) - see GSMA IR.88 [67].

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.10.3.5	Triggering of BBIFF-U from BBIFF-C over LI_T3

##### 7.10.3.5.1	General

With HR LI Phase-1, the user plane packets from the IMS signaling channel are delivered to the LMISF-IRI for all inbound roaming UEs with home-routed roaming.

When BBIFF is separated into BBIFF-C and BBIFF-U, these user plane packets are captured at the BBIFF-U. In order to enable the BBIFF-U to do that function, the BBIFF-C triggers the BBIFF-U over the LI_T3 interface.

The trigger sent from the BBIFF-C to the BBIFF-U shall contain the information necessary to isolate the identified N9HR or S8HR signaling flows. The BBIFF-C shall send a trigger each time the BBIFF-C detects that a new N9HR or S8HR flow has been established as described in clauses 7.10.3.5.2 and 7.10.3.5.3.

The BBIFF-U delivers the user plane from the IMS signaling channel over the LI_X3_LITE-S interface to the LMISF-IRI.

##### 7.10.3.5.2	N9HR LI

When the BBIFF-C present in the SMF detects that a PDU session with an IMS signaling related QoS Flow is for an inbound roaming UE with home-routed roaming (referenced in this clause as an N9HR session), it shall send a trigger to the BBIFF-U present in the UPF over the LI_T3 interface as described in clause 5.2.5 with the additional details shown in table 7.10.3.5.2-1.

The exact point at which the trigger is sent is left to the implementation (preferably, when the SMF receives the N4: PFCP Session Establishment/Modification Response from the UPF, see TS 29.244 [15] clauses 6.3.2 and 6.3.3), however, the following criteria shall be used to determine an N9HR session has been established:

:

-	Then conditions for generating xIRI messages out of the BBIFF-C (see clause 7.10.3.3.1) are met

AND

- The 5QI value associated with the QoS Flow is 5 – see GSMA NG.114 [68].

If the PDU session for IMS services is already established for an inbound roaming UE with Home-Routed based roaming when the above indicated ActivateTask is received, then the BBIFF-C shall send the trigger at the time Activation Task is received from the LIPF.

The details of ActivateTask sent to the BBIFF-U are shown in table 7.10.3.5.2-1.

Table 7.10.3.5.2-1: Additional details for the ActivateTask message for triggering the BBIFF-U in the UPF

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| XID | Shall be set to the XID allocated by the BBIFF-C. | M |
| TargetIdentifiers | Packet detection criteria as determined by the BBIFF-C in the SMF, which enables the BBIFF-U to isolate user-plane packets. The BBIFF-U in the UPF shall support the identifier types given in table 6.2.3.3.1-2. The BBIFF-C shall populate the TargetIdentifiers as described below: - If the SMF has created a PDR sufficient for isolating the signalling flow, the BBIFF-C shall populate the TargetIdentifiers using a PDR ID set to the value of the PDR ID of that PDR. - If no sufficient PDR exists the BBIFF-C shall use populate the TargetIdentifiers with both: - a PDR with: - A PDR ID allocated by the BBIFF-C that is valid only for the BBIFF-U. - PDIs populated with information sufficient for the BBIFF-U to isolate the N9HR signaling flow. - A PDR ID TargetIdentifier type populated with the PDR ID of the PDR created above. | M |
| DeliveryType | Set to "X3Only". | M |
| CorrelationID | Correlation ID to assign to X3 PDUs generated by the BBIFF-U in the UPF. This field is populated with the same CorrelationID the BBIFF-C in the SMF uses for the associated xIRI. | M |
| ProductID | Shall be set to the XID of the Task Object associated with the interception at the BBIFF-C. | M |

When the BBIFF-C present in the SMF detects that the PDU session is released (e.g. when SMF receives the N4: PFCP Session Deletion Response from the UPF, see TS 29.244 [15] clause 6.3.4), it shall send a deactivation message to the BBIFF-U present in the UPF over the LI_T3 interface, if the task is still active in the BBIFF-U.

The BBIFF-C shall also send the deactivation message to the BBIFF-U when a DeactivateTask is received from the LIPF for the XID if the task is still active in the BBIFF-U.

##### 7.10.3.5.3	S8HR LI

When the BBIFF-C present in the SGW-C detects that the default bearer used for IMS signaling is activated on the PDN connection for an inbound roaming UE with home-routed roaming, it shall send a trigger to the BBIFF-U present in the SGW-U over the LI_T3 interface as described in clause 5.2.5 with the additional details shown in table 7.10.3.5.3-1.

The exact point at which the trigger is sent is left to the implementation (preferably, when the SGW-C receives the Sx: Session Establishment/Modification Response from the SGW-U). However,  the following criteria shall be used to determine an S8HR session has been established:

-	The conditions for generating xIRI messages out of the BBIFF-C (see clause 7.10.3.3.1) are met

AND

- The QCI value associated with the default bearer is 5 – see GSMA NG.114 [68].

If the default bearer (for IMS signaling bearer) on the PDN connection is already established for an inbound roaming UE with Home-Routed based roaming when the above indicated ActivateTask is received, then the BBIFF-C shall send the trigger at the time Activation Task is received from the LIPF.

The details of ActivateTask sent to the BBIFF-U present in the SGW-U are shown in table 7.10.3.5.3-1.

Table 7.10.3.5.3-1: ActivateTask message for triggering the BBIFF-U in the SGW-U

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| XID | Shall be set to the XID allocated by the BBIFF-C. | M |
| TargetIdentifiers | Packet detection criteria as determined by the BBIFF-C in the SGW-C, which enables the BBIFF-U in SGW-U to isolate user-plane packets. The BBIFF-U in the SGW-U shall support the identifier types given in table 6.2.3.3.1-2. The target identity type of PDR ID shall be mandatory. The BBIFF-C shall populate the TargetIdentifiers as described below: - If the SGW-C has created a PDR sufficient for isolating the signalling flow, the BBIFF-C shall populate the TargetIdentifiers using a PDR ID set to the value of the PDR ID of that PDR. - If no sufficient PDR exists the BBIFF-C shall use populate the TargetIdentifiers with both: - a PDR with: - A PDR ID allocated by the BBIFF-C that is valid only for the BBIFF-U. - PDIs populated with information sufficient for the BBIFF-U to isolate the S8HR signaling flow. - A PDR ID TargetIdentifier type populated with the PDR ID of the PDR created above. | M |
| DeliveryType | Set to "X3Only". | M |
| CorrelationID | Correlation ID to assign to X3 PDUs generated by the BBIFF-U in the SGW-U. This field is populated with the same CorrelationID the BBIFF-C in the SGW-C uses for the associated xIRI. | M |
| ProductID | Shall be set to the XID of the Task Object associated with the interception at the BBIFF-C. | M |

When the BBIFF-C present in the SGW-C detects that the PDN connection is released (e.g. when SGW-C receives the Sx: Session Release Response from the SGW-U), it shall send a deactivation message to the BBIFF-U present in the SGW-U over the LI_T3 interface, if the task is still active in the BBIFF-U.

The BBIFF-C present in the SGW-C shall also send the deactivation message to the BBIFF-U present in the SGW-U when a DeactivateTask is received from the LIPF for the XID if the task is still active in the BBIFF-U.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

##### 7.10.3.6.1	BBIFF-U

The BBIFF-U in UPF and the BBIFF-U in SGW-U shall send the xCC over LI_X3_LITE_S for each of the packet matching the criteria specified in the Triggering message (i.e. Activate Task message) received over the LI_T3 from the BBIFF-C.

If a PDR is included in the list of TargetIdentifiers sent in the ActivateTask message, the BBIFF-U shall use that PDR and not any PDR matching the provisioned PDR ID that was created by the SMF.

##### 7.10.3.6.2	BBIFF

The BBIFF present in the SGW shall send the xCC over LI_X3_LITE_S for each of the packet from the default bearer with the QCI value of 5 (GSMA NG.114 [68]) with following other conditions:

-	If the received ActivateTask includes an APN:

-	the APN used for the IMS-based service matches the APN in the ActivateTask.

-	If the ActivateTask doesn’t include an APN:

-	the MCC + MNC of the Operator Identifier field of the APN (for S8HR) is different from the MCC+MNC configured in the SGW - see TS 29.502 [16] clause 6.1.6.2.2 and 23.003 [19] clause 9.1.2.

AND

-	the Network Identifier field or APN contains "IMS" (IMS services) - see GSMA IR.88 [67].

The BBIFF in SGW uses the QCI value of 5 (GSMA NG.114 [68]) to identify that the packets are from the IMS signaling bearer.

## **** END OF MAIN DOCUMENT CHANGES ****

## **** FIRST ATTACHMENT CHANGE (urn_3GPP_ns_li_3GPPX1Extensions.xsd) ****

---a/33128/r19/urn_3GPP_ns_li_3GPPX1Extensions.xsd
+++b/33128/r19/urn_3GPP_ns_li_3GPPX1Extensions.xsd

@@ -59,6 +59,7 @@

59	59		      <xs:element name="NetworkInstance" type="xs:hexBinary"/>

60	60		      <xs:element name="GTPTunnelDirection" type="GTPTunnelDirection"/>

61	61		      <xs:element name="FTEID" type="FTEID"/>

62	+	      <xs:element name="PDR" type="xs:hexBinary"/>

62	63		    </xs:choice>

63	64		  </xs:complexType>

64	65

## **** END OF ALL CHANGES ****