3GPP SA3#99-LI	S3i250659

# 15-18 November 4-7, 2025, Overland Park (US)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0792 | rev | - | Current version: | 17.18.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional PTC corrections (R17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI16 |  |  |  |  |  | Date: |  |  | 2025-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR provides additional corrections to the PTC related text in 33.128. The trigger conditions for some of the xIRIs may not happen as presumed in the specification. The text corrected to reflect the actual triggers. The TBCP: Release is from the PTC Client to the PTC Server. The TS 33.128 says the other way. When a participant joins the chat-group PTC session, according to OMA specification, no NOTIFY is sent to the other participants. The TS 33.128 says it does and defines an xIRI. PartyJoin and xIRIPartyDrop to be generated when the participating PTC Server sees the SIP: NOTIFY. The PTC server hosting the chat-group provides the LI when the chat-group ID is the target. When one of the PTC clients is a target, the participating PTC server that provides the LI for that target do not generate the xIRI PartyJoin and PartyDrop. Therefore, only ChatGroupID as a possible target identity has to be retained for PartyJoin and PartyDrop |  |  |  |  |  |  |  |  |
| Summary of change: |  | FloorControl xIRI is corrected to reflect the above indicated correction. The PartyJoin and PartyDrop are corrected to reflect the above change. . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect parameter value in an xIRI. Incorrect trigger definition for TBCP: Release and xRI PartyJoin and xIRIPartyDrop. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.9, 7.5.2.10, 7.5.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### ** First Change **

#### 7.5.2.14	PTC floor control

The IRI-POI present in the PTC server shall generate an xIRI containing a PTCFloorControl record when the IRI-POI present in the PTC server detects when the PTC target requests floor control (i.e. send media), when floor control is granted to PTC target, when floor control request from the PTC target is rejected/released, when the floor becomes open (e.g. idle), when the floor control request from the PTC target is queued, when the floor control request from the PTC target is dequeued, or when the floor control request is revoked. In addition, when the PTC chat group is the PTC target, the IRI-POI present in the PTC server hosting the PTC chat group shall generate an xIRI containing a PTCFloorControl record when the IRI-POI present in the PTC server detects any of the previously mentioned scenarios for all PTC participants participating in the PTC chat group session. Accordingly, the IRI-POI in the PTC server generates the xIRI when the following events are detected:

-	when the PTC server receives a TBCP Talk Burst Request from the PTC target.

-	when the PTC server hosting the PTC chat group, where the PTC chat group is the PTC target, receives a TBCP Talk Burst Request from a PTC participant.

-	when the PTC server sends a TBCP Talk Burst Granted to a PTC target.

-	when the PTC server hosting the PTC chat group, where the PTC chat group is the PTC target, sends a TBCP Talk Burst Granted to a PTC participant.

-	when the PTC server sends a TBCP Talk Burst Taken to a PTC target.

-	when the PTC server hosting the PTC chat group, where the PTC chat group is the PTC target, sends a TBCP Talk Burst Taken to a PTC participant.

-	when the PTC server sends a TBCP Talk Burst Deny to a PTC target.

-	when the PTC server hosting the PTC chat group, where the PTC chat group is the PTC target, sends a TBCP Talk Burst Deny to a PTC participant.

-	when the PTC server receives a TBCP Talk Burst Release from a PTC target.

-	when the PTC server hosting the PTC chat group, where the PTC chat group is the PTC target, receives a TBCP Talk Burst Release from a PTC participant.

-	when the PTC server sends a TBCP Talk Burst Idle to a PTC target.

-	when the PTC server hosting the PTC chat group, where the PTC chat group is the PTC target, sends a TBCP Talk Burst Idle to a PTC participant.

-	when the PTC server sends a TBCP Talk Burst Request Queue Status Response to a PTC target.

-	when the PTC server hosting the PTC chat group, where the PTC chat group is the PTC target, sends a TBCP Talk Burst Request Queue Status Response to a PTC participant.

-	when the PTC server receives a TBCP Talk Burst Cancel from a PTC target.

-	when the PTC server hosting the PTC chat group, where the PTC chat group is the PTC target, receives a TBCP Talk Burst Cancel from a PTC participant.

-	when the PTC server sends a TBCP Talk Burst Revoke to a PTC target.

-	when the PTC server hosting the PTC chat group, where the PTC chat group is the PTC target, sends a TBCP Talk Burst Revoke to a PTC participant.

Table 7.5.2-14: Payload for PTCFloorControl record

| Field name | Description | M/C/O |
| --- | --- | --- |
| pTCTargetInformation | Provide PTC target identity. At least one among MCPTT ID, IMPU, IMPI, InstanceIdentifierURN and PTCChatGroupID shall be provided for PTCTargetInformation. | M |
| pTCDirection | Indicates the direction of the session relative to the target: "toTarget" or "fromTarget." | M |
| pTCSessioninfo | Shall provide PTC session information such as PTC Session URI and PTC Session type (e.g. on-demand, pre-established, ad-hoc, pre-arranged, group session). | M |
| pTCFloorActivity | Sequence of: a) "TBCP_Request": Received by the PTC server to request permission for the PTC target or PTC participant to send a talk burst. b) "TBCP_Granted": Used by the PTC server to notify the PTC target or PTC participant that it has been granted permission to send a talk burst. c) "TBCP_Deny": Used by the PTC server to notify a PTC target or PTC participant that it has been denied permission to send a talk burst. d) "TBCP_Idle": Used by the PTC server to notify the PTC target or PTC participant that no one has the permission to send a Talk Burst at the moment and that it may accept the TBCP talk burst request message. e) "TBCP_Taken": Used by the PTC server to notify the PTC target or PTC participant that another PTC participant has been given permission to send a talk burst. f) "TBCP_Revoke": Used by the PTC server to revoke the media resource from the PTC target or PTC participant and can be used for pre-emption functionality but is also used by the system to prevent overly long use of the media resource. g) "TBCP_Queued": Indicates the request to talk is queued, if queued floor control is supported. Include identification of the PTC target or PTC participant that has the queued talk burst, if known. h) "TBCP_Release": Indicates the request to talk has completed. | M |
| pTCFloorSpeakerID | Include identification of the PTC participant that has initiated the talk burst, if known. | C |
| pTCMaxTBTime | Include the maximum duration value for the talk burst before the permission is revoked. This parameter is defined in seconds. Provide when known. | C |
| pTCQueuedFloorControl | Indicates if queuing is supported by the PTC server and the PTC target's device. | C |
| pTCQueuedPosition | Include if queue position in the TBCP is detected by the IRI-POI. | C |
| pTCTalkBurstPriority | If more than one level of priority is supported, indicates the talk burst priority level of the PTC target. | C |
| pTCTalkBurstReason | The reason for the denial or revoke of a Talk Burst. Provide when known. | C |

### ** Next Change **

#### 7.5.2.9	PTC party join

The IRI-POI present in the PTC server hosting the PTC chat group session when the PTC chat group is the PTC target, shall generate an xIRI containing a PTCPartyJoin record when the IRI-POI present in that PTC server detects when a PTC participant joins (or re-joins) an on-going PTC chat group session. . Accordingly, the IRI-POI in the participating PTC server generates the xIRI when the following event is detected:

-	when the PTC server hosting a PTC chat group session sends a SIP 200 OK in response to a SIP INVITE indicating a PTC participant joining the PTC chat group session.

Table 7.5.2-9: Payload for PTCPartyJoin record

| Field name | Description | M/C/O |
| --- | --- | --- |
| pTCTargetInformation | Provide PTC target identity. PTCChatGroupID shall be provided for PTCTargetInformation. | M |
| pTCDirection | Indicates the direction of the session relative to the target: "toTarget" or "fromTarget." | M |
| pTCSessionInfo | Shall provide PTC session information such as PTC Session URI and PTC Session type (e.g. on-demand, pre-established, ad-hoc, pre-arranged, group session). | M |
| pTCParticipants | Shall identify the individual PTC participants of the communication session, when known. | C |
| participantPresenceStatus | Shall provide the Participant Presence Status, which is a list of: - PresenceID: Identity of PTC client(s) or PTC group, when known. - PresenceType: Identifies type of ID [PTC client(s) or PTC group]. - PresenceStatus: Presence state of each ID. True indicates PTC client is available, while false indicates PTC client is unavailable. Report when the Presence functionality is supported by the PTC server and the PTC server assumes the role of the watcher on behalf of the PTC target. | C |
| pTCMediaStreamAvail | Shall include this parameter to indicate if the PTC target is able/not able to receive media streams immediately. True indicates available for media, while false indicates not able to accept media. | M |
| pTCBearerCapability | Shall provide when known the media characteristics information elements of the PTC session, encoded in SDP format as per RFC 4566 [43] clause 5. | C |

NOTE:	The PartyJoin xIRI can only be generated by the PTC server hosting the chat-group session. Only the PTCChatGroupID applicable for the PTC server hosting the chatgroup

#### 7.5.2.10	PTC party drop

The IRI-POI present in the PTC server hosting the PTC chat group session, when the PTC chat group is the PTC target, shall generate an xIRI containing a PTCPartyDrop record when the IRI-POI present in that PTC server detects that a PTC participant leaves the PTC chat group session that still remains active with other PTC participants. Accordingly, the IRI-POI in the participating PTC server generates the xIRI when the following event is detected:

-	when the PTC server hosting a PTC chat group session, where the PTC chat group is the target, sends a SIP 200 OK in response to a SIP BYE with the PTC chat group session remaining active with other PTC participants.

-	when the participating PTC server of a PTC target forwards a SIP NOTIFY (received from the PTC server hosting the PTC chat group session) to the PTC target containing information about a PTC participant leaving the PTC chat group session.

Table 7.5.2-10: Payload for PTCPartyDrop record

| Field name | Description | M/C/O |
| --- | --- | --- |
| pTCTargetInformation | Provide PTC target identity. PTCChatGroupID shall be provided for PTCTargetInformation. | M |
| pTCDirection | Indicates the direction of the session relative to the target: "toTarget" or "fromTarget." | M |
| pTCSessionInfo | Shall provide PTC session information such as PTC Session URI and PTC Session type (e.g. on-demand, pre-established, ad-hoc, pre-arranged, group session). | M |
| pTCPartyDrop | Shall provide the identity of the participant that leaves the PTC session. | M |
| pTCParticipantPresenceStatus | Shall provide the Participant Presence Status, which is a list of: - PresenceID: Identity of PTC client(s) or PTC group, when known. - PresenceType: Identifies type of ID [PTC client(s) or PTC group]. - PresenceStatus: Presence state of each ID. True indicates PTC client is available, while false indicates PTC client is unavailable. Report when the Presence functionality is supported by the PTC server and the PTC server assumes the role of the watcher on behalf of the PTC target. | C |

NOTE:	The PartyJoin xIRI can only be generated by the PTC server hosting the chat-group session. Only the PTCChatGroupID applicable for the PTC server hosting the chatgroup

### ** End of all Changes **