Source:	Bytedance

Title:	[DaCAS] Propose adding device mesh in target device template

Document for:	Discussion and Agreement

Agenda Item:	1.6

# 1 Introduction

In TR 26.933 - Study on Diverse Audio Capturing Systems, the necessity of an immersive audio capturing system in UEs has been mentioned [1]. This is a challenging task because end-user devices carry different microphone geometries. During SA4#131 meeting in Geneva, a template for target device definition is proposed [2], as well as example solutions [3][4].

The source noticed that these example solutions can benefit from having detailed shape information of target devices, for example, their triangle meshes. Both [3] and [4] used FEM simulation to compute the Ambisonic encoding matrix, which would prefer an accurate triangle mesh of the device.

Apart from help building the example solutions, having a collective database that has mesh information of diverse device type can help testing the example solutions in a more comprehensive manner, since availability of devices are different across companies.

However, in current template for target device definition [2], we haven’t included triangle mesh information. Thus, the source would like to propose a way adding it.

# 2 Example triangle mesh information

A widely used file format for triangle meshes is obj wavefront. It’s a specially formatted text file, in which vertices coordinates, triangle definitions, and colour texture can be defined. In our use case, we can only include vertices and triangle definitions. Here’s an example triangle mesh stored in an obj file, which represents a 10cm x 3cm square:

# List of geometric vertices, with (x, y, z, [w]) coordinates, w is optional and defaults to 1.0.

v 50 5 15 1.0

v 50 5 -15 1.0

v -50 5 -15 1.0

v -50 5 15 1.0

...

# Triangle face element (see below)

f 1 2 3

f 3 4 1

…

With the following requirements

- Uses right-hand coordinate.

- The coordinate origin is at device reference point.

- The unit of the vertices coordinates is millimetres.

- To ensure stability for FEM simulation:

  - The triangle mesh has needs to be manifold [6]

  - Euler characteristic [7] equals 2.

  - Maximum edge length is 5mm, which is slightly below 1/3 of the wavelength of 20kHz.

  - Maximum error of vertices position is 5mm, which is slightly below 1/3 of the wavelength of 20kHz.

We provided an example triangle mesh in the attachment, which is a 10cm * 0.5cm * 5cm cube, stored in an obj file.

Figure 1. Example triangle mesh, visualized using Blender [8]

## 2.1 Example workflow of creating conformant device mesh.

We here give an example workflow of how to get a conformant mesh with the help of structure engineers, from the perspective of a device manufacturer:

- Ask structure engineer to export stl file from their favorite CAD software [9] [10]

- Import STI file into Blender [8]

- Move device reference point to coordinate origin.

- Apply remeshing and triangularization modifiers to provide a manifold triangle mesh. Remeshing accuracy can be set to hide ID details that’s irrelevant to FEM simulation, as long as the resulting mesh meets our a-for-mentioned requirements.

- Export to .obj [11]

| 2. Imported STL | 3. Recentered | 4. Remeshed and triangularized |
| --- | --- | --- |

Figure 2. Example workflow of creating conformant mesh (ID detail is censored using color block)

# 3 Proposal

The source suggests adding device triangle mesh as part of target device template, with the above requirements.

# References

[1] TS 26.933 Study on Diverse Audio Capturing systems

[2] S4-250139 Templates for potential target devices and database

[3] S4-250125 [DaCAS] Scene-based audio capturing example solution

[4] S4-250140 Multi-microphone capture to Ambisonics format conversion

[5] Wavefront .obj file https://en.wikipedia.org/wiki/Wavefront_.obj_file

[6] https://en.wikipedia.org/wiki/Manifold

[7] Euler characteristic https://en.wikipedia.org/wiki/Euler_characteristic

[8] Blender https://www.blender.org/

[9] AutoCAD: To Export 3D Solids to an STL File https://help.autodesk.com/view/ACD/2023/ENU/?guid=GUID-E40AB085-5A19-4E3E-A74C-82F62DCD0DDB

[10] SolidWorks: Exporting 3D Print Files https://help.solidworks.com/2021/english/SolidWorks/sldworks/t_exporting_3d_print_files_impexp.htm

[11] Blender: Wavefront OBJ https://docs.blender.org/manual/en/3.1/files/import_export/obj.html#wavefront-obj