3GPP TSG-SA WG4 Meeting post 132	S4aA250030

online, June 4th 2025

Source:	vivo

Title:	Parameter updates on end to end simulation

Spec:	3GPP TR 26.940 v0.2.0

Agenda item:	1.9

Document for:	Discussion/Agreement

1. Introduction

At the recent SA4#132 meeting, the framework of how to obtain delay-error profiles and how to generate the error trace to reflect the realistic GEO channel by RAN simulations are agreed. However, some simulation assumptions are still need more discussions to be determined, this contribution will provide more detail analysis on following aspects:

- Link budget analysis to determine the target SNR

Link budget is calculated based on SET-1 GEO parameters and handheld UE parameters with different UE transmit power considerations, and then the target SNR are chosen from some of the link budget results as 5 dB, 8 dB, 10 dB, [12 dB].

- TBS considerations

The brackets of candidate TBS can be removed and TBS are evaluated with 15kHz of SCS since most of the NPUSCH duration time with 3.75kHz SCS are larger than bundling time with HD-FDD consideration.

2. Reason for Change

The present document provides how to determine the target SNR and TBS determinations to derive the error trace to reflect the realistic GEO channel.

- Link budget analysis

In the TR 38.821[1], link budget is represented by Carrier-to-noise-and-interference ratio (CNIR) of transmission link between satellite and UE or carrier-to-noise ratio (CNR) if interference is not considered. For link level simulation, CNR can be used as target SNR to determine whether the target BLER can be achieved, as shown in Figure 1, if the corresponding BLER at the target SNR is higher than target BLER, the TB would be failed to be decoded.

Figure 1 Relationship of target SNR and target BLER based on the SNR-BLER curve of link-level simulation

Based on the link budget analysis in TR38.821, the formula for CNR calculation is

CNR dB=EIRP [dBW]+GT[dB/K]-k [dBW/K/Hz]-PLFS [dB]-PLA [dB]-PLSM [dB]-PLSL[dB]-PLAD [dB]-B [dBHZ]

- EIRP

EIRP is effective isotropic radiated power (EIRP), which can be calculated by

EIRP dBW=PT dBW-LC dB+GT [dBi]

For UL, PT is the antenna transmit power of UE, the value usually be 23dBm for handheld UE since it was categorized as the power class 3. In addition, high power UE other than PC3 has been agreed to be feasible for Rel-19 IoT-NTN in RAN4, e.g., PC2, PC1.5, PC1, which means that the maximum transmit power can be as large as 31dBm. Moreover, higher transmit power such as 33dBm can also be considered based on RAN study outcome.  GT is the antenna gain of transmitter, which can be considered as 0dBi for handheld UE.

- GT

GT is antenna-gain-to-noise-temperature, which is 19 dB K-1 for Set-1 GEO satellite in S-band.

- k

k is Boltzmann constant and equals to -228.6 dBW/K/Hz.

- PLFS

The free space path loss (FSPL) in dB for a separation distance d in meter and frequency fc in GHz is given by[2]

For a ground terminal, the distance d  can be determined by the satellite/HAPS altitude  and elevation angle α by

where  denotes Earth radius. In general, the minimum elevation angle of GEO is 12.5 degree considered as the edge of edge beam.

- PLA

Attenuation by atmospheric gases which is entirely caused by absorption depends mainly on frequency, elevation angle, altitude above sea level and water vapour density (absolute humidity). Only if elevation angles below 10 degrees and frequency above 1 GHz it is recommended that the calculation is performed, otherwise, it may normally be neglected.

- PLSM

PLSM is shadowing margin, which can be considered as 3dB.

- PLSL

PLSL is is scintillation loss, for simulations below 6 GHz, PLS is equal to 2.2dB for latitudes of maximum ±20°. For latitudes above ±20°of latitude, PLS=0.

- PLAD

PLAD is additional loss, for example degradation due to feeder links in case of non-regenerative systems, which is considered as 0dB. Specially, for S band, we assume that the User Equipment has an omni-directional antenna of linear polarization, while the antenna on board space-borne or airborne platforms features typically employs circular polarization. Hence a polarization mismatch of 3 dB has to be taken into account for the radio link budget computation.

- B

B is channel bandwidth, which is depends on the SCS and resource allocation.

Based on the above analysis of per parameters, we list all the candidate values of parameters and the corresponding CNR in Table 1.

Table 1 UL link budget analysis

|  | Parameter | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Satellite parameters | G/T | 19 dB K-1 |  |  |  |  |
|  | Antenna polarization | Circular polarization |  |  |  |  |
|  | Satellite altitude | 35786km |  |  |  |  |
| UE parameters | Antenna polarization | Linear polarization |  |  |  |  |
|  | Antenna gain | 0dBi |  |  |  |  |
|  | Elevation angle | 12.5 degree |  |  |  |  |
|  | UE power | 23dBm | 26dBm | 29dBm | 31dBm | [33] dBm |
| System parameters | Carrier frequency | 2GHz |  |  |  |  |
|  | SCS | 15kHz |  |  |  |  |
|  | BW | 15kHz |  |  |  |  |
|  | PLFS | 190.6 dB |  |  |  |  |
|  | PLA | 0 dB |  |  |  |  |
|  | PLSM | 3 dB |  |  |  |  |
|  | PLSL (high latitude) | 0 dB |  |  |  |  |
|  | PLAD (Polarization loss) | 3dB |  |  |  |  |
| CNR |  | 2.26 dB | 5.26 dB | 8.26 dB | 10.26 dB | [12.26] dB |

Based on the CNR results in above table, considering different UE transmit power, the minimum and maximum values are 2.26dB and 10.26dB (12.26dB if 33dBm of transmit power is supported by RAN4), respectively.  For the target SNR determination, CNR values result from high UE transmit power of 26dBm, 29dBm, 31dBm, [33dBm] can be used since uplink usually suffer as bottleneck.

Proposal#1: based on the above calculation, the target SNR are considered as 5 dB, 8 dB, 10 dB, [12 dB] resulting from round of CNR values for simplification. Specially, [12dB] depends on RAN4 specifications.

- TBS determination

For TBS determination, based on Table 16.5.1.2-2 of TBS table for NPUSCH in TS 36.213[3], it is determined by two parameters: IRU and ITBS, IRU represents the resource unit (RU) index, which is corresponding to the number of RU, and ITBS represents the TBS index, which is corresponding to the MCS index. Note that the modulation is 2/pi BPSK and 16QAM for ITBS=0,2 and  ITBS=14~21, respectively, otherwise, the modulation is QPSK. Therefore, all the possible combinations of IRU and ITBS for different TBS are provided in following sections.

Furthermore, the duration time without repetition for different SCS is calculated. Considering that IoT UE works as HD-FDD mode, only part of the bundling time can be used for uplink transmission, and the rest time should be used for downlink transmission and DL/UL switching time. For example, for a certain bundling time of 80ms, if regardless of the DL control signal transmission and DL/UL switching time, the realistic time for uplink and downlink can distributed as half and half, which means only 40ms can be used for NPUSCH transmission. Therefore, an initial comparison is done between the realistic time for uplink with the duration time of NPUSCH when half of the bundling time is used for uplink transmission.

In the following tables, values in gray colour means that the duration time cannot suit the bundling time restriction.

Observation#1: It can be observed that 3.75kHz of SCS cannot satisfy the bundling time restriction in most of the TBS cases, thus the candidate TBS can be evaluated with 15kHz of SCS.

- 80ms bundling time

Table 2-1 Combinations of TBS 144bit w/ 80ms bundling time

| TBS | IRU | Number of RU | ITBS (MCS) | Duration Time w/o repetition |
| --- | --- | --- | --- | --- |
| 144 | 3 | 4 | 1 | 3.75scs: 4*32 =128ms 15scs： 4 * 8 = 32ms |
|  | 1 | 2 | 5 | 3.75scs: 2*32 =64ms 15scs： 2 * 8 = 16ms |
|  | 0 | 1 | 10 | 3.75scs: 1*32 =32ms 15scs： 1 * 8 = 8ms |

Table 2-2 Combinations of TBS 256bit w/ 80ms bundling time

| TBS | IRU | Number of RU | ITBS (MCS) | Duration Time w/o repetition |
| --- | --- | --- | --- | --- |
| 256 | 6 | 8 | 1 | 3.75scs: 8*32 =256ms 15scs： 8 * 8 = 64ms |
|  | 4 | 5 | 3 | 3.75scs: 5*32 =160ms 15scs： 5 * 8 = 40ms |
|  | 3 | 4 | 4 | 3.75scs: 4*32 =128ms 15scs： 4 * 8 = 32ms |
|  | 2 | 3 | 6 | 3.75scs: 3*32 =96ms 15scs： 3 * 8 = 24ms |
|  | 1 | 2 | 8 | 3.75scs: 2*32 =64ms 15scs： 2 * 8 = 16ms |

Table 2-3 Combinations of TBS 328bit w/ 80ms bundling time

| TBS | IRU | Number of RU | ITBS (MCS) | Duration Time w/o repetition |
| --- | --- | --- | --- | --- |
| 328 | 5 | 6 | 3 | 3.75scs: 6*32 =192ms 15scs： 6 * 8 = 48ms |
|  | 4 | 5 | 4 | 3.75scs: 5*32 =160ms 15scs： 5 * 8 = 40ms |
|  | 3 | 4 | 5 | 3.75scs: 4*32 =128ms 15scs： 4 * 8 = 32ms |
|  | 2 | 3 | 7 | 3.75scs: 3*32 =96ms 15scs： 3 * 8 = 24ms |
|  | 1 | 2 | 10 | 3.75scs: 2*32 =64ms 15scs： 2 * 8 = 16ms |

Table 2-4 Combinations of TBS 424bit w/ 80ms bundling time

| TBS | IRU | Number of RU | ITBS (MCS) | Duration Time w/o repetition |
| --- | --- | --- | --- | --- |
| 424 | 4 | 5 | 5 | 3.75scs: 5*32 =160ms 15scs： 5 * 8 = 40ms |

- 160ms bundling time

Table 3-1 Combinations of TBS 208bit w/ 160ms bundling time

| TBS | IRU | Number of RU | ITBS (MCS) | Duration Time w/o repetition |
| --- | --- | --- | --- | --- |
| 208 | 5 | 6 | 1 | 3.75scs: 6*32 =192ms 15scs： 6 * 8 = 48ms |
|  | 3 | 4 | 3 | 3.75scs: 4*32 =128ms 15scs： 4 * 8 = 32ms |
|  | 2 | 3 | 4 | 3.75scs: 3*32 =96ms 15scs： 3 * 8 = 24ms |
|  | 0 | 1 | 12 | 3.75scs: 1*32 =32ms 15scs： 1 * 8 = 8ms |

Table 3-2 Combinations of TBS 424bit w/ 160ms bundling time

| TBS | IRU | Number of RU | ITBS (MCS) | Duration Time w/o repetition |
| --- | --- | --- | --- | --- |
| 424 | 4 | 5 | 5 | 3.75scs: 5*32 =160ms 15scs： 5 * 8 = 40ms |

Table 3-3 Combinations of TBS 600bit w/ 160ms bundling time

| TBS | IRU | Number of RU | ITBS (MCS) | Duration Time w/o repetition |
| --- | --- | --- | --- | --- |
| 600 | 5 | 6 | 6 | 3.75scs: 6*32 =192ms 15scs： 6 * 8 = 48ms |

Table 3-4 Combinations of TBS 808bit w/ 160ms bundling time

| TBS | IRU | Number of RU | ITBS (MCS) | Duration Time w/o repetition |
| --- | --- | --- | --- | --- |
| 808 | 6 | 8 | 6 | 3.75scs: 8*32 =256ms 15scs： 8 * 8 = 64ms |
|  | 5 | 6 | 8 | 3.75scs: 6*32 =192ms 15scs： 6 * 8 = 48ms |

- 320ms bundling time

Table 4-1 Combinations of TBS 328bit w/ 320ms bundling time

| TBS | IRU | Number of RU | ITBS (MCS) | Duration Time w/o repetition |
| --- | --- | --- | --- | --- |
| 328 | 5 | 6 | 3 | 3.75scs: 6*32 =192ms 15scs： 6 * 8 = 48ms |
|  | 4 | 5 | 4 | 3.75scs: 5*32 =160ms 15scs： 5 * 8 = 40ms |
|  | 3 | 4 | 5 | 3.75scs: 4*32 =128ms 15scs： 4 * 8 = 32ms |
|  | 2 | 3 | 7 | 3.75scs: 3*32 =96ms 15scs： 3 * 8 = 24ms |
|  | 1 | 2 | 10 | 3.75scs: 2*32 =64ms 15scs： 2 * 8 = 16ms |
|  | 5 | 6 | 3 | 3.75scs: 6*32 =192ms 15scs： 6 * 8 = 48ms |

Table 4-2 Combinations of TBS 776bit w/ 320ms bundling time

| TBS | IRU | Number of RU | ITBS (MCS) | Duration Time w/o repetition |
| --- | --- | --- | --- | --- |
| 776 | 4 | 5 | 9 | 3.75scs: 5*32 =160ms 15scs： 5 * 8 = 40ms |
|  | 3 | 4 | 11 | 3.75scs: 4*32 =128ms 15scs： 4 * 8 = 32ms |

Table 4-3 Combinations of TBS 1000bit w/ 320ms bundling time

| TBS | IRU | Number of RU | ITBS (MCS) | Duration Time w/o repetition |
| --- | --- | --- | --- | --- |
| 1000 | 7 | 10 | 6 | 3.75scs: 10*32 =320ms 15scs： 10 * 8 = 80ms |
|  | 6 | 8 | 7 | 3.75scs: 8*32 =256ms 15scs： 8 * 8 = 64ms |
|  | 5 | 6 | 10 | 3.75scs: 6*32 =192ms 15scs： 6 * 8 = 48ms |
|  | 4 | 5 | 11 | 3.75scs: 5*32 =160ms 15scs： 5 * 8 = 40ms |
|  | 3 | 4 | 12 | 3.75scs: 4*32 =128ms 15scs： 4 * 8 = 32ms |

Table 4-4 Combinations of TBS 1544bit w/ 320ms bundling time

| TBS | IRU | Number of RU | ITBS (MCS) | Duration Time w/o repetition |
| --- | --- | --- | --- | --- |
| 1544 | 7 | 10 | 9 | 3.75scs: 10*32 =320ms 15scs： 10 * 8 = 80ms |

Proposal#2: Based on the above analysis, the TBS value as listed in Table 1,2,3 in TR 26.940 are valid and it is suggested to delete the brackets.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.940 [4].

* * * First Change * * *

### 5.2.2	Simulation Model to derive error traces

The NTN link consists of a service link (between the UE and the satellite) and a feeder link (between the satellite and the ground station). The bottleneck is the  link due to the limited TX power and small antenna at the UE. The feeder link is typically characterized by large capacity and high reliability and can be abstracted as an ideal link in the end-to-end simulation. The RAN simulation addresses the service link only.

The objective is to generate multiple loss traces for a combination of frame loss rate (target BLER), raw bitrate (TBS), voice bundling period and Doppler spread, while maintaining channel consistency among different combinations.

The multiple loss traces are the result of using multiple random seeds, and the number is 10. For each combination, all 10 seeds are used in generating the error traces.

Each trace represents a duration of 400 seconds (or 6.67 minutes). Therefore, for 80ms bundling, there are 5000 TBs, and for 160ms bundling there are 2500 TBs.

The following parameters are for the uplink of the service link.

Channel model: NTN-TDL-C [38811]

Modulation: QPSK

SCS: 3.75kHz, 15kHz

Number of tones: 1

Voice bundling period: 80ms, 160ms, 320ms

NOTE: the 40ms bundling is not considered because for SCS 3.75kHz the minimum time-domain allocation is 32ms and it leaves insufficient time for downlink data (NPDSCH) and control (NPDCCH) transmissions in the same 40ms time interval.

Doppler spread: 1Hz, 5 Hz

Target BLER: 1%, 2%, 6%, 10%

Target SNR:  5 dB, 8 dB, 10 dB, [12 dB] .

Editor’s Note: Achieving 12 dB requires a transmit power of 33 dBm, which is dependent on RAN4

TBS values and PHY bitrates: The TBS values are selected from table 16.5.1.2-2 for NB-IoT for NPUSCH in TS36.213 and the corresponding PHY bitrates and codec bitrate (assuming 8 bytes of packet header with RoHC) are calculated for each bundling period.

Table 1 TBS and PHY bitrate for 80ms bundling

| TBS (bits) | 144 | 256 | 328 | 424 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.8 | 3.2 | 4.1 | 5.3 |
| Codec bitrate (kbps) | 1.0 | 2.4 | 3.3 | 4.5 |

Table 2 TBS and PHY bitrate for 160ms bundling

| TBS (bits) | 208 | 424 | 600 | 808 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.30 | 2.65 | 3.85 | 5.05 |
| Codec bitrate (kbps) | 0.9 | 2.25 | 3.45 | 4.65 |

Table 3 TBS and PHY bitrate for 320ms bundling

| TBS (bits) | 328 | 776 | 1000 | 1544 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.025 | 2.425 | 3.125 | 4.825 |
| Codec bitrate (kbps) | 0.8025 | 2.225 | 2.925 | 4.625 |

NOTE: A PHY bitrate is considered feasible if the required SNR to meet a target BLER is lower than the target SNR.

Channel consistency: The same set of channel realizations are used across all combinations.

* * * End of Changes * * *

[1] TR 38.821, " Solutions for NR to support non-terrestrial networks (NTN)".

[2] TR 38.811, " Study on New Radio (NR) to support non-terrestrial networks ".

[3] TS 36.213, " Physical layer procedures"