Source:	Samsung Electronics Co., Ltd.

Title:	[FS_ULBC] GEO satellite channel parameters for FS_ULBC

## Document for:	Discussion and agreement

## Agenda Item:	1.9

# 1		Introduction

At the SA4#132 meeting, the initial analysis on the channel characteristics evaluation for ULBC was discussed [1]. It provides the helpful preliminary operation ranges, such as codec bitrate, Ptime, and BLER target to be considered as the good starting points for simulation parameters within SA4. While those values would be confirmed by RAN working groups in the future, it is beneficial to figure out the practical ranges first in SA4 before sending LS to other groups.

RAN1 had carried out the link budget analysis on NB-IoT for Non-Terrestrial Networks (NTN), including GEO satellite, and published the results in TR 36.763 [2]. Those link budget results were carefully verified by 15 contributing companies, thus provisioning the practical ranges for C/N values for various satellite scenarios. This paper is introducing the results specified in TR 36.763 [2] and providing the preliminary estimates on bitrate with comparison to S4-250818 [1].

2		Discussions

2.1		Link budget results for C/N range from RAN1 work

TR 36.763 [2] and TR 38.821 [3] specify the satellite parameters for various scenarios, as named as Set-1, Set-2, Set-3, Set-4, and Set-5. Among the all Sets, only Set-1/2/3 are relevant to GEO satellite case, as addressed in Table 1.

Table 1: GEO satellite parameters for simulation calibration (based on clause 6.1.1.1 of TR 38.821, clause 6.2.1 of TR 36.763)

| GEO Satellite orbit |  | Set-1 | Set-2 | Set-3 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 35786 km | 35786 km |
| Satellite antenna pattern |  | Clause 6.4.1 in TR 38.811 | Clause 6.4.1 in TR 38.811 | Clause 6.4.1 in TR 38.811 |
| Central beam edge elevation |  | 2.3 degrees | 11.0 degrees | 12.5 degree |
| Central beam centre elevation |  | 12.5 degrees | 20 degrees | 20.9 degree |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture | S-band (i.e. 2 GHz) | 22 m | 12 m | 12 m |
| Satellite EIRP density |  | 59 dBW/MHz | 53.5 dBW/MHz | 59.8 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 45.5 dBi | 45.7 dBi |
| 3dB beamwidth |  | 0.4011 deg | 0.7353 degrees | 0.7353 degree |
| Satellite beam diameter |  | 250 km | 450 km | 459km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture | S-band (i.e. 2 GHz) | 22 m | 12 m | 12 m |
| G/T |  | 19 dB/K | 14 dB/K | 16.7 dB/K |
| Satellite Rx max Gain |  | 51 dBi | 45.5 dBi | 45.7 dBi |

Based on the parameters above, RAN1 had conducted the calibration works of link budget results from 15 contributing companies (Huawei, Xiaomi, OPPO, Ericsson, Vivo, Qualcomm, CATT, Apple, MediaTek, Samsung, Nokia, SONY, CMCC, Sateliot, ZTE) and agreed the link budget results as shown in Table 2.

Table 2: Link budget results for GEO satellite case (excerpted from clause 6.2.2 of TR 36.763)

| PC3 (23 dBm), NF=7 dB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Cases | EIRP Density | EIRP per spot | DL C/N | G/T | UL C/N 180 kHz / 90 kHz / 45 kHz / 30 kHz / 15 kHz / 3.75 kHz |
| Set-1 | 59 dBW/MHz | 81.6 dBm | -3.3 dB | 19 dB/K | -14.2 dB / -11.2 dB / -8.2 dB / -6.5 dB / -3.5 dB / 2.6 dB |
| Set-2 | 53.5 dBW/MHz | 76.1 dBm | -8.5 dB | 14 dB/K | -19.0 dB / -16.0 dB / -13.0 dB / -11.2 dB / -8.2 dB / -2.2 dB |
| Set-3 | 59.8 dBW/MHz | 84.4 dBm | -2.2 dB | 16.7 dB/K | -16.3 dB / -13.3 dB / -10.3 dB / -8.5 dB / -5.5 dB / 0.6 dB |

| PC5 (20 dBm), NF=9 dB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Cases | EIRP Density | EIRP per spot | DL C/N | G/T | UL C/N 180 kHz / 90 kHz / 45 kHz / 30 kHz / 15 kHz / 3.75 kHz |
| Set-1 | 59 dBW/MHz | 81.6 dBm | -5.3 dB | 19 dB/K | -17.2 dB / -14.2 dB / -11.2 dB / -9.5 dB / -6.5 dB / -0.4 dB |
| Set-2 | 53.5 dBW/MHz | 76.1 dBm | -10.5 dB | 14 dB/K | -22.0 dB / -19.0 dB / -16.0 dB / -14.2 dB / -11.2 dB / -5.2 dB |
| Set-3 | 59.8 dBW/MHz | 84.4 dBm | -4.2 dB | 16.7 dB/K | -19.3 dB / -16.3 dB / -13.3 dB / -11.5 dB / -8.5 dB / -2.4 dB |

The table shows the C/N values for device type with PC3 or PC5 and the noise figure of 7 dB or 9 dB. For channel bandwidth, NB-IoT 180kHz for DL, and up to 180kHz with all permissible smaller resource allocations for UL are assumed.

In summary, the following ranges of C/N are agreed during RAN1 discussion

- For DL, C/N range : -10.5 dB (min) ~ -2.2 dB (max)

- For UL, C/N range : -22.0 dB (min) ~ +2.6 dB (max)

From the results, it is observed that C/N values for most cases are less than 0dB, which represents the very severe conditions. Although not every case should be considered in our study, but it is obvious that we should primarily focus on < 0dB cases in ULBC design in order to correspond to the severe GEO satellite conditions. The detailed C/N range could be further elaborated depending on the choice of simulation parameters (e.g., noise figure, device type, channel bandwidth allocation, etc)

Observation 1) we should primarily focus on < 0dB cases in ULBC design in order to correspond to the severe GEO satellite conditions

2.2		Calculation of practical bitrate

There are many other parameters that should be considered to calculate the practical ULBC bitrate from C/N ranges captured above, such as target BLER, duplexing, DL/UL ratio, etc. Those parameters should be carefully chosen upon the agreements, but it is worthwhile to address the recent input S4-250818 [1] to SA4#132 meeting. In the analysis of [1], the Table 3 was provided;

Table 3. Relationship between radio parameter settings and ULBC bitrates (from S4-250818)

The source specified the ULBC bitrate calculated from C/N value. Among various combinations for DL/UL C/N, we highlight that the first combination is within the C/N range in TR 36.763, which eventually result in ULBC bitrate of 300 ~ 500 bps. Of course, this bitrate is based on the assumption of full-duplex which should be revisited upon agreement, however it provides the snapshot showing how much bitrate should be considered in GEO scenario. For more accurate estimation, we should figure out more practical assumptions for GEO case.

Observation 2) To correspond to severe C/N range for GEO case, ULBC bitrate of several hundreds bps (less than 1kbps) would be our primary consideration.

3		Proposal

We propose the followings;

- Put the context of clause 2.1 in this document into the permanent documents (as related works in other 3GPP working groups)

- Work on the simulation parameters for practical GEO satellite scenario in order to provide more accurate target bitrate

References

[1] Huawei, Tdoc S4-250818, "[FS_ULBC] On channel characteristics evaluation for ULBC".

[2] 3GPP TR 36.763, “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)”.

[3] 3GPP TR 38.821, “Solutions for NR to support Non-Terrestrial Networks (NTN)”.