Source:	Beijing Xiaomi Mobile Software

Title:	Simulation results for FS_ULBC

## Document for:	Discussion & Agreement.

## Agenda Item:	1.9

# Introduction

During the June 4, 2025 Audio SWG post 132 meeting, there were a lot of discussions on “Link budget analysis” and T“TBS” for the simulation model, many remaining parameters are still for further discussion, however, in order to save time, interest company can start to simulate the common numbers of the proposals listed in black text in S4aA250038[1], and the simulations will follow the NTN-TDL-C5 channel model as specified in TS 36.102[2]. The source has done some simulations for several cases with assumed simulation parameters based on the discussions. The details are described in the following clauses.

# Discussion

## Link budget analysis

The CNR is calculated as follows, refer to TR38.821[3]

CNR dB=EIRP [dBW]+GT[dB/K]-k [dBW/K/Hz]-PLFS [dB]-PLA [dB]-PLSM [dB]-PLSL[dB]-PLAD [dB]-B [dBHZ]

Table 2.1-1 Basic Link budget parameters

|  | Parameter | Value |
| --- | --- | --- |
|  | EIRP density | 59 dBW/MHz |
| Satellite parameters | G/T | 19 dB K-1 |
|  | Antenna polarization | Circular polarization |
|  | Satellite altitude | 35786km |
| UE parameters | Antenna polarization | Linear polarization |
|  | Antenna configuration | 1Tx |
|  | Antenna gain | 0dBi |
|  | Elevation angle | 12.5 degree |
|  | UE power | 23dBm, 26dBm, 29dBm, 31dBm |
|  | Noise Figure | 7dB |
| System parameters | Carrier frequency | 2GHz |
|  | SCS(BW) | 3.75kHz, 15kHz |
|  | PLFS | 190.8 dB |
|  | PLA | 0.2 dB |
|  | PLSM | 3 dB |
|  | PLSL | 2.2dB |
|  | PLAD | 0 dB |

Table 2.1-2 CNR results under different parameter assumptions

|  | SCS/BW | UE power | UL CNR |
| --- | --- | --- | --- |
| Config. 1 | 3.75kHz/3.75kHz | 23dBm | 8.66 dB |
| Config. 2 | 15kHz/15kHz | 23dBm | 2.64 dB |
| Config. 3 | 3.75kHz/3.75kHz | 26dBm | 11.66 dB |
| Config. 4 | 15kHz/15kHz | 26dBm | 5.64 dB |
| Config. 5 | 3.75kHz/3.75kHz | 29dBm | 14.66dB |
| Config. 6 | 15kHz/15kHz | 29dBm | 8.64dB |
| Config. 7 | 3.75kHz/3.75kHz | 31dBm | 16.66dB |
| Config. 8 | 15kHz/15kHz | 31dBm | 10.64dB |

## Simulation results

The common simulation parameters are in Table 2.2-1

Table 2.2-1 Common simulation parameters

|  | Parameter | value |
| --- | --- | --- |
| scenario | orbit | GEO |
|  | Elevation angle | 12.5 degree |
| Channel and impairments | carrier frequency | 2GHz |
|  | Channel model | NTN-TDL-C5 The channels from different UE are independent. |
|  | Delay spread | 5ns |
|  | TO/FO | Not assumed |
| transmitter | Physical channel | NPUSCH format 1 |
|  | SCS | 3.75KHz and 15KHz |
|  | Number of tones | Single tone |
|  | Waveform | SC-FDMA |
|  | Antenna number | 1 |
|  | MIMO scheme | SISO |
|  | DMRS configuration | OS#4 per slot for 3.75kHz OS#3 per slot for 15kHz |
|  | Velocity of UE | 3km/h |
|  | #symbols per slot | 7 |
|  | #Slots per RU | 16 |
|  | Modulation order | Pi/4-QPSK |
| receiver | Antenna number | 1 |
|  | Receiver algorithm | MMSE |
|  | Channel estimation | Real channel estimation |
| KPI | SNR at target BLER | Target BLER: 1%, 2%, 6%, 10% |

In this simulation, the assumption of the determination of repetition number is to ensure the total transmission delay between uplink and downlink meeting the bundling time constraint, while allocating specific time-domain resources for downlink transmission, under this assumption to get the maximum repetition number for uplink.

- 80ms bundling time

  - 144 bits

Table 2.2-2 Combinations of TBS 144bit w/ 80ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 144 | 15 | 2 | 4 | 2 | 1.8 | 1.0 |
| Case 2 |  | 3.75 | 10 | 1 | 2 |  |  |
| Case 3 |  | 15 | 10 | 1 | 4 |  |  |

Table 2.2-3 SNR at target BLER with TBS 144bit w/ 80ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case1) | -4.97 | -4.83 | -4.54 | -4.35 |
| SNR(case2) | 1.5 | 1.71 | 2.11 | 2.30 |
| SNR(case3) | -4.64 | -4.46 | -4.07 | -3.90 |

  - 256 bits

Table 2.2-4 Combinations of TBS 256bit w/ 80ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 4 | 256 | 15 | 3 | 5 | 1 | 3.2 | 2.4 |
| Case 5 |  | 3.75 | 8 | 2 | 1 |  |  |
| Case 6 |  | 15 | 8 | 2 | 4 |  |  |

Table 2.2-5 SNR at target BLER with TBS 256bit w/ 80ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case4) | -0.89 | -0.77 | -0.5 | -0.33 |
| SNR(case5) | 5.02 | 5.18 | 5.51 | 5.73 |
| SNR(case6) | -2.84 | -2.71 | -2.45 | -2.28 |

  - 328 bits

Table 2.2-6 Combinations of TBS 328bit w/ 80ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 7 | 328 | 15 | 4 | 5 | 1 | 4.1 | 3.3 |
| Case 8 |  | 3.75 | 10 | 2 | 1 |  |  |
| Case 9 |  | 15 | 10 | 2 | 4 |  |  |

Table  2.2-7 SNR at target BLER with TBS 328bit w/ 80ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case7) | 0.43 | 0.66 | 0.90 | 0.95 |
| SNR(case8) | 8.26 | 8.53 | 9.00 | 9.45 |
| SNR(case9) | -1.53 | -1.34 | -1.09 | -1.01 |

  - 424 bits

Table  2.2-8 Combinations of TBS 424bit w/ 80ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 10 | 424 | 15 | 5 | 5 | 1 | 5.3 | 4.5 |

Table 2.2-9 SNR at target BLER with TBS 424bit w/ 80ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case10) | 1.44 | 1.58 | 1.88 | 2.05 |

- 160ms bundling time

  - 208 bits

Table  2.2-10 Combinations of TBS 208bit w/ 160ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 11 | 208 | 15 | 2 | 6 | 2 | 1.3 | 0.9 |
| Case 12 |  | 3.75 | 4 | 3 | 1 |  |  |
| Case 13 |  | 15 | 4 | 3 | 4 |  |  |

Table  2.2-11 SNR at target BLER with TBS 208bit w/ 160ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case11) | -5.56 | -5.4 | -5.04 | -4.89 |
| SNR(case12) | 0.91 | 1.07 | 1.36 | 1.53 |
| SNR(case13) | -5.41 | -5.21 | -4.9 | -4.76 |

  - 424 bits

Table 2.2-12 Combinations of TBS 424bit w/ 160ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps)） | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 14 | 424 | 15 | 5 | 5 | 2 | 2.65 | 2.25 |

Table  2.2-13 SNR at target BLER with TBS 424bit w/ 160ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case14) | -1.95 | -1.85 | -1.65 | -1.52 |

  - 600 bits

Table  2.2-14 Combinations of TBS 600bit w/ 160ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 15 | 600 | 15 | 6 | 6 | 2 | 3.75 | 3.35 |

Table  2.2-15 SNR at target BLER with TBS 600bit w/ 160ms bundling time

| BLER |  | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- | --- |
| SNR(case15) | -1.38 | -1.28 | -1.08 | -0.97 |

  - 808 bits

Table  2.2-16 Combinations of TBS 808bit w/ 160ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 16 | 808 | 15 | 6 | 8 | 2 | 5.05 | 4.65 |
| Case 17 |  | 15 | 8 | 6 | 2 |  |  |

Table  2.2-17  SNR at target BLER with TBS 808bit w/ 160ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case16) | -1.42 | -1.35 | -1.18 | -1.07 |
| SNR(case17) | -0.15 | -0.03 | 0.12 | 0.21 |

- 320ms bundling time

  - 328 bits

Table  2.2-18 Combinations of TBS 328bit w/ 320ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 18 | 328 | 3.75 | 3 | 6 | 1 | 1.025 | 0.8250 |
| Case 19 |  | 15 | 3 | 6 | 4 |  |  |
| Case 20 |  | 3.75 | 4 | 5 | 1 |  |  |
| Case 21 |  | 15 | 4 | 5 | 4 |  |  |
| Case 22 |  | 3.75 | 10 | 2 | 4 |  |  |
| Case 23 |  | 15 | 10 | 2 | 8 |  |  |

Table  2.2-19 SNR at target BLER with TBS 328bit w/ 320ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case18) | -0.74 | -0.63 | -0.38 | -0.22 |
| SNR(case19) | -6.80 | -6.69 | -6.47 | -6.32 |
| SNR(case20) | 0.21 | 0.32 | 0.56 | 0.72 |
| SNR(case21) | -5.93 | -5.83 | -5.62 | -5.49 |
| SNR(case22) | -1.47 | -1.31 | -0.98 | -0.84 |
| SNR(case23) | -4.61 | -4.47 | -4.17 | -3.99 |

  - 776 bits

Table  2.2-20 Combinations of TBS 776bit w/ 320ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 24 | 776 | 3.75 | 9 | 5 | 1 | 2.425 | 2.225 |
| Case 25 |  | 15 | 9 | 5 | 4 |  |  |

Table 2.2-21 SNR at target BLER with TBS 776bit w/ 320ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case24) | 6.02 | 6.12 | 6.33 | 6.46 |
| SNR(case25) | -2.48 | -2.35 | -2.08 | -1.94 |

  - 1000 bits

Table  2.2-22 Combinations of TBS 1000bit w/ 320ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 26 | 1000 | 15 | 6 | 10 | 2 | 3.125 | 2.925 |
| Case 27 |  | 3.75 | 10 | 6 | 1 |  |  |
| Case 28 |  | 15 | 10 | 6 | 4 |  |  |

Table  2.2-23 SNR at target BLER with TBS 1000bit w/ 320ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case26) | -1.5 | -1.44 | -1.31 | -1.23 |
| SNR(case27) | 6.97 | 7.09 | 7.33 | 7.47 |
| SNR(case28) | -1.95 | -1.89 | -1.75 | -1.66 |

  - 1544 bits

Table  2.2-24 Combinations of TBS 1544bit w/ 320ms bundling time

| scheme | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) | Codec bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 29 | 1544 | 15 | 9 | 10 | 2 | 4.825 | 4.625 |

Table  2.2-25 SNR at target BLER with TBS 1544bit w/ 320ms bundling time

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR(case29) | 0.48 | 0.58 | 0.69 | 0.76 |

# Conclusion

It is recommended to include the simulation results into TR 26.940 for determining the design constraints for ULBC.

# References

[1] S4aA250038, Qualcomm, “Calibration of contributions on RAN simulation assumptions for ULBC”.

[2] TS 36.102,” Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access”.

[3] TS 38.821," Solutions for NR to support non-terrestrial networks (NTN)".

# ANNEX A : Figures of relationship between BLER and SNR

A.1

A.2

A.3

A.4

A.5

A.6

A.7

A.8

A.9

A.10

A.11

A.12