Source:	IVAS-11 Editor (Dolby Laboratories Inc)

Title:	IVAS Non-BE Conformance (IVAS-11)

Version:	0.2.0

Agenda item:	7.5 - IVAS_Codec_Ph2

Document for:	Agreement

Revision history:

| Date | Meeting | Subject/Comment | Old | New |
| --- | --- | --- | --- | --- |
| 2025-05-22 | SA4#132 | Initial version including S4-250854 and time plan from S4-251074. | N/A | 0.0.1 |
| 2025-05-22 | SA4#132 | Initial version including S4-250854 and time plan from S4-251074. | 0.0.1 | 0.1.0 |
| 2025-06-16 | Audio Telco Post SA4#132 | Decoder only tests | 0.1.0 | 0.2.0 |

- Key Acronyms

HOA	           Higher Order Ambisonics

IVAS	           Immersive Voice and Audio Services

ISM	           Independent Streams with Metadata

MASA	           Metadata-Assisted Spatial Audio

MC	           Multi Channel

FFT                Fast Fourier Transform

EVS                Enhanced Voice Services

WMOPS            Weighted Million Operations Per Second

MCPS              Million Cycles Per Second

WID                Work Item description

BE                  Bit-exact

Non-BE             Non Bit-exact

# Introduction

During SA #107, IVAS phase 2 WID objectives were updated [1] to develop the conformance process (including tools and test vectors) of IVAS for non-BE implementations as highlighted below.

- Enhancements to codec conformance test procedures and criteria. Under this objective, a conformance process (including tools and test vectors) of IVAS for non-bit-exact floating-point implementations shall be developed, aligned with the conclusions of TR 26.843. More specifically, this work includes the following steps:

  - Specification of tools and conformance test vectors suitable for performing IVAS conformance testing.

  - Definition of relevant testing processes and criteria, based on the latest IVAS floating-point reference code.

    - The conformance testing processes and criteria shall be tight enough to ensure equivalent quality and interoperability with

      - implementation of the floating-point reference code that meets the bit-exact conformance requirements for this code specified in TS 26.252 and

      - implementation of the fixed-point reference code that meets the bit-exact conformance requirements for this code specified in TS 26.252.

    - At the same time, the coverage of the conformance testing processes and criteria shall be sufficient to avoid interoperability issues between implementations found conformant based on the non-bit-exact criteria and conformant implementations based on the bit-exact criteria.

  - Investigation on the applicability of the testing processes and criteria, which includes:

    - Robustness testing with validation that inadequate optimizations are properly detected while adequate optimizations still pass the criteria.

  - Definition of a mandatory non-bit-exact IVAS conformance testing process to be included in TS 26.252 using the tools, conformance criteria and conformance test vectors developed under this work item objective.

To come up with conformance criteria, similar process can be followed as EVS non-BE conformance [3]. However, given that IVAS supports immersive input/output formats, certain changes may be required to the conformance methods and criteria to ensure equivalent quality as compared to the reference outputs. In this permanent document, the objective analysis data is gathered by running IVAS source code on various platforms and compilers using multiple compiler optimizations.

# Objective Methods

## Description

IVAS codec internally uses core coding switches similar to EVS and may encode an audio frame using LPC based coding or MDCT transform based coding depending on the outcome of various classifiers in the source code. Moreover, stereo format employs switching mechanism to switch between various stereo downmix techniques at low bitrates. Due to these dynamic switching mechanisms, the output of IVAS encoder+decoder may vary significantly even if there is a minor precision change at the encoder. For this reason, in the conformance criteria, it may be desired to use objective tools that rely on perceptual quality analysis. The impact of floating point precision change across different platforms and compilers may be minimal in case of IVAS external renderer and certain modes of IVAS decoder and hence signal based objective methods may be desirable to define conformance criteria for such cases.

Various signal based and perceptual based methods that can potentially be used for IVAS non-BE conformance are described below. All the methods/tools described below are used for mono audio comparison. For output formats with more than one channel (i.e. all output configurations except MONO), the tools perform a pairwise comparison of each channel from the test and reference audio samples.

## Signal based methods

### Overview

### Segmental Signal-to-Noise Ratio (SSNR)

The Segmental Signal-to-Noise Ratio is evaluated as defined in as defined in ISO/IEC 14496-4. The segment length is 20 ms and the minimum SSNR of all the segments across all channels is reported. This objective measure has been extensively used in IVAS fixed point development process and can be considered for IVAS non-BE conformance as well. The results with SSNR method with IVAS floating point code are present in clause 3.3.2.

### RMS error threshold

This tool is described in [3] and can be considered for IVAS non-BE conformance as well. This tool is yet to be tested with IVAS tests set described in clause 3.

## Perceptually based methods

### Overview

### Maximum Loudness Difference (MLD)

MLD is described in [3]. For MLD, the maximum number among all the channels is reported. This objective measure has been extensively used in IVAS fixed point development process and can be considered for IVAS non-BE conformance as well. The results with MLD method with IVAS floating point code are present in clause 3.3.1.

### MOS-LQO Validation

This tool is described in [3] and can be considered for IVAS non-BE conformance as well. This tool is yet to be tested with IVAS tests set described in clause 3.

### Perceptual Evaluation of Audio Quality (PEAQ)

PEAQ is fully complaint to ITU-R BS.1387 and can be considered for IVAS non-BE conformance as well. This tool is yet to be tested with IVAS tests set described in clause 3.

IVAS floating point Non-BE conformance

## IVAS decoder tests

### Summary of experiments

Experiments were done by generating reference bitstream and decoded outputs on a reference system. The reference bitstreams were then decoded on various test systems and the decoded outputs were compared with reference decoded outputs. Following experiments were done:

- Experiment 1: Using windows MSVS reference as mentioned in Table 3.1.1

  - Test outputs generated on windows and Linux platform with various compiler settings as shown in Table 3.1.2.

- Experiment 2: Using Linux GCC reference as mentioned in Table 3.1.1

  - Test outputs generated on windows and Linux platform with various compiler settings as shown in Table 3.1.3

- Experiment 3: Using Linux GCC reference as mentioned in Table 3.1.1

  - Test outputs generated on MacOS with various compiler settings as shown in Table 3.1.4.

### Test vector set and test running method

#### Test vector set

- IVAS encoder and decoder tests: Short test vector (STV) set from [6].

- IVAS external renderer tests: Test set from [6].

- IVAS Split renderer tests: Test set from [6].

- The test set used for all three test types mentioned above has the following characteristics:

  - Input audio and accompanying metadata spanning all formats.

  - Speech and mixed speech/music content

  - Length: 1-30 seconds, average 10.5 seconds

#### Test running method and command lines

- Pytest framework is used to run the STV set.

- For IVAS encoder + decoder tests, the following steps were followed:

  - First bitstream and decoded output references (audio files) were generated using reference platform and compiler settings as mentioned in clause 3.1.3. The following pytest command was used to generate reference outputs.

python -m pytest tests/codec_be_on_mr_nonselection -v -n auto --update_ref 1 --create_ref --keep_files

  - The reference bitstreams and decoded outputs were stored on the test system using the same directory/file structure as on the reference system.

  - Then IVAS float source code is compiled on the test system with its specific compiler setting.

  - Then “decoder only” test outputs were generated using following pytest command

python -m pytest tests/codec_be_on_mr_nonselection -v -n auto --keep_files --create_cut  --decoder_only. This step takes reference bitstream and decodes it using test decoder compiled on the test system. Furthermore, this step uses tools like MLD and SSNR to compare the test outputs and the reference outputs.

### Reference platform and compiler settings

|  | Processor | Platform | Compiler | Compiler settings and optimization flags |
| --- | --- | --- | --- | --- |
| 1 | Intel(R) Xeon(R) W-1290 CPU | Windows 11, SDK: 10.0.17763.0 | Microsoft Visual Studio Toolset: Visual Studio 2017 (v141) ToolsVersion="15.0" | Debug, Win32, Od (disabled), no optimizations |
| 2 | Intel(R) Xeon(R) W-1290 CPU | Linux, Ubuntu 22.04.1, x86_64 | GCC 11.3 | O0, Makefile [8] Default setting with make -j |

Table 3.1.1 Reference platform and compiler settings

### Results

#### MLD and SSNR

##### Experiment 1

In this experiment, windows MSVS reference is used (platform/compiler settings as mentioned in Table 3.1.1). Test outputs are generated on windows and Linux platform with various compiler settings.

Total number of tests: 626

| Platform | Compiler | Compiler settings and optimization flags | Max MLD | % tests < 5 MLD | % tests < 2 MLD | Min SSNR | % tests > 50 SSNR | crashes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Windows 11, SDK: 10.0.17763.0 | Microsoft Visual Studio Toolset: Visual Studio 2017 (v141) ToolsVersion="15.0" | Release, Win32, O2 (Favor speed) | 1.26 | 100 | 100 | 31.88 | 99.84 | 0 |
| Windows 11, SDK: 10.0.17763.0 | Microsoft Visual Studio Toolset: Visual Studio 2017 (v141) ToolsVersion="15.0" | Release, Win32, O1 (Favor size) | 3.54 (Note 1) | 100 | 99.84 | 54.98 | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | GCC 11.3 | O0, Makefile [8] Default setting with make -j | 0.8 | 100 | 100 | 54.22 | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | GCC 11.3 | Makefile [8] Modifications: O3 optimization | 0.8 | 100 | 100 | 54.22 | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: Clang and O2 optimization | 0.8 | 100 | 100 | 54.22 | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: Clang and O3 optimization | 0.8 | 100 | 100 | 54.22 | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: GCC and Ofast optimization | 44.85 (Note 2) | 99.5 | 99.33 | 29.17 | 82.45 | 22 (Note 2) |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: GCC and O2 and -ffast_math optimization | 45.07 (Note 3) | 99.67 | 99.50 | 40.97 | 86.25 | 22 (Note 3) |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: GCC and O2 and -funsafe-math-optimizations optimization | 45.07 (Note 3) | 99.67 | 99.50 | 40.93 | 86.58 | 22 (Note 3) |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: GCC and O2 and -frounding-math -fsignaling-nans optimization | 0.8 | 100 | 100 | 54.22 | 100 | 0 |

Table 3.1.2 Experiment 1 results (MLD and SSNR scores with IVAS decoder only tests)

- Note 1: 1 ISM bitrate switching test with MLD 3.84, rest all tests with MLD < 2

Note 2: Out of 626 tests, 604 tests ran w/o crash, 22 tests crashed (including both BR switching and non-BR switching tests). Out of 604, 2 SBA tests with MLD ~45, 1 OMASA JBM test with MLD ~11, 1 ISM BR switch test with MLD ~3, rest all tests with MLD < 2. The issue in tests with MLD ~45 is noticed in only one frame.  There seems to be an indication of corruption leading to garbage in the output during one of the frames. Rest of the output file seems very close to reference. Minimum SSNR result for this test is 55.51 dB which is clearly too good considering the obvious issue and it raises concerns to the usability of SSNR as a metric for conformance.

Note 3: Similar to issues described in Note 2. OMASA JBM case is not seen here. So out of 604, 2 SBA tests with MLD ~45, 1 ISM BR switch test with MLD ~3

##### Experiment 2

In this experiment, Linux GCC reference is used (platform/compiler settings as mentioned in Table 3.1.1). Test outputs generated on windows and Linux platform with various compiler settings.

Total number of tests: 626

| Platform | Compiler | Compiler settings and optimization flags | Max MLD | % tests < 5 MLD | % tests < 2 MLD | Min SSNR | % tests > 50 SSNR | crashes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Windows 11, SDK: 10.0.17763.0 | Microsoft Visual Studio Toolset: Visual Studio 2017 (v141) ToolsVersion="15.0" | Debug, Win32, Od (disabled), no optimizations | 0.8 | 100 | 100 | 54.22 | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | GCC 11.3 | Makefile [8] Modifications: O3 optimization | 0 | 100 | 100 | inf | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: Clang and O2 optimization | 0 | 100 | 100 | inf | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: Clang and O3 optimization | 0 | 100 | 100 | inf | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: GCC and Ofast optimization | 44.85 (Note 1) | 99.5 | 99.33 | 29.15 | 82.6 | 22 (Note 1) |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: GCC and O2 and -ffast_math optimization | 45.07 (Note 2) | 99.67 | 99.50 | 40.97 | 86.42 | 22 (Note 2) |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: GCC and O2 and -funsafe-math-optimizations optimization | 45.07 (Note 2) | 99.67 | 99.50 | 40.93 | 86.58 | 22 (Note 2) |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: GCC and O2 and -frounding-math -fsignaling-nans optimization | 0.009 | 100 | 100 | 60.47 | 100 | 0 |

Table 3.1.3 Experiment 2 results (MLD and SSNR scores with IVAS decoder only tests)

Note 1: Refer to Note 2 of Table 3.1.2

Note 2: Refer to Note 3 of Table 3.1.2

##### Experiment 3

In this experiment, Linux GCC reference is used (platform/compiler settings as mentioned in Table 3.1.1). Test outputs were generated on Mac OSX with various compiler settings.

Total number of tests: 627

| Platform | Compiler | Compiler settings and optimization flags | Max MLD | % tests < 5 MLD | % tests < 2 MLD | Min SSNR | % tests > 50 SSNR | crashes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MacOS Sequoia V15.5, Arm64-apple-darwin24.5.0 | Apple Clang version 17.0.0 | O0, Makefile [8] Default setting with make -j | 1.8 | 100 | 100 | 27.44 | 99.36 | 0 |
| MacOS Sequoia V15.5, Arm64-apple-darwin24.5.0 | Apple Clang version 17.0.0 | Makefile [8] Modifications: O3 optimization | 1.8 | 100 | 100 | 27.44 | 99.36 | 0 |
| MacOS Sequoia V15.5, Arm64-apple-darwin24.5.0 | Apple Clang version 17.0.0 | Makefile [8] Modifications: Clang and O2 optimization | 1.8 | 100 | 100 | 27.44 | 99.36 | 0 |
| MacOS Sequoia V15.5, Arm64-apple-darwin24.5.0 | Apple Clang version 17.0.0 | Makefile [8] Modifications: Clang and O3 optimization | 1.8 | 100 | 100 | 27.44 | 99.36 | 0 |
| MacOS Sequoia V15.5, Arm64-apple-darwin24.5.0 | Apple Clang version 17.0.0 | Makefile [8] Modifications: GCC and Ofast optimization | Deprecated |  |  |  |  |  |
| MacOS Sequoia V15.5, Arm64-apple-darwin24.5.0 | Apple Clang version 17.0.0 | Makefile [8] Modifications: GCC and O2 and -ffast_math optimization | 82.28 (Note 1) | 98.4 | 98.24 | -10.75 | 86.9 | 0 |
| MacOS Sequoia V15.5, Arm64-apple-darwin24.5.0 | Apple Clang version 17.0.0 | Makefile [8] Modifications: GCC and O2 and -funsafe-math-optimizations optimization | 85.42 (Note 2) | 97.44 | 97.28 | 13.12 | 86.9 | 0 |
| MacOS Sequoia V15.5, Arm64-apple-darwin24.5.0 | Apple Clang version 17.0.0 | Makefile [8] Modifications: GCC and O2 and -frounding-math -fsignaling-nans optimization | 1.8 | 100 | 100 | 27.44 | 99.36 | 0 |

Table 3.1.4 Experiment 3 results (MLD and SSNR scores with IVAS decoder only tests)

Note 1: Out of 627 tests, 0 crashes, 7 tests with MLD > 50 (ISM normal, BR switch and JBM), 1 ISM test with MLD 15.5, 1 ISM JBM with MLD 6.2, 1 ISM JBM with MLD 5.2

Note 2: Out of 627 tests, 0 crashes, 6 tests (5 ISM and 1 MC) with MLD > 50, 2 tests (MC) with 20 < MLD < 30, 3 tests (MC and ISM) with 10 < MLD < 20, 4 tests (MC and ISM) with 5 < MLD < 10.

### Observations and conclusions

- Without unsafe float optimizations (Ofast, funsafe-math-optimizations, ffast-math), the MLD and SSNR scores show that the outputs are very similar to reference on various platforms.

- With unsafe optimizations, very high MLD scores were observed. The MLD scores are in sync with the quality issues observed when comparing DUT and Reference outputs. On various platforms, these optimizations are leading to crashes and corruption. So there are interoperability issues when bitstream is generated without unsafe optimizations and decoded with unsafe optimizations.

- Ofast, –ffast-math and -funsafe-math-optimizations may not be ISO or IEEE complaint [10] and may be considered unsafe on many platforms. While the sources recommend debugging of the interoperability issues highlighted in Table 3.1.2, 3.1.3 and 3.1.4, these flags may not be suitable for IVAS implementation and [shall] not be considered while defining non-BE conformance threshold and criteria.

- Furthermore, it is seen that using SSNR as a metric may give misleading results (as shown in Note 2 of Table 3.1.2). Therefore, SSNR shall not be used as a standalone metric for non-BE conformance. It may be combined with MLD to define conformance thresholds.

- Without unsafe optimizations, MLD threshold of 2 may be considered as a threshold for Non-BE floating point conformance. An exception can be made for JBM, BR switching and PLC tests where the MLD threshold may be relaxed to 5.

## IVAS encoder tests

### Summary of experiments

### Test vector set and test running method

#### Test vector set

#### Test running method and command lines

### Reference platform and compiler settings

### Results

### Observations and conclusions

## IVAS encoder + decoder tests

### Summary of experiments

Experiments were done by generating reference bitstreams and decoded outputs on a reference system. The test outputs were generated by running IVAS encoder + decoder on various test systems and the decoded outputs were compared with reference decoded outputs. Following experiments were done:

- Experiment 1: Using windows MSVS reference as mentioned in Table 3.3.1

  - Test outputs generated on windows and Linux platform with various compiler settings as shown in Table 3.3.2.

- Experiment 2: Using Linux GCC reference as mentioned in Table 3.3.1

  - Test outputs generated on windows and Linux platform with various compiler settings as shown in Table 3.3.2.

### Test vector set and test running method

#### Test vector set

The stv set described in clause 3.1.2.1 was used for these experiments.

#### Test running method and command lines

- Pytest framework is used to run the STV set.

- For IVAS encoder + decoder tests, the following steps were followed:

  - First bitstream and decoded output references (audio files) were generated using reference platform and compiler settings as mentioned in clause 3.3.3. The following pytest command was used to generate reference outputs.

python -m pytest tests/codec_be_on_mr_nonselection -v -n auto --update_ref 1 --create_ref --keep_files

  - The reference bitstreams and decoded outputs were stored on the test system using the same directory/file structure as on the reference system.

  - Then IVAS float source code is compiled on the test system with its specific compiler setting.

  - Then test outputs were generated using following pytest command

python -m pytest tests/codec_be_on_mr_nonselection -v -n auto --keep_files --create_cut. This step generates test bitstreams and decoded outputs using the IVAS encoder and decoder compiled on the test system. Furthermore, this step uses tools like MLD and SSNR to compare the test outputs and the reference outputs.

### Reference platform and compiler settings

|  | Processor | Platform | Compiler | Compiler settings and optimization flags |
| --- | --- | --- | --- | --- |
| 1 | Intel(R) Xeon(R) W-1290 CPU | Windows 11, SDK: 10.0.17763.0 | Microsoft Visual Studio Toolset: Visual Studio 2017 (v141) ToolsVersion="15.0" | Debug, Win32, Od (disabled), no optimizations |
| 2 | Intel(R) Xeon(R) Gold 6152 CPU | Linux, RHEL 8.4 | GCC 9.4.0 | O0, makefile [8] Default setting with make -j |

Table 3.3.1 Reference platform and compiler settings

### Results

#### MLD and SSNR

##### Experiment 1

Experiment 1 as per clause 3.3.1.

Total number of tests: 627

| Platform | Compiler | Compiler settings and optimization flags | Max MLD | % tests < 5 MLD | Min SSNR (dB) | % tests > 50 SSNR | Crashes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Windows 11, SDK: 10.0.17763.0 | Microsoft Visual Studio Toolset: Visual Studio 2017 (v141) ToolsVersion="15.0" | Release, Win32, O2 (Favor speed) | 1.5984988 | 100 | 58.83 | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | GCC 11.3 | O0, Makefile [8] Default setting with make -j | 5.37 | 99.84 | 46.81 | 99.84 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | GCC 11.3 | Makefile [8] Modifications: O3 optimization | 3.7 | 100 | 33.35 | 99.84 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: Clang and O2 optimization | 3.7 | 100 | 33.35 | 99.84 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: Clang and O3 optimization | 3.7 | 100 | 33.35 | 99.84 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: GCC and Ofast optimization | 16.534607 | 86.9 | 2.66 | 24 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: Clang and O3 and -ffast_math optimization | 20.8804 | 85.9 | 10.13 | 26 | 0 |

Table 3.3.2 Experiment 1 results ( IVAS encoder and decoder using MLD and SSNR tools)

##### Experiment 2

Experiment 2 as per clause 3.3.1.

Total number of tests: 627

| Platform | Compiler | Compiler settings and optimization flags | Max MLD | % tests < 5 MLD | Min SSNR (dB) | % tests > 50 SSNR | Crashes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RHEL 8.4 | llvm-v18.1.6d1rh84 -O0 | Makefile [8] Modifications: Clang and O0 | WIP | WIP | inf | 100 | 0 |
| RHEL 8.4 | gcc v9.4.0 -O3 | Makefile [8] Modifications: GCC and O3 | WIP | WIP | 44.7 | 99.82 | 0 |
| RHEL 8.4 | llvm-v18.1.6d1rh84 -O3 | Makefile [8] Modifications: Clang and O3 | WIP | WIP | 44.7 | 99.82 | 0 |
| Cadence xt-clang | xt-clang RJ-2024.3 tools | Makefile [8] Modifications: xt-clang and O0 | WIP | WIP | WIP | WIP | 0 |

Table 3.3.3 Experiment 2 results ( IVAS encoder and decoder using MLD and SSNR tools

### Observations and conclusions

With MLD and SSNR Objective methods, the following observations can be made:

- In the IVAS encoder+decoder tests, low MLD (MLD < 5 in all cases except 1 test) values are observed with O0, O2 and O3 compiler optimization settings on various platforms with both GCC and clang compiler. MLD >10 values are observed with compiler optimization flags -Ofast and -ffast-math.

- With O0, O2 and O3 compiler optimization settings on various platforms with both GCC and clang compiler, about 99.84 % tests report the worst case SSNR > 50 dB (only 1 test case out of 627 tests report SSNR less than 50 dB). With -Ofast and -ffast-math optimization SSNR performance can be considered poor.

- More experiments are needed to draw conclusions on non-BE conformance criteria

- Cases with -Ofast and -ffast-math should be debugged further wherein perceptual differences can be heard in informal listening in the test runs that correspond to high MLD values.

## IVAS external renderer tests

### Summary of experiments

Experiments were done by generating reference IVAS renderer outputs on a reference system. The test outputs were generated by running IVAS external renderer on various test systems and the rendered outputs were compared with the reference rendered outputs. Following experiments were done:

- Experiment 1: Using windows MSVS reference as mentioned in Table 3.4.1

  - Test outputs generated on windows and Linux platform with various compiler settings as shown in Table 3.4.2.

- Experiment 2: Using Linux GCC reference as mentioned in Table 3.4.1

  - Test outputs generated on windows and Linux platform with various compiler settings as shown in Table 3.4.3.

### Test vector set and test running method

#### Test vector set

#### Test running method and command lines

- For IVAS external renderer tests, the following steps were followed:

- First references were generated using reference platform and compiler settings as mentioned in clause 4. The following pytest command was used to generate reference outputs

python -m pytest tests/renderer/test_renderer.py  -v -n auto --update_ref 1 --create_ref --keep_files

- The reference outputs were stored on test Operating systems/platform using the same directory/file structure as on the reference system.

- Then IVAS floating source code is compiled on test platform and compilation setting.

- Then test outputs were generated using following pytest command

python -m pytest tests/renderer/test_renderer.py  -v -n auto --keep_files --create_cut

### Reference platform and compiler settings

|  | Processor | Platform | Compiler | Compiler settings and optimization flags |
| --- | --- | --- | --- | --- |
| 1 | Intel(R) Xeon(R) W-1290 CPU | Windows 11, SDK: 10.0.17763.0 | Microsoft Visual Studio Toolset: Visual Studio 2017 (v141) ToolsVersion="15.0" | Debug, Win32, Od (disabled), no optimizations |
| 2 | Intel(R) Xeon(R) Gold 6152 CPU | Linux, RHEL 8.4 | GCC 9.4.0 | O0, makefile [8] Default setting with make -j |

Table 3.4.1 Reference platform and compiler settings

### Results

#### MLD and SSNR

##### Experiment 1

Experiment 1 as per clause 3.4.1.

Total number of tests: 658

| Platform | Compiler | Compiler settings and optimization flags | Max MLD | % tests < 5 MLD | Min SSNR (dB) | % tests > 50 MLD | Crashes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Windows 11, SDK: 10.0.17763.0 | Microsoft Visual Studio Toolset: Visual Studio 2017 (v141) ToolsVersion="15.0" | Release, Win32, O2 (Favor speed) | 0.002 | 100 | 96.5 | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | GCC 11.3 | O0, Makefile [8] Default setting with make -j | 0.59 | 100 | 81.58 | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | GCC 11.3 | Makefile [8] Modifications: O3 optimization | 0.59 | 100 | 85.62 | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: Clang and O2 optimization | 0.59 | 100 | 85.62 | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: Clang and O3 optimization | 0.59 | 100 | 85.62 | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: GCC and Ofast optimization | 0.59 | 100 | 67.08 | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: Clang and O3 and -ffast_math optimization | 0.59 | 100 | 65.43 | 100 | 0 |

Table 3.4.2 Experiment 1 results (IVAS external renderer using MLD and SSNR tools)

##### Experiment 2

Experiment 2 as per clause 3.4.1.

Total number of tests: 658

| Platform | Compiler | Compiler settings and optimization flags | Max MLD | % tests < 5 MLD | Min SSNR (dB) | % tests > 50 MLD | Crashes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RHEL 8.4 | llvm-v18.1.6d1rh84 -O0 | Makefile [8] Modifications: Clang and O0 | WIP | WIP | inf | 100 | 0 |
| RHEL 8.4 | gcc v9.4.0 -O3 | Makefile [8] Modifications: GCC and O3 | WIP | WIP | 81.98 | 100 | 0 |
| RHEL 8.4 | llvm-v18.1.6d1rh84 -O3 | Makefile [8] Modifications: Clang and O3 | WIP | WIP | 81.98 | 100 | 0 |
| Cadence xt-clang | xt-clang RJ-2024.3 tools | Makefile [8] Modifications: xt-clang and O0 | WIP | WIP | 83.3 | 100 | 0 |

Table 3.4.3 Experiment 2 results (IVAS external renderer using MLD and SSNR tools)

### Observations and conclusions

With MLD and SSNR Objective methods, the following observations can be made from the IVAS external renderer experiments:

- Very low MLD (< 1) and high SSNR (> 65 dB) can be observed across platforms and compilers.

- More experiments are needed to draw conclusions on non-BE conformance criteria.

## IVAS split renderer tests

### Summary of experiments

Experiments were done by generating reference IVAS end to end split renderer outputs on reference system. The test outputs were generated using reference bitstreams and decoding/rendering them using IVAS decoder and split renderer on various test systems and the rendered outputs were compared with reference outputs. Following experiments were done:

- Experiment 1: Using windows MSVS reference as mentioned in Table 3.5.1

  - Test outputs generated on windows and Linux platform with various compiler settings as shown in Table 3.5.2.

- Experiment 2: Using Linux GCC reference as mentioned in Table 3.5.1

  - Test outputs generated on windows and Linux platform with various compiler settings as shown in Table 3.5.2.

### Test vector set and test running method

#### Test vector set

#### Test running method and command lines

- For IVAS split renderer tests, the following steps were followed:

- First references were generated using reference platform and compiler settings as mentioned in clause 4. The following pytest command was used to generate reference outputs

python -m pytest tests/split_rendering/test_split_rendering.py  -v -n auto --update_ref 1 --create_ref --keep_files

- The reference outputs were stored on test Operating systems/platform using the same directory/file structure as on the reference system.

- Then IVAS floating source code is compiled on test platform and compilation setting.

- Then test outputs were generated using following pytest command

python -m pytest tests/split_rendering/test_split_rendering.py-v -n auto --keep_files --create_cut

- Reference encoded bitstreams were used for split rendering tests.

### Reference platform and compiler settings

|  | Processor | Platform | Compiler | Compiler settings and optimization flags |
| --- | --- | --- | --- | --- |
| 1 | Intel(R) Xeon(R) W-1290 CPU | Windows 11, SDK: 10.0.17763.0 | Microsoft Visual Studio Toolset: Visual Studio 2017 (v141) ToolsVersion="15.0" | Debug, Win32, Od (disabled), no optimizations |
| 2 | Intel(R) Xeon(R) Gold 6152 CPU | Linux, RHEL 8.4 | GCC 9.4.0 | O0, makefile [8] Default setting with make -j |

Table 3.5.1 Reference platform and compiler settings

### Results

#### MLD and SSNR

##### Experiment 1

Experiment 1 as per clause 3.5.1.

Total number of tests: 658

| Platform | Compiler | Compiler settings and optimization flags | Max MLD | % tests < 5 MLD | Min SSNR | % tests > 50 SSNR | Crashes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Linux, Ubuntu 22.04.1, x86_64 | GCC 11.3 | O0, Makefile [8] Default setting with make -j | 0.9 | 100 | 81.58 | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | GCC 11.3 | Makefile [8] Modifications: O3 optimization | 0.9 | 100 | 85.62 | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: Clang and O2 optimization | 0.9 | 100 | 85.62 | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: Clang and O3 optimization | 0.9 | 100 | 85.62 | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: GCC and Ofast optimization | 11.858449 | 97.4 | 67.08 | 100 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: Clang and O3 and -ffast_math optimization | 12.11 | 97.5 | 65.43 | 100 | 0 |

Table 3.5.2 Experiment 1 results (IVAS split renderer using MLD and SSNR tools)

##### Experiment 2

Experiment 1 as per clause 3.5.1.

Total number of tests: 658

| Platform | Compiler | Compiler settings and optimization flags | Max MLD | % tests < 5 MLD | Min SSNR | % tests > 50 SSNR | Crashes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RHEL 8.4 | llvm-v18.1.6d1rh84 -O0 | Makefile [8] Modifications: Clang and O0 | WIP | WIP | inf | 100 | 0 |
| RHEL 8.4 | gcc v9.4.0 -O3 | Makefile [8] Modifications: GCC and O3 | WIP | WIP | 81.98 | 100 | 0 |
| RHEL 8.4 | llvm-v18.1.6d1rh84 -O3 | Makefile [8] Modifications: Clang and O3 | WIP | WIP | 81.98 | 100 | 0 |
| Cadence xt-clang | xt-clang RJ-2024.3 tools | Makefile [8] Modifications: xt-clang and O0 | WIP | WIP | 83.3 | 100 | 0 |

Table 3.5.3 Experiment 2 results (IVAS split renderer using MLD and SSNR tools)

### Observations and conclusions

- For IVAS split renderer tests, the following steps were followed:

- First references were generated using reference platform and compiler settings as mentioned in clause 4. The following pytest command was used to generate reference outputs

python -m pytest tests/split_rendering/test_split_rendering.py  -v -n auto --update_ref 1 --create_ref --keep_files

- The reference outputs were stored on test Operating systems/platform using the same directory/file structure as on the reference system.

- Then IVAS floating source code is compiled on test platform and compilation setting.

- Then test outputs were generated using following pytest command

python -m pytest tests/split_rendering/test_split_rendering.py-v -n auto --keep_files --create_cut

- Reference encoded bitstreams were used for split rendering tests.

With MLD and SSNR Objective methods, the following observations can be made from IVAS split rendering experiments:

- Very low MLD (< 1) is observed across platforms and compilers without unsafe optimization flags. MLD >10 values are observed with -ffast-math and -Ofast optimization flags.

- Very high Min SSNR is observed without unsafe optimization flags (Min SSNR > 80 in all cases). With -Ofast and fast-math optimization, 97% of tests report the worst case SSNR of greater than 50 dB.

- More experiments are needed to draw conclusions on non-BE conformance criteria.

# IVAS Fixed/Float interoperable Non-BE conformance

## Test vector set and test running method

## Results

## Observations

# IVAS Fixed point Non-BE conformance

## Test vector set and test running method

## Results

## Observations

# Time plan

The time plan of the IVAS BASOP verification is covered in [9].

References

SP-250262: “Revised WID on EVS Codec Extension for Immersive Voice and Audio Services, Phase 2”

TS 26.444: “Test sequences (EVS)”

TR 26.843: “Study on non-bit-exact conformance criteria and tools for floating-point EVS codec”

TS 26.443: “Codec for Enhanced Voice Services (EVS); ANSI C code (floating-point)”

TS 26.258: “Codec for Immersive Voice and Audio Services (IVAS); C code (floating-point)”

IVAS floating point source code: “IVAS Codec Public Collaboration / IVAS Codec · GitLab”

IVAS-10 IVAS BASOP Verification (IVAS-10), S4-242071

IVAS floating point Makefile: “Makefile · main · IVAS Codec Public Collaboration / IVAS Codec · GitLab”

IVAS Permanent document IVAS-2b: IVAS_Codec_Ph2 Project Plan

Optimize Options (Using the GNU Compiler Collection (GCC))

Rishabh Tyagi, Dolby Laboratories Inc. ; email : rishabh.tyagi@dolby.com