3GPP TSG-SA WG4 Audio SWG post 132	S4aA250076

Online, July 11,  2025                                                                   revision of S4aA250074

Source:	Dolby Laboratories, Inc., Nokia, Novamint, EDF

Title:	[FS_ULBC] On ULBC bit rate spacing

Agenda item:	1.9

Document for:	Agreement

- Introduction

The FS_ULBC Permanent Document [1] currently contains a proposal for several Transport Block Sizes (TBSs) potentially available for transmission over NTN NB-IoT channels supporting IMS Voice Calls via GEO satellites.

The current proposal and configuration of relevant Transport Format (TF) parameters are provided below for reference:

Subcarrier Spacing (SCS):

UL: 3.75kHz, 15kHz

DL: 15 kHz

Number of tones:

UL: 1 for 3.75kHz SCS and 15kHz SCS

DL: 12

Voice bundling period: 80ms, 160ms, 320ms

TBS values and PHY bitrates: The TBS values are selected from Table 16.5.1.2-2 for NB-IoT NPUSCH in TS36.213 [2]. The corresponding PHY bitrates and codec bitrates (assuming 7 bytes of packet header) are calculated for each bundling period.

Table 1 TBS and PHY bitrate for 80ms bundling

| TBS (bits) | 144 | 256 | 328 | 424 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.8 | 3.2 | 4.1 | 5.3 |
| Codec bitrate (kbps) | 1.1 | 2.5 | 3.4 | 4.6 |

Table 2 TBS and PHY bitrate for 160ms bundling

| TBS (bits) | 208 | 424 | 600 | 808 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.30 | 2.65 | 3.75 | 5.05 |
| Codec bitrate (kbps) | 0.95 | 2.30 | 3.40 | 4.70 |

Table 3 TBS and PHY bitrate for 320ms bundling

| TBS (bits) | 328 | 776 | 1096 | 1544 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.025 | 2.425 | 3.425 | 4.825 |
| Codec bitrate (kbps) | 0.850 | 2.250 | 3.250 | 4.650 |

- Observation

The available TBS values and resulting source bit rates that could potentially be used by candidate ULBC solutions cover a range of approximately 1–4.7 kbps. While this range is desirable, a significant limitation exists: the set of available bit rates for each bundling period is sparse and unevenly spaced. This deficiency could severely restrict the design freedom codec proponents require to develop optimal solutions for IMS Voice Call services over GEO satellites.

- Proposal

To overcome this limitation, we propose selecting additional TBSs from Table 16.5.1.2-2 for NB-IoT NPUSCH in TS36.213. A reasonable objective providing sufficient design freedom would be a spacing of approximately 500 bits/s between available bit rates.

The following tables demonstrate the proposed selection of TBSs and how this spacing objective is achieved:

Table 4 TBS and PHY bitrate for 80ms bundling

| TBS (bits) | 144 | 176 | 208 | 256 | 296 | 328 | 376 | 424 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.8 | 2.2 | 2.6 | 3.2 | 3.7 | 4.1 | 4.7 | 5.3 |
| Codec bitrate (kbps) | 1.1 | 1.5 | 1.9 | 2.5 | 3 | 3.4 | 4 | 4.6 |
| tx-time (UL) (ms) | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 40 |

Table 5 TBS and PHY bitrate for 160ms bundling

| TBS (bits) | 208 | 256 | 328 | 408 | 504 | 600 | 680 | 776 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.3 | 1.6 | 2.05 | 2.55 | 3.15 | 3.75 | 4.25 | 4.85 |
| Codec bitrate (kbps) | 0.95 | 1.25 | 1.7 | 2.2 | 2.8 | 3.4 | 3.9 | 4.5 |
| tx-time (UL) (ms) | 128 | 128 | 128 | 128 | 128 | 128 | 128 | 128 |

Table 6 TBS and PHY bitrate for 320ms bundling

| TBS (bits) | 328 | 440 | 600 | 776 | 904 | 1032 | 1224 | 1384 | 1544 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.025 | 1.375 | 1.875 | 2.425 | 2.825 | 3.225 | 3.825 | 4.325 | 4.825 |
| Codec bitrate (kbps) | 0.85 | 1.2 | 1.7 | 2.25 | 2.65 | 3.05 | 3.65 | 4.15 | 4.65 |
| tx-time (UL) (ms) | 256 | 256 | 256 | 256 | 256 | 256 | 256 | 256 | 256 |

It should be noted that not all Modulation and Coding Schemes (MCSs) are available for all TBSs, and the number of Resource Units (RUs) depends on the selected MCS. The transmission time (tx-time) within the bundling period depends on both the number of RUs and the number of repetitions NRep. Consequently, only a limited number of TF parameter configurations are feasible – specifically those where the resulting tx-time falls within the bounds of the considered bundling period while leaving sufficient room for transmission in the opposite direction.

To optimize use of available transmission resources, tx-time should be maximized within these constraints. For the bundling periods of 80, 160, and 320 ms under consideration, the number of resource units NRU should preferably be a power of 2, resulting in preferred tx-times for UL transmission of 64, 128, and 256 ms respectively. The tables above display the resulting tx-times for UL transmission.

Additionally, multiple TF parameter configurations (SCS, MCS, and NRep) may be suitable for a given bundling period, TBS, and tx-time. Table 7 illustrates this by showing different possible configurations with TBS=144 bits that achieve a tx-time of 64 ms on UL for an 80 ms bundling period.

Table 7 Example with TBS=144 bits and tx-time=64 ms

| Bundling time | TBS | SCS | MCS | NRU | Nrep | tx-time | PHY rate | Net rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 80 | 144 | 15 | 1 | 4 | 2 | 64 | 1800 | 1100 |
| 80 | 144 | 15 | 5 | 2 | 4 | 64 | 1800 | 1100 |
| 80 | 144 | 15 | 10 | 1 | 8 | 64 | 1800 | 1100 |
| 80 | 144 | 3.75 | 5 | 2 | 1 | 64 | 1800 | 1100 |
| 80 | 144 | 3.75 | 10 | 1 | 2 | 64 | 1800 | 1100 |

It is essential to identify the most suitable TF parameter configurations for all potential bundling times and TBSs to ensure the most robust transmission. Packet loss patterns should subsequently be generated for these optimal configurations. We believe this selection can be determined through BLER vs. SNR simulations for the different configurations, as suggested in [1].

- Conclusion

We recommend adopting the TBSs presented in Tables 4–6, which would provide codec proponents with sufficient design freedom to develop optimal ULBC solutions for IMS Voice Calls over GEO satellites.

Furthermore, the most suitable TF parameter configurations should be identified for all potential bundling times and TBSs, and corresponding packet loss patterns should be made available.

These assumptions should be documented in the Technical Report, and SA4 should seek confirmation from RAN1.

References

[1] 	Tdoc S4-251154: Permanent Document FS_ULBC, v1.1

[2] 3GPP TS36.213: Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures

[3] Tdoc S4aA250058: Simulation results for FS_ULBC, Beijing Xiaomi Mobile Software