3GPP TSG-SA WG4 Audio SWG post 132	S4aA250079

Online, July 11, 2025	revision of S4aA250044

Source:	Skylo Technologies

Title:	IMS voice call via Non-IP Data Delivery over GEO

Spec:	3GPP TR 26.940

Agenda item:	1.9

Document for:	Agreement

1. Introduction

NB-IoT channels are subject to strict bandwidth limitations, with only 180 kHz of usable spectrum. These constraints significantly limit the number of simultaneous voice calls over a non-anchor carrier, particularly when operating over GEO satellites. To maximize transmission efficiency in such narrowband and high-latency environments, it is essential to employ a data delivery mechanism with minimal protocol header overhead.

Non-IP Data Delivery (NIDD), when operating over the user plane (UP), is well-suited for this purpose. It not only minimizes protocol overhead, as shown in Table 1, but also supports the use of AS-level security mechanisms. While NB-IoT does not support dedicated data radio bearers, UP NIDD utilizes the default bearer, which can be configured for radio-level optimization. For example, RLC Acknowledged Mode (AM) may be used for signaling, and RLC Unacknowledged Mode (UM) for media delivery, allowing for improved reliability and latency management.

Table 1: Data Delivery Overhead Comparisons

| Bytes | RTP | UDP | IP | PDCP | RRC | NAS | RLC | MAC |  | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPv4 | 12 | 8 | 20 | 1 |  |  | 2 | 1 |  | 44 |
| IP with RoHC |  |  | 3 | 1 |  |  | 2 | 1 |  | 7 |
| UP NIDD |  |  |  | 1 |  |  | 1 | 1 |  | 3 |
| CP NIDD |  |  |  |  | 2 | 11 | 2 | 1 |  | 16 |

IPv4 is not ideal in this context due to the relatively high overhead of protocol headers. IP with RoHC adds additional complexity and CPU processing for both the UE and the network. This makes it less efficient for NB-IoT, where devices are constrained by limited processing capability, power consumption, and memory footprint.

2. Reason for Change

This contribution adds the necessary description for supporting IMS voice over UP NIDD in GEO-based NTN. It enables the integration of NIDD into the end-to-end simulation model, which is essential for deriving accurate delay and error profiles under realistic conditions. This forms the basis for evaluating performance and ensuring alignment with protocol behavior in constrained satellite environments.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 26.940 or to the permanent document.

* * * First Change * * * *

### 5.2.1	Scenario 1:  IMS Voice Call over GEO

#### 5.2.1.1	Introduction

This clause introduces the methodology of obtaining channel characteristics and results for developing design constraints and performance requirements for a codec supporting the main scenario as documented in Clause 4.2.1: IMS Voice Call over GEO.

#### 5.2.1.2	Delay error profiles

The delay-error profile is a model used to describe the network impairments—particularly delay and packet loss—that can impact real-time conversational services such as IMS voice call. Such profile typically reveals the GEO satellite channel characteristics and will be used to evaluate codec robustness, guide jitter buffer design and ensure a fair and comparable testing.

#### 5.2.1.3	End to end simulation model to derive delay error profiles

The intention of this methodology is to reuse the simulation model defined in Annex E of TS 26.132 [26132] to produce the delay error profile.

This Annex E reference LTE access scenario is illustrated in Figure 5.2.1.3-1. Building on the main scenario defined in Clause 4.2.1, the corresponding end-to-end GEO access scenario is shown in Figure 5.2.1.3-2 and Figure 5.2.1.3-2a. The primary distinction between the reference LTE scenario and the GEO voice main scenario lies in the introduction of the “new GEO channel” and the potential inclusion of the Non-IP Data Delivery option in the protocol stack as illustrated in Figure 5.2.1.3-2a.

Fig.5.2.1.3-1: End-to-end channel of VoLTE using LTE access

Fig.5.2.1.3-2: End-to-end channel of main scenario for IMS voice call using NB-IoT (GEO) satellite access

Fig.5.2.1.3-2a: End-to-end channel of main scenario for IMS voice call using NB-IoT (GEO) satellite access with Non-IP Data Delivery

Based on the functional description in Table E.1 of TS 26.132, the following input parameters are required to implement the simulation model:

BLER_tx / BLER_rx:

These parameters are required to simulate block error rates in both uplink and downlink.

NOTE: the resulted error trace based on Clause 5.2.2 will be used to serve as the BLER_tx/BLER_rx.

[max_tx / max_rx:

These define the maximum number of HARQ retransmissions for uplink and downlink respectively, which fall under RAN2 scope. In current specifications, NB-IoT supports at most two HARQ processes, which face constraints in high-latency GEO satellite scenarios. For IMS voice over GEO, HARQ feedback is suggested to be disabled per the standard of Release 18 [5].]

drx_cycle_length:

This parameter represents the duration of the DRX (Discontinuous Reception) cycle in milliseconds. It determines how frequently the device wakes up to monitoring possible scheduling grant. This parameter affects packet scheduling and transmission timing in the simulation context Annex E of TS 26.132. In addition, the values for LTE are 20-40ms, whether these values are suitable for GEO scenarios should be confirmed with RAN2.

mis_eNB1_eNB2:

This parameter represents the scheduling time mis-align between the two eNBs. In GEO scenarios, it indicates how long packets wait in the buffer before the next transmission opportunity. This should be determined primarily by RAN2 (responsible for dynamic scheduling or Semi-Persistent Scheduling) with possible input from RAN1 about physical layer timing relationship aspects.

[max_net_delay / min_net_delay:

These represent the delay range between eNB1 and eNB2. For GEO voice, they are considered similar to the LTE scenario, and legacy parameter values can be reused.]

Editor’s NOTE: 	whether the model for the delay between eNB1 and eNB2 for LTE scenarios well reflects the 		delay in deployment is FFS.

nFrames:

This refers to the number of frames for the simulation. In the reference LTE scenario, one IP packet corresponds to 20 ms of speech. In contrast, the GEO voice scenario introduces additional considerations shown as follows due to the propagation delay from GEO satellite altitude.

- Speech sequence (frame length): For GEO, a longer frame length may be used. The maximum frame length of 80 ms, as defined by 3GPP, is assumed in this simulation. Final confirmation is expected from SA4.

- Voice packet size: This depends on the protocol overhead as illustrated in Figure 5.2.1.3-3 for the reference LTE access scenario and Figure 5.2.1.3-4 for the GEO voice main scenario. The exact overhead depends on the transport path of the voice packets, user plane or control plane, via IP or via Non-IP (NIDD), and must be confirmed by RAN2 and SA2. The RTP layer in Figure 5.2.1.3-4 may use a simplified RTP header. The feasibility of such a simplified header may be specified by SA4. For simulation purpose, the assumed range is as follows:

-	Lower bound: ~3 bytes. Assuming user plane with Non-IP Data Delivery (NIDD)and NB-IoT protocol layers: PDCP (1 byte), RLC (1 byte), MAC (1 bytes);

-	Upper bound: ~52 bytes. Assuming control plane with IPv4/UDP/RTP (20/8/12 = 40 bytes), NAS reduction (7 bytes, optimized by CT1), and NB-IoT layers: RRC (2 bytes), RLC (2 bytes), MAC (1 byte);

Fig. 5.2.1.3-3: VoIP RTP packet in reference LTE access scenario

Fig.5.2.1.3-4: Example of RTP packet in GEO voice main scenario

-	RTP Payload Size: This is computed as the product of frame length and codec bit rate.

Editor’s Note: whether the size of RTP payload affects the delay-error profile is FFS.

Once the parameters regarding GEO channel are confirmed, the simulation methodology as described in Table E.1 will be updated with these new parameters and used to produce the required delay-error profiles.

* * * End of Changes * * * *