3GPP TSG SA4 Meeting SA4-e (AH) Audio SWG post 133-e	S4aA250099

Online, 05 September 2025	revision of S4aA250090

Source:	HEAD acoustics GmbH

Title:	Updates to Motion-to-Sound Latency Test Method and Measurement Results

Agenda item:	1.6 (ATIAS_Ph2)

Document for:	Agreement

# Objective

The source provides an update to the Motion-to-Sound (M2S) Latency Test Method proposed for TS 26.260 in Clause 5.2 of the current ATIAS_Ph2 permanent document [1]. The current status of the proposal is “Progress: Initial proposal / pending validation”. In order to progress the work, the present document provides

- measurement results from the field for commercially off-the-shelf devices to validate the measurement method,

- proposals for minor changes to the test method, and

- a discussion of what the current measurement method addresses and what is out of scope.

This document is a continuation of the discussion on input documents S4-241964 [2] and SA4aA250001 [3].

# Proposed Changes to the Test Method and Field Results

The source has recently conducted a field test with a test method (“constant-speed measurement”), which is largely identical to the method defined in the ATIAS_Ph2 permanent document. The results have already been presented to the experts in the field at the AES International Conference on Headphone Technology 2025 [4]. The present input document recaps relevant findings of the publication and identifies need for action.

## Method Refinement

The current test method in [1] uses a narrowband sine signal. However, experiments with commercial off-the-shelf DUTs showed that narrowband features in the DUTs lead to undesired fluctuations in the interaural level differences. By using a multitone test signal, the resulting ILD curve is smoother and less prone to peculiarities in the HRTF sets used in the DUTs, ultimately resulting in a more robust test method [4].

Therefore, it is proposed to use the multitone signal from [4] (also called "perfect sweep"). Furthermore, the motor noise reduction filter described in the test procedure has to be adjusted to the new test signal. A Python demo script to generate this type of sweep signal is provided as an electronic attachment to this document.

Moreover, increased robustness was achieved by increasing the moving average smoothing length from P=101 to P=201.

## Method Validation

In order to validate the M2S latency test method, an experiment has been conducted in [4], in which the method was applied with no actual DUT but a loudspeaker in front of the motorized HATS. During head movement, the source position remains fixed in the world coordinate system, which is exactly what devices simulate with head-tracked binaural rendering. With an actual loudspeaker, there is no M2S latency such that the measurements should ideally yield a latency of 0 ms. The experiment results are shown in the column “no DUT” in Figure 1. As expected, the measured M2S latency is close to 0 ms. The median is only -0.1 ms off.

## Field Test with Commercial Off-the-shelf DUTs

In [3] and [5], the test method was applied to a self-build demo system featuring a real-time implementation of the IVAS renderer. While this was a successful proof-of-concept, validation with more realistic DUTs was still pending.

Currently, there is no IVAS-UE available. However, there are other spatial audio solutions in the market, which are expected to be comparable in terms of their system architecture to what can potentially be expected from future IVAS-UE. In [4], the M2S latency test method is applied to three commercial off-the-shelf DUTs. Each DUT is a combination of spatial-audio-enabled hearables and smartphone, wirelessly connected via Bluetooth. In the following, the DUTS are denoted as DUT 1, DUT 2 and DUT 3. The test signal is injected into the center channel of a 5.1 surround audio signal, which is encoded with a lossless codec and is played with the smartphones.

The measurement results are shown in Figure 1.

Figure 1: M2S latency measurement results for the validation measurement (“no DUT”) and for commercial off-the-shelf devices. Results for each motion pass 𝑛=1,…,𝑁 are shown as scattered points with median and mean values with red and grey bars, respectively. The numbers shown below the plot are median M2S latency values. Source: [4]

DUT 1 scores the lowest median M2S latency with 34 ms, DUT 2 scores 71 ms, and DUT 3 scores a significantly higher latency of 158 ms, which is above the perceptual threshold.

In the DUT 1 results, an outlier is present that departs the mean value (grey bar) from the median (red bar). Given that such outliers are measurement artefacts, this motivates to prefer the median over the mean to obtain a single value M2S latency estimate.

For DUT 2, systematic differences in the left-to-right and right-to-left results are eminent. These can be attributed to a mismatch of zero ILD in the calibration and the measurement phase, most likely due to sensor drift, as was already discussed in [2] and [5].

For further details, the reader is referred to [4]. In [4], the method is also compared to some variants and further details are given, suggesting that the test method works as intended.

# Discussion: Additional Methods and Head-Tracking Performance

During past meetings, it was hypothesized that the proposed test method measures effective M2S latency in a specific operation point but not necessarily the underlying physical delay of the system. This hypothesis was confirmed in [4]. It could be shown that DUT 1 performs motion prediction to improve head tracking despite a physical system delay that exceeds what is measured with the constant-speed test method.

It was discussed whether it is possible to measure the physical M2S latency. Indeed, [4] proposes and evaluates a test variant for physical M2S latency based on detection of the motion onset. However, the method turned out to be less robust and less accurate than the constant-speed method. Moreover, the “prototype matching method” is proposed in [4] as a compromise that measures an operation point that leaves less room for motion prediction while still delivering robust results.

Although there are publications of perceptual studies on the perception threshold of M2S latency, there is no perceptual study known to the source, which specifically addresses the perception of time-variant M2S latency due to motion prediction. It is expected that motion prediction is able to reduce the perceived M2S latency to some extent but that perception might be most sensitive to motion onsets and offsets exactly where motion prediction is challenging. However, at the time being, there is no scientific evidence yet that show that other test methods outperform the proposed constant-speed test method in terms of correlation with human perception. Therefore, in the scope of ATIAS_Ph2, the source proposes to adopt the proposed test method for TS 26.260 as it has proven to be more robust and accurate than the other methods. The source considers robustness, reproducibility, and accuracy most relevant for incorporation into TS 26.260. Perceptual findings in the future may be incorporated in a future version of TS 26.260, but are out of scope for ATIAS_Ph2.

Moreover, for future refinement of test methods for head-tracked binaural-rendering beyond ATIAS_Ph2, it is suggested to consider “M2S latency” and “head-tracking performance” two separate requirements. This is based on the observations that some devices have a low M2S latency but poor tracking behavior (e.g., sensor drift, the orientation changes over time without motion) or the other way around. Potential approaches could be as follows:

- Trajectory nullification (as proposed by Fraunhofer in [6]): a source is rendered with the same trajectory as the motorized HATS orientation trajectory such that the source appears in front of the user. It can then be checked if binaural cues remain stable and requirements can be stated for fluctuations of the binaural cues.

- The “prototype matching method” in [4] calculates an error between the actual and a target ILD curve (Equation (7) in [4]), which could serve as a basis to assess tracking performance.

# Summary and Proposal

The present document has presented minor improvements to the existing test method in the current ATIAS_Ph2 PDoc and has summarized field test results with commercial off-the-shelf spatial audio devices, which have been published in [4]. The results serve as a validation for the test method.

The source proposes to consider the amendment from Annex A.1 to be incorporated into the ATIAS_Ph2 PDoc. Furthermore, the source suggests to tag the proposal mature for adoption into TS 26.260.

# References

- Rapporteur ATIAS_Ph2, “Permanent Document ATIAS-2 v0.5”, 3GPP input document S4-251364 to 3GPP TSG SA WG4 Meeting SA4#133-e, online, May 2025.

- HEAD acoustics GmbH, “Test methods for UE headtracking and binaural rendering”, 3GPP input document S4-241964 to 3GPP TSG-SA WG4 Meeting #130, USA, Orlando, November 2024.

- HEAD acoustics GmbH, “Test Method for Motion-to-Sound Latency”, 3GPP input document SA4aA250001 to Conference Call SA4-e (AH) Audio SWG post 130, January 2025.

- M. Kentgens, M. Bárány, A. Hallmann, C. Nelke, “Head-Tracked Spatial Audio: What is the Motion-to-Sound Latency of your Device?”, AES International Conference on Headphone Technology, Espoo, Finland, August 2025. URL: https://aes2.org/publications/elibrary-page/?id=22945.

- M. Kentgens, M. T. Bárány, J. Reimes, H. Steingrobe, “Measuring Motion-to-Sound Latency with Motorized HATS,” Proceedings of the German Annual Conference on Acoustics (DAGA), Copenhagen, Denmark, March 2025. DOI: 10.71568/dasdaga2025.094.

- Fraunhofer IIS, “Evaluating IVAS Rendering”, 3GPP input document S4-231709 to 3GPP TSG SA WG4 Meeting SA4#126, Chicago, USA, November 2023.

- Annex

## Proposed Changes to the ATIAS_Ph2 PDoc

The following changes to Clause 5.2 of the current ATIAS_Ph2 permanent document [1] are proposed:

### Changes for TS 26.260

Start change	[

5.7.5	Motion-to-Sound Latency

The test method applies to headset UEs providing headtracking functionality.

The test setup is as described in clause 5.4.3.2. A motorized HATS or comparable apparatus suitable for dynamic head-tracking measurements as described in clause 4.02 is used. The measurement apparatus shall be capable of realizing the below-stated movement trajectory. It shall feature a sensing mechanism that tracks the point in time at which the apparatus passes 0° orientation with a tolerance of ±2 ms. Moreover, its noise emission shall not exceed -70 dBPa measured at the artificial ears after signal filtering. An example setup is illustrated in Figure X.

Figure X: Setup for measuring M2S latency using a motorized HATS

The test method evaluates interaural level differences that are calculated from the artificial ear signals with the following algorithm:

1)	The left and right ear signals are each filtered with a narrow zero-phase filter that is designed to suppress noise of the measurement apparatus while not impacting the test signal. The filtered signals are denoted as xll and xrl with sample index l and shall be made available at a sampling rate of fs=48 kHz.

2)	The interaural level difference (ILD) is determined on the pre-processed left and right ear signals xll and xrl, respectively. The ILD is calculated on short frames with index λ and can be stated as
	ILDλ=max-ILDmax,minILDmax, 10log10l=0L-1xr2Rλ+ll=0L-1xl2Rλ+ldB.

For the calculation, frame length L=[96] and hop size R=[24] are used. The ILD is limited to
-ILDmin≤ILD(λ)≤ILDmax, with ILDmin/max=±12 dB.

3)	The ILD is then smoothed over P=1201 frames using a moving average with
	ILDλ=1Pr=-P-12P-12ILDλ+r.

Time instances per frame index λ are determined by
	tλ=1fsRλ+L-12.

The motion-to-sound (M2S) latency measurement shall be conducted as follows:

1)	The test signal to be used for the measurements shall be the periodic multitone signal with period length 1 ms as used in [XXX]. The signal shall be virtually positioned to 0° azimuth and 0° elevation (frontal position for 0° head orientation) as described in clause 5.5.2. The signal shall be calibrated to -20 LFKS as described in clause 5.5.1.

2)	The UE and the reference client are setup according to clauses 5.4.2 and 5.3.2, the source signal is encoded by the reference client and inserted at the POI to the UE. In case a volume control is provided in receive direction, it shall be set to nominal setting as identified per clause 5.7.2.

3)	The motorized HATS is oriented to 0°. For ILD calibration, the ear signals are captured for a duration of 2.5 s without applying any motion to the UE. Then, ILDλ is determined for the ILD calibration sequence and an average ILD (denoted as ILD0°) is calculated as the mean of ILDλ.

4)	The dynamic part of the measurement (in motion) is conducted as follows:

a)	The motorized HATS rotates continuously and repeatedly between [+60°] (start orientation) and -60°. The 0° orientation shall be passed Nmeas=[124] times, i.e., 7 times for each direction of rotation, so that a total of Nmeas partial estimates of M2S latency are determined. The duration between two zero-crossings shall be less than 2.0 s.

b)	In the angle range between -10° and +10°, the angular velocity shall be constant with a nominal angular velocity of 108°/s.

c)	During motion, the left and right ear signals and the time stamps t0(i) (where the motorized HATS passes 0° orientation) are captured for i=1,…, Nmeas.

5)	After the last orientation is reached, the motorized HATS is immediately oriented to 0°. Step 3) is repeated to obtain the verification ILD, denoted as ILD0°,v.

6)	The M2S latency time is calculated as follows for each zero-crossing i=1,…, Nmeas of the orientation:

a)	The smoothed ILD versus time ILD(i)λ is determined.

b)	The time stamp t1(i) of the sound signal passing 0° orientation is determined by identifying the zero-crossing of ILD(i)λ- ILD0°.

c)	The M2S latency for the i-th measurement is determined by T(i)=t1(i)-t0(i).

7)	The overall M2S latency is determined by calculating the median of T(i).

The principle of motion-to-sound analysis and determination of t1(i) and t0(i) is illustrated in Figure X+1.

NOTE 1:	Before starting the measurement, the UE shall be initialized to match its reference (or zero) orientation with the HATS oriented to 0° and the UE shall be in active headtracking mode.

NOTE 2: The measurement method presented requires a constant reference orientation. In some IVAS UE use-cases, a DUT may intentionally adapt its reference orientation over time, for instance, based on the average of past orientations. The measurement method is not suited for such cases.

Figure X+1: Example setup for measuring M2S latency using a motorized HATS

T(i)

]	End change

### Changes for TS 26.261

Start change	[

6.5.3	Motion-to-Sound Latency

The M2S latency measured with the test method from clause 5.7.5 from TS°26.260 should not exceed [80 ms] and shall not exceed [100 ms].

The absolute difference between the initial ILD calibration (ILD0°) and the verification ILD (ILD0°,v) shall not exceed 0.5 dB.

]	End change

### Changes to the references in Clause 3 of the PDoc

Start change	[

[…]

[XXX]	M. Kentgens, M. Bárány, A. Hallmann, C. Nelke, “Head-Tracked Spatial Audio: What is the Motion-to-Sound Latency of your Device?”, AES International Conference on Headphone Technology, Espoo, Finland, August 2025.

[…]

]	End change