SA4 (AH) Audio SWG post 133-e					                  S4aA250243

F2F Erlangen, 23 – 25 Sept 2025

Source:	Dolby Laboratories Inc.

Title:	[FS_ULBC] On ULBC latency budget

Agenda item:	7.9

Document for:	Discussion and Agreement

# Background

Latency is a crucial parameter of communications systems with fundamental impact on voice service quality. In the context of the ongoing FS_ULBC SID study [1] a latency breakdown will become part of TR 26.940.

This contribution aims to put mouth-to-ear latency into both a service quality and a listening-only quality perspective. Based on conclusions of the discussion of this document it is proposed to update the latency breakdown of draft TR 26.940 [2].

# Discussion

A first breakdown of mouth-to-ear latency is included in draft TR 26.940. Given the recent discussions on link-level transmission parameters for the UE to GEO satellite, this first breakdown is outdated. Two recent contributions from meeting SA4-133e address latency, namely Tdocs S4-251304 [3] and S4-251382 [4].

  - Considerations on S4-251304 [3]

This contribution draws a connection to ITU-T recommendation P.114 [5] and its recommendations for one-way transmission time. The impact of mouth-to-ear latency (=one-way transmission time) on user satisfaction of the voice service is shown in the below figure. The fact that the mouth-to-ear latency will even in the best case significantly exceed 500 ms means that users of a ULBC voice service over GEO satellite will generally be dissatisfied with the service, if it is used under the assumption that it is a regular voice service. In its conclusion, contribution [3] proposed to evaluate resulting quality level in a human factor test.

3GPP/SA4 will conduct such tests to identify the best of potentially several candidate solutions. When doing such tests, different kinds of subjective quality evaluations can be considered, which can coarsely be categorized as listening-only tests and conversation-opinion tests. Draft TR 26.940 already contains a discussion of these categories.

In the view of the source, SA4 should focus on listening-only tests. The reason is that no matter how a specific candidate solution potentially optimises latency, there is no possibility to leave the red area of the E-model curve indicating that nearly all users are dissatisfied. However, candidate solutions may significantly vary in terms of offered listening-only quality. Moreover, since the conversational quality is in any case in the red area, candidate solutions should be given the freedom within reasonable limits to use extra delay if this helps to enhance the listening-only quality. One example where such a solution-specific delay budget may lead to substantial improvements of the listening-only quality is interleaved transmission, as shown in [6]. Other examples may be additional codec delay (look-ahead) or (partial) redundancy transmission of previous frame data. The value of such a reasonable limit should be for further discussion and would have to be set together with latency design constraints.

  - Considerations on S4-251382 [4]

This contribution proposed to update the mouth-to-ear latency breakdown of draft TR 26.940. The merit of the contribution is that it considers the present assumptions on useable bundling times which are 80, 160 and 320 ms. The current breakdown shows too low figures since bundling was not accounted for.

The contribution further proposed to make certain assumptions about the processing time of encoder and decoder. The proposal was to assume that the processing time both for encoder and for decoder would range between 80% and 95% of the coding frame length.

The source is of the opinion that such an assumption needs to be based on facts, which the contribution did not provide. Given the large span of complexity figures of different codecs as well as substantial potential differences of their implementations on different platforms, it is felt that it is impossible to make strong assumptions on the processing time. Encoder and decoder processing times should rather be referred to as a vendor specific encoder/decoder processing delay, and the processing times should span between the theoretical extremes 0 and 100% of the codec frame length. Table 5.1.2-2 of contribution [4] is accordingly updated and shown below. The vendor delay budget that originally comprises a codec processing delay component of 20 ms is reduced by that amount.

To connect to the discussion in clause 2.1 and the suggested additional solution specific delay budget, the table is further updated by this component. As explained above, a maximum for this value should be set as part of ULBC delay design constraints. However, the source is of the opinion that a reasonable limit could be 160 ms.

Table 5.1.2-2 UE delay estimation for ULBC

| Voice bundling period | Codec frame size | UE delay excluding solution specific delay (Note 1) |  |
| --- | --- | --- | --- |
|  |  | lower bound | upper bound |
| 80 | 20 | 268 | 355 |
|  | 40 |  | 395 |
|  | 80 |  | 475 |
| 160 | 20 | 428 | 515 |
|  | 40 |  | 555 |
|  | 80 |  | 635 |
|  | 160 |  | 795 |
| 320 | 20 | 748 | 835 |
|  | 40 |  | 875 |
|  | 80 |  | 955 |
|  | 160 |  | 1115 |
|  | 320 |  | 1435 |
| Note 1: UE_delay excluding the solution specific delay = 2x voice bundling period + 2x vendor specific encoder/decoder processing delay + vendor delay budget + JBM + codec alg. Delay Note 2: Solution specific delay X = [TBD ms] Editor’s note: The above TBD value is characteristic of the solution. It will be set once the solution is selected. |  |  |  |

The resulting table of the mouth-to-ear latency is hence updated as follows:

| Voice bundling period | Codec frame size | Mouth to ear delay main scenario in ms (GEO - TN) (Note 1) |  | Mouth to ear delay sub-scenario in ms (GEO - GEO) (Note 2) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | lower bound | upper bound | lower bound | upper bound |
| 80 | 20 | 548 + X | 872 + X | 804 + X | 1315 + X |
|  | 40 |  | 912 + X |  | 1355 + X |
|  | 80 |  | 992 + X |  | 1435 + X |
| 160 | 20 | 708 + X | 1032 + X | 964 + X | 1475 + X |
|  | 40 |  | 1072 + X |  | 1515 + X |
|  | 80 |  | 1152 + X |  | 1595 + X |
|  | 160 |  | 1312 + X |  | 1755 + X |
| 320 | 20 | 1028 + X | 1352 + X | 1284 + X | 1795 + X |
|  | 40 |  | 1392 + X |  | 1835 + X |
|  | 80 |  | 1472 + X |  | 1915 + X |
|  | 160 |  | 1632 + X |  | 2075 + X |
|  | 320 |  | 1952 + X |  | 2395 + X |
| Note 1: UE delay+GEO transmission+Delay_GSCN+Delay_eNBCN + Solution specific delay X Note 2: UE delay +2x GEO transmission+2x Delay_GSCN + Solution specific delay X |  |  |  |  |  |

- Conclusion and Proposal

In this contribution the source has made a proposal to optimize listening-only quality of ULBC codec solutions even though the conversational voice service quality according to ITU-T G.114 is expected to lead to a level where nearly all users are dissatisfied. It has been proposed to give candidate ULBC solutions an extra latency allowance to enable improvements of the listening-only quality. A corresponding solution specific delay parameter X has been proposed to be added to the mouth-to-ear latency breakdown of draft TR 26.940.

In addition, the contribution has reviewed the mouth-to-ear latency breakdown of [4] with the conclusion that there is not enough evidence that would allow an assumption according to which encoder and decoder processing delay would be in the range of 80 – 95 % of the codec frame length. Instead, the source proposes to refer to vendor specific encoder/decoder processing delay and to work with the assumption that this delay will be within 0 and 100% of the codec frame length.

An implementation of the proposal in draft TR 26.940 V0.2.0 is provided below.

* * * First Change * * * *

## 5.1	Estimation of mouth to ear delay for GEO scenarios

### 5.1.1	Overview

This clause estimates the mouth to ear (M2E) delay for IMS voice call over GEO satellites based on the application scenario introduced in clause 4.2. Two sub-scenarios are considered:

-	Main Scenario (see clause 4.2.2.2): UE1 is connected via satellite while UE2 is connected via terrestrial network which corresponds to the signal flow UE1 àGEO satellite àGround stationàCore networkà eNodeB àUE2

-	Sub-Scenario 1 (see clause 4.2.2.3): Both UEs are connected to a GEO satellite which corresponds to the signal flow UE1 àGEO satellite àGround stationàCore networkàGround stationàGEO satelliteàUE2

This approach aims to estimate the maximum and minimum delay components in the signal flow and finally to estimate a range of the. mouth-to-ear delay accordingly. The estimation assumes jitter free case and no network congestion.

NOTE: In practical deployments, various jitter and network conditions can arise.

Editor’s note: The scenarios and the terminology of this clause needs to be aligned with clause 4.) “Application Scenario” where a detailed description of the call scenarios is expected.

### 5.1.2	Delay components

#### 5.1.2.1	Overview

In this clause, the individual delay components that contribute to the mouth-to-ear delay are introduced and derived. The derived values are independent of the signal flow direction.

#### 5.1.2.2	UE Delay considering IMS codecs

TS 26.131 [D.1] defines the internal UE delay requirements and objectives depending on the components codec (frame size and algorithmic delay), air interface, jitter buffer depth and vendor specific delay budget. The UE delays in sending (UE1) and receiving directions (UE2) are not separated in TS 26.131, however the sum of the sending and receiving delays can be considered together.

The jitter buffer delay budget contains 40ms if the packet duration is 20ms and it includes the expected jitter profiles for terrestrial network transmission. In case of 40ms packet duration, the budget is doubled to 80ms, which is not further discussed. The value for the air interface in [D.1] just reflects the delay between UE and measurement equipment (2 ms) and needs be replaced by the expected delay for real air interface, i.e. air interface to GEO satellite or terrestrial network.

For MTSI-based speech only services with LTE and NR, the UE delay is outlined in the following table:

Table 5.1.2-1 UE delay components

|  | UE delay in ms (Performance objective) (Note 2) | UE delay in ms (Maximum requirement) (Note 2) |
| --- | --- | --- |
| Frame size (Note 1) | 20 | 20 |
| alg. Codec Delay (Note 1) | 5 | 12 |
| JBM (jitter free) (Note 3) | 40 | 40 |
| Vendor specific budget (Note 4) | 83 | 123 |
| UE delay Ts+Tr | 148 | 195 |
| Note 1: Values reflect the IMS codecs AMR/AMR-WB/EVS Note 2: Requirements and Performance Objectives apply to the UE delay only (sum of send (Ts) and receive (Tr) delays) and only for MTSI-based speech-only with LTE, NR or WLAN access in error and jitter free conditions. Note 3: JBM delay is considered as constant independent of the frame size. Note 4: Vendor specific budget of TS 26.131 may change for GEO satellite connectivity |  |  |

Editor’s note: This table assumes LTE/NR air interface and needs to be updated for GEO satellite access air interface.

For ULBC, the air interface delay for GEO depends on the selected voice bundling periods for the ULBC transmission where 80ms, 160ms or 320ms are considered. It is assumed, that the available time budget is entirely used for RUs or repetitions to maximize the spectral efficiency. The delay for the air interface excluding the propagation delay is therefore identical to the voice bundling period.

The frame size of the codec in Table 5.1.2-1 represents the audio capturing delay which is for GEO identical to the voice bundling period.

The codec frame size determines the processing delay of the encoder and decoder. Encoder and decoder processing times are referred to as a vendor specific encoder/decoder processing delay, and the processing times should span between the theoretical extremes from 0 to 100% of the codec frame length.

The vendor specific budget that originally comprises a codec processing delay component of 20 ms is reduced by that amount.

This leads to a UE delay estimation for voice bundling periods of 80, 160 and 320 ms and codec frame sizes of 20, 40, 80, 160 and 320 ms, according to the following table:

Table 5.1.2-2 UE delay estimation for ULBC

| Voice bundling period | Codec frame size | UE delay excluding solution specific delay (Note 1) |  |
| --- | --- | --- | --- |
|  |  | lower bound | upper bound |
| 80 | 20 | 268 | 355 |
|  | 40 |  | 395 |
|  | 80 |  | 475 |
| 160 | 20 | 428 | 515 |
|  | 40 |  | 555 |
|  | 80 |  | 635 |
|  | 160 |  | 795 |
| 320 | 20 | 748 | 835 |
|  | 40 |  | 875 |
|  | 80 |  | 955 |
|  | 160 |  | 1115 |
|  | 320 |  | 1435 |
| Note 1: UE_delay excluding the solution specific delay = 2x voice bundling period + 2x vendor specific encoder/decoder processing delay + vendor delay budget + JBM + codec alg. Delay Note 2: Solution specific delay X = [TBD ms] Editor’s note: The above TBD value is characteristic of the solution. It will be set once the solution is selected. |  |  |  |

#### 5.1.2.3	Core network delay

The delay contribution of the core network consists of the packet transmission delay between two network entities, e.g. ground station to core network or core network to eNodeB. In case of the interop scenario GEO NTN to TN network, an additional delay component for transcoding needs to be considered. Assuming the frame size of both codecs is identical or a multiple of each other, only the algorithmic codec delay contributes to the transcoding delay, i.e. 5ms for AMR/AMR-WB or 12ms for EVS, and an additional delay margin for the processing of the transcoding (2 ms). This means, transcoding with AMR/AMR-WB adds 7ms and with EVS adds 14ms.

Table Y.5.1.2.3-1 Core network delay components

|  | Minimum delay in ms | Maximum delay in ms |
| --- | --- | --- |
| Network delay ground station to core network (Delay_GSCN) | [ 5 Note1-1 , 20 Note1-2] | [200 Note2] |
| Network delay eNodeB to core network (Delay_eNBCN) | 5 | 20 |
| Transcoding | 7 | 14 |
| [Note1-1: In [D.2] 5 ms network latency is assumed] [Note1-2: TS 23.501 assumes a static delay value for the CN PDB of 20ms between a UPF and 5G-AN. ] [Note2: In some NTN deployments, the core network may need to be located far from the ground station due to factors like user distribution, geography, or other practical considerations. As a result, latency can increase, ping statistics between continents, for example, can reach up to 200ms.] |  |  |

#### 5.2.2.4	Transmission delay UE – GEO - Ground station

Clause 7.4.2 of [D.2] defines the KPI requirement for GEO based satellite access, i.e. 280ms. TR 36.763 clause 7.1.1 describes the max. and min. propagation delay contribution which depends on the location of the UE within the beam. As a result, the round-trip-delay can differ by 64ms which corresponds 32ms for one-way transmission. It is proposed to consider the 280ms as the max. transmission delay and consequently 248ms (280ms – 32ms) as the minimal transmission time. This assumes no retransmission over the GEO satellite link.

Table 5.1.2.4-1	Transmission delay GEO satellite

|  | Minimum delay in ms | Maximum delay in ms |
| --- | --- | --- |
| GEO transmission delay | 248 | 280 |
| Note: Transmission delay ground station to core network counted in 5.1.2.3-1. |  |  |

#### 5.2.2.5	ULBC Delay components

Table 5.1.2.2-1 lists the algorithmic delay for the IMS codecs AMR and EVS, i.e. in range of 5ms to 12ms. For ULBC, different delay values may result from codec processing delays as well as algorithmic delays. Exact numbers are for further study.

### 5.1.3	Estimation of Mouth-to-ear delay

Given the values in 5.1.2 the mouth-to-ear delay for scenario can be estimated for the two scenarios outlined in 5.1.1 by summing up the delay components according to the signal flow to derive a lower (minimum values as in tables 5.1.2.2-1, 5.1.2.3-1, 5.1.2.4-1) and an upper bound (maximum values as in tables 5.1.2.2-1, 5.1.2.3-1, 5.1.2.4-1).

As the bitrate for GEO satellite link is very restricted, options for minimizing the protocol overhead need to be considered. One option to reduce the protocol overhead are larger frame sizes or a larger voice bundling period or frame aggregation as the protocol stack is transmitted less often. Therefore, the following table outlines the delay values for codec frame sizes of 20ms, 40ms, 80ms, 160ms and 320ms and the bundling periods of 80ms, 160ms and 320ms.

Editor’s Note: Current values assume algorithmic delay of AMR and EVS as given in 5.1.2.2-1. ULBC Delay components documented in 5.1.2 still need to be addressed. For the min. Delay_GSCN, 20ms is assumed.

Table 5.1.3-1	 Mouth-to-ear delay estimation depending on codec frame size

| Voice bundling period | Codec frame size | Mouth to ear delay main scenario in ms (GEO - TN) (Note 1) |  | Mouth to ear delay sub-scenario in ms (GEO - GEO) (Note 2) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | lower bound | upper bound | lower bound | upper bound |
| 80 | 20 | 548 + X | 872 + X | 804 + X | 1315 + X |
|  | 40 |  | 912 + X |  | 1355 + X |
|  | 80 |  | 992 + X |  | 1435 + X |
| 160 | 20 | 708 + X | 1032 + X | 964 + X | 1475 + X |
|  | 40 |  | 1072 + X |  | 1515 + X |
|  | 80 |  | 1152 + X |  | 1595 + X |
|  | 160 |  | 1312 + X |  | 1755 + X |
| 320 | 20 | 1028 + X | 1352 + X | 1284 + X | 1795 + X |
|  | 40 |  | 1392 + X |  | 1835 + X |
|  | 80 |  | 1472 + X |  | 1915 + X |
|  | 160 |  | 1632 + X |  | 2075 + X |
|  | 320 |  | 1952 + X |  | 2395 + X |
| Note 1: UE delay+GEO transmission+Delay_GSCN+Delay_eNBCN + Solution specific delay X Note 2: UE delay +2x GEO transmission+2x Delay_GSCN + Solution specific delay X |  |  |  |  |  |

Editor’s note: The scenarios and the terminology of this clause needs to be aligned with clause 4.) “Application Scenario” where a detailed description of the call scenarios is expected.

* * * End of Changes * * * *

References

[1] SP-250378 “New SID on Ultra Low Bitrate Speech Codec”

[2] Draft TR 26.940: Study on Ultra Low Bit rate Speech Codecs, v. 0.2.0

[3] Tdoc S4-251304, “[FS_ULBC] On bundling time determination”, Huawei Technologies Co., Ltd.

[4] Tdoc S4-251382, “Revision of GEO voice service delay analysis” Fraunhofer IIS

[5] ITU-T Recommendation P.114: “One-way transmission time  ”

[6] Tdoc S4aA250239, “[FS_ULBC] Packet Loss Concealment with existing AI based codec DAC”, Dolby Laboratories Inc., Ericsson LM