3GPP SA4 WG4 SA4-e (AH) post 134	S4aI260017r01

Adhoc telco 29th January 2026	revision of S4aI250209

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0014 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Solution for KI5 Client-based Media Application Server selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addtion of a solution to Key Issue 5 on Media Application Server Energy management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a clause 7.13 adding a solution to Key Issue #5 on Media Application Server Energy management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KI#5 will not be solved. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.1, 7.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

## 2	References

…

[36300]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[28558]	3GPP TS 28.558: “Management and orchestration; UE level measurements for 5G system”

[23288]	3GPP TS 23.288: “Architecture enhancements for 5G System (5GS) to support network data analytics services”

Key Issue mapping

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solutions | Solution title |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #1 |  |  |  |  | X |  |  |  |
| #2 |  |  |  |  | X |  |  |  |
| #3 |  |  |  | X |  |  |  |  |
| #4 |  |  | X |  |  |  |  |  |
| #5 |  |  | X |  |  |  |  |  |
| #6 |  |  | X | X |  |  |  |  |
| #7 |  |  | X |  |  |  |  |  |
| #8 |  |  |  | X |  |  |  |  |
| #9 |  |  | X |  |  |  |  |  |
| #11(b) | Client-based Media AS service endpoint reselection based on QoE reporting by the QMC framework and asynchronous notification to the Media Client |  |  |  |  |  | X | X |

These solutions are simply candidate solutions. Their inclusion in the following clauses does not imply that they have been agreed upon or endorsed. Any decisions and work to be done for the normative work will be detailed in the conclusions of this Technical Report.

Candidate Solution
(All new text)

## 7.13	Solution #11(b): Client-based Media AS service endpoint reselection based on QoE reporting by the QMC framework and asynchronous notification to the Media Client

### 7.13.1	Key Issue mapping

This solution candidate addresses Key Issue #5 (Media Application Server Energy management) described in clause 6.5 as well as Key Issue #6 (Client-driven management of media delivery service energy optimisation) described in clause 6.6.

### 7.13.2	Functional description

This Candidate Solution proposes a method that allows re-selection of Media AS service locations during a media delivery session when content is delivered between the UE Media Client and the Media AS via reference point M4, based on the UE application QoE requirements (e.g., resolution, bit rate, etc.) while also taking into account the various characteristics (e.g. Media AS load, Media AS energy consumption, etc.) of individual service locations of the Media AS that are being used to support media delivery.

The solution makes use of the QMC-based QoE reporting framework summarised in clause 	4.2.2.2 and is based on option 1 from Solution #6 proposed in clause 7.7.3.1.2. As stated under clause 4.2.2.2 of the present document, OAM acts as a MnS producer of standardized services such as fault supervision, performance management, trace reporting, etc. For example, data collected by OAM (KPIs, performance metrics, energy data, etc.) can be exposed to authorized consumers using the SBMA framework defined in TS 28.533 and the generic services in TS 28.532. in. This means that an Application Function (AF) could be configured/authorized by the operator or network domain to consume certain OAM management services (e.g., performance KPIs, fault reports, analytics, etc.) through the SBMA interfaces, assuming the OAM’s MnS producer exposes them to authorized consumers. The use of RAN-based QoE metrics in this context does not imply the definition of new OAM measurements, management interfaces, or exposure mechanisms. Existing OAM capabilities and information as specified in 3GPP SA2 and SA5 are assumed.

This solution leverages interfaces existing reference points in the Media Delivery System that support communication between a Media Application Provider and the 5G System, particularly for exchanging Quality of Experience (QoE) data. The interfaces at these reference points are enhanced by the solution with network energy-related characteristics, allowing the Media Client to use this information for Media AS service location reselection.

### 7.13.3	Collaboration scenarios

This solution is applicable to all collaboration scenarios requiring access to energy-related information from a UE and a Media AS.

### 7.13.4	Architecture mapping

The architecture shown in figure 7.13.4-1 represents the case of a UE client accessing two different Media AS service locations. The Energy Information AF instantiated in the Media AFand the Energy Information Collector instantiated in the Media Session Handler of the UE Media Client correspond to the architecture nstantiation in clause 7.6.2.4.

Figure 7.13.4-1: Illustration of Media AS service location reselection based on energy characteristics

Media delivery is provisioned by the Media Application Provider creating a Provisioning Session and other needed resources (e.g. Content Hosting Configuration for downlink media streaming) in the Media AF. The Media Application Provider provisions the Media AF via reference point M1, including an Energy Information exposure configuration intended for the Energy Information AF instantiated in the Media AF.

The Media Application Provider configures Service Access Information for use by the Media-aware Application via reference point M8. The Service Access Information delivered to the Media-aware Application includes information about different Media AS service locations

During the course of a media delivery session, the UE Media Client switches to a service location based on the QoE requirements of the UE application in combination with energy usage/characteristics of the available Media AS service locations. For example, imagine a scenario in which the Media Client is accessing media content at service location #1. The Energy Information AF instantiated in the Media AF now suggests that the Media Client switches to service location of #2 based on: (a) the energy consumption of the available service locations and (b) the QoE requirements of the application. One way to tackle this by via QMC based method detailed in clause 4.2.2.2 of the present document. The RAN-based QoE metrics derived from OAM systems as specified by SA5 (e.g. “as specified in TS 28.558” [28558]), and service experience information as defined in SA2 (e.g. “as specified in TS 23.288 [23.288]”, could be made available to the Energy Information Application Function. The Energy Information AF may take these QoE metrics into account when evaluating energy consumption and energy efficiency in the context of ongoing media services. In particular, the Energy Information AF correlates QoE information with service-level and media-level energy information in order to determine whether energy consumption is proportional to the experienced QoE. This process is explicitly detailed in clause 7.13.6. of the present document. Based on this evaluation, the Energy Information AF may derive energy-related information that reflects both energy efficiency and QoE conditions. This information may then be conveyed via the E5 interface to a UE-based Energy Information Collector. The conveyed information may include recommendations that consider the combined impact of QoE, and energy consumption. In addition, the Energy Information AF instantiated in the Media AF subscribes to Application Server (AS) Energy Report when providing energy-related information in the context of media services.

### 7.13.5	Energy-related information

#### 7.13.5.1	Energy-related provisioning information

Editor’s Note: Define the parameters that need to be provisioned in the Energy Information AF via M1.

The Media Application Provider may provision energy-related parameters to the Energy Information Application Function (Energy Information AF) via the M1 interface. These parameters follow common M1 provisioning patterns used in SA4 specifications, and enable energy-aware behavior and interaction with other media-related application functions. The list provided is non-exhaustive.

| Abstract information element |  | Semantics / constraints |  |
| --- | --- | --- | --- |
| EnergyPolicyId |  | Identifies the energy policy instance provisioned via M1. The identifier may be referenced by other M1-provisioned entities or by internal AF interactions. |  |
| EnergyMonitoringConfiguration | Specifies the scope and granularity of energy monitoring (e.g. per service, per session, per media component). |  |  |
| EnergyReportingConfiguration | Specifies reporting conditions for energy-related information, including reporting frequency or threshold-based triggers. |  |  |
| EnergyThreshold | Specifies threshold values related to energy consumption or energy efficiency. Crossing a threshold may trigger internal evaluation or notifications. |  |  |
| EnergyAdaptationGuidance | Specifies recommended energy-aware adaptation actions that may be considered when energy thresholds are crossed or when QoS degradation occurs. |  |  |
| EnergySuspensionPolicy | Specifies conditions under which media components or media sessions may be suspended in order to reduce energy consumption. |  |  |
| EnergyRecoveryPolicy | Specifies conditions under which increased energy consumption is permitted, e.g. to restore higher media quality after constrained operation. |  |  |
| EnergyReportingRecipient | Identifies entities to which energy-related reports or notifications are provided (e.g. Media Application Provider, Media AF). |  |  |
| ServiceEnergyProfile | Identifies the energy efficiency profile associated with a service or application. The Energy Information AF may apply differentiated behavior based on this profile. |  |  |

NOTE: The Energy Information AF may use the energy-related parameters provisioned via M1 to evaluate energy consumption and efficiency in the context of ongoing media services. When energy-related conditions are met (e.g. energy thresholds are crossed or energy budgets are exhausted), the Energy Information AF may provide energy-related indications or recommendations to the Media AF. The Media AF may combine this information with QoS-related input (e.g. QoS degradation notifications from the 5G Core) when selecting media adaptation actions.

#### 7.13.5.2	Client Energy reporting information

Editor’s Note: Define the parameters that are passed by Energy Information AF to the Energy Information Collector via E5.

The Energy Information Application Function may convey energy-related information to the Energy Information Collector via the E5 interface in order to support energy monitoring, analysis, and aggregation across media services. The parameters listed below are provided as examples. The list provided is non-exhaustive.

| Abstract information element |  | Semantics / constraints |  |
| --- | --- | --- | --- |
| EnergyPolicyId |  | Identifies the energy policy under which the reported information was generated. This identifier enables correlation with M1-provisioned energy policies. |  |
| ServiceIdentifier |  | Identifies the service, application, or media session group to which the reported energy information applies. |  |
| EnergyMeasurementScope |  | Indicates the scope of the reported energy information (e.g. per service, per session, per media component). |  |
| EnergyMeasurementTime |  | Indicates the time or time interval over which the reported energy information was measured or estimated. |  |
| EnergyThresholdStatus |  | Indicates whether configured energy thresholds have been crossed. If applicable, the direction of crossing may be indicated. |  |
| EnergyBudgetStatus |  | Indicates the current status of the configured energy consumption budget (e.g. within budget, approaching limit, exceeded). |  |
| EnergyAdaptationStatus |  | Indicates whether energy-aware adaptation actions are currently applied by the Energy Information AF or related media functions. |  |
| EnergyReportingReason |  | Indicates the reason for reporting (e.g. periodic reporting, threshold crossing, policy change). |  |
| EnergyReportValidityTime |  | Indicates the time period during which the reported information is considered valid. |  |
| EnergyInformationTimestamp |  | Provides the timestamp at which the energy information was generated by the Energy Information AF. |  |

NOTE: The Energy Information Collector may use the information received via E5 for aggregation, long-term analysis, correlation with QoS or service information, or for providing feedback to the Media Application Provider. The Collector does not directly influence media adaptation decisions, which remain under the control of the Media Application Function.

### 7.13.6	Procedures

Figure 7.13.6-1 below details the different steps for application service endpoint re-selection process.

Figure 7.13.6-1: Procedures for Media AS service location re-selection based on energy characteristics

Assumption: The Media Application Provider may deliver Service Access Information through reference point M8. The Service Access Information delivered to the media aware Application may have information about different Service Operation Points accessible through each of the DNs.

In the above call flow, steps 1 to 8 areto provision Energy Information Collection:

.

1. The 5GMS Application Provider provisions the 5GMS AF via reference point M1, including a Provisioning Session resource, any other required resources and an Energy Information exposure configuration intended for the Energy Information AF instantiated in the 5GMS AF. The Energy Information exposure configuration may identify application(s) that fall within its scope.

At some later point, a media delivery session is initiated:

2.	The 5GMS-Aware Application initiates a new media delivery session with the Media Session Handler via reference point M6, including a request to enable energy-related information collection and reporting.

3.	The Media Session Handler obtains Service Access Information from the 5GMS AF, including access details of the Energy Information AF.

4.	As a consequence of the previous step, the Media Session Handler creates a new energy-related information collection and reporting context with the Energy Information Collector instantiated in it.

5.	The Energy Information Collector subscribes to Network Energy Information reporting from Energy Information AF via reference point E5, if relevant, and receives in response a UE Energy Information collection configuration.

6.	The Energy Information Collector shares the UE Energy Information collection configuration with the Media Session Handler.

7. The Energy Information Collector subscribes to Network Energy Information reporting from Energy Information AF via reference point E5, if relevant, and receives in response a UE Energy Information collection configuration.

8. The EIAF configures AS Energy Information report via reference point E3. The AS Energy Information report may include energy consumption information of different granularities, e.g. UE, PDU Session and/or QoS Flow, as described in clause 5.51.2.3 of TS 23.501 [72].

9.  The 5GMS AS may submit an AS Energy Information report to the Energy Information AF via reference point E3 using the callback endpoint.

Energy information configuration: The Media Session Handler creates a context in its Energy Information Collector (step 4) and the Energy Information Collector obtains a UE Energy Information collection configuration from the Energy Information AF (step 5).

Initial Energy Information subscription and reporting: Steps 6 to 10 are not optional in this solution. In step 6, the Energy Information Collector subscribes to receive energy-related information from the Energy Information AF. Step 7 is omitted because this Candidate Solution does not involve the EIF. In steps 8 and 9, the Energy Information AF configures the Media AS to provide periodic AS Energy Reports and receives an initial AS Energy Report immediately.

10.	Based on the information in the AS Energy Report, the Energy Information AF and the enclosing Media AF now internally processes the received energy-related information about the Media AS service locations.

Application Layer QoE measurement configuration: Additional steps 10a to 10d involve collecting application layer QoE metrics following the procedure defined in clause 21.2.1 of TS 38.300 [38300]. The feature is activated in the gNodeB either by direct configuration from the OAM system (management-based activation), or by signalling from the OAM via the 5GC (signalling-based activation). The present solution the above call flows is by direct configuration from the OAM system (management-based activation). For management-based QMC activation, the OAM sends one or more QoE measurement configurations directly to the gNodeB. The QoE measurement configuration for management-based QMC activation also includes an application layer measurement configuration list and the corresponding information for QoE measurement collection.

10a.	The OAM sends a QoE configuration request to the RAN/gNode B including an application layer measurement configuration.

10b. The gNodeB forwards this application layer measurement configuration to the UE as measConfig‌App‌Layer‌Container in the RRCReconfiguration message as specified in in clause 21.2.1 of TS 38.300.XYZ [38.300].

10c.	The Media Session Handler subscribes to receive QoE metrics from the Media Access Function at reference point M11 and the Media Access Function returns an initial set of QoE metrics.

10d.	Application layer measurement reports received from UE's application layer are encapsulated in a transparent container and sent to the network in the MeasurementReport‌AppLayer message, as specified in TS 38.331 [38331]. The UE can send multiple application layer measurement reports to the gNodeB in one such message. The UE may additionally be configured by the gNodeB to indicate to the gNodeB when a QoE measurement session starts or stops for a certain application layer measurement configuration.

10e. The gNodeB forwards this message to the OAM.

10f. Upon the Media AF optionally subscribing to the OAM to receive such information (if desired) the OAM forwards the UE Application Layer QoE measurement to the Media AF.

NOTE:	According to TS SS.XYZ [SSXYZ], the OAM may forward this information to any MnS Consumer (e.g. an Application Function).

10g.	The Energy Information AF and Media AF map the UEs with the Media AS service locations, based on the BaseURL of the service location of the Media AS as well as based on the reported UE-related metrics and the information in the Media AS Energy Report about service locations.

11.	The Energy Information AF sends initial configuration information to the UE Media Client (Energy Information Collector of the Media session Handler), with the list of newly available Media AS service location endpoints, per clause 5.19.3.3 of the present document. This is sent as metadata to the UE client, via M5 interface via Service Access Information which may include Media Player Entry URLs and BaseURL of the new Media AS service location endpoint as per clause of TS 26.501 [23].

Steps 12, 13 and 14 are not required by this solution.

In step 15, the Media Entry Points are returned to the Media-aware Application.

The choice of a Media Entry Point by the Media-aware Application in step 16 in this solution is based on the list of newly available Media AS service location endpoints, per clause 5.19.3.3 of the present document. This is sent as metadata to the Media-aware Application by the Energy Information Collector of the Media session Handler which may include Media Player Entry URLs and BaseURL of the new Media AS service location endpoint as per clause of TS 26.501 [23].

In steps 17 to 19, media delivery is started and the Media Entry Point is retrieved from one of the available Media AS service locations ("SL1", for example).

Reporting Application Server related metrics.

30. The 5GMS AS may expose an AS Energy Report to the Energy Information AF via reference point E3. This AS Energy Report includes energy consumption information of different granularities, including Media AS service locations.

31.	 Based on the information in the Media AS Energy Report, the Energy Information AF and the enclosing Media AF now internally processes the received energy-related information about the Media AS service locations.

Steps 20 to 27 are not required by this solution.

28.	Media is delivered between the Media Access Function and service location "SL1" of the Media AS in step 28.

31a. Meanwhile, the Media-aware application could start publishing application layer measurement reports to the network in the MeasurementReport‌AppLayer message, as specified in TS 38.331 [38331].

31b. The gNodeB forwards this message to the OAM.

31c. Upon the Media AF optionally subscribing to the OAM to receive such information (if desired) the OAM forwards the UE Application Layer QoE measurement to the Media AF.

31d. The Energy Information AF and Media AF map the UEs with the Media AS service locations, based on the BaseURL of the service location of the Media AS as well as based on the reported UE-related metrics and the information in the Media AS Energy Report about service locations.

Media streaming session adaptation based on energy information:

28.	Media is delivered between the Media Access Function and service location "SL1" of the Media AS in step 28.

In steps 19 to 25 the Media AS service location is reselected based on the reported energy-related characteristics.

32.	The Energy Information AF sends new configuration information to the UE Media Client (Energy Information Collector of the Media session Handler), with the list of newly available Media AS service location endpoints, per clause 5.19.3.3 of the present document. This is sent as metadata to the UE client, via M5 interface via Service Access Information which may include Media Player Entry URLs and BaseURL of the new Media AS service location endpoint as per clause of TS 26.501 [23].

35.

35a.	The Media Session Handler selects a new Media AS service location for its current session without modifying the on-going session.

35b.	The Media Session Handler requests the Media Access Function to switch to new Media AS service location.

35c.	A new media delivery transport session is established between the Media Access Function and the "SL2" service location of the Media AS.

35d.	The Media Access Function notifies the Media Session Handler at reference point M11 of the successful establishment of the new media delivery transport session in the previous step.

35e:	The Media Session Handler relays this notification to its Energy Information Collector via an internal API.

35f. This results in the Energy Information Collector sending a notification to the Energy Information AF about the modification of Media AS service location endpoint.

35g.		Media is delivered to/from the new Media AS service location "SL2".

The exposure of energy information to the Media-aware Application (step 36) and subsequently to the Media Application Provider (step 37) are omitted from this solution.

### 7.13.7	Gap analysis

The following gaps in Release 19 normative specifications are highlighted by the high-level procedure proposed in clause 7.13.6:

- In reference to step 2, the ability for the Media-aware Application to indicate a user preference for the current energy mode to the Media Access Function (via reference point M7/M11) and to the Media Session Handler (via reference point M6/M11) when initiating a new media delivery session.

- In reference to Step 10/10g, the ability for the Energy Information AF and Media AF map the UEs with the Media AS service locations, based on the BaseURL of the service location of the Media AS as well as based on the reported UE-related metrics and the information in the Media AS Energy Report about service locations

3.	In reference to step 11 and 12, the ability for the Energy Information AF to send initial configuration information to the UE Media Client (Energy Information Collector of the Media session Handler), with the list of newly available Media AS service location endpoints and fom EIC to MSH.

4.	In reference to step 15, the ability for the Media Session Handler to provide Energy Information related to each Media Entry Point when those are transmitted to the Media-aware Application at reference point M6.

5.	In reference to step 16, the ability for the Media-aware Application to select a Media Entry point according to energy-related characteristics.

6.  In reference to step 35, the ability for the Media Stream Handler to select a Service Operation Point based on its current configuration provided to it via the Media Session Handler.

### 7.13.8	Proposed normative changes

The potential normative work required by this Candidate involves the Media-Aware Application and the Media Access Function to adapt the media delivery session based on energy-related information received to respect the energy mode configured by the use.  In this regard the potential identified changes are the below:

1.	New clauses in TS 26.501 [23] defining:

a.	The instantiation of the generic architecture defined in point 1 above in the 5G Media Streaming System, based on clause 7.6.2.3 of the present document.

b.	Collaboration scenarios extending the generic procedures defined in point 1 above for particular Use Cases relevant to 5G Media Streaming.

2.	New clauses in TS 26.506 [59] defining:

a.	The instantiation of the generic architecture defined in (1) above in the RTC System.

b.	Collaboration scenarios extending the generic procedures defined in point 1 above for particular Use Cases relevant to RTC.

3.	New clauses in TS 26.510 [26510] specifying:

a.	Extensions (as needed) to the procedures and service-based interfaces at reference point M5, in particular with regard to additional energy-related information provided in the Service Access Information response.

4.	New clauses in TS 26.512 [26512] specifying:

a.	Extensions (as needed) to the procedures and service-based interfaces at reference points E1, E3 and E5 that pertain to the 5G Media Streaming System, in particular with regard to additional energy-related information provided by the 5GMS AS in relation to media streaming sessions.

b.	Extensions (as needed) to the client API exposed by the Energy Information Collector instantiated in the Media Session Handler to the 5GMS-Aware Application at reference point M6 that pertain to the 5GMS System.

c.	Provisioning the Energy Information AF instantiated in the 5GMS AF via reference point M1.

### 7.13.9	Summary

This Candidate Solution describes how energy-related information about different service locations could be exposed by the Media AS to the Media AF, which then could expose this information to the UE application during a media delivery session. This solution leverages the Energy Information AF with its associated reference points, allowing the Media AS  Energy Information to be delivered to the UE application. This Candidate Solution also proposes a method for dynamically reselecting Media AS service locations during an ongoing media session over reference point M4, based on UE application QoE requirements and energy-realted characteristics reported by the Media AS service locations. It introduces a mechanism for enabling the Media Client to dynamically select the most energy-efficient Media AS service location. This is done based on the interaction between the Energy Information AF and Media Client that jointly make this selection based on energy-related information received from the network and from the UE. The proposed methods are applicable to downlink media sessions, but also for uplink media sessions and RTC communication sessions. This is based on the following factors:

1.	UE application and characterisation data: Information pertaining to the User Equipment (UE), factors such as, current QoE, and application-specific needs (e.g., resolution, bit rate).

2.	Energy efficiency metrics from Network Operator: Energy-related characteristics of the Media AS whose service locations are deployed in the Data Network. These metrics, including server power consumption profiles and energy performance indicators, are transmitted from individual Media AS deployment instances to the Energy and Infrastructure Application Function (EIAF) within the Media Application Function (Media AF).

This approach aims to optimise the energy consumption of multimedia content delivery, particularly video streaming services, while maintaining service quality within defined thresholds by enabling client-based Media Application Server selection based on asynchronous notification to the Media Client as specified in TS 26.512.

The candidate solution operates entirely within the network layer, which involves the UE, QoE characteristics as the steering and optimisation mechanisms are fully supported by standard adaptive streaming frameworks. This ensures compatibility with existing adaptive streaming ecosystems, preserving end-user experience while reducing the environmental footprint of content delivery.

End of changes