3GPP SA4 WG4 meeting #134	S4aI250209 r01

Dallas, Texas, 17-21 November 2025

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0014 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Solution for KI5 Client-based Media Application Server selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addtion of a solution to Key Issue 5 on Media Application Server Energy management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a clause 7.13 adding a solution to Key Issue #5 on Media Application Server Energy management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KI#5 will not be solved. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.1, 7.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

## 2	References

…

[26941]	3GPP TS 26.941: "Network Slicing Extensions for 5G media services".

[36300]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

Key Issue mapping

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solutions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #1 |  |  | X |  |  |  |
| #2 |  |  | X |  |  |  |
| #3 |  | X |  |  |  |  |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  |  |  |  |
| #6 | X | X |  |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 | X |  |  |  |  |  |
| #11(b) |  |  |  |  | X | X |

These solutions are simply candidate solutions. Their inclusion in the following clauses does not imply that they have been agreed upon or endorsed. Any decisions and work to be done for the normative work will be detailed in the conclusions of this Technical Report.

Candidate Solution
(All new text)

## 7.13	Solution #11(b): Client-based Media AS service endpoint reselection based on asynchronous notification to the Media Client

### 7.13.1	Key Issue mapping

This solution candidate addresses Key Issue #5 (Media Application Server Energy management) described in clause 6.5 as well as Key Issue #6 (Client-driven management of media delivery service energy optimisation) described in clause 6.6.

### 7.13.2	Functional description

#### 7.13.2.1	Introduction

This Candidate Solution proposes a method that allows re-selection of Media AS service locations during a media delivery session when content is delivered between the UE Media Client and the Media AS via reference point M4, based on the UE application QoE requirements (e.g., resolution, bit rate, etc.) while also taking into account the various characteristics (e.g. Media AS load, Media AS energy consumption, current Media AS QoE metrics, etc.) of individual service locations of the Media AS that are being used to support media delivery.

This solution leverages interfaces existing reference points in the Media Delivery System that support communication between a Media Application Provider and the 5G System, particularly for exchanging Quality of Experience (QoE) data. The interfaces at these reference points are enhanced by the solution with network energy-related characteristics, allowing the Media Application Provider to use this information for service location reselection. In particular the solution leverages extensions for edge processing defined for the 5GMS System in clause 4.5 of TS 26.501 [23] and for the RTC System in clause 6 of TS 26.506 [59] in which instances of the Media AS are deployed as Edge Application Server (EAS) instances and they offer different service locations to the Media Client.

#### 7.13.2.2	Architecture mapping

The architecture shown in figure 7.13.2.2-1 represents the case of a UE client accessing two different Media AS service locations. The Energy Information AF instantiated in the Media AFand the Energy Information Collector instantiated in the Media Session Handler of the UE Media Client  correspond to the architecture nstantiation in clause 7.6.2.4.

Figure 7.13.2.2-1: Illustration of Media AS service location reselection based on energy characteristics

Media delivery is provisioned by the Media Application Provider creating a Provisioning Session and other needed resources (e.g. Content Hosting Configuration for downlink media streaming) in the Media AF. The Media Application Provider then negotiates with the MNO to provision a network slice as described in clause 4.3 of TR 26.941 [26941]. The 5GMS Application Provider provisions the Media AF via reference point M1, including an Energy Information exposure configuration intended for the Energy Information AF instantiated in the Media AF.

The Media Application Provider configures Service Access Information for use by the Media-aware Application via reference point M8. The Service Access Information delivered to the Media-aware Application includes information about different service locations accessible through each of the DNs..

During the course of a media delivery session, the Media Client switches to a service location based on the QoE requirements of the UE in combination with energy usage/characteristics of the available Media AS service locations. For example, imagine a scenario in which the Media Client is accessing media content at service location AS#1. Can the Energy Information AF instantiated in the Media AF suggest that the UE Media Client now switches to a different service location ofAS#2 based on: (a) the energy consumption of the available service locations and (b) the QoE requirements of the application?

### 7.13.3	Procedures

Figure 7.12.3-2 below details the different steps for application service endpoint re-selection process.

Figure 7.13.3-1: Procedures for Media AS service location re-selection based on energy characteristics

Assumption: The Application Provider may deliver Service Access Information through reference point M8. The Service Access Information delivered to the media aware Application may have information about different Service Operation Points accessible through each of the DNs. For example, an enterprise may utilise an enterprise-specific service location of the Media AS (e.g., hosting enterprise-related video tutorials). 5G Media Streaming sessions for enterprise-related video tutorials use the PDU Session terminating in the enterprise-specific service location of the Media AS, while some other video tutorial requests are sent through PDU Sessions terminating in a different service location of the Media AS.

In the below call flow, steps 1 to 8 are retained from the baseline procedure for downlink media streaming found in clause 5.9 of TS 26.501 [26501], which is common steps followed to establish any media downlink session establishment. However, as stated earlier, the proposed solution is applicable for uplink, downlink and RTC architectures as well.

These steps follow the usual setup while establishing a downlink media delivery session between the UE and the 5GMS AS. However, they are adapted for the generalised Media Delivery architecture The Media-aware Application requests a media delivery session, following which a media delivery session is established, and the media is transferred between the Media Client and the Media Application Provider via the Media AS. The media delivery session starts at step 8. Based on step 1 to 8, an on-going media delivery session is established.

9.	The Energy Information AF subscribes to EIF to be informed and to receive energy-related information from the different NFs.

Steps 10-15: Reporting UE-related and apps-related QoE metrics.The goal is to report the Media Player QoE metrics to the OAM, based on the already existing state of the art QoE metrics reporting techniques, for instance as described in clause 23.16 of TS 36.300 [36300].

10.	The OAM periodically requests the UE to report its current QoE. In order to do this, the OAM first reaches the 5G Core entities, such as PCF and SMF via N5 and N7 interfaces respectively.

11.	The PCF forwards this request to the Media AF.

12. The Media AF requests the latest QoE metrics of the Media Player of the current session from the Media Session Handler.

13.	The  MSH in return forwards this to Media AF (after querying the current QoE information from Media Access Function).

1.	The AF now forwards this information to the OAM. This way, the UE periodically reports the QoE metrics upon request from the OAM.

NOTE:	This is based on the QMC framework, as described in clause 23.16 of TS 36.300 [36300], where UE provides application relevant metrics, limited to a certain duration of X ms, in order to protect the user privacy.

15.	The NWDAF retrieves this information from OAM for further analysis.This is optionally done in order to perform further network analytics.

Steps 16-20: Reporting Application Server related metrics.

16.

16.	EIAF collects (or retrieves) the energy-related information of each Media AS service location from the Media AS.

17. Energy-related information of each Media AS service location is reported to the EIAF.	EIAF reports this energy-related information to the EIF.

18.	Based on this information, the Energy Information AF and the enclosing Media AF now internally processes the received energy-related information about the Media AS service locations.

19.	The Energy Information AF and Media AF map the UEs with the Media AS service locations, based on the BaseURL of the service location of the Media AS as well as based on the reported UE-related metrics and the service location of Media AS energy-related information.

In steps 20 to 26 the Media AS service location is reselected based on the reported energy-related characteristics.

20.	The Energy Information AF sends the new configuration information to the UE client (EIC of the Media session Handler), with the list of newly available Media AS service location endpoints, per clause 5.19.3.3 of the present document. This is sent as metadata to the UE client, via M5 interface via Service Access Information which may include Media Player Entry URLs and BaseURL of the new Media AS service location endpoint as per clause of TS 26.501.

22.	The Media Session Handler selects a new Media AS service location for its current session without modifying the on-going session.

23.This results in the Media Session Handler sending a notification to the Media AF about the modification of Media AS service location endpoint.

24.	The Media Session Handler requests the Media Access Function to switch to new Media AS service location.

25.	A new media delivery transport session is established with the "EAS2" service location of the Media AS.

26.	The Media Access Function notifies the Media Session Handler of the successful establishment of the new media delivery transport session in the previous step.

27.	Media delivery session is established between Media Session handler and Media Access Function.	Media delivery starts via the new Media AS service location "Service Location 2 (SL2)".

### 7.13.4	Summary

This Candidate Solution introduces a mechanism for enabling to dynamically select the most energy-efficient Media AS service location. This is done based on the interaction between the Energy Information AF and Media Client that jointly make this selection based on energy-related information received from the network and from the UE. The proposed methods are applicable to downlink media sessions, but also for uplink media sessions and RTC communication sessions. This is based on the following factors:

1.	UE application and characterisation data: Information pertaining to the User Equipment (UE), factors such as, current QoE, and application-specific needs (e.g., resolution, bit rate).

2.	Energy efficiency metrics from Network Operator: Energy-related characteristics of the Media AS whose service locations are deployed in different Edge Data Networks. These metrics, including server power consumption profiles and energy performance indicators, are transmitted from individual Media AS service locations to the Energy and Infrastructure Application Function (EIAF) within the Media Application Function (Media AF).

This approach aims to optimise the energy consumption of multimedia content delivery, particularly video streaming services, while maintaining service quality within defined thresholds by enabling client-based Media Application Server selection based on asynchronous notification to the Media Client as specified in TS 26.512.

The candidate solution operates entirely within the network layer, which involves the UE, QoE characteristics as the steering and optimisation mechanisms are fully supported by standard adaptive streaming frameworks. This ensures compatibility with existing adaptive streaming ecosystems, preserving end-user experience while reducing the environmental footprint of content delivery.

End of changes