Source: 	Nokia Corporation

Title: 	Proposed changes to draft CR - Overlays

Document for	Agreement

Agenda item: 	4.5

Introduction

The document proposes changes related to the requirements and functionality of the overlay feature to draft CR as proposed in Tdoc S4-201538. The changes are marked with revision marks, which use S4-201538 as the original text.

Changes to draft CR

Annex X (normative):
Immersive Teleconferencing and Telepresence for Remote Terminals (ITT4RT)

# X.1	General

The MTSI terminal may support the Immersive Teleconferencing and Telepresence for Remote Terminals (ITT4RT) feature as defined in this clause. MTSI clients supporting the ITT4RT feature shall be referred to as ITT4RT clients.

ITT4RT functionality for MTSI enables support of an immersive experience for remote terminals joining teleconferencing and telepresence sessions. It addresses scenarios with two-way audio and one-way immersive 360-degree video, e.g., a remote single user wearing an HMD participating in a conference will send audio and optionally 2D video (e.g., of a presentation, screen sharing and/or a capture of the user itself), but receives stereo or immersive voice/audio and immersive 360-degree video captured by an omnidirectional camera in a conference room connected to a fixed network.

Since immersive 360-degree video support for ITT4RT is unidirectional, ITT4RT clients supporting immersive 360-degree video are further classified into two types to distinguish between the capabilities for sending or receiving immersive video: (i) ITT4RT-Tx client, which is an ITT4RT client only capable of sending immersive 360-degree video, and (ii) ITT4RT-Rx client, which is an ITT4RT client only capable of receiving immersive 360-degree video. Such a classification does not apply for ITT4RT clients supporting immersive speech/audio, since the support for immersive speech/audio is expected to be bi-directional. It should also be noted that a terminal containing ITT4RT-Tx or ITT4RT-Rx client capabilities may also contain further MTSI client capabilities to support bi-directional 2D video.

MTSI gateways supporting ITT4RT functionality are referred to as ITT4RT MRF, which is an ITT4RT client implemented by functionality included in the ITT4RT MRF. An ITT4RT MRF supporting immersive 360-degree video contains both ITT4RT-Tx and ITT4RT-Rx clients.

# X.2	Architecture and Interfaces

Definitions, reference and coordinate systems, video signal representation and audio signal representation as described in clause 4.1 of TS 26.118 [R2] are applicable.

Figure X.1 provides a possible sender architecture that produces the RTP streams containing 360-degree video and immersive speech/audio as applicable to an ITT4RT client in terminal. VR content acquisition includes capture of 360-degree video and immersive speech/audio, as well as other relevant content such as overlays. Following VR content pre-processing and encoding of 360-degree video and immersive speech/audio components, the corresponding elementary streams are generated. For 360-degree projected video, pre-processing may include video stitching, rotation or other translations, and the pre-processed 360-degree video is then passed into the projection functionality in order to map 360-degree video into 2D textures using a mathematically specified projection format. Optionally, the resulting projected video may be further mapped region-wise onto a packed video. For 360-degree fisheye video, circular videos captured by fisheye lenses are not stitched, but directly mapped onto a 2D texture, without the use of the projection and region-wise packing functionalities (as described in clause 4.3 of ISO/IEC 23090-2 [R1]). In this case, pre-processing may include arranging the circular images captured by fisheye lenses onto 2D textures, and the functionality for projection and mapping is not needed. For audio, no stitching process is needed, since the captured signals are inherently immersive and omnidirectional. Followed by the HEVC/AVC encoding of the 2D textures and EVS encoding of immersive speech/audio along with the relevant immersive media metadata (e.g., SEI messages), the consequent video and audio elementary streams are encapsulated into respective RTP streams and transmitted.

RTP stream for 360-Degree Video

Video pre-processing (e.g., stitiching, rotation, etc.)

Elementary stream (ES)

EVS Encoder

Acquisition

Video ES (including metadata)

RTP Encapsulation

Audio pre-processing

RTP Encapsulation

HEVC or AVC Encoder

RTP stream for Immersive Speech/Audio

Projection & Mapping

Audio ES

Figure X.1 - Reference sender architecture for ITT4RT client in terminal

Figure X.2 provides an overview of a possible receiver architecture that reconstructs the 360-degree video and immersive speech/audio in an ITT4RT client in terminal. Note that this figure does not represent an actual implementation, but a logical set of receiver functions. Based on one or more received RTP media streams, the UE parses, possibly decrypts and feeds the elementary video stream into the HEVC/AVC decoder and the speech/audio stream into the EVS decoder. The HEVC/AVC decoder obtains the decoder output signal, referred to as the "2D texture", as well as the decoder metadata. Likewise the EVS decoder output signal contains the immersive speech/audio. The decoder metadata for video contains the Supplemental Enhancement Information (SEI) messages, i.e., information carried in the omnidirectional video specific SEI messages, to be used in the rendering phase. In particular, the decoder metadata may be used by the Texture-to-Sphere Mapping function to generate a 360-degree video (or part thereof) based on the decoded output signal, i.e., the texture. The viewport is then generated from the 360-degree video signal (or part thereof) by taking into account the pose information from sensors, display characteristics as well as possibly other metadata.

For 360-degree video, the following components are applicable:

-	The RTP stream contains an HEVC or an AVC bitstream with omnidirectional video specific SEI messages. In particular, the omnidirectional video specific SEI messages as defined in ISO/IEC 23008-2 [119] and ISO/IEC 14496-10 [24] may be present.

-	The video elementary stream(s) areencoded following the requirements in clause X.3

Audio ES

RTP streams

RTP Receiver

Video ES (with metadata)

Decoder Metadata

HEVC or AVC Decoder

Texture to Sphere Mapping

Decoder output signal (texture)

Viewport Rendering for Immersive Video

EVS Decoder

Decoder output signal

Immersive Voice/audio Rendering

360-Degree Video

Figure X.2 - Reference receiver architecture for ITT4RT- client in terminal

The output signal, i.e., the decoded picture or "texture", is then rendered using the Decoder Metadata information in relevant SEI messages contained in the video elementary streams as well as the relevant information signalled at the RTP/RTCP level (in the viewport-dependent case). The Decoder Metadata is used when performing rendering operations such as region-wise unpacking, projection de-mapping and rotation for 360-degree projected video, or fisheye video information for 360-degree fisheye video) toward creating spherical content for each eye.  Details of such sample location remapping process operations are described in clause D.3.41.7 of ISO/IEC 23008-2 [119].

<Editor’s Note: Produce another reference client architetecture for the VDP case with new arrows for RTCP etc.>

Viewport-dependent 360-degree video processing could be supported for both point-to-point conversational sessions and multiparty conferencing scenarios and can be achieved by sending from the ITT4RT-Rx client RTCP feedback  messages with  viewport information and then encoding and sending the corresponding viewport by the ITT4RT-Tx client or by the ITT4RT-MRF. This is expected to deliver resolutions higher than the viewport independent approach for the desired viewport. The transmitted RTP stream from the ITT4RT-Tx client or ITT4RT-MRF may also include the information on the region of the 360-degree video encoded in higher quality in an RTP header extension message as the video generated, encoded and streamed by the ITT4RT-Tx client may cover a larger area than the desired viewport. Viewport-dependent processing is  realized via RTP/RTCP based protocols that are supported by ITT4RT clients. The use of RTP/RTCP based protocols for viewport-dependent processing is further described in clause X.6.2.

# X.3	Immersive 360-Degree Video Support

ITT4RT-Rx clients in terminals offering video communication shall support decoding capabilities based on:

-	H.264 (AVC) [24] Constrained High Profile, Level 5.1 with the following additional restrictions and requirements on the bitstream:

-    the maximum VCL Bit Rate is constrained to be 120 Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively.

-    the bitstream does not contain more than 10 slices per picture.

-	H.265 (HEVC) [119] Main 10 Profile, Main Tier, Level 5.1.

In addition, ITT4RT-Rx clients in terminals may support:

-	H.265 (HEVC) [R4] Screen-Extended Main 10 Profile, Main Tier, Level 5.1.

-	H.265 (HEVC) [R4] Screen-Extended Main 4:4:4 10 Profile, Main Tier, Level 5.1.

ITT4RT-Tx clients in terminals offering video communication shall support encoding up to the maximum capabilities (e.g., color bit-depth, luma samples per second, luma picture size, frames per second) compatible with decoders compliant with the following on the bitstream:

-	H.264 (AVC) [24] Constrained High Profile, Level 5.1 with the following additional restrictions and requirements:

-    the maximum VCL Bit Rate is constrained to be 120 Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively.

-    the bitstream does not contain more than 10 slices per picture.

-	H.265 (HEVC) [119] Main 10 Profile, Main Tier, Level 5.1.

In addition, ITT4RT-Tx clients in terminals may support:

-	H.265 (HEVC) [R4] Screen-Extended Main 10 Profile, Main Tier, Level 5.1.

-	H.265 (HEVC) [R4] Screen-Extended Main 4:4:4 10 Profile, Main Tier, Level 5.1.

Hence, for a Bitstream conforming to the H.264 (AVC) [24] Constrained High Profile, Level 5.1 delivered from an ITT4RT-Tx client to the ITT4RT-Rx client, the following restrictions apply:

-	The profile_idc shall be set to 100 indicating the High profile.

-	The constraint_set0_flag, constraint_set1_flag, constraint_set2_flag and constraint_set3_flag shall all be set to 0, and constraint_set4_flag and constraint_set5_flag shall be set to 1.

-	The value of level_idc shall not be greater than 51 (corresponding to the level 5.1) and should indicate the lowest level to which the Bitstream conforms.

Furthermore, for a Bitstream conforming to the H.265 (HEVC) [119] Main 10 Profile, Main Tier, Level 5.1 delivered from an ITT4RT-Tx client to the ITT4RT-Rx client, the following restrictions apply:

-	The general_profile_idc shall be set to 2 indicating the Main10 profile.

-	The general_tier_flag shall be set to 0 indicating the Main tier.

-	The value of level_idc shall not be greater than 153 (corresponding to the Level 5.1) and should indicate the lowest level to which the Bitstream conforms.

For 360-degree video delivery across ITT4RT clients, the following components are applicable:

-	The RTP stream shall contain an HEVC or an AVC bitstream with possible presence of omnidirectional video specific SEI messages. In particular, the omnidirectional video specific SEI messages as defined in clause D.2.41 of ISO/IEC 23008-2 [119] or ISO/IEC 14496-10 [24] may be present for the respective HEVC or AVC bitstreams.

-	The video elementary stream(s) shall be encoded following the requirements in the Omnidirectional Media Format (OMAF) specification ISO/IEC 23090-2 [R1], clauses 10.1.2.2 (viewport-independent case) or 10.1.3.2 (viewport-dependent case) for HEVC bitstreams and clause 10.1.4.2 for AVC bitstreams. Furthermore, the general video codec requirements for AVC and HEVC in clause 5.2.2 of TS 26.114 also apply.

ITT4RT-Rx clients are expected to be able to process the VR metadata carried in SEI messages for rendering 360-degree video according to the relevant processes. Relevant SEI messages contained in the elementary stream(s) with decoder rendering metadata may include the following information for the relevant processes as per clause D.3.41 of ISO/IEC 23008-2 [119] and ISO/IEC 14496-10 [24]:

-	Projection mapping information (indicating the projection format in use, e.g., Equirectangular projection (ERP) or Cubemap projection (CMP)), for the projection sample location remapping process as specified in clauses 7.5.1.3 and 5.2 of ISO/IEC 23090-2 [R1]

-	Region-wise packing information (carrying region-wise packing format indication, any coverage restrictions or padding/guard region information in ithe packed picture), for the inverse processes of the region-wise packing as specified in clauses 7.5.1.2 and 5.4 of ISO/IEC 23090-2 [R1]

-	Sphere rotation information (indicating the amount of sphere rotation, if any, applied to the sphere signal before projection and region-wise packing at the encoder side), for the coordinate axes conversion process as specified in clause 5.3 of ISO/IEC 23090-2 [R1]

-	Frame packing arrangement (indicating the frame packing format for stereoscopic content), for the processes as specified in D.3.16 of ISO/IEC 23008-2 [119]

-	Fisheye video information (indicating that the picture is a fisheye video picture containing a number of active areas captured by fisheye camera lens), for the fisheye sample location remapping process as specified in clause D.3.41.7.5 of ISO/IEC 23008-2 [119]

The exchange of SEI messages carrying VR metadata for rendering 360-degree video or fisheye video shall be performed using one of the following types of signalling:

1-  Bitstream-level: SEI messages shall be present in the respective video elementary streams corresponding to the HEVC or AVC bitstreams carrying 360-degree video or fisheye video from the ITT4RT-Tx client to the ITT4RT-Rx client, as per ISO/IEC 23008-2 [119] or ISO/IEC 14496-10 [24]. As expressed more clearly below, the mandatory inclusion of the specific SEI messages in the bitstream by the ITT4RT-Tx client and their decoder and rendering processing by the ITT4RT-Rx client is conditional upon successful SDP-based negotiation of the corresponding 360-degree video or fisheye video capabilities.

2-  SDP-level: SEI messages shall be signalled in the SDP using the ‘sprop-sei’ parameter based on the procedures specified in IETF RFC 7798 [120] or via other SDP-based means, in the corresponding SDP offer or answer from the ITT4RT-Tx client to the ITTRT-Rx client, during the session setup involving media negotiations for 360-degree video or fisheye video. It should be noted that the signalling based on the ‘sprop-sei’ parameter is only available for the HEVC-based RTP payload formats and is not supported in AVC-based RTP payload formats as defined in IETF RFC 6184 [25].

Editor’s Note: The details of which one of the above signalling types shall be used for specific SEI messages is FFS. The frequency of the signalling of each SEI message is FFS.

In particular, using one of the above two signalling types (bitstream-level or SDP-level), the ITT4RT-Tx client supporting 360-degree video for viewport-independent processing shall signal the equirectangular projection SEI message (payloadType equal to 150) to the ITT4RT-Rx client, with the erp_guard_band_flag set to 0.

If viewport-dependent processing (VDP) capability is successfully negotiated by the ITT4RT-Tx client and ITT4RT-Rx client for the exchange of 360-degree video, then, using one of the above two signalling types (bitstream-level or SDP-level), the ITT4RT-Tx client shall signal to the ITT4RT-Rx client either:

-	the equirectangular projection SEI message (payloadType equal to 150) with the erp_guard_band_flag set to 0, or

-	the cubemap projection SEI message (payloadType equal to 151).

In order to optimize the spatial resolution of specific viewports, the ITT4RT-Tx client and ITT4RT-Rx client may negotiate the use of region-wise packing as part of the exchange of 360-degree video. If this is the case, the region-wise packing SEI message (payloadType equal to 155) shall also be signalled by the ITT4RT-Tx client to the ITT4RT-Rx client using one of the above two signalling types (bitstream-level or SDP-level).

If stereoscopic video capability is successfully negotiated by the ITT4RT-Tx client and ITT4RT-Rx client as part of the exchange of 360-degree video, then the frame packing arrangement SEI message (payloadType equal to 45) shall also be signalled by the ITT4RT-Tx client to the ITT4RT-Rx client using one of the above two signalling types (bitstream-level or SDP-level), with the following restrictions:

-	The value of frame_packing_arrangement_cancel_flag is equal to 0.

-	The value of frame_packing_arrangement_type is equal to 4.

-	The value of quincunx_sampling_flag is equal to 0.

-	The value of spatial_flipping_flag is equal to 0.

-	The value of field_views_flag is equal to 0.

-	The value of frame0_grid_position_x is equal to 0.

-	The value of frame0_grid_position_y is equal to 0.

-	The value of frame1_grid_position_x is equal to 0.

-	The value of frame1_grid_position_y is equal to 0.

Furthermore, ITT4RT-Tx clients supporting 360-degree fisheye video shall signal the fisheye video information SEI message (payloadType equal to 152) to the ITT4RT-Rx clients, using one of the above two signalling types (bitstream-level or SDP-level).

# X.4	Immersive Voice/Audio Support

TBD

# X.5	Overlay Support

An overlay is defined as a piece of visual media, rendered over omnidirectional video or on the viewport. An ITT4RT-Tx client supporting the ‘Overlay’ feature shall add real-time overlays on top of a 360-degree background and offer this capability in the SDP as part of the the initial offer-answer negotiation. An overlay in ITT4RT is characterized by the overlay source and its rendering configuration. ITT4RT supports both 2D and spherical overlays.

An overlay source specifies the image or video to be used as the content for the overlay. The overlay source is a bitstream that may be delivered as

an overlay video encoded and delivered as an RTP stream.

ITT4RT clients supporting the ‘Overlay’ feature have the following requirements:

- an ITT4RT-Tx client may be capable to provide a bitstream consisting of an overlay with a video source that conforms to the requirements of X.3 or the video bitstream requirements of MTSI.

- an ITT4RT-Rx client may be capable to decode and render a 360-degree video bitstream and further decode and render one or more overlay streams on top of the 360-degree video.

The rendering configuration defines how the overlay is to be rendered in the spherical scene. It is possible for an overlay to be delivered without any rendering configuration, in which case, the ITT4RT-Rx client should render the content as per the default configuration described in section X.6.4.3. Alternatively, the rendering configuration for one or more overlays may be negotiated during session establishment to ensure uniform operation for different receivers.

Editor’s note: The audio/image aspect is FFS.

# X.6	Media configuration

## X.6.1	General

Based on the architecture described in clause X.2, an SDP framework for immersive video and immersive voice/audio exchange for ITT4RT is presented to negotiate codec support, SEI messages for decoder rendering metadata, as well as RTP/RTCP signaling necessary for viewport dependent processing.

The SDP attributes 3gpp_360video, 3gpp_overlay, 3gpp_360bg shall be used to indicate respectively a 360-degree video stream, a spherical overlay and a 360-degree background image (or series of images or video). The detailed definition and usage of these SDP attributes are presented in the clauses below.

## X.6.2	Main 360-degree video

A new SDP attribute 3gpp_360video is defined with the following ABNF syntax:

3gpp_360video = "a=3gpp_video:" [SP "VDP" SP "Stereo" SP Projection SP FOV SP PPM SP FOV_CENTER SP viewport_ctrl]

Editor’s Note: The above ABNF is only a placeholder and should be revised to indicate the agreements from the text below. A basic set of minimum requirements for interop should be clearly defined. Furthermore, if the SDP negotiations become too complex, defining profiles can be considered.

The semantics of the above attribute and parameters is provided below.  Unsupported parameters of the 3gpp_360video attribute may be ignored.

An ITT4RT client supporting the 3gpp_360video attribute using viewport-independent processing or stereoscopic video for video shall support the following procedures:

- when sending an SDP offer, the ITT4RT client includes the 3gpp_360video attribute in the media description for video in the SDP offer

- when sending an SDP answer, the ITT4RT client includes the 3gpp_360video attribute in the media description for video in the SDP answer if the 3gpp_360video attribute was received in an SDP offer

- after successful negotiation of the 3gpp_360video attribute in the SDP, for the video streams based on the HEVC or AVC codec, the ITT4RT clients exchange an RTP-based video stream containing an HEVC or AVC bitstream [with omnidirectional video specific SEI messages] as defined in clause X.3

An ITT4RT client  supporting the 3gpp_360video attribute supporting use of viewport-dependent processing (VDP) shall include the VDP parameter in the SDP offer and answer. Depending on the value indicated by the VDP parameter, the ITT4RT client shall further support the following procedures:

- the RTCP feedback (FB) message described in clause X.6.2 of type ‘Viewport’ to carry requested viewport information during the RTP streaming of media (signalled from the ITT4RT-Rx client to the ITT4RT-Tx client).

An ITT4RT client shall not include VDP parameter in the SDP answer if the SDP offer contains the 3gpp_360video attribute without the VDP parameter.

[Editor’s Note: Additional viewport-dependent processing capabilities from the PD may be added later, including signaling of desired ROI, viewport margin and actually transmitted viewport. ]

[An ITT4RT client in terminal supporting the 3gpp_360video attribute with stereoscopic video support shall include the Stereo parameter and shall further support frame packing arrangement SEI message as in ISO/IEC 23090-2 [119] for HEVC and ISO/IEC 14496-10 [24] – further details TBD. ]

X.5.2.1 Projection

An ITT4RT- client supporting the 3gpp_360video attribute with VDP supporting projection may include the Projection parameter indicating the types of projection (e.g. ERP, CMP) it prefers (in the order of preference) in the SDP.  An ITT4RT client may respond to an SDP offer with multiple options indicated in the Projection parameter with the agreed option. An ITT4RT-Tx client is not required to provide the preferred form of projection indicated by an ITT4RT-Rx client but may do so when possible.

The ABNF syntax is defined as follows:

<insert ABNF>

X.5.2.2 FOV

An ITT4RT-Tx client may support sending a limited 360-degree video.

  - An ITT4RT-Tx client supporting the 3gpp_360video attribute capable of sending a limited 360-degree video shall include the parameter FOV in its SDP offer to indicate the cfov (Capture FoV) as the extent (range) of the 360-degree video with respect to the unit sphere. The range is expressed in unit of degrees with an x parameter for azimuth range and a y parameter for elevation range, sent as a comma-seperated tuple. In the absence of cfov, the default value of x and y are 360 and 180 degrees, respectively.

  - An ITT4RT-Rx client supporting the 3gpp_360video attribute capable that wants to receive a limited 360-degree video shall include the parameter FOV in its SDP offer/answer to indicate the pfov (Preferred FoV), where pfov <= cfov in one or both the x and y dimensions when cfov is known. The pfov range is expressed in unit of degrees with an x parameter for azimuth range and a y parameter for elevation range, sent as a comma-seperated tuple. In the absence of pfov, it should be assumed to be the same as cfov.

  - An ITT4RT-Tx client that has received an SDP offer from an ITT4RT-Rx client with the parameter FOV shall include in its SDP answer the parameter FOV to indicate the range of the 360-degree video it will provide. The value is the same as the FOV in the SDP offer or different based on the ITT4RT-Tx client capabilities.

An ITT4RT client supporting the 3gpp_360video attribute with the FOV parameter may include the paramater FOV_CENTER in the SDP. FOV_CENTER is expressed as a comma-separated tuple <x,y>, where x is the azimuth (in degrees) and y is the elevation (in degrees) with respect to the global coordinates such that the range defined FOV bypasses through the coordinates defined by FOV_CENTER.

The imageattr attribute indicates the resolution of the delivered content based on the cfov and pfov options.

a=imageattr:100 send [x=1920,y=1080]

The full ABNF syntax is defined as follows:

<insert ABNF>

X.5.2.3 Picture Packing

An ITT4RT-Tx client may support sending the 360-degree video using tiled encoding, e.g., HEVC motion-constrained tiling. An ITT4RT-Tx client using the tiled encoding and supporting the 3gpp_360video attribute shall include in its SDP offer the parameter PPM.

- A PPM value of 1 indicates mixed-quality tiled encoding is used

- A PPM value of 2 indicates mixed-resolution tiled encoding is used

An ITT4RT-Tx client sending low-quality background 360-degree video frame-packed with a high-quality viewport (possibly with margins) shall include in its SDP offer the parameter PPM.In this case the PPM is an ordered comma-seperated list of the following six fields:

- PPWHQ defines packed_picture_width of the high-quality region in pixels

- PPHHQ defines packed_picture_height of the high-quality region in pixels

- TRHQ defines transform operations applied on the high-quality region.

- PPWLQ defines packed_picture_width of the low-quality region in pixels

- PPHLQ defines packed_picture_height of the low-quality region in pixels

- TRLQ defines transform operations applied on the low-quality region

The transform operations have a value of 0-7 where the values correspond to the following operations:

| Transform value | Transform operation |
| --- | --- |
| 0 | no transorm |
| 1 | mirrored horizontally |
| 2 | rotation by 180 degrees (counter-clockwise) |
| 3 | rotation by 180 degrees (counter-clockwise) before mirroring horizontally |
| 4 | rotation by 90 degrees (counter-clockwise) before mirroring horizontally |
| 5 | rotation by 90 degrees (counter-clockwise) |
| 6 | rotation by 270 degrees (counter-clockwise) before mirroring horizontally |
| 7 | rotation by 270 degrees (counter-clockwise) |

Further details on viewport-dependent processing options and details on PPM can be found in the informational section X.8.

The full ABNF syntax is defined as follows:

PPM = <PPWHQ>,<PPHHQ>,<TRHQ>,<PPWLQ>,<PPHLQ>,<TRLQ> Note: The size of the viewport and fov attributes define the size of projected regions. It should be considered if the two values should be included explicitly here

X.5.2.4 Viewport Control

An ITT4RT client that supports the 3gpp_360video with VDP shall in its SDP offer include the parameter viewport-ctrl with one or more of the following control options:

- device_controlled if it will provide VDP based on the viewport received in the viewport information RTCP feedback from the corresponding ITT4RT-Rx client.

- recommended_viewport if it will provide VDP with the help of a recommendation/prediction engine.

- viewport-ctrl with the value presenter_viewport if it can provide VDP based on the viewport of an ITT4RT-Rx client other than the one the SDP offer is being sent to.

Multiple options are provided as a comma-separated list. An ITT4RT client that receives an SDP offer with multiple viewport-ctrl options may include its preferred viewport-ctrl option in the SDP answer. If no options are given in the answer, the sender shall use the first option in the list.

The ABNF syntax is defined as follows:

<insert ABNF>

X.5.2.5 Overlay and 360-degree video

An ITT4RT-Tx client that sends an SDP offer with more than one 360-degree video/audio and at least one overlay [shall] include in SDP the attribute itt4rt_group before any media lines. The itt4rt_group attribute is used to group 360 media and overlays using the mid attribute as defined according to the ABNF below:

a = itt4rt_group: <mid1> SP <mid2> SP <mid3> …

The mid attribute with the appropriate value as defined in the other parts of the SDP shall be included in the media description for the relevant 360 audio/video and overlay when the itt4rt_group attribute is used. The order of the media included in the itt4rt_group indicates the synchronization source with the first media always being the synchronization anchor when synchronization is required.

<Additional aspects from the PD to be incorporated>

[

- Offering 360-degree video in multiple versions

During a VR conference call, a 360-degree video sender may offer an overlay and a 2D version of the ominidirectional video for receivers that are not viewing the content immersively. For example, in the case of multiple 360-degree sender, receivers in of sender A for instance can receive the stream for sender B as an overlay. The SDP offer is similar to the way MTSI offers multiple encoding options, so each receiver would choose only one of these proposed formats.

## X.6.3	Still Background

- Background area

A background stream may be offered for a 360-degree video as a separate stream using the parameter 3gpp_360bg. For now, the ITT4RT use cases only propose using a still background image. However, the same parameter can be used for a background video.

Note: Additional information may be needed to define the background stream that is FFS,  e.g., sphere-locking, transparency.

]

## X.6.4	Overlays

ITT4RT clients supporting the ‘Overlay’ feature may define an overlay source and overlay configuration in the SDP. An ITT4RT client that supports overlays

- shall support a video stream indicated by a media line in the SDP as the source of an overlay.

- may support a conditional overlay of a region from a 360-degree video as the source of an overlay.

- [may support an externally specified overlay source, referenced by a URI.]

### X.6.4.1 Video Media

### Any visual media that is defined with the ‘m=video’ line, includes the attribute id and does not have the attribute a=3gpp_360video in the SDP may be rendered as an overlay by an ITT4RT-Rx client. An ITT4RT client shall include the attribute mid in the overlay media description. If an overlay is to be associated with a particular overlay configuration, the mid shall be used to associate the overlay media description to the the overlay configuration, which is later described in X.6.4.3.

### X.6.4.2 Conditional overlay

A conditional overlay is defined as an overlay that is activated (i.e., transmitted by the ITT4RT-Tx client) as an overlay media stream when a certain condition is met, and deactivated (i.e., not transmitted) when the condition is no longer met. ITT4RT clients may use a conditional overlay for displaying a region of interest of the 360-degree video when the viewport is not overlapping with that region of interest. This capability may be used, for example, for extending the FoV in devices when two areas of interest do not fit within the FoV of the device.

An ITT4RT client supporting conditional overlay shall include under the media description for the 360-degree video, the attribute 3gpp_conditional_overlay with the following ABNF syntax:

3gpp-conditional-overlay = “a = 3gpp_conditional_overlay: “ SP 1*(conditional_overlay_id)

The conditional_overlay_id is a 32-bit unsigned integer expressed in decimal that is unique for each conditional overlay for the entire session and shall not have the same value as any other mid in the session; the number of conditional_overlay_id fields depends on the number of conditional overlays that are supported with a unique id for each.

If conditional overlays are successfully negotiated, the ITT4RT-Rx client may enable a conditional overlay during a session by sending the conditional overlay RTCP feedback message with a conditional_overlay_id, condition and region as described in section X.7.2.

### X.6.4.3 Overlay Configuration

## Editor’s note: There should be a default configuration for overlay when explicit configuration is not provided to ensure that multiple receivers have similar experience.

[

ISO/IEC 23090-2 defines an overlay as a piece of visual media rendered over omnidirectional video or image item or over a viewport.

The SDP signaling semantics in Section 9.6 currently defined as a potential solution for predefined region signaling will have the following applicability:

- Allow adding real-time overlay on the top of a pre-defined region. Client devices may use the pre-defined regions as hints for personalized overlay operations. Compared to handling overlays for all audience on the server side, a client device may utilize its own computing power to generate overlays on pre-defined regions.

- Content to be overlaid on the predefined region may be encoded and transported separately with higher quality.

An MTSI sender supporting the ‘Overlay’ feature can allow adding real-time overlay on top of a 360 background and offer this capability in the SDP as part of the the initial offer-answer negotiation. Regions for overlays can be offered by including the "a=overlay" attribute under the relevant media line corresponding to the related 360-degree video and overlay images. The following parameters can be provided in the attribute for each overlay:

Editor’s Note: The parameters below are aligned with “Sphere-relative two-dimensional overlay” specification in OMAF.

- Overlay_ID – identifies the offered  region for overlay

- Overlay_azimuth: Specifies the azimuth angle of the centre of the offered  overlay region on the unit sphere in units of 2−16 degrees relative to the global coordinate axes.

- Overlay_elevation: Specifies the elevation angle of the centre of the offered  overlay region on the unit sphere in units of 2−16 degrees relative to the global coordinate axes.

- Overlay_tilt: Specifies the tilt angle of the offered overlay region, in units of 2−16 degrees, relative to the global coordinate axes.

- Overlay_azimuth_range: Specifies the azimuth range of the offered region corresponding to the 2D plane on which the overlay is rendered through the centre point of the overlay region in units of 2−16 degrees.

- Overlay_elevation_range: Specifies the elevation range of the offered region corresponding to to the 2D plane on which the overlay is rendered  through the centre point of the overlay region in units of 2−16 degrees.

- Overlay_rot_yaw, Overlay_rot_pitch, and Overlay_rot_roll specify the rotation of the 2D plane on which the overlay is rendered. Prior to rendering the 2D plane, it may be rotated as specified by overlay_rot_yaw, overlay_rot_pitch and overlay_rot_yaw and placed on a certain distance as specified by region_depth_minus1. The rotations are relative to the coordinate system as specified in clause 5.1 of ISO/IEC 23090-2 in which the origin of the coordinate system is in the centre of the overlay region, the X axis is towards the origin of the global coordinate axes, the Y axis is towards the point on the plane that corresponds to cAzimuth1 in Figure 7-4 of ISO/IEC 23090-2, and the Z axis is towards the point on the plane that corresponds to cElevation2 in Figure 7-4 of ISO/IEC 23090-2. overlay_rot_yaw expresses a rotation around the Z axis, overlay_rot_pitch rotates around the Y axis, and overlay_rot_roll rotates around the X axis. Rotations are extrinsic, i.e., around X, Y, and Z fixed reference axes. The angles increase clockwise when looking from the origin towards the positive end of an axis. The rotations are applied starting from overlay_rot_yaw, followed by overlay_rot_pitch, and ending with overlay_rot_roll.

- region_depth_minus1 - indicates the depth (z-value) of the region on which the overlay is to be rendered. The depth value is the norm of the normal vector of the overlay region. region_depth_minus1 + 1 specifies the depth value relative to a unit sphere in units of 2−16.

- timeline_change_flag equal to 1 specifies that the overlay content playback shall pause if the overlay is not in the user's current viewport, and when the overlay is back in the user's viewport the overlay content playback shall resume with the global presentation timeline of the content. The content in the intermediate interval is skipped. timeline_change_flag equal to 0 specifies that the overlay content playback shall pause if the overlay is not in the user's current viewport, and when the overlay is back in the user's viewport the overlay content playback resumes from the paused sample. This prevents loss of any content due to the overlay being away from the user's current viewport.

- Name- specifies the name of the offered  region for overlay.

Editor’s Note: The parameters below are aligned with “Viewport-relative overlay” specification in OMAF.

- Overlay_ID – identifies the offered region for overlay

- Overlay_rect_left_percent: Specifies the x-coordinate of the top-left corner of the rectangular region of the overlay to be rendered on the viewport in per cents relative to the width and height of the viewport. The values are indicated in units of 2-16 in the range of 0 (indicating 0%), inclusive, up to but excluding 65536 (that indicates 100%).

- Overlay_rect_top_percent: Specifies the y-coordinate of the top-left corner of the rectangular region of the overlay to be rendered on the viewport in per cents relative to the width and height of the viewport. The values are indicated in units of 2-16 in the range of 0 (indicating 0%), inclusive, up to but excluding 65536 (that indicates 100%).

- Overlay_rect_width_percent: Specifies the width of the top-left corner of the rectangular region of the overlay to be rendered on the viewport in per cents relative to the width and height of the viewport. The values are indicated in units of 2-16 in the range of 0 (indicating 0%), inclusive, up to but excluding 65536 (that indicates 100%).

- Overlay_rect_height_percent: Specifies the height of the top-left corner of the rectangular region of the overlay to be rendered on the viewport in per cents relative to the width and height of the viewport. The values are indicated in units of 2-16 in the range of 0 (indicating 0%), inclusive, up to but excluding 65536 (that indicates 100%).

NOTE:	The size of overlay region over the viewport changes according to the viewport resolution and aspect ratio. However, the aspect ratio of the overlaid media is not intended to be changed.

- Relative_disparity_flag indicates whether the disparity is provided as a percentage value of the width of the display window for one view (when the value is equal to 1) or as a number of pixels (when the value is equal to 0). This applies for the case when there is a monoscopic overlay.

- Disparity_in_percent: Specifies the disparity, in units of 2−16, as a fraction of the width of the display window for one view. The value may be negative, in which case the displacement direction is reversed. This value is used to displace the region to the left on the left eye view and to the right on the right eye view. This applies for the case when there is a monoscopic overlay and stereoscopic background visual media.

- Disparity_in_pixels indicates the disparity in pixels. The value may be negative, in which case the displacement direction is reversed. This value is used to displace the region to the left on the left eye view and to the right on the right eye view. This applies for the case when there is a monoscopic overlay and stereoscopic background visual media.

- Name- specifies the name of the offered region for overlay.

Editor’s Note: Other overlay definitions in OMAF are not excluded from ITT4RT. Which overlay definition(s) from OMAF are adopted for overlays in ITT4RT is currently TBD.

Editor’s Note: For both of the above overlay types from OMAF, incorporate from the spec further details about the order of operations for overlay rendering (in particular order of translation and rotation).

As controls for user interaction with the overlays, the a=overlay" signalling can optionally include the following additional parameters:

- change_position_flag, when set to 1, specifies that users are allowed to move the overlay window to any location on the viewing sphere or the viewport.

- change_depth_flag, when set to 1, specifies that the depth of overlay can be chosen by user interaction. When both change_position_flag and change_depth_flag are set to 1 then the X,Y,Z position of the overlay can be freely choosen by user interaction.

- switch_on_off_flag, when set to 1, specifies that the user is allowed to switch ON/OFF the overlay.

- change_opacity_flag, when set to 1, specifies that the user is allowed to change the opacity of the overlay.

- resize_flag, when set to 1, specifies that the user is allowed to resize the overlay window. The field-of-view of the resized overlay window shall be same as that of original overlay window.

- rotation_flag, when set to 1, specifies that the user is allowed to rotate the overlay window to different directions. The field-of-view of the rotated overlay window shall be same as that of original overlay window.

- change_position_flag, change_depth_flag, switch_on_off_flag, change_opacity_flag, resize_flag, or rotation_flag when set to 0, specifies that the user is disallowed to perform the respective operation on the overlay.

Editor’s Note: Default behaviour when the control parameters are not signalled should be explained. One interpretation is that if not signalled renderer is completely free to perform the above operations.

Editor’s Note: Consider the possibility of packing all the flags into a single field with different bit values corresponding to the flags above.

For the negotiated 360-degree video, a=overlay" attribute is expected to contain all of the above parameters for overlays under the m= line for “Viewport-relative overlay” or “Sphere-relative two-dimensional overlay” (controls for user interactivity are optional), each overlay image is expected to only contain the corresponding Overlay_ID parameter as part of a=overlay" attribute.

In response to the SDP offer with the set of offered regions provided using the "a=overlay" line(s), an MTSI client accepting ‘Overlay’ can provide an SDP answer using the "a=overlay" line(s) containing the accepted set of regions. The accepted set of  regions for overlays would be a subset of the offered set of overlay regions.

A new SDP offer-answer negotiation can be performed to modify the set of regions for overlays. The MTSI sender may update all the content of overlay regions, including the total number of overlay regions, and the spherical coordinates and name of each of the overlay regions.

- Overlays definition

The following aspects are considered for SDP signalling when sending an overlay:

- A media level attribute a=3gpp_overlay indicates the presence of an overlay. The attribute further defines aspects of the overlay including flags and position.

- flags: set of eight flags related to overlay positioning, moveability, on/off behavior and resizing is included for overlay media streams.

- position: position of the overlay on the spherical region if it is to be defined. The fields for position are defined in section 6.3 of PD. Depending on the values within position, a spherical and 2D overlay can be differentiated.

| Bit Position | Flag and Value |
| --- | --- |
| 0 | 0 = Viewport Relative, 1=Sphere relative |
| 1 | Change_position_flag |
| 2 | Switch_on_off_flag |
| 3 | Resize_flag |
| 4 | Rotation_flag |
| 5 | Opacity_flag |
| 6 | Timeline_change_flag |
| 7 | Reserved |

## X.6.5	Fisheye Video

### X.6.5.1	Identifying the 360-degree fisheye video stream

The SDP attribute 3gpp_fisheye is used to indicate a 360-degree fisheye video stream.

ITT4RT clients supporting 360-degree fisheye video shall support the 3gpp_fisheye attribute and shall support the following procedures:

- when sending an SDP offer, the ITT4RT-Tx client includes the 3gpp_fisheye attribute in the media description for video in the SDP offer

- when sending an SDP answer, the ITT4RT-Rx client includes the 3gpp_fisheye attribute in the media description for video in the SDP answer if the 3gpp_fisheye attribute was received in an SDP offer

- after successful negotiation of the 3gpp_fisheye attribute in the SDP, for the video streams based on the HEVC codec, the MTSI clients exchange an RTP-based video stream containing an HEVC or AVC bitstream with fisheye omnidirectional video specific SEI messages as defined in clause X.3

ITT4RT-Tx clients that support both 360-degree projected video and 360-degree fisheye video may include both 3gpp_360video and 3gpp_fisheye attributes as alternatives in the SDP offer, but an ITT4RT-Rx client shall include only one attribute (either 3gpp_360video or 3gpp_fisheye, based on support or selection) in the SDP answer.

### X.6.5.2	360-degree fisheye video SDP attribute parameters

Media-line level parameters are defined in order to aid session establishment between the ITT4RT-Tx and ITT4RT-Rx clients for 360-degree fisheye video, as well as to describe the fisheye video stream as identified by the 3gpp_fisheye attribute.

- Total number of fisheye circular videos at the capturing terminal

Depending on the camera configuration of the sending terminal, the 360-degree fisheye video may be comprised of multiple different fisheye circular videos, each captured through a different fisheye lens.

- <fisheye>: this parameter indicates the total number of fisheye circular videos output by the camera configuration at the terminal

- Fisheye circular video static parameters

In order to enable the quick selection of desired fisheye circular videos by the ITT4RT-Rx client during SDP negotiation, the following static parameters are defined for each fisheye circular video.  These parameters are defined from the video bitstream fisheye video information SEI  message as defined in ISO/IEC 23008-2 [119] and ISO/IEC 23090-2 [R1].

- <fisheye-img-X> = [<id-X> <azi> <ele> <til> <fov>]

  - <id>: an identifier for the fisheye video

  - <azi>, <ele>: azimuth and elevation indicating the spherical coordinates that correspond to the centre of the circular region that contains the fisheye video, in units of 2-16 degrees.  The values for azimuth shall be in the range of −180 * 216 (i.e., −11 796 480) to 180 * 216 − 1 (i.e., 11 796 479), inclusive, and the values for elevation shall be in the range of −90 * 216 (i.e., −5 898 240) to 90 * 216 (i.e., 5 898 240), inclusive

  - <til>: tilt indicating the tilt angle of the sphere regions that corresponds to the fisheye video, in units of 2−16 degrees. The values for tilt shall be in the range of −180 * 216 (i.e., −11 796 480) to 180 * 216 − 1 (i.e., 11 796 479), inclusive

  - <fov>: specifies the field of view of the lens that corresponds to the fisheye video in the coded picture, in units of 2−16 degrees.  The field of view shall be in the range of 0 to 360 * 216 (i.e., 23 592 960), inclusive

- Stream packing of fisheye circular videos

Depending on the terminal device capabilities and bandwidth availability, the packing of fisheye circular videos within the stream can be negotiated between the sending and receiving terminals.

- <maxpack>: this parameter indicates the maximum supported number of fisheye videos which can be packed into the stream by the terminal device

An example SDP offer is shown in table X.A.

Table X.A: Example SDP offer with 360-degree fisheye video attribute parameters

SDP offer

m=video 49154 RTP/AVP 99 a=tcap:1 RTP/AVPF a=pcfg:1 t=1 b=AS:10000 b=RS:0 b=RR:2500 a=rtpmap:99 H265/90000 a=fmtp:99 profile-id=1; level-id=93; a=imageattr:99 send [x=1920,y=1080] [x=1280,y=720] a=3gpp_fisheye:99 fisheye=4 [id=1 azi ele til fov] [id=2 azi ele til fov] [id=3 azi ele til fov] [id=4 azi ele til fov]; maxpack=2; a=sendonly

As an example, a receiving terminal which only receives 360-degree fisheye video (and possibly sends a 2D video to the sender) replies with an SDP answer containing only the selected fisheye videos equal to the number as selected by the value of maxpack in the corresponding m-line, which is set to recvonly.

### X.6.5.3	Viewport dependent delivery of fisheye video

By exposing the coverage information of each fisheye circular video using the parameters in section X.4.5.2, the collective multitude of which makes up the whole 360-degree video, a ITT4RT-Rx client can opt to select only the required fisheye circular videos needed to render the current viewport of the user.

Through the parameters defined in section X.4.5.2, a ITT4RT-Rx client can select the desired fisheye packing configuration of the video stream during SDP negotiation, as well as the initial desired fisheye videos using the id parameter.

## X.6.6	Camera Calibration for Network-based Stitching

The SDP syntax for a=3gpp-flus-media-configuration is defined with the following ABNF:

3gpp-flus-media-configuration = "a=3gpp-flus-media-configuration:" [SP "Param 1" SP "Param 2" SP ……. SP "Param K"]

where “Param 1”, …. , “Param K” may be the set of intrinsic and extrinsic camera parameters.

3GPP-specific source system description conveyed using the urn identifier such as urn:org:3gpp:itt4rt:default:2020 may imply such an SDP syntax to communicate camera calibration parameters. If the set of parameters to be conveyed is found to considerably increase the SDP size, another alternative may be considered where the camera calibration parameters may be described by a JSON document embedded in the SDP.

For each 2D video capture to be used for network-based stitching, the SDP attribute 3gpp-flus-media-configuration may be included under the m= line to signal the camera calibration parameters associated with that particular media stream. Session-level camera configuration parameters may be signalled using 3gpp-flus-configuration.

The set of camera calibration parameters to be signaled can include lens numbers, layouts, positions, angles, radius and resolutions.  More specifically, detailed camera calibration parameters based on ISO/IEC 23008-2 [3] are provided below, considering the multi-view acquisition information SEI message for HEVC. With these specifications, a 3-dimensional world point, wP = [ x y z ] is mapped to a 2-dimensional camera point, cP[ i ] = [ u v 1 ], for the i-th camera according to:

s * cP[ i ] = A[ i ] * R−1[ i ] * ( wP − T[ i ] )	(eqn. 1)

where A[ i ] denotes the intrinsic camera parameter matrix, R−1[ i ] denotes the inverse of the rotation matrix R[ i ], T[ i ] denotes the translation vector, and s (a scalar value) is an arbitrary scale factor chosen to make the third coordinate of cP[ i ] equal to 1. The elements of A[ i ], R[ i ], T[ i ] are determined according to the syntax elements signalled in this SEI message and as specified below.

Earlier equation can be extended to incorporate the entrance pupil variation proposed in [10]. As a result, we propose to correct the incidence ray of  cP[ i ] = [ u v 1 ] to always pass through the camera optical center. Due to this process the distortion is removed. By adapting the proposal in [10], we propose E[i] as a 1x1 vector that is individually incorporated as follows:

s * cP[ i ] = A[ i ] * R−1[ i ] * ( (wP + E[i]) − T[ i ] )												(eqn. 2)

where wP + E[i]) = [ x y z+E[i] ], E[i] = e1* 𝞡3 + e2* 𝞡5 + e3* 𝞡7 + e4* 𝞡9, 𝞡 is the incidence angle pertaining to each ray formed by the pixel cP[ i ] = [ u v 1 ], and [e1, e2, e3, e4] are entrance pupil coefficients. In addition, the accuracy of these entrance pupil parameters have an influence of the accuracy of estimated extrinsic parameters and thus improve the future imaging tasks. If not available, vector E is considered as 0 and a fallback to eqn. 1 is expected.

More specifically, the following intrinsic camera parameters can be signalled in the SDP for each camera as per ISO/IEC 23008-2 [3]:

focalLengthX[ i ] specifies the focal length of the i-th camera in the horizontal direction as a signed floating-point number.

focalLengthY[ i ] specifies the focal length of the i-th camera in the vertical direction as a signed floating-point number.

principalPointX[ i ] specifies the principal point of the i-th camera in the horizontal direction as a signed floating-point number.

principalPointY[ i ] specifies the principal point of the i-th camera in the vertical direction as a signed floating-point number.

skewFactor[ i ] specifies the skew factor of the i-th camera as a signed floating-point number.

The intrinsic matrix A[ i ] for i-th camera is represented by:

It is possible that the intrinsic camera parameters are equal for all of the cameras. In that case, only one set of values based on the above parameters would need to be signalled, e.g., via SDP signalling at the session level.

Furthermore, the following extrinsic camera parameters can be signalled in the SDP for each camera as per ISO/IEC 23008-2 [3]:

rE[ i ][ j ][ k ] specifies the ( j, k ) component of the rotation matrix for the i-th camera as a signed floating-point number.

The rotation matrix R[ i ] for i-th camera is represented as follows:

tE[ i ][ j ] specifies the j-th component of the translation vector for the i-th camera as a signed floating-point number.

The translation vector T[ i ] for the i-th camera is represented by:

E[ i ][ j ] specifies the j-th component of the entrance pupil vector E[i] for the i-th camera as a signed floating-point number, as per eqn (2) above.

]

# X.7	Media transport

## X.7.1	RTP

[

When viewport-dependent processing (VDP) is successfully negotiated between the MTSI-Tx client and ITT4RT-Rx client as per the SDP-based procedures described in clause X.4.2, the RTP header extension message type ‘High Quality Region (HQR)’ may carry information on the region encoded in high quality in the video bitstream delivered from the ITT4RT-Tx client to the ITT4RT-Rx client as part of the RTP transmission of media.

‘HQR’ involves signalling from the ITT4RT-Tx client to the ITT4RT-Rx client and this helps the ITT4RT-Rx client to know the high quality region corresponding to the 360 video transmitted by the ITT4RT-Tx client, i.e., which may or may not agree with the viewport requested by the ITT4RT-Rx client.

The signalling of ‘HQR’ shall use RTP header extensions as specified in IETF 5285 [X] and shall carry the HQR_azimuth, HQR_elevation, HQR_tilt, HQR_azimuth_range and HQR_elevation_range parameters corresponding to the region encoded in high quality in the video bitstream delivered as part of the RTP transmission of media.

- HQR_azimuth: Specifies the azimuth of the centre point of the sphere region corresponding to HQR in units of 2−16 degrees relative to the global coordinate axes in 32 bits.

- HQR_elevation: Specifies the elevation of the centre point of the sphere region corresponding to the HQR in units of 2−16 degrees relative to the global coordinate axes in 32 bits.

- HQR_tilt: Specifies the tilt angle of the sphere region corresponding to the HQR, in units of 2−16 degrees, relative to the global coordinate axes in 32 bits.

- HQR_azimuth_range: Specifies the azimuth range of the sphere region corresponding to the HQR through the centre point of the sphere region in units of 2−16 degrees in 32 bits.

- HQR_elevation_range: Specifies the elevation range of the sphere region corresponding to the HQR through the centre point of the sphere region in units of 2−16 degrees in 32 bits.

- HQR_stereoscopic: Included if the HQR is indicated for stereoscopic video. Value 0 indicates monoscopic content, value 1 indicates that HQR corresponds to the left view of a stereoscopic content, value 2 indicates HQR corresponds to the right view of a stereoscopic content, and value 3 indicates that the HQR corresponds to the center view.

The one-byte form of the header should be used. The values for the parameters HQR_azimuth, HQR_elevation, HQR_tilt, HQR_azimuth_range and HQR_elevation_range shall each be indicated using 4 bytes, with the following format:

0                   1                   2                   3
 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
| 									  HQR_azimuth                              |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|   								  HQR_elevation                            |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
| 									  HQR_tilt     	                          |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|									  HQR_azimuth_range                        |              +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+               |									  HQR_elevation_range                      |              +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+            |s+t+									  zero padding                        |              +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

For each 4-byte indication of the HQ_azimuth, HQ_elevation, HQ_tilt, HQ_azimuth_range and HQ_elevation_range, the bytes will be ordered in the order of importance such that high bytes shall be followed by the low bytes, where the highest byte holds the most significant bits and lowest byte holds the least significant bits.

The bits s and t store stereoscopic video information such that the ‘st’ value ‘00’ indicates monoscopic content, ‘st’ value ‘01’ indicates that the HQ is on the left view of a stereoscopic content, ‘st’ value ‘10’ indicates the HQ is on the right view of a stereoscopic content, and ‘st’ value ‘11’ indicates that the HQ is on center view.

]

## X.7.2	RTCP

When viewport-dependent processing (VDP) of 360-degree video is successfully negotiated between the ITT4RT-Tx client and ITT4RT-Rx client as per the SDP-based procedures described in clause X.4.2, the RTCP feedback (FB) message type ‘Viewport’ shall carry requested viewport information during the RTP transmission of media signalled from the ITT4RT-Rx client to the ITT4RT-Tx client.

The signalling of ‘Viewport’ requests shall use RTCP feedback messages as specified in IETF 4585. The RTCP feedback message is identified by PT (payload type) = PSFB (206) which refers to payload-specific feedback message.  FMT (feedback message type) shall be set to the value ‘X’ for Viewport feedback messages.  The IANA registration information for the FMT value for ‘Viewport’ is provided in Annex R.1 of TS 26.114. The RTCP feedback method may involve signalling of viewport information in both of the immediate feedback and early RTCP modes.

The FCI (feedback control information) format for Viewport shall be as follows. The FCI shall contain exactly one viewport. The signalled desired viewport information in the RTCP feedback message for ‘Viewport’ is composed of the following parameters (as defined as shape type value equal to 0 in clause 7.5.6 of ISO/IEC 23090-2 [R1]):):

- Viewport_azimuth: Specifies the azimuth of the centre point of the sphere region corresponding to the requested viewport in units of 2−16 degrees relative to the global coordinate axes expressed in signed 32-bit integer format, in the range of −180 * 216 to 180 * 216 − 1, inclusive..

- Viewport_elevation: Specifies the elevation of the centre point of the sphere region corresponding to the requested viewport in units of 2−16 degrees relative to the global coordinate axes expressed in signed 32-bit integer format, in the range of −90 * 216 to 90 * 216, inclusive.

- Viewport_tilt: Specifies the tilt angle of the sphere region corresponding to the requested viewport, in units of 2−16 degrees, relative to the global coordinate axes expressed in signed 32-bit integer format, in the range of −180 * 216 to 180 * 216 − 1, inclusive.

- Viewport_azimuth_range: Specifies the azimuth range of the sphere region corresponding to the requested viewport through the centre point of the sphere region in units of 2−16 degrees expressed in unsigned 32-bit integer format, in the range of 0 to 180 * 216, inclusive.

- Viewport_elevation_range: Specifies the elevation range of the sphere region corresponding to the requested viewport through the centre point of the sphere region in units of 2−16 degrees expressed in unsigned 32-bit integer format, in the range of 0 to 180 * 216, inclusive.

The RTCP feedback message for ‘Viewport’ shall contain all of the parameters Viewport_azimuth, Viewport_elevation, Viewport_tilt, Viewport_azimuth_range and Viewport_elevation_range. The values for the each of the parameters Viewport_azimuth, Viewport_elevation, Viewport_tilt, Viewport_azimuth_range and Viewport_elevation_range shall each be indicated using four bytes.

The FCI for the RTCP feedback message for ‘Viewport’ shall follow the following format:

0                   1                   2                   3
 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
| 									  Viewport_azimuth                         |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|   								  Viewport_elevation                       |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
| 									  Viewport_tilt	                          |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|									  Viewport_azimuth_range                   |              +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+               |									  Viewport_elevation_range                 |              +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

For each 4-byte indication of the Viewport_azimuth, Viewport_elevation, Viewport_tilt, Viewport_azimuth_range and Viewport_elevation_range, the bytes will be ordered in the order of importance such that high bytes shall be followed by the low bytes, where the highest byte holds the most significant bits and lowest byte holds the least significant bits.

<Conditional overlay RTCP Feedback should be added >

# X.8	Quality of Experience

<Integrate text from PD and expand>

# X.9	Viewport-Dependent Processing (Informative)

<Integrate text from PD and expand>

# X.10	Example SDP  (Informative)

Proposal

The proposal is to move the agreed changes to the draft CR.

Contact: Igor Curcio, Saba Ahsan, Nokia Technologies, Finland. Emails: igor.curcio, saba.ahsan@nokia.com.