3GPP TSG-SA WG4 adhoc post #134	       S4aR260001

28 January 2026, online

Source:	Nokia

Title:	[FS_Q4RTC_MED] pCR on QUIC-based media delivery protocols

Agenda item:	4.7

Document for:	Agreement

1. Introduction

The study item FS_Q4RTC_MED was agreed in SA4#134 and approved in SA#110 [SP-251661].

One of the objectives of FS_Q4RTC_MED is:

- Identify existing and emerging QUIC-based media delivery protocols suitable for real-time communication and document their features, benefits, limitations and current applications.

This contribution provides information on the existing QUIC-based media delivery protocols that are potentially suitable for real-time communication scenarios, describing their features, benefits/limitations, as well as their current applications.

2. Discussion

QUIC is a user-space UDP-based transport protocol developed by IETF with built‑in encryption (TLS 1.3), connection migration, stream multiplexing, pluggable congestion control (runs in user space), and optional unreliable datagrams defined in an extension to the core protocol.

The core QUIC specifications are:

- RFC 9000 (QUIC: A UDP-Based Multiplexed and Secure Transport): The foundational document defining QUIC's core mechanisms.

- RFC 9001 (Using TLS to Secure QUIC) specifies how TLS 1.3 is integrated for security.

- RFC 9002 (QUIC Loss Detection and Congestion Control) specifies how QUIC handles packet loss and manages network congestion.

- RFC 8999 (Version-Independent Properties of QUIC) describes properties that don't change between QUIC versions.

Beyond the core specifications, IETF has specified multiple extensions such as HTTP/3 (RFC 9114) and QPACK (RFC 9204), as well as operational specifications relating to applicability (RFC 9308) and manageability (RFC 9312) of the QUIC protocol.

Ongoing QUIC related work in IETF includes extensions to the UDP proxying protocol (connect-udp) [RFC 9298] for QUIC aware proxying [draft-ietf-masque-quic-proxy] developed in the MASQUE WG. Additionally, the SCONE WG is developing a solution that allows networks to indicate to endpoints the maximum available sustained throughput, or throughput advice, related to the UDP flows that QUIC exchanges, allowing applications to adapt their send rates accordingly [draft-ietf-scone-protocol].

As QUIC has seen broader adoption in recent years, its role in improving media transport compared to earlier TCP and UDP-based methods has become increasingly significant. QUIC-based media transport protocols can benefit from QUIC’s security, multiplexing, low-latency features to deliver media with finer control over reliability and ordering. Some key motivations to use QUIC for media transport are:

Lower latency and faster start-up: 1-RTT handshake with optional 0-RTT resumption shortens join time for live/interactive sessions. User-space pacing algorithms can be applied to space packets over a round-trip time (RTT) rather than sending them at once, minimizing burstiness and thus potentially reducing jitter.

Independent stream processing: Independent streams prevent one stalled media flow (e.g. a large video frame) from blocking others (e.g. audio, timed metadata), ensuring no head‑of‑line blocking across different streams. Support of DATAGRAM frames [RFC 9221] allows best‑effort delivery (no retransmission) for latency‑critical content.

Selective reliability and prioritization: QUIC stream priority allows applications to signal the importance of different data flows, enabling the transport layer to allocate resources effectively. Applications can mix reliable streams carrying critical data with datagrams for unreliable delivery of non-critical and delay-sensitive data.

Always-on security: Due to the built-in TLS 1.3 encryption and authentication, no separate DTLS layer is needed. Connection IDs (CIDs) and encrypted headers improve privacy and resillience to middlebox ossification.

Better mobility and robustness: Connection migration enables IP/port changes (e.g. Wi‑Fi to cellular) without call drops that lead to renegotiations and disrupt audio/video continuity.

Currently, various QUIC-based application protocols are being standardized by IETF to address different application scenarios including real-time communication. Those protocols are:

- Media over QUIC Transport (MOQT): https://datatracker.ietf.org/doc/draft-ietf-moq-transport/

- RTP over QUIC (ROQ): https://datatracker.ietf.org/doc/draft-ietf-avtcore-rtp-over-quic/

- WebTransport: https://datatracker.ietf.org/doc/draft-ietf-webtrans-overview/

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.836.

* * * First Change (all new) * * * *

## X.1	Media over QUIC Transport (MOQT)

### X.1.1	Introduction

Media over QUIC Transport (MOQT) [x0] is a binary data transport protocol that is under development by the IETF MOQ Working Group since 2023, designed to run either directly over QUIC [x1] or via WebTransport [x2]. Although the protocol was originally created for media applications as its name suggests, its flexible design also allows it to transmit various other types of data.

In separate Internet Drafts, the MOQ WG also develops container formats that specify encapsulation of MOQT data and media streaming formats operating over MOQT that specify media packaging as well as signaling mechanisms for negotiation between MOQT endpoints. Examples of such formats include the Low Overhead Media Container (LOC) [x3] and the MOQT Streaming Format (MSF, formerly known as WARP) [x4].

### X.1.2	Features

Below is a non-exhaustive summary of MOQT's key features. For further details, please refer to the latest version of the MOQT draft under: https://datatracker.ietf.org/doc/draft-ietf-moq-transport/

Object-based data model:

MOQT’s data model represents each schedulable unit of content as an Object within a named Track, organized into Groups and Subgroups. Objects are identified by an Object ID within a Group/Subgroup and carry metadata including Track Alias, Group ID, Object IDs, Publisher Priority, and optional extension headers.

A Track is a named content stream published under a specific Track Namespace. Tracks are referenced on the wire by a numeric Track Alias, which is assigned when a subscription is established. Publishers may provide multiple tracks for identical content, offering alternatives such as varying quality levels.

A Track is a collection of Groups that are temporal sequences of Objects. Groups are typically aligned with a codec’s synchronization point, e.g., a MOQT Group can be a Group of Pictures (GOP) defined in the H.264/AVC and H.265/HEVC video specifications. So, Group boundaries act as random access points such that subscribers can join and begin decoding without requiring information from earlier Groups.

Publish/Subscribe workflow:

MOQT’s publish/subscribe workflow enables publishers to make media Tracks available within designated namespaces, allowing subscribers to select only the only the namespaces and Tracks they need.

Publishers are endpoints that handles subscriptions by sending requested Objects from the requested Track; the initial publisher of a given track is called the original publisher. Subscribers are endpoints that subscribes to and receives tracks. Relays are protocol entities that efficiently cache and route content, acting as intermediaries that handle subscription requests, retrieve data from upstream sources, and distribute it to downstream subscribers.

Publishers and subscribers are decoupled, enabling relays to fan out to large audiences; the original publisher (i.e. the initial publisher of a given track) need not push to every endpoint. Publishers generate Tracks carrying different media content (e.g. audio, video or metadata) and transmit them to subscribers/relays either by responding to SUBSCRIBE and FETCH messages from subscribers/relays or by initiating subscriptions themselves using PUBLISH messages. SUBSCRIBE only requests newly published or received Objects while Objects from the past are retrieved using FETCH.

MOQT relies on out-of-band mechanisms for discovery of MOQT servers. However, an in-band means of discovery of publishers for a given namespace is provided by SUBSCRIBE_NAMESPACE, PUBLISH and PUBLISH_NAMESPACE messages. A subscriber can send a SUBSCRIBE_NAMESPACE message to a publisher to request the current set of matching published namespaces and/or subscriptions, as well as future updates to the set. A publisher can send a PUBLISH_NAMESPACE message to advertise that it has tracks available within a given Track Namespace.

Subscribers can also specify subscription filters indicating to the publisher which Objects to send, such as defining a starting point or choosing an exact range of Objects.

Data transport over streams and datagrams:

Objects are transmitted either on QUIC streams (reliable, ordered) or QUIC DATAGRAM frames (unreliable, unordered) as defined in RFC 9221 [x5]. The choice may affect media latency and robustness, as datagrams are typically not retransmitted upon loss and are not subject to head‑of‑line blocking, in contrast to streams.

Grouping mutually dependent Objects together in a QUIC stream can provide operational advantages, such as improved prioritization. This is achieved by using Subgroups that are distinct segments within a Group, with each Subgroup delivered over a single QUIC stream. Each Object has an associated Forwarding Preference, specifying whether it is sent in a Subgroup (reliable stream) or datagram (unreliable); this is a property of an individual Object and can vary among the Objects in the same Track.

When Objects are sent in datagrams, each is encapsulated within a variable-length OBJECT_DATAGRAM message containing a single Object. When QUIC streams are used, Objects are sent on unidirectional streams that start with a variable-length integer indicating the stream type (SUBGROUP_HEADER or FETCH_HEADER). The specification only specifies two uses of bidirectional streams: the control stream which begins with a CLIENT_SETUP message and stream carrying SUBSCRIBE_NAMESPACE messages.

Editor’s note: It is expected that future versions of the draft will allow sending other control messages in bidirectional streams: https://github.com/moq-wg/moq-transport/pull/1389

Relay behavior and scalability:

Relays acquire Tracks once and serve many subscribers, thereby facilitating scalable distribution in a manner analogous to Content Delivery Networks (CDNs). Furthermore, relays function as policy enforcement points by validating both subscription and publication requests at the network edge.

Objects and Tracks can have relay-visible fields by means of Extension Headers that allow publishers to inform relays about publisher preferences/settings (e.g. delivery timeout, max cache duration) and Object metadata (e.g. publisher priority). As endpoints, relays terminate QUIC transport sessions, providing visibility into Object metadata including data carried in optional extension headers that might be necessary for distribution. However, Object payload may be encrypted, and relays are required to treat the Object payload as opaque; they are not allowed to modify, split or combine payloads.

Relays may cache recent Objects to reduce load of the original publisher, enable quicker late joins and recovery from loss.

### X.1.3	Benefits and limitations

Benefits:

-	Leverage the features of QUIC for media delivery including:

-	Independent stream processing: QUIC’s per stream independence ensures that a blocked stream does not stall others and cause Head-of-Line (HoL) blocking.

-	Selective reliability and prioritization: Allows applications to signal the importance of different data flows.

-	Security and mobility: QUIC provides encryption (TLS 1.3) and connection migration.

-	Lower latency and faster start-up: 1-RTT handshake with optional 0-RTT resumption shortens join time for live/interactive sessions

-	Convergence to a single protocol suitable for ingest to distribution reduces the need for intermediary servers to repackage content and service providers to maintain multiple workflows.

-	Scalable pub-sub architecture that uses relays to fan out Objects to many subscribers, with the aim of enabling independent scaling of producers and consumers as well as efficient content distribution.

Objects and Tracks can have relay-visible fields by means of Extension Headers that allow publishers to inform relays about publisher preferences/settings (e.g. delivery timeout, max cache duration) and Object metadata (e.g. publisher priority

Limitations:

-	MOQT is still evolving (the IETF draft is not yet finalized), meaning limited production implementations, debugging tools, and operational experience.

-	Initial deployment costs for the new architecture and protocol implementation

-	Additional testing and operational experience are needed to validate scalability.

### X.1.4	Current applications

-	Several open-source implementations of the IETF draft exist. A non-exhaustive list is given below:

-	Google’s production-ready implementation supports MOQT: https://github.com/google/quiche

-	Meta provides an experimental MOQT relay and live encoder/player designed to work with it:

-	Relay: https://github.com/facebookexperimental/moxygen

-	Encoder/player: https://github.com/facebookexperimental/moq-encoder-player

-	Ozyegin University provides a MOQT library with publisher, subscriber and relay components, featuring various live and on-demand demo applications using the LOC and CMAF formats: https://moqtail.dev/

-	Cloudflare relay network implementation – Deployed on Cloudflare datacenter servers. https://blog.cloudflare.com/moq/

- 	Bitmovin web player– Integrated MOQT playback, bringing sub-second latency to modern web browsers. Leverages WebTransport and the WebCodecs API. https://bitmovin.com/blog/sub-second-streaming-bitmovin-player-web-x-moq-playback/

-	Vindral live streaming solution – Integrated MOQT into their low-latency live streaming platform. https://vindral.com/live/features/moq/

- 	Red5 announced upcoming support for MOQT-based live streaming in their cloud-based live video streaming and real-time communications solutions in early 2026. https://www.red5.net/media-over-quic-moq/

* * * Next Change (all new) * * * *

## X.2	RTP over QUIC (ROQ)

### X.2.1	Introduction

RTP over QUIC (ROQ) [x6] is a protocol that has been under development by the IETF AVTCORE (Audio/Video Transport Core Maintenance) WG since 2022. ROQ specifies a minimal mapping for encapsulating Real-time Transport Protocol (RTP) [x7] and RTP Control Protocol (RTCP) packets within the QUIC protocol.

The latest version of the ROQ draft can be found under: https://datatracker.ietf.org/doc/draft-ietf-avtcore-rtp-over-quic/

### X.2.2	Features

ROQ allows multiplexing multiple RTP and RTCP streams over a single QUIC connection using flow identifiers. For example, one flow ID can be assigned to audio-related RTP/RTCP packets and another to video stream packets.

ROQ offers two ways to map RTP/RTCP packets onto QUIC:

1.	Use QUIC streams: Multiple RTP packets are sent within the same QUIC stream using ROQ’s in-stream framing (each packet is length-prefixed for delineation).

2.	Use QUIC datagrams: Each QUIC DATAGRAM frame carries one RTP/RTCP packet—only the flow ID is needed, no additional length field is required.

Selecting between streams and datagrams, as well as determining the number of streams, depends on application needs and expected impact of HoL blocking.

ROQ aims to minimize RTCP traffic by utilizing data already accessible in QUIC. For example, QUIC acknowledgments can be used to compute the lost packet statistics, which are typically derived from RTCP reports in RTP applications.

RTP applications often rely on the Session Description Protocol (SDP) [x8] as their signaling protocol to establish connections and media negotiation. For ROQ, [x9] describes how SDP Offer/Answer can be used to set up an RTP connection using QUIC.

### X.2.3	Benefits and limitations

Benefits:

-	Reuse established RTP payload formats, media semantics and timing, A/V lip-sync across multiple streams, etc.

-	Built-in authentication and encryption via QUIC/TLS 1.3 (no need for separate DTLS-SRTP).

Limitations:

-	Flow identifiers introduce some overhead in addition to the header overhead of RTP and QUIC.

-	RTP applications often implement application-layer rate control (e.g. Google Congestion Control) while QUIC enforces transport-level congestion control (e.g. CUBIC, BBR). Coordination between the two is necessary to prevent conflicts and performance degradation. Currently, there is no API defined for communication between app-layer rate control and transport-layer congestion control algorithms.

-	Not suitable for point-to-multipoint topologies, as QUIC is not defined for multicast operation.

-	Limited ecosystem and adoption in the industry so far.

### X.2.4	Current applications

No known commercial deployment.

Open-source implementations exist:

-	Go implementation by TUM: https://github.com/mengelbart/roq

-	Gstreamer plugin by BBC: https://github.com/bbc/gst-roq

-	Meetecho C library imquic implements ROQ in addition to MOQT: https://github.com/meetecho/imquic/

No commercial deployments have been identified, further exploration is FFS.

* * * Next Change (all new) * * * *

## X.3	WebTransport

### X.3.1	Introduction

WebTransport is a modern web API and protocol framework that enables secure, low‑latency, bidirectional communication between browsers or web apps and servers. The IETF defines the WebTransport protocol framework [x2], including the HTTP/2 [x10] and HTTP/3 [x11] mappings found in [x12] and  [x13], respectively, while the W3C specifies the web API [x14] that allow data to be exchanged between a browser and server.

The protocol and API are designed to work within the Web security model (origin verification, TLS), and are aimed at real-time scenarios such as gaming, interactive streaming, collaboration, and IoT telemetry. Non-browser clients, such as native apps, might instead benefit from using QUIC directly for greater control and efficiency.

### X.3.2	Features

WebTransport utilizes QUIC to provide both unidirectional and bidirectional streams, ensuring reliable and ordered delivery of byte streams. It also allows for unreliable delivery using QUIC datagrams. The API exposes readable/writable streams and datagrams to developers.

When layered over HTTP/3, a WebTransport session is established via an HTTP/3 CONNECT with :protocol=webtransport and negotiated SETTINGS. All data then flows over QUIC streams/datagrams. Applications gain high-level access to QUIC’s capabilities through the WebTransport API. However, arbitrary crafting of QUIC packets or frames is not allowed; the developers interact with QUIC through the WebTransport API’s streams and datagrams mapped onto QUIC.

Congestion control is hinted via an API preference (“throughput” or “low-latency”) but not directly selectable or configurable; actual algorithms are defined by the user agent.

### X.3.3	Benefits and limitations

Benefits:

-	Leverage benefits of QUIC within the standard web security model providing browser support

-	More control and lower latency than WebSockets (which is TCP‑only and ordered/reliable)

-	Simpler deployment compared to WebRTC DataChannels (no ICE/STUN/TURN)

Limitations:

-	Browser support is evolving; no support in Safari as of early 2026.

-	An application-layer protocol is required to deliver media, utilizing either a custom frame format with message boundaries and headers or MOQT. Same for per-stream prioritization.

-	Intentionally exposes only a high-level subset of QUIC features considered safe within the web security model, not allowing full control over QUIC configuration.

### X.3.4	Current applications

MOQT can be layered on top of WebTransport.

Many open-source implementations exist. Some examples:

-	Google QUICHE supports WebTransport: https://github.com/google/quiche

-	Rust WebTransport library: https://github.com/moq-dev/web-transport

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x0]		S. Nandakumar, V. Vasiliev, I. Swett, A. Frindell; draft-ietf-moq-transport-16, “Media over QUIC Transport“, Work in Progress, Internet-Draft, 13 January 2026.

[x1]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport", May 2021.

[x2]	E. Kinnear and V. Vasiliev; draft-ietf-webtrans-overview-11, "The WebTransport Protocol Framework", Work in Progress, Internet-Draft, 20 October 2025.

[x3]	M. Zanaty, S. Nandakumar and P. Thatcher, draft-mzanaty-moq-loc-05, “Low Overhead Media Container”, Work in Progress, Internet-Draft, 3 March 2025.

[x4]	W. Law, draft-ietf-moq-msf-00, “MOQT Streaming Format”, Work in Progress, Internet-Draft, 19 January 2026.

[x5]	IETF RFC 9221: "An Unreliable Datagram Extension to QUIC", March 2022.

[x6]	M. Engelbart, J. Ott and S. Dawkins, draft-ietf-avtcore-rtp-over-quic-14, “RTP over QUIC (RoQ)”, Work in Progress, Internet-Draft, 20 March 2025.

[x7]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications", July 2003.

[x8]	IETF RFC 8866, “SDP: Session Description Protocol”, January 2021.

[x9]	S. Dawkins and V.Pascual, draft-ietf-avtcore-sdp-roq-00, “SDP Offer/Answer for RTP over QUIC (RoQ)”, Work in Progress, Internet-Draft, 11 October 2025.

[x10]	IETF RFC 9113: "HTTP/2", June 2022.

[x11]	IETF RFC 9114: "HTTP/3", June 2022.

[x12]	A. Frindell et al., draft-ietf-webtrans-http2-07, “WebTransport over HTTP/2“, Work in Progress, Internet-Draft, 20 October 2025.

[x13]	A. Frindell, E. Kinnear and V. Vasiliev, draft-ietf-webtrans-http3-14, “WebTransport over HTTP/3”, Work in Progress, Internet-Draft, 20 October 2025.

[x14]	N. Jaju, V. Vasiliev, J. Bruaroey, “WebTransport”, W3C Working Draft, 17 December 2025.

* * * End of Changes * * * *