3GPP TSG- SA4 Video SWG Conf Call on VRStream 	AHVIC-127

# 21 March 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.118 | CR | CRNum | rev | 0 | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Comments on Video Operation Points |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fraunhofer HHI |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | VRStream |  |  |  |  |  |  | Date: |  | 2018-19-03 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 5	Video Enablers

# 5.1	Video Operation Points

<Provides video operation points, describing content formats following the parameter in clause 4.1.2>

## 5.1.1	Definition of Operation Point

For the purpose to define interfaces to a conforming video decoder, video operation points are defined. In this case the following definitions hold:

- Operation Point: A collection of discrete combinations of different content formats including spatial and temporal resolutions, colour mapping, transfer functions, rendering metadata etc. and the encoding format.

- Receiver: A receiver that can decode and render any bitstream that is conforming to a certain Operation Point.

- Bitstream: A video bitstream that conforms to a video encoding format and certain Operation Point including VR Rendering Metadata

Figure X Video Operation Points

This clause focuses on the interoperability point to a media decoder as indicated in Figure X. This clause does not deal with the access engine and file parser which addresses aspects how the video bitstream is delivered.

In all video operation points, the VR Presentation is rendered using a single media decoder which provides decoded signals and rendering metadata by decoding relevant SEI messages.

## 5.1.2	Parameters of an Operation Point

Editor’s Note: At this stage the following parameters are identified, and the following options are considered.

Video Decoder

The video decoder expresses a receiver that is capable to decode a bitstream following profile/level constraints as well as additional restrictions

- V1) H.264/AVC Progressive High Profile Level 5.1 [X] for H.264/AVC with the following additional restrictions

- the maximum VCL Bit Rate is constrained to be 120Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively.

- No support for Flexible macro-block order (FMO), Arbitrary slice ordering (ASO), Redundant slices (RS) and Data partition

- A maximum of 10 slices per picture

- V2) H.264/AVC Progressive High Profile Level 5.2 [X] for H.264/AVC with the following additional restrictions

- the maximum VCL Bit Rate is constrained to be 120Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively.

- No support for Flexible macro-block order (FMO), Arbitrary slice ordering (ASO), Redundant slices (RS) and Data partition

- A maximum of 10 slices per picture

-	V3) H.265/HEVC Main-10 Profile Main Tier Profile Level 5.1 [X] for HEVC

Spatial Resolution of decoded video signal

This defines the permitted resolution of the decoded video signal.

For the spatial resolution of the decoded video all permitted resolutions following video profile/level restriction are permitted.

NOTE: Please check if any further restrictions are beneficial

Picture Aspect ratio

Limited by profile and level, with the codecs it is max of  8:1 or 1:8.

NOTE: Please check if any further restrictions are beneficial

Frame Rates of the decoded video signal

This defines the permitted frame rates of the decoded video signal.

The following frame rates are permitted:  24; 25; 30; 24/1.001; 30/1.001; 50; 60; 60/1.001  Hz.

In addition, the maximum number of macroblocks per second is restricted and certain spatial resolutions may then also imply restrictions on the frame rate.

NOTE: Please check if any further restrictions or extension are beneficial. This should be based on use cases and/or implementation issues

Chroma Subsampling

For all operation points, 4:2:0 chroma sub-sampling shall be used.

Chroma Format

Y'CbCr (non-constant luminance) as the Chroma Format shall be used.

Colour space formats

The following colour space formats may be used depending on the Operation Point for the decoded video signal:

-	ITU-R BT.709 [3]

-	ITU-R BT.2020 [4]

Transfer characteristics

The following transfer characteristics formats may be used depending on the Operation Point for the decoded video signal:

- BT.709 [3]

- BT.2020 [4] non-constant luminance transfer characteristics or the electro-optical transfer function as defined in Recommendation ITU-R BT.2100 [11] for the Perceptual Quantization (PQ) system.

Bit depth:

The following bit depth for the signal may be used:

- 8 bit

- 10 bit

Projection Formats:

The following projection formats may be used depending on the Operation Point:

- ERP, signalled by the equirectangular projection SEI message (payloadType equal to 150)

- CMP, signalled by he cubemap projection SEI message (payloadType equal to 151)

Sphere Rotation

Depending on the Operation Point, the sphere rotation SEI message (payloadType equal to 154)

- may be present and shall be processed by the receiver, or

- shall not be present.

Director’s Cut

Depending on the Operation Point,

- the receiver is may ignore processing the omnidirectional viewport SEI message (payloadType equal to 156).

- the receiver is required to process the omnidirectional viewport SEI message (payloadType equal to 156) with omni_viewport_id set to 0 ("director’s cut")

Region-wise Packing

Depending on the Operation Point,

- the region-wise packing SEI message (payloadType equal to 155) shall not be present

- the region-wise packing SEI message (payloadType equal to 155) may be present in an unrestricted manner and shall be processed by the receiver

- the region-wise packing SEI message (payloadType equal to 155) may be present with the following restrictions (to support coverage restriction):

  - If monoscopic or frame_packing_arrangement_type is set to 5 the num_packed_regions is set to 1, and frame_packing_arrangement_type is set to 3 or 4 the  num_packed_regions is set to 1 and constituent_picture_matching_flag is set to 1 or num_packed_regions is set to 2 and constituent_picture_matching_flag is set to 0

  - The transform_type[ i ] value is set to 0 only (no transform)

  - The value of packed_region_width[i] is set to the same value as proj_region_width[i]

  - The value of packed_region_height[i] is set to the same value as proj_region_height[i]

and shall be processed by the receiver

NOTE: Please check if any other restrictions and subsets are necessary, i.e. adding a new mode. This should be based on use cases (for extensions) and implementation problems (for restrictions)

Stereo signals and Frame-Packing Arrangement SEI

Depending on the Operation Point,

- the frame-packing arrangement (payloadType equal to XXX) shall not be present

- the frame packing arrangement SEI message may be present with

  - the syntax element frame_packing_arrangement_type set to 4 for Top-and-Bottom.

  - the syntax element quincunx_sampling_flag is equal to 0;

  - The syntax element content_interpretation_type is equal to 1;

  - The syntax elements spatial_flipping_flag is equal to 0;

  - The syntax element field_views_flag is equal to 0;

and shall be processed by the receiver

- the frame packing arrangement SEI message may be present with

  - the syntax element frame_packing_arrangement_type set to 3 for SbS.

  - the syntax element quincunx_sampling_flag is equal to 0;

  - The syntax element content_interpretation_type is equal to 1;

  - The syntax elements spatial_flipping_flag is equal to 0;

  - The syntax element field_views_flag is equal to 0;

and shall be processed by the receiver

- the frame packing arrangement SEI message may be present with

  - the syntax element frame_packing_arrangement_type set to 5 for temporal interleaving.

  - the syntax element quincunx_sampling_flag is equal to 0;

  - The syntax element content_interpretation_type is equal to 1;

  - The syntax elements spatial_flipping_flag is equal to 0;

  - The syntax element field_views_flag is equal to 0;

and shall be processed by the receiver

NOTE: Please check if any other restrictions and subsets are necessary, i.e. adding a new mode. This should be based on use cases (for extensions) and implementation problems (for restrictions)

SEI Message Dynamics

Depending on the Operation Point and SEI message type, the SEI message may follow the following rules

- May change for every picture

- May change for every coded video sequence

- May not change at all

## 5.1.3	Operation Point Summary

The following table summarizes the Operation points

| Operation Point name | Decoder | Bit depth | Colour space format | Transfer Characteristics | Projection | Rotation | Director’s Cut | RWP | Stereo |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H.264/AVC vanilla | H.264/AVC HP@L5.1 | 8 | BT.709 | BT.709 | ERP | No | No | Restricted (Coverage) | No |
| H.264/AVC banana-split | H.264/AVC HP@L5.2 | 8 | BT.709 | BT.709 | ERP | No | No | Full | TaB |
| H.265/HEVC chocolate | H.265/HEVC MP10@L5.1 | 8, 10 | BT.709 BT.2020 | BT.709, BT.2100 PQ | ERP | No | No | Restricted (Coverage) | No |
| H.265/HEVC Stracciatella | H.265/HEVC MP10@L5.1 | 8, 10 | BT.709 BT.2020 | BT.709, BT.2100 PQ | ERP CMP | Yes | Yes | Full | TaB SbS Temp |

## 5.1.4	Vanilla H.264/AVC

This operation targets simple deployments and legacy receivers at basic quality.

Video Decoder

The video decoder expresses a receiver that is capable to decode a bitstream following profile/level constraints as well as additional restrictions

- H.264/AVC Progressive High Profile Level 5.1 [X] for H.264/AVC with the following additional restrictions

- the maximum VCL Bit Rate is constrained to be 120Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively.

- No support for Flexible macro-block order (FMO), Arbitrary slice ordering (ASO), Redundant slices (RS) and Data partition

- A maximum of 10 slices per picture

Spatial Resolution of decoded video signal

For the spatial resolution of the decoded video all permitted resolutions following video profile/level restriction are permitted.

NOTE: Please check if any further restrictions are beneficial

Picture Aspect ratio

Limited by profile and level, with the codecs it is max of  8:1 or 1:8.

NOTE: Please check if any further restrictions are beneficial

Frame Rates of the decoded video signal

This defines the permitted frame rates of the decoded video signal.

The following frame rates are permitted:  24; 25; 30; 24/1.001; 30/1.001; 50; 60; 60/1.001  Hz.

In addition, the maximum number of macroblocks per second is restricted and certain spatial resolutions may then also imply restrictions on the frame rate.The combiniation of luma samples and maximum frate rates is restricted by video profile/level restriction.

NOTE: Please check if any further restrictions or extension are beneficial. This should be based on use cases and/or implementation issues

Chroma Subsampling

For all operation points, 4:2:0 chroma sub-sampling shall be used.

Chroma Format

Y'CbCr (non-constant luminance) as the Chroma Format shall be used.

Colour space formats

The following colour space formats may be used depending on the Operation Point for the decoded video signal:

-	ITU-R BT.709 [3]

Transfer characteristics

The following transfer characteristics formats may be used depending on the Operation Point for the decoded video signal:

- BT.709 [3]

Bit depth:

The following bit depth for the signal may be used:

- 8 bit

Projection Formats:

The following projection formats may be used depending on the Operation Point:

- ERP, signalled by the equirectangular projection SEI message (payloadType equal to 150)

Sphere Rotation

The sphere rotation SEI message (payloadType equal to 154) shall not be present.

Director’s Cut

The receiver is may ignore processing the omnidirectional viewport SEI message (payloadType equal to 156).

Region-wise Packing

The region-wise packing SEI message (payloadType equal to 155) may be present with following constraints (to support coverage):

  - If monoscopic or frame_packing_arrangement_type is set to 5 the num_packed_regions is set to 1, and frame_packing_arrangement_type is set to 3 or 4 the  num_packed_regions is set to 1 and constituent_picture_matching_flag is set to 1 or num_packed_regions is set to 2 and constituent_picture_matching_flag is set to 0

  - The transform_type[ i ] value is set to 0 only (no transform)

  - The value of packed_region_width[i] is set to the same value as proj_region_width[i]

  - The value of packed_region_height[i] is set to the same value as proj_region_height[i]

and shall be processed by the receiver

NOTE: Addting option his should be based on use case (for extensions) and implementation problems (for restrictions)

Stereo signals and Frame-Packing Arrangement SEI

- the frame-packing arrangement (payloadType equal to XXX) shall not be present

SEI Message Dynamics

The SEI messages shall not be changed in CVS.

## 5.1.5	Banana-split H.264/AVC

This operation targets advanced H.264/AVC based deployments.

Video Decoder

The video decoder expresses a receiver that is capable to decode a bitstream following profile/level constraints as well as additional restrictions

- H.264/AVC Progressive High Profile Level 5.2 [X] for H.264/AVC with the following additional restrictions

- the maximum VCL Bit Rate is constrained to be 120Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively.

- No support for Flexible macro-block order (FMO), Arbitrary slice ordering (ASO), Redundant slices (RS) and Data partition

- A maximum of 10 slices per picture

NOTE: Check for support of 5.2. in devices

Spatial Resolution of decoded video signal

This defines the permitted resolution of the decoded video signal.

For the spatial resolution of the decoded video all permitted resolutions following video profile/level restriction are permitted.

NOTE: Please check if any further restrictions are beneficial

Picture Aspect ratio

Limited by profile and level, with the codecs it is max of  8:1 or 1:8.

NOTE: Please check if any further restrictions are beneficial

Frame Rates of the decoded video signal

This defines the permitted frame rates of the decoded video signal.

The following frame rates are permitted:  24; 25; 30; 24/1.001; 30/1.001; 50; 60; 60/1.001  Hz.

In addition, the maximum number of macroblocks per second is restricted and certain spatial resolutions may then also imply restrictions on the frame rate.

NOTE: Please check if any further restrictions or extension are beneficial. This should be based on use cases and/or implementation issues

Chroma Subsampling

For all operation points, 4:2:0 chroma sub-sampling shall be used.

Chroma Format

Y'CbCr (non-constant luminance) as the Chroma Format shall be used.

Colour space formats

The following colour space formats may be used depending on the Operation Point for the decoded video signal:

-	ITU-R BT.709 [3]

Transfer characteristics

The following transfer characteristics formats may be used depending on the Operation Point for the decoded video signal:

- BT.709 [3]

Bit depth:

The following bit depth for the signal may be used:

- 8 bit

Projection Formats:

The following projection formats may be used depending on the Operation Point:

- ERP, signalled by the equirectangular projection SEI message (payloadType equal to 150)

Sphere Rotation

The sphere rotation SEI message (payloadType equal to 154) shall not be present.

[Alternative: The sphere rotation SEI message (payloadType equal to 154) may be present and shall be processed by the receiver] Bring use Case, if this is important

Director’s Cut

The receiver may ignore processing the omnidirectional viewport SEI message (payloadType equal to 156).

Region-wise Packing

The region-wise packing SEI message (payloadType equal to 155) may be present in an unrestricted manner and shall be processed by the receiver.

The SEI message shall be present in RAP only.

NOTE: Please check if any other restrictions and subsets are necessary, i.e. adding a new mode. This should be based on use cases (for extensions) and implementation problems (for restrictions)

Stereo signals and Frame-Packing Arrangement SEI

The frame packing arrangement SEI message may be present with

  - the syntax element frame_packing_arrangement_type set to 4 for Top-and-Bottom.

  - the syntax element quincunx_sampling_flag is equal to 0;

  - The syntax element content_interpretation_type is equal to 1;

  - The syntax elements spatial_flipping_flag is equal to 0;

  - The syntax element field_views_flag is equal to 0;

and shall be processed by the receiver

Any of the frame packing SEI messages shall present only in RAPs.

SEI Message Dynamics

The SEI messages shall not be changed in CVS.

## 5.1.6	Chocolate H.265/HEVC

This operation targets simple and efficient deployments based on H.265/HEVC.

Video Decoder

The video decoder expresses a receiver that is capable to decode a bitstream following profile/level constraints as well as additional restrictions

-	V3) H.265/HEVC Main-10 Profile Main Tier Profile Level 5.1 [X] for HEVC

Spatial Resolution of decoded video signal

This defines the permitted resolution of the decoded video signal.

For the spatial resolution of the decoded video all permitted resolutions following video profile/level restriction are permitted.

NOTE: Please check if any further restrictions are beneficial

Picture Aspect ratio

Limited by profile and level, with the codecs it is max of  8:1 or 1:8.

NOTE: Please check if any further restrictions are beneficial

Frame Rates of the decoded video signal

This defines the permitted frame rates of the decoded video signal.

The following frame rates are permitted:  24; 25; 30; 24/1.001; 30/1.001; 50; 60; 60/1.001  Hz.

In addition, the maximum number of macroblocks per second is restricted and certain spatial resolutions may then also imply restrictions on the frame rate.

NOTE: Please check if any further restrictions or extension are beneficial. This should be based on use cases and/or implementation issues

Chroma Subsampling

For all operation points, 4:2:0 chroma sub-sampling shall be used.

Chroma Format

Y'CbCr (non-constant luminance) as the Chroma Format shall be used.

Colour space formats

The following colour space formats may be used depending on the Operation Point for the decoded video signal:

-	ITU-R BT.709 [3]

-	ITU-R BT.2020 [4]

NOTE: Please check if there are any implementation issus with processing BT.2020

Transfer characteristics

The following transfer characteristics formats may be used depending on the Operation Point for the decoded video signal:

- BT.709 [3]

- BT.2020 [4] non-constant luminance transfer characteristics or the electro-optical transfer function as defined in Recommendation ITU-R BT.2100 [11] for the Perceptual Quantization (PQ) system.

NOTE: Please check if there are any implementation issus with processing BT.2100

Bit depth:

The following bit depth for the signal may be used:

- 8 bit

- 10 bit

Projection Formats:

The following projection formats may be used depending on the Operation Point:

- ERP, signalled by the equirectangular projection SEI message (payloadType equal to 150)

Sphere Rotation

The sphere rotation SEI message (payloadType equal to 154) shall not be present.

Director’s Cut

The receiver is may ignore processing the omnidirectional viewport SEI message (payloadType equal to 156).

Region-wise Packing

The region-wise packing SEI message (payloadType equal to 155) may be present with following constraints (to support coverage):

  - If monoscopic or frame_packing_arrangement_type is set to 5 the num_packed_regions is set to 1, and frame_packing_arrangement_type is set to 3 or 4 the  num_packed_regions is set to 1 and constituent_picture_matching_flag is set to 1 or num_packed_regions is set to 2 and constituent_picture_matching_flag is set to 0

  - The transform_type[ i ] value is set to 0 only (no transform)

  - The value of packed_region_width[i] is set to the same value as proj_region_width[i]

  - The value of packed_region_height[i] is set to the same value as proj_region_height[i]

and shall be processed by the receiver

NOTE: Addting option his should be based on use case (for extensions) and implementation problems (for restrictions)

Stereo signals and Frame-Packing Arrangement SEI

The frame-packing arrangement (payloadType equal to XXX) shall not be present

SEI Message Dynamics

The SEI messages shall not be changed in CVS.

## 5.1.7	Stracciatella H.265/HEVC

This operation targets advanced deployments based on H.265/HEVC.

Video Decoder

The video decoder expresses a receiver that is capable to decode a bitstream following profile/level constraints as well as additional restrictions

-	V3) H.265/HEVC Main-10 Profile Main Tier Profile Level 5.1 [X] for HEVC

Spatial Resolution of decoded video signal

This defines the permitted resolution of the decoded video signal.

For the spatial resolution of the decoded video all permitted resolutions following video profile/level restriction are permitted.

NOTE: Please check if any further restrictions are beneficial

Picture Aspect ratio

Limited by profile and level, with the codecs it is max of  8:1 or 1:8.

NOTE: Please check if any further restrictions are beneficial

Frame Rates of the decoded video signal

This defines the permitted frame rates of the decoded video signal.

The following frame rates are permitted:  24; 25; 30; 24/1.001; 30/1.001; 50; 60; 60/1.001  Hz.

In addition, the maximum number of macroblocks per second is restricted and certain spatial resolutions may then also imply restrictions on the frame rate.

NOTE: Please check if any further restrictions or extension are beneficial. This should be based on use cases and/or implementation issues

Chroma Subsampling

For all operation points, 4:2:0 chroma sub-sampling shall be used.

Chroma Format

Y'CbCr (non-constant luminance) as the Chroma Format shall be used.

Colour space formats

The following colour space formats may be used depending on the Operation Point for the decoded video signal:

-	ITU-R BT.709 [3]

-	ITU-R BT.2020 [4]

Transfer characteristics

The following transfer characteristics formats may be used depending on the Operation Point for the decoded video signal:

- BT.709 [3]

- BT.2020 [4] non-constant luminance transfer characteristics or the electro-optical transfer function as defined in Recommendation ITU-R BT.2100 [11] for the Perceptual Quantization (PQ) system.

Bit depth:

The following bit depth for the signal may be used:

- 8 bit

- 10 bit

Projection Formats:

The following projection formats may be used depending on the Operation Point:

- ERP, signalled by the equirectangular projection SEI message (payloadType equal to 150)

- CMP, signalled by he cubemap projection SEI message (payloadType equal to 151)

Sphere Rotation

the sphere rotation SEI message (payloadType equal to 154) may be present and shall be processed by the receiver.

Director’s Cut

The receiver is required to process the omnidirectional viewport SEI message (payloadType equal to 156) with omni_viewport_id set to 0 ("director’s cut")

Region-wise Packing

The region-wise packing SEI message (payloadType equal to 155) may be present in an unrestricted manner and shall be processed by the receiver.

The SEI message may change with every CVS.

NOTE: Please check if any other restrictions and subsets are necessary, i.e. adding a new mode. This should be based on use cases (for extensions) and implementation problems (for restrictions)

Stereo signals and Frame-Packing Arrangement SEI

- The frame packing arrangement SEI message may be present with

  - the syntax element frame_packing_arrangement_type set to 4 for Top-and-Bottom.

  - the syntax element quincunx_sampling_flag is equal to 0;

  - The syntax element content_interpretation_type is equal to 1;

  - The syntax elements spatial_flipping_flag is equal to 0;

  - The syntax element field_views_flag is equal to 0;

and shall be processed by the receiver

- the frame packing arrangement SEI message may be present with

  - the syntax element frame_packing_arrangement_type set to 3 for SbS.

  - the syntax element quincunx_sampling_flag is equal to 0;

  - The syntax element content_interpretation_type is equal to 1;

  - The syntax elements spatial_flipping_flag is equal to 0;

  - The syntax element field_views_flag is equal to 0;

and shall be processed by the receiver

- the frame packing arrangement SEI message may be present with

  - the syntax element frame_packing_arrangement_type set to 5 for temporal interleaving.

  - the syntax element quincunx_sampling_flag is equal to 0;

  - The syntax element content_interpretation_type is equal to 1;

  - The syntax elements spatial_flipping_flag is equal to 0;

  - The syntax element field_views_flag is equal to 0;

and shall be processed by the receiver

NOTE: Please check if any other restrictions and subsets are necessary, i.e. adding a new mode. This should be based on use cases (for extensions) and implementation problems (for restrictions)

SEI Message Dynamics

The SEI messages shall not be changed in CVS.