3GPP TSG-SA4 Meeting #103 	S4-190364

# Newport Beach, CA, USA, Apr 8 - 12, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.929 | CR | - | rev | - | Current version: | 0.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VR QoE metrics additions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_QoE_VR |  |  |  |  |  | Date: |  |  | 2019-04-01 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Missing text in some clauses. Conclusion not finalized. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add relevant text where empty. Finalize conclusions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Uncomplete study. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.1, 8.1, 12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

================================= Start of first change =================================

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AV	Audio/video

3DOF	3 Degrees of freedom

6DOF	6 Degrees of freedom

DASH	Dynamic Adaptive Streaming over HTTP

FOV	Field of view

HMD	Head Mounted Display

MCC		Metrics collection and computation

MPD		Media Presentation Description

QoE		Quality of Experience

VR	Virtual Reality

UL	Up-link

================================= Start of next change =================================

## 4.1	Introduction

The VR service is much more complex than traditional 2D video streaming, and thus defining relevant metrics is also more challenging. The following clauses address some of these challenges.

================================= Start of next change =================================

## 8.1	Introduction

While transmission-related factors (such as delay) certainly play an important role for the VR experience, the VR content itself will also have some impact. The following clause specifically investigates the content aspect.

================================= Start of next change =================================

# 12	Conclusions

It is evident that VR streaming services add a lot of complexity as compared to traditional streaming services. While the latter are more statically consumed, for VR services the user actions and movements becomes an important part of the total user experience. Also the actual content delivery will often depend on what the user is doing, which presents new challenges, mostly in terms of throughput and delay.

While the exact relationship between potentially measurable metrics and the complete user experience will probably not be fully researched and understood until VR services are becoming more widely used, we need to address at least the basic issues now, and add further improvements and more elaborate metrics later.

Some existing QoE metrics, such as throughput, buffer level and playlist are useful also for a VR use case, and can probably be kept more or less as-is, as described in clauses 9.2.1 to 9.2.3.

A very basic addition would be to define new device metrics, as outlined in clause 9. These are necessary to understand the context of the VR service, as well as limitations and other characteristics of the device.

A more challenging task is to define metrics which describe the interaction between user actions and movements, and the responsiveness and quality of the content. Due to the complexity of the VR service it might be useful to define scaleable interaction metrics, e.g. as outlined in clause 9.2.4, so that a lower-accuracy metric can be measured with a relatively simple client implementation, while higher-accuracy variants could be available if the client supports more advanced metric features.

Very advanced metrics, such as relations to content complexity as outlined in clause 8.2, most likely need further understanding before any implementable metrics can be defined.

To get as wide support as possible for new VR metrics, any metrics defined in a future normative work should ideally be coordinated with other standardization fora (for instance MPEG as the current traditional QoE metrics have a more or less a full MPEG alignment).

Configuration and reporting should as far as possible re-use and extend existing mechanisms, as described in clause 11, preferrably in a backward-compatible way.

To facilitate a timely development of VR metrics, it is recommended that a normative work item is started. The work item should at least implement:

-	A client reference model for VR QoE measurements.

- Basic VR device metrics.

- Metrics describing the interaction delay.

- Expanded configuration and reporting schemas to cover the above.