3GPP TSG-SA4 Meeting #105	S4-190921
Ljubljana, Slovenia ,  August 12 - 17, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.118 | CR | <CR#> | rev | <Rev#> | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adoption of MPEG-I VR QoE Metrics in TS 26.118 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fraunhofer HHI, InterDigital Communications, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | VRQoE |  |  |  |  |  | Date: |  |  | 2019-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | With approval of the work item VRQoE, adding new QoE VR metrics to TS 26.118 that are aligned with metrics from MPEG-I Part 6 ensures coordination and interoperability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Aligning with MPEG-I VR metrics |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It becomes challenging for a VR streaming service provider to implement and maintain different sets of metrics defined in two different standards |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Introduction

In TR 26.929, the 3GPP SA4 group has studied the importance of VR QoE metrics and their impact on the overall experience in VR services. In S4-190323, the immersive media metrics adopted in MPEG-I Part 6 were introduced and added to the TR 26.929 at SA4#103. Later at SA4#104, in S4-190781, a new VR Metrics clause was agreed to be added to the TS 26.118. However, no specific VR QoE metrics are defined yet. In this document we propose to align VR QoE metrics in TS 26.118 to the metrics defined in MPEG-I Part 6 (ISO/IEC 23090-6) in order to facilitate interoperability.

This document is structured as follows. Section 2 discusses various aspects of VR metrics defined in ISO/IEC 23090-6, while Section 3 presents the proposed changes to TS 26.118 with the definition of VR metrics based on ISO/IEC 23090-6.

# Discussion

In TR 26.929, it was concluded that metrics defined in future normative work should ideally be coordinated with other standardizations to achieve wide support. The adoption of VR metrics from ISO/IEC 23090-6 achieves that goal and is inline with the similarities between the defined observation points in the two specifications. As shown in Figure 1 the observation points defined in ISO/IEC 23090-6 align with observation points from TS 26.118.

Figure 1: Comparison of observation points defined in MPEG and 3GPP

In Section 3 we present the proposed changes to TS 26.118 in order to adopt the metrics from ISO/IEC 23090-6. The following VR metrics as defined in ISO/IEC 23090-6 are proposed:

- Display information set metric

- Rendered FOV set metric

- Rendered viewports metric

- Comparable quality viewport switching latency metric

Further details on each metric are provided in the remainder of this section.

Display information set metric

Display information set metric allows to gather a display characteristics of VR-capable devices used in a service. Display resolution, its pixel density and refresh rate are considered to be the most important ones. With this metric, a service provider can get information about the display properties of their customers and quickly adapt its content (or improve other parts of its servcice) accordingly.

On a native implementation of a VR client application, it might be easy to get information about the display properties (e.g., through APIs such as DisplayMetrics[1] on Android, and UIScreen[2] on iOS). However, in a browser-based application, this might be more difficult. For example, a pixel (or reference pixel as defined in W3C specification) is defined as the visual angle of one pixel on a deivce with a pixel density of 96 dpi and a distance of an arm’s length from the reader. Hence calculating the actual pixel density would be inconsistent.

Rendered FOV set metric

2D displays and programmable HMD devices have adjustable FOVs. The rendered FoV set metric includes the horizontal and vertical element of the rendered FOV in unit of degree. A record of rendered FOV may provide information about device capabilities and the user’s most comfortable FOV during the VR playback. This information could be useful to optimise QoE for a service using the Advanced Video Media profile. In a JavaScript implementation using three.js library, the FOV can be set by changing the parameters of the perspective camera.

Rendered viewports metric

The rendered viewports metric reports a list of viewports that have been rendered. The time instance when a viewport is rendered and the duration of the rendered viewport playback are also included in the metric. For example, services using the Advanced Video Media profile may get the paramerters for this metric which are available in the extractor track descriptor in an MPD file. The report might also include the time instance when the content corresponding to the selected extractor track was played and the duration of the playback. An analysis which aggregates rendered viewport metrics from multiple users can be used to identify the popular region(s) within the VR content. This information might be helpful in assisting the content distributor to optimise caching in the CDNs.

Comparable quality viewport switching latency metric

The comparable quality viewport switching latency metric reports the latency experienced by the user when switching from one viewport to another viewport until the presentation quality of the new viewport reaches a similar presentation quality as the first viewport. This metric includes the time instance when a viewport switching event is identified and assigns the time value to the ‘t’ field defined in the metric. The latency is measured until the quality of the second viewport is comparable to that of the first viewport just before the time indicated by the parameter ‘t’. This metric can be used to assess the overall QoE for an immersive media streaming service. The comparable quality viewport switching latency can be affected by various factors such as bandwidth, segment duration, decoder buffer size, buffer fullness, the segment scheduling algorithm, and etc. For example, in a service using the Advanced Video Media profile, the user can change the head position from one viewport position to another viewport position. A content request for the new viewport position is made. The system might experience some delay in achieving a similar quality for the second viewport compared to that of the first viewport due to network delay, buffer fullness, etc.

The four ISO/IEC 23090-6 VR metrics listed above have been implemented in the JavaScript player[3]. The project includes an HTML5 player capable of playing content compliant to 3GPP TS 26.118 Advanced Media Profile. The player will allow gathering VR metrics as defined in in ISO/IEC 23090-6, which are useful for quality-related feedback and analysis. The intention of this new release is to show that the VR metrics which are defined in ISO/IEC 23090-6 are implementable and also valuable. At the moment of writing of this document the implementation is contained in a separate branch called ‘metrics’. After testing it will be included in the next software release.

Figure 2 shows a screenshot of the JavaScript player along with information on some of the VR metrics (note that by the time this document is presented, the implementation will look differently).

Figure 2. Screenshot of the Javascript player with VR metrics

# Proposed changes to TS 26.118

========================== Start of changes ================================

### 9.3.1	General

As the VR metrics functionality is based on the DASH QoE metrics, metrics already defined in TS 26.247 are valid also for a VR client. Therefore, the following sub-clauses only define additional VR-related metrics.

A new data type, ViewportDataType, is defined as shown in Table 1. ViewportDataType is an Object with six Integer keys that identify a viewport. The six keys are: viewpoint_id, centre_azimuth, centre_elevation, centre_tilt, azimuth_range, and elevation_range, respectively.

Table 1 ViewportDataType

| Key |  | Type | Description |
| --- | --- | --- | --- |
| ViewportDataType |  | Object |  |
|  | viewpoint_id | Integer | Specifies the identifier of the viewpoint to which the viewport belongs. |
|  | centre_azimuth | Integer | Specifies the azimuth of the centre of the viewport in units of 2−16 degrees. The value shall be in the range of −180 * 216 to 180 * 216 − 1, inclusive. |
|  | centre_elevation | Integer | Specifies the elevation of the centre of the viewport in units of 2−16 degrees. The value shall be in the range of −90 * 216 to 90 * 216, inclusive. |
|  | centre_tilt | Integer | Specifies the tilt angle of the viewport in units of 2−16 degrees. The value shall be in the range of −180 * 216 to 180 * 216 − 1, inclusive. |
|  | azimuth_range | Integer | Specifies the azimuth range of the viewport through the centre point of the viewport, in units of 2−16 degrees. |
|  | elevation_range | Integer | Specifies the elevation range of the viewport through the centre point of the viewport, in units of 2−16 degrees. |

## 9.3.2 Display information set metric

The display information set metric reports the characteristics of the display used by the client for rendering the VR content, including display resolution, pixel density, and refresh rate. The display information set metric is specified in Table 2.

Table 2 Display information set metric

| Key |  |  | Type | Description |
| --- | --- | --- | --- | --- |
| DisplayInfoSet |  |  | Set | Set of display information |
|  | Entry |  | Object |  |
|  |  | displayResolution | String | Display resolution, in units of pixels |
|  |  | displayPixelDensity | Integer | Display pixel density, in units of PPI |
|  |  | displayRefreshRate | Integer | Display refresh rate, in units of Hz |

This metric is a generic metric that also applies to other visual media applications than VR applications.

## 9.3.3 Rendered FOV set metric

The rendered FOV set metric reports a set of FOVs rendered by VR client devices and is specified in Table 3.

Table 3 Rendered FOV set metric

| Key |  |  | Type | Description |
| --- | --- | --- | --- | --- |
| RenderedFovSet |  |  | Set | Set of rendered FOVs |
|  | Entry |  | Object |  |
|  |  | renderedFovH | Integer | The horizontal element of the rendered FOV, in units of degrees |
|  |  | renderedFovV | Integer | The vertical element of the rendered FOV, in units of degrees |

## Rendered viewports metric

The rendered viewports metric reports a list of viewports that have been rendered longest time at particular intervals of media presentation times and is specified in Table 4.

Table 4 Rendered viewports metric

| Key |  |  | Type | Description |
| --- | --- | --- | --- | --- |
| RenderedViewports |  |  | List | List of rendered viewports |
|  | Entry |  | Object |  |
|  |  | startTime | Media-Time | Specifies the media presentation time of the first played out media sample when the viewport indicated in the current entry is rendered starting from this media sample. |
|  |  | duration | Integer | The time duration, in units of milliseconds, of the continuously presented media samples when the viewport indicated in the current entry is rendered starting from the media sample indicated by startTime. "Continuously presented" means that the media clock continued to advance at the playout speed throughout the interval. |
|  |  | viewport | ViewportDataType | Indicates the region of the omnidirectional media corresponding to the viewport that is rendered longest time starting from the media sample indicated by startTime. |

## Comparable quality viewport switching latency

The comparable quality viewport switching latency metric reports the latency experienced by the user when switching from a first viewport to a second viewport until the presentation quality of the second viewport reaches a comparable presentation quality as the first viewport.

The comparable quality viewport switching latency metric is specified in Table 5.

Table 5 Comparable quality viewport switching latency metric

| Key |  |  |  |  | Type | Description |
| --- | --- | --- | --- | --- | --- | --- |
| CQViewportSwitchingLatency |  |  |  |  | List | List of comparable quality viewport switching latencies |
|  | Entry |  |  |  | Object |  |
|  |  | firstViewport |  |  | ViewportDataType | Specifies the spherical region corresponding to the first viewport (i.e., before the switching). |
|  |  | secondViewport |  |  | ViewportDataType | Specifies the spherical region corresponding to the second viewport (i.e., after the switching). |
|  |  | firstViewportQuality |  |  | Integer | Specifies the quality value of the first viewport. |
|  |  | secondViewportQuality |  |  | Integer | Specifies the quality value of the second viewport. |
|  |  | t |  |  | Real-Time | Specifies the measurement time of the viewport switching latency in wall-clock time. |
|  |  | latency |  |  | Integer | Specifies the delay in milliseconds between the time a user movement from first viewport to second viewport and the time when the presentation quality of the second viewport reaches a comparable presentation quality as the first viewport. |
|  |  | reason |  |  | List | Specifies a list of possible causes for the latency. |
|  |  |  | Entry |  | Object |  |
|  |  |  |  | code | Enum | A possible cause for the latency. The value is equal to one of the following: - 0: Segment duration - 1: Buffer fullness - 2: Availability of comparable quality segment |

========================== End of changes ================================

# References

[1] Android API reference (DisplayMetrics class), https://developer.android.com/reference/android/util/DisplayMetrics.html

[2] UIKit Framework (UIScreen class), https://developer.apple.com/documentation/uikit/uiscreen

[3] HTML5 MSE Playback of MPEG 360 VR Tiled Streaming, https://github.com/fraunhoferhhi/omaf.js