3GPP TSG-SA4 Meeting #108-e	S4-200594

# Electronic,  , 6th–9th April 2020	revision of S4-200512

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.501 | CR | TBA | rev | – | Current version: | 16.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add sample percentage to 5GMSA stage 2 Consumption Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BBC, Enensys |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 2020-03-XX |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | - Reduce impact of consumption reporting mechanism on the network. - Harmonisation of consumption reporting design with metrics reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a “Sample percentage” parameter to table 4.2.3-2. Remove duplicate table 5.6.2-1 and provide cross-referencing back to table 4.2.3-2. Note that the new parameter is specified in such a way that it is backwards-compatible with any existing implementation, i.e. omitting the parameter is equivalent to 100% of clients reporting consumption. Clarified in clause 4.2.3 that the baseline Service Access Information applies irrespective of which method has been used to acquire it. Fixed formatting in the tables to use the correct ETSI template styles. Minor editorial improvements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | With the current design, there is a risk that the 5GMSd Application Function will become swamped by an excessive volume of media consumption reports from UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3, 5.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-AHI930, S4-200512 |  |  |  |  |  |  |  |  |

================================START OF FIRST CHANGE==============================

### 4.2.3	Service Access Information

The Service Access Information is the set of parameters and addresses which are needed by the 5GMSd Client to activate the reception of a downlink streaming session.

The Service Access Information may be provided together with other service announcement information using M8d. Alternatively, the 5GMSd Client fetches the Service Access Information from the 5GMSd AF. Regardless of how it is provided, the Service Access Information contains at least the baseline information in Table 4.2.3-1. Additional information is added, depending on offered features.

Table 4.2.3-1: Parameters of baseline service access information

| Parameters | Description |
| --- | --- |
| Media player entry | A document or a pointer to a document that defines a media presentation e.g. MPD for DASH content or URL to a video clip file. |
| URL signing information | …. |
| 5GMSd Application Service Configuration Id | Unique identification of the M1d Provisioning Session. |

When the consumption reporting feature is activated for a downlink streaming session the parameters from Table 4.2.3-2 below are additionally present.

Table 4.2.3-2: Parameters for consumption reporting configuration

| Parameters | Description |
| --- | --- |
| Reporting interval | Identifies the interval between consumption reports being sent by the 5GMS Media Session Handler. |
| Server address | A list of 5GMSd AF addresses where the consumption reportis are sent by the 5GMS Media Session Handler. |
| Sample percentage | The proportion of clients that shall report media consumption. If not specified, all clients shall send reports. |
| Location reporting | A flag identifying whether the 5GMS Media Session Handler provides location data to the 5GMSd AF (in case of MNO or trusted third parties). |

When the dynamic policy invocation feature is activated for a downlink streaming session the parameters from Table 4.2.3-3 below are additionally present.

Table 4.2.3-3: Parameters for dynamic policy invocation configuration

| Parameters | Description |
| --- | --- |
| Server address | A list of 5GMSd AF addresses which offer the APIs for dynamic policy invocation sent by the 5GMS Media Session Handler. (Opaque URL, following the 5GMSA URL format.) |
| Policy Template Ids | A list of Policy Template Ids which the 5GMSd Client is authorized to use. |

When the QoE metrics reporting feature is activated for a downlink streaming session the parameters from Table 4.2.3-4 below are additionally present.

Table 4.2.3-4: Parameters for metrics configuration

| Parameters | Description |
| --- | --- |
| Server address | A list of 5GMSd AF addresses to which metrics reports shall be sent. |
| DNN | The DNN which shall be used when sending metrics report. If not specified, the default APN shall be used. |
| Reporting interval | The sending interval between metrics reports. If not specified, a single final report shall be sent after the streaming session has ended. |
| Sample percentage | The proportion of clients that shall report metrics. If not specified, all clients shall send reports. |
| Streaming source filter | A list of URL patterns for which metrics reporting shall be done. If not specified, reporting shall be done for all sessions. |
| Metrics | A list of metrics which shall be reported. |

When Network Assistance is activated for a downlink streaming session the parameters from Table 4.2.3-5 below are additionally present.

Table 4.2.3-5: Parameters for Network Assistance configuration

FFS

=================================END OF FIRST CHANGE===============================

===============================START OF SECOND CHANGE=============================

### 5.6.2	Consumption reporting parameters

Table 4.2.3-2 describes the baseline parameters used in step 4 of clause 5.6.1.

Table 5.6.2-2 below describes the additional parameters used in steps 12 or 15 of clause 5.6.1.

Table 5.6.2-2: Additional consumption reporting parameters

| Parameters | Description |
| --- | --- |
| Media player entry pointer | Identify the Media player entry. In case of DASH, the media player entry pointer can be a URL of the MPD. |
| Consumption reporting client ID | Identify the identifier of the UE that consumes the data. |
| Location | Identify the UE location. This parameter is only used when the location reporting is enabled for the UE or for the Downlink Streaming session with a condition that the UE allows to share its location within operator’s trust domain. |
| Media consumed | Identify the media consumed. In case of DASH, the AdaptationSet@id may be used. |
| Start time | Define the time when the consumption starts. |
| Duration | Define the duration of the consumption of the media consumed. |

================================END OF SECOND CHANGE==============================