Agenda Item:	8.3

Source:	Ericsson LM

Title:	Discussion on LS on method for collection of data from the UE

Document for:	Discussion and Agreement

## 1. Background

In the LS S2-2006292, SA2 asks SA4 to comment on certain aspects regarding how the NWDAF can collect information from UE applications. Two different top-level architectures (solutions #27 and #64) are sketched, and SA2 asks for feedback regarding feasibility for these. Both architectures are based on the following SA2 interim conclusions (see TR 23.700-91, clause 8.8):

- The user plane based solution is agreed for UE data collection. NWDAF interacts with an AF in the MNO domain or an AF external to MNO domain to collect UE data as an input for analytics generation. The data provided by the MNO AF or external AF to NWDAF is based on collected data from UE over user plane.

- The UE Application is configured with the address of the AF to contact. The UE Application may be configured to contact an AF in MNO domain (i.e. a trusted AF) or an AF external to the MNO domain (i.e. an untrusted AF). How the new UE data is collected by the AF may be developed by SA WG4 and feedback from SA WG4 will be considered if there is any SA WG2 work to be done.

- The NWDAF to (MNO) AF interface based on Naf_EventExposure is reused and should be extended if new input data from the UE based on user plane data collection as above is needed.

## 1.1 Solution #27

"Solution #27" is described in TR 23.700-91: The UE application exchanges information with an MNO AF, which in turn communicates with the NWDAF, as shown in Figure 6.27.1.1-1 below.

Figure 6.27.1.1-1: Schematic picture of the setup

## 1.2 Solution #64

"Solution #64" is also described in TR 23.700-91 (and was proposed during SA2#140e): The UE application exchanges information with an ASP Server, which in turn communicates with an MNO-AF (also called DC-AF) over a (possibly enhanced) M1d interface, which then communicates with the NWDAF. It is also specified that "the communication between ASP client and ASP server is out of 3GPP scope". Figure 6.64.1.1-1 below shows the proposal.

Figure 6.64.1.1-1

## 1.3 Other

SA2 also asks:

- For solution #27, if the IP address of the UE can be acquired by the MNO AF, and included in the exchanged information for correlation purposes.

- If the solutions are applicable to both 3GPP and Non-3GPP services.

## 2. Discussion

## 2.1 Architecture solutions

The current 5GMS architecture and UE<->Network metrics information exchange protocols defined in TS 26.501 are shown in Figure 5.5.3-1 below (see CR S4-201218 which was agreed at SA4 #110 but has not yet implemented):

Figure 5.5.3-1: Metrics collection and reporting via 5GMSd AF-based configuration

For SA2 solution #27, data from the UE would go via arrows #15 above (Media Session Handler on the UE to a trusted MNO 5GMSd AF) and #18 (MNO 5GMSd AF to an OAM Server), where the OAM Server in this case would be replaced by an NWDAF. Note that SA2 treats the UE Application as a monolith, so the Media Session Handler and Media Player are not explicitly described in their solution.

For SA2 solution #64 (although it states that the communication between ASP client and ASP server is out of 3GPP scope) this communication could in principle go via arrow #13 (Media Session Handler on the UE to an untrusted AF), and then possibly further via arrow #17 (untrusted AF to the ASP). Or it could go totally over-the-top from the UE to the ASP as well. However, the next path from the ASP to the MNO AF (DC-AF) via the M1d interface is currently not described in TS 26.501, as client data is assumed to reach the AF via M5d.

Regarding configuration of the UE Application (i.e. what to measure and report) in 5GMS, this is contained in the Service Access Information (SAI). The SAI can either be included when the UE Application is first configured by the ASP (see step #1 in Figure 5.2-1 from TS 26.501 below), or the UE Application might only get a reference to the AF which has the SAI (and fetching is then done at step #4).

Figure 5.2-1: High Level Procedure for progressive download for on-demand media

This is similar to the SA2 solution #27 (see figures 6.27.1.2-1 and 3-1 below) , where the UE Application is first given an address to the AF, and later fetches the configuration of what to report.

Figure 6.27.1.2-1: UE ASP Application Client connects to MNO AF for data collection

Figure 6.27.1.3-1: UE ASP Application Client Data provided to NWDAF

For solution #64 the configuration of the UE is described as being out of 3GPP scope, so no comparison with 5GMS and the SAI seems to be relevant.

Going down from the architecture level into more details, some subtle differences can be noted:

- SA2 proposes that SLA-related authentication information exchange is mandatory in the provisioning data, both for the MNO-AF and the UE App, while such authentication is currently not explicitly mentioned in 5GMS. Such authentication should probably be described also in 26.501, and not only for the metrics reporting functionality.

- SA2 proposes that the UE App provides the UE IP address/prefix and an external Application Id, while the NWDAF provides the SUPI and the internal Application Id. In 5GMS we currently do not specify any such application id:s in the reporting (the closest match would be either the metricsReportConfigurationId, or the scheme URI, see Table 7.8.3-1 below).

- The SAI contains much more than only the metric reporting configuration, so it might need some adaptation so that the metrics part can be used "more stand-alone" for SA2 purposes.

## 2.2 IP address

Regarding the UE IP address, the most straightforward way for SA2 solution #27 is that the IP address is read from the IP headers by the MNO AF. 5GMSd AFs in the external domain might not be able to see the real UE IP address (due to a possible presence of a NAT between the UE and the UPF), but this should not be an issue for the MNO AF.

## 2.3 Non-3GPP services

The current 5GMS architecture already describes a potential non-3GPP usage via an "URI tag", but exactly how this will be done is not yet described. One possibility is that black-box configuration and reporting containers are exchanged, but in such a case it is not easy to see how this information could be made useful for the AF and the NWDAF.

However, the principles of SA2 architecture #27 can be equally well applied to 3GPP and non-3GPP services, and it seems that if 5GMS is extended with Application-ID it would support both 3GPP and non-3GPP cases.

## 3. Conclusions and possible normative work

So the conclusions regarding the SA2 questions would be:

- The analysis by SA4 has been based on the current 5GMS architecture, and SA4 notes that if 5GMS is selected as a basis some adaptions are required.

- Solution #27 is feasible and seems to fit well into the 5GMS architecture, although some enhancements are likely needed (add authentication, add application id, specify how to connect NWDAF and MNO AF, adapt the SAI).

- Solution #64, where most of the data path is formally outside scope of 3GPP, is also feasible, and could possibly re-use either the M1d or the M5d interfaces to feed client data to the MNO AF. But probably some enhancements are needed (adapt M1d or M5d, specify how to connect NWDAF and MNO AF).

- The IP address could just be read from the IP header by the MNO AF.

- 3GPP and non-3GPP services can be supported.

Normative work would likely be needed to:

- Adjust the SAI concept so that also non-streaming 3GPP services can use the relevant parts of 5GMS for client reporting purposes.

- Specify how authentication is handled between the 5GMS entities.

- Add relevant identities (application id, IP address etc.) where needed in the 5GMS reporting structure.

- Describe the usage of non-3GPP services in more detail.

- Describe how the MNO AF can serve NWDAF and possible other entities with information. This could also be related to the discussion proposal in S4aI201072 "AF Exposure" by Qualcomm.

## 4. Proposal

We propose that:

- The reply to SA2 is based on the positive feasibility conclusions as described above.

- That a study item (or possibly a work item) is started to look into the above issues.