SA WG4 Meeting #S4-112E	S4- 210235

1 - 10 Febrary, 2021, Electronic meeting

Title:		Draft LS on Server Domain Name Usage for Application Traffic Detection

Response to: 	—

Release:	Rel-16

Work Item:	5GMS3

Source:	SA4

To:		TSG SA2, TSG CT3

Cc:

Contact person:	Qi Pan <panqi8@huawei.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

SA4 has specified the 5G Media Streaming (5GMS) architecure within Release 16, which enables usage of some Network Features using N5 or N33.

For dynamic policy invocation, an ASP can configure one or more Policy Templates within the 5GMSd AF, which can then be individually invoked by a 5GMSd Client. The 5GMSd Client provides a Service Data Flow description (IP Packet Filter Set or server domain name(s)) when activating a dynamic policy to be applied by the 5GMSd AF. The 5GMSd AF then acts as AF towards the PCF or NEF.

For using server domain names within a Service Data Flow description, SA4 understands that the 5GMSd AF first needs to provision a Packet Flow Description (PFD) containing a domain name using the PFD Management API, and then it selects the provisioned PFD (part of a PfdData object) using either the N5 Npcf_PolicyAuthorization API or else via the N33 AfSessionWithQos or ChargeableParty APIs. Note that a single 5GMSd AF might provision multiple PfdData objects, each containing one or more PFDs.

Furthermore, SA4 understands that in the subsequent dynamic policy invocation procedures defined for the 5FMSd AF, previously provisioned PFDs can be referenced as an alternative traffic detection parameter to full flow descriptions.

- When the Policy Template relates to QoS, the 5GMSd AF may use the Nnef_AFsessionWithQoS service as defined in TS 23.502 [3] clause 5.2.6.9. (The complete call flow is described in TS 23.502 clause 4.15.6.6);

- When the Policy Template relates to a different charging scheme, the 5GMSd AF may use the Nnef_ChargeableParty service as defined in TS 23.502 [3] clause 5.2.6.8.

Based on the above assumptions, SA4 would like to ask the following clarification questions to SA2 and CT3:

1:	When invoking the Nnef_AFSessionWithQoS method, is it possible to use the application identifier (as a reference to a set of PFDs) instead of the flow description?

2:	What is the relationship between the AF Identifer (SCS/AS ID in 4G) and the AF application ID and/or the application identifier?

3:	When invoking a particular policy using Nnef_AFSessionWithQoS, is it possible to provide the DNN, the S-NSSAI, and/or a reference to the server domain name for the purposes of traffic identification?

NOTE:	The same questions apply to Nnef_ChargeableThird Party API.

# 2	Actions

To SA2

ACTION: 	SA4 kindly asks SA2 to provide feedback for question 1 and question 3.

To CT3

ACTION: 	SA4 kindly asks CT3 to provide feedback for question 2.

# 3	Dates of next TSG SA WG4 meetings

SA4#113E		6 April – 14 April 2021				             Electronic meeting

SA4#114E		19 May – 28 May 2021					Electronic meeting