Source:	Huawei

Title:	AR media type classification and transporting solution

## Document for:	Discussion and Agreement

## Agenda Item:	9.5

# 1	Introduction

At the 3GPP-SA4#120-e, the Permanent Document for MeCAR v3.0.0 [1] was approved. The Permanent Document specifies AR/MR media type definition and real-time characteristics.

This contribution provides AR/MR media type classification and transporting solution.

# 2	AR media type classification

According to clause 3.7.3 of MeCAR v3.0.0 PD, the AR media can be classified to three categories according to its real time characteristics:

1) Non timed: Scene Description/3D Model/FOV/Sensor information.

2) Sparsely timed (event based): Spatial Description/AR Anchor/Camera information/Projection information.

3) Continuous: User Pose/Viewport/Gesture/Body action/Facial expression.

Therefore, the AR media transporting solution should support unreliable, partial reliable and reliable for each categories.

# 3	AR media transporting solution

In line with clause 2, there are different transporting requirements for each AR/MR media, we consider two solutions, RTP and IMS (or WebRTC) data channel.

## 3.1 RTP/RTCP Solution

RTP/RTCP over UDP is used for real-time media transporting. Currently, RTP/RTCP is mainly used for audio/video media transporting, it is very mature.

### 3.1.1. RTP header extension

According to clause 5.3.1 of RFC3550[3], 16 bits can be used for extension, the maximum length used for AR/MR media can be 65535 bytes. AR/MR media can be carried together with audio RTP packet because video may not be available.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      defined by profile       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                        header extension                       |

|                             ....                              |

### 3.1.2 RTCP extension

According to clause 6.7 of RFC3550[3], RTCP packet with PT=APP can be used to send AR/MR media, which specifies subtype with the AR/MR media type. The maximum length can be 65535 bytes.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=2|P| subtype |   PT=APP=204  |             length            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                           SSRC/CSRC                           |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                          name (ASCII)                         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   application-dependent data                ...

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

### 3.1.3 New RTP media channel

All AR/MR medias transmission will use this new media channel beyond audio/video channel. Considering different AR/MR media types may have different transporting frequency, a dynamic payload type is used for the new channel. To distinguish AR/MR media type, an extension on RTP header extension is needed. Here is an example:

SDP:

m=application 3238 UDP 100

a=rtpmap:100 xr-media

RTP:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      defined by profile       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|             header extension: AR/MR media type                |

|                             ....                              |

### 3.1.4 Summary

|  | Solution1: RTP header extension | Solution2: RTCP extension | Solution3: New RTP media connection |
| --- | --- | --- | --- |
| Adv. | Easy to extend | 1.Easy to extend 2. No need extra transmission path. | 1.Independent channel beyond audio and video. 2.All AR/MR media data transmission can reuse the new connection. |
| Dis. | 1.It will make the RTP header to be too big, which may bring new problems to audio packet transmission. 2. Some AR/MR media data such as scene description and 3D model are too large to fit into one RTP packet, and needs to be divided into multiple pieces for transmission. 3. The application layer needs to add a flag in front of the AR/MR media data content to indicate the AR/MR media type. 4. The AR/MR media transmission frequency should be the same with audio, which can’t meet the requirements for AR/MR media. | 1. RTCP packet will send with a limited interval (0.5s to 1.5s), can’t meet the requirement of AR/MR media transmission, for example the tracking frequency for gesture should be at least 1000Hz according to clause 4.5.1 of 3GPP TR26.998[2]. 2. Some AR/MR media data such as scene description and 3D model are too large to fit into one RTCP packet, and needs to be divided into multiple pieces for transmission. 3. RTCP packet will stop sending when the network is in congestion. | 1. Application level needs to map RTP header extension tag to AR/MR media type. |

From the comparison above, it is easily to achieve a conclusion the only solution over RTP is using a new RTP media connection for AR/MR media transmission.

## 3.2 Data Channel Solution

According to clause 6.5.5 of 3GPP TR 26.998[2], IMS data channel can be used for AR/MR media (AR-specific data) transmission. IMS data channel over SCTP is derived from WebRTC data channel which is widely used web real-time communication.

Data channel can set flexible reliability modes for different streams id, by setting “max-retr” and “max-time” parameters (defined in RFC8864[5]) in each a=dcmap during data channel establishment.

Application can use stream id for different AR/MR media type identification. Here is an example:

m=application 1002 UDP/DTLS/SCTP webrtc-datachannel

a=max-message-size:100000

a=sctp-port:5000

a=setup:actpass

a=fingerprint:SHA-1 \

4A:AD:B9:B1:3F:82:18:3B:54:02:12:DF:3E:5D:49:6B:19:E5:7C:AB

a=tls-id:abc3de65cddef001be82

a=dcmap:1001 subprotocol="http"; max-retr=3; max-time=10 //scene description

a=dcmap:1002 subprotocol="http"; max-retr=5; max-time=15 // AR Anchor

a=dcmap:1003 subprotocol="http"; max-retr=0; max-time=0 // User Pose

Note: “max-retr” parameter indicates the maximal number of times a user message will be retransmitted. “max-time” parameter indicates that a user message will no longer be transmitted or retransmitted after a specified lifetime. If both of the value are set to 0, it indicates the unreliable mode.

For the scenario that different AR/MR media type (for example, both no timed and partial timed media are used in an application) have different Qos requirements, the application can set different “3gpp-qos-hint” attribute defined in 3GPP TS 26.114[4] for different stream id with different m line, an example is shown as follows:

m=application 1002 UDP/DTLS/SCTP webrtc-datachannel

a=max-message-size:100000

a=sctp-port:5000

a=setup:actpass

a=fingerprint:SHA-1 \

4A:AD:B9:B1:3F:82:18:3B:54:02:12:DF:3E:5D:49:6B:19:E5:7C:AB

a=tls-id:abc3de65cddef001be82

a=dcmap:1001 subprotocol="http"; max-retr=3; max-time=10

a=3gpp-qos-hint:loss=0.001;latency=300

m=application 1002 UDP/DTLS/SCTP webrtc-datachannel

a=max-message-size:100000

a=sctp-port:5000

a=setup:actpass

a=dcmap:1002 subprotocol="http"; max-retr=5; max-time=15

a=3gpp-qos-hint:loss=0.002;latency=400

## 3.3 Summary

From the analysis above, both RTP and data channel solutions for AR/MR media transmission are available, a comparison is shown as below:

|  | RTP Solution: New RTP media connection | Data Chanel Solution |
| --- | --- | --- |
| Adv. | RTP protocol is mature for real time media transmission with low latency and high reliability mechanism | 1. The application can be decoupled from the terminal architecture, and the application can customize the transmitted data and data format, which can be expanded flexibly without terminal changes; 2. The application does not need to be pre-installed or built-in on the terminal. |
| Dis. | RTP extension is difficult, it needs chip support and to be open to upper layer applications. | 1. The delay of SCTP link establishment is large due to its 4RTTs interaction. 2. The forced SCTP over DTLS encryption and decryption performance is poor. 3.SCTP has no FEC mechanism. |

# 4	Proposal

Due to rapid progress of AR technology, AR/MR media data will expand frequently. In order to avoid the continuous expansion of RTP protocol, considering that SA2 is studying IMS data channel architecture, it appears advantageous to use data channel to transmit AR/MR media.

We propose to agree to include relevant parts of Clause 2 and Clause 3 of this contribution in the MeCAR PD.

# References

- 3GPP TSG SA WG4 S4-221150, “MeCAR Permanent Document v3.0”, August 2022

- 3GPP TR 26.998: "Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices ".

- RFC3550: "A Transport Protocol for Real-Time Applications".

- 3GPP TS 26.114: “IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction”

- RFC 8864: “Negotiation Data Channels Using the Session Description Protocol (SDP)”.