Source:	Nokia Corporation

Title:	Clarifications for ATIAS FOA audio tests

Document for:	Discussion

Agenda Item:	7.6 – ATIAS

# 1. Introduction

The ATIAS work item develops test specifications for objective characterization of terminals for 3GPP immersive services, including conversational services and non-conversational services. Dolby input [1] proposes acoustic performance tests for FOA audio.

In this document, we present some questions for clarification. It is the intention of the source to, e.g., replicate some of the measurements according to Dolby’s proposal to allow for verifying the ATIAS test methods and developing them further.

# 2. Points for clarification

We identified the following aspects in [1], where some further clarifications would be useful.

# “Suggested test method and parameters to evaluate”

The test signal characteristics are suggested for evaluating the spatial capture accuracy. However, the attributes of the proposed signal are ambiguous or not explained in detail.

The proposed signal is characterised to have a harmonic structure in a frequency domain. In the subclause 4, the spacing of harmonic frequency components is said to be based on 1/3rd octaves, and the tone should cover the range 250 Hz – 19 kHz. However, in Figure 4, the spacing of the frequency components is based on 1/4th octaves. Furthermore, by utilizing the 1/3rd octave spacing, the signal would cover the range up to 20 kHz, while with the 1/4th octave spacing the range would match with the suggested range.

In the document, it is suggested that the test signal is AM and FM modulated. It is left unclear whether the signal is constructed by utilizing e.g., FM modulation or is the test signal modulated afterwards. Moreover, the modulation frequencies as well as other signal parameters are not presented in the document in numeric form or otherwise in sufficient detail. Even if the exact parameters are not considered to be relevant, some initial values would help to further assess the necessity and the impact of the proposed modulation schemes for the suggested test method.

In addition to the presented power spectra of the proposed test signal, the signal’s time domain properties would help to evaluate the applicability of the suggested test signal further. The parameters of interest would be at least the signal length and the length of possible fade-in/fade-out.

It is stated that the presented test signal is inspired by Recommendation ITU-T P.501 [2]. Is the intention to further study the possibility of using the test signals presented in [2] or is the suggested test signal already derived from [2]?

# “Level measurement”

The level difference of measured components is proposed to be evaluated according to Table 1 in Document [1]. The target level differences were left to be further decided. In order to evaluate whether the proposed assessment method is appropriate and replicable, it would be helpful to obtain measured and calculated level difference results of a previously performed test.

It is proposed that the level of the signals is calculated by summing the power of the selected bins. In Figure 5, the selecting of the bins is done by applying masks on the FFT bins. It is further stated that the power spectra should be calculated by utilizing 64k long FFT at 48 kHz sampling rate with the Hann window with 50% overlap. Could the purpose of the windowing be further clarified?

# 3. Summary

We have identified some points for clarification to allow for replicating the test configuration in [1].

# References

[1]	Tdoc S4-220729: On ATIAS acoustic performance testing for FOA audio, Dolby Laboratories, Inc.

[2]	Recommendation ITU-T P.501, Test signals for use in telephony and other speech-based applications