3GPP TSG-SA WG4 Meeting #127bis-e 	S4-240593

# Online, April 08-12 2024

Source:				Fraunhofer IIS

Title:					[ISAR] Fraunhofer Listening Lab Report

Document for:		Information & Agreement

Agenda Item:		7.7

## 1	Overall Summary

This listening lab report provides the listening test results for the ISAR candidate, conducted by the solution proponent laboratory "Fraunhofer IIS".

Contained are listening test results for BS1534-2 (Multi-Channel 7.1+4) and BS1534-3 (Objects). The system under test meets the performance requirements and achieves mean results in the “Excellent" range for all tested items. For test BS1534-2 confidence intervals for the system under test and the hidden reference overlap for each item and overall.

The source proposes to complement the listening test results, proposed by the proponent of the profile with the results of the tests and document them, along with the proponent’s results.

## 2	Test Setup

### 2.1	BS1534-2

The subjective listening test was carried out based on the processed listening test material provided by Dolby on box.com. The test procedure was a “MUlti Stimulus test with Hidden Reference and Anchor (MUSHRA)” based on Recommendation ITU-R BS.1534-3 [1] for the subjective assessment of intermediate quality audio for assessing the ‘Basic Audio Quality’ attribute described in [1].

The test was conducted over headphones as defined in [2]. Tests were run in acoustically isolated rooms using STAX electrostatic headphones.

In each test presentation of test signals, the presentation order was randomized for each listener.

Due to limited availability of assesors that could participate at on-site test, some assesors participated also in test BS1534-3. Further information on the test panel is provided in clause 3.

### 2.1	BS1534-2

The subjective listening test was carried out based on the processed listening test material provided by Dolby on box.com. The test procedure was a “MUlti Stimulus test with Hidden Reference and Anchor (MUSHRA)” based on Recommendation ITU-R BS.1534-3 [1] for the subjective assessment of intermediate quality audio for assessing the ‘Basic Audio Quality’ attribute described in [1].

The test was conducted over headphones as defined in [2]. Tests were run in acoustically isolated rooms using STAX electrostatic headphones.

In each test presentation of test signals, the presentation order was randomized for each listener.

Due to limited availability of assesors that could participate at on-site test, some assesors participated also in test BS1534-3. Further information on the test panel is provided in clause 3.

## 3	Test Panel

Assessors were recruited from the staff of Fraunhofer IIS. All listeners have a background in audio processing and high levels of experience in MUSHRA listening tests. No assessor was involved in developing the split rendering features in the system under test. To ensure unbiased testing, only participants were chosen that were considered to be expert listeners and the test was conducted in a blind fashion, providing as only guidance the following summary of the project:

Technical Background: When devices have to be power-efficient (e.g. wireless headphones) complexity matters. Complexity when decoding and rendering head-tracked immersive audio could be offloaded to another device, e.g. a smartphone, and the binaural stereo signal is just sent. Optimizing this connection is the scope of the ISAR work. You will listen to binaural audio.

Of the listeners 3 were female, 14 male. 5 listeners participated in both tests. Listener age was not asked for data protection reasons.

In compliance with BS.1534-3 we used following rules for post-screening in Tests BS1534-2 and BS1534-3:

- none of the hidden references are supposed to be rated below 90

Table 1: Listeners per Test

| Test | Listeners | After Post Screening |
| --- | --- | --- |
| BS1534-2 | 11 | 10 |
| BS1534-3 | 11 | 10 |

## 4	Results

### 4.2	BS1534-2 (Multi-Channel 7.1+4)

#### 4.2.1	Ratings

Figure 1 Mean rating with 95% confidence interval of MUSHRA scores for Test BS1534-2

Figure 1 shows mean ratings with 95 % confidence interval (computed assuming normally distributed data and applying a t-distribution) for each item as well as pooled over all items. For each item, the system under test shows a mean score in the “Excellent” range and not worse than the hidden reference.

Table 2: Mean values and confidence intervals for Test BS1534-2

| Condition | Mean | Ci_high | Ci_low |
| --- | --- | --- | --- |
| Hidden Reference | 99.1 | 99.4 | 98.7 |
| LP7 Anchor | 26.7 | 28.8 | 24.7 |
| 0-DOF native transcoding reference | 66.6 | 69.3 | 63.9 |
| System under test | 98.5 | 99.2 | 97.8 |

#### 4.2.2	Evaluation

The system under test achieves mean results in the “Excellent" range for all tested items. Confidence intervals for the system under test and the hidden reference overlap for each item and overall.

### 4.3	BS1534-3 (Objects)

#### 4.3.1	Ratings

Figure 2 Mean rating with 95% confidence interval of MUSHRA scores for Test BS1534-3

Figure 2 shows mean ratings with 95 % confidence interval (computed assuming normally distributed data and applying a t-distribution) for each item as well as pooled over all items. For each item, the system under test shows a mean score in the “Excellent” range.

Table 3: Mean values and confidence intervals for Test BS1534-3

| Condition | Mean | Ci_high | Ci_low |
| --- | --- | --- | --- |
| Hidden Reference | 99.2 | 99.5 | 98.8 |
| LP7 anchor | 32.3 | 34.2 | 30.3 |
| 0-DOF native transcoding reference | 67.8 | 70.6 | 65.0 |
| System under test | 94.4 | 95.8 | 93.1 |

#### 4.3.2	Evaluation

The system under test achieves mean results in the “Excellent" range for all tested items.

## References

[1]		ITU-R recommendation BS.1534

[2]		3GPP Tdoc S4sA240013: ISAR Pdoc on Testing Aspects for Phase/Track/ 2/a