Source:	Nokia Corporation

Title:	Example design of parametric spatial audio capture for multi-microphone devices

Document for:	Discussion & Agreement

Agenda Item:	7.8 – FS_DaCED

# 1. Introduction

Providing immersive voice and audio services by end-user devices is becoming increasingly practicable with the development of 4G/5G technologies. For example, smartphones can capture immersive audio by utilizing multiple microphone inputs; head-tracked headphones provide the capability to present binaural audio with high immersion level to end users; and the 3GPP IVAS codec handles encoding, decoding, and rendering of several different immersive formats and immersive format combinations.

Due to their physical constraints relating to shape and size, the end-user devices are typically configured, e.g., with different numbers of microphones and different microphone array layouts. Therefore, different audio capturing capabilities are in practice realized for different devices.

This input document describes an example design of practical spatial audio capture for a multi-microphone smartphone device based on parametric spatial audio capture. The example can be generalized for other device form factors and UE types. The example is also directly applicable, e.g., for generation of the MASA format encoder input for the 3GPP IVAS codec.

The source proposes to include the description in draft TR 26.933 under clause 8 (Example audio capture processing solutions).

# 2. Example design

This clause describes the proposed example design. The following numbered subclauses are proposed for inclusion in draft TR 26.933 under clause 8.

# x.1 Overview

Support of immersive voice and audio services by end-user devices requires successful combination of several audio technologies and appropriate product design for taking full advantage of the new immersive capabilities. Relevant devices (UE) can come in many shapes and sizes (form factors) for different use cases, and even traditionally dominant UE form factors such as mobile devices can be expected to be used in new ways, e.g., multiple orientations (landscape and portrait) in different use cases and applications that provide immersive voice and audio communication.

Practical application of spatial audio capture technologies includes multiple processing stages that need to fit together to achieve the desired audio capture performance. The following clause discusses technology HW adaptation aspects that relate to different UE designs. This is relevant because device HW and transducers have significant impact on the captured audio signals. These differences can be addressed with audio front-end processing that compensates HW differences for parametric audio capture processing.

# x.2 Device (UE) characteristics

## x.2.1 General

Support of immersive communication requires favorable integration of electroacoustic transducers, namely microphones and loudspeakers. Smartphones are currently defined by a large display and a slim body, which is a challenging shape for spatial audio capture devices. A device’s shape influences the spatial capture characteristics and favorable spatial capture requires typically parametric audio capture techniques to support more uniform performance for different sound source directions. Current smartphones typically have at least 2 microphones, which is sufficient to support immersive voice and audio capture for a wide range of use cases and applications.

While immersive voice and audio playback can be based, e.g., on rendering for headphones or external loudspeaker systems, stereo and immersive playback are possible to support also independently of any accessory devices on the mobile device itself. The number of loudspeakers and their locations in the device together with the rendering processing have an impact on the perceived immersion of the loudspeaker playback. Loudspeaker placement relative to the microphone locations influences both the (spatial) audio capture performance and requirements of the Acoustical Echo Cancellation (AEC) solution.

## x.2.2 Number and placement of microphones

Spatial audio capture requires at least two microphones, but for rich capture of audio scenes more microphones are recommended. Two microphones can be used to support stereo recording, and it can be recommendable to implement symmetrical microphone integration and placement at the ends of the device for achieving a symmetric stereo image.

Spatial audio capture supporting 360-degree spatial capture in 2D plane can be supported with 3 microphones. It is common that stereo recording microphones are complemented with a third microphone integrated on the back cover of the device supporting spatial audio capture aligned with the optical axis of the main camera.

The practical aspects related to microphone placement include the detailed mobile device form factor (geometry), which is related to how the device is used, and how user’s hands might interfere audio capture, e.g., either by blocking or disturbing any of the microphone inputs.  Acoustic performance of 360-degree capture is dependent on microphone placement to ensure robust direction analysis for different sound source directions.

For immersive voice and audio use cases with higher spatial fidelity and flexibility, 4 or more microphones can be considered for supporting a full 3D spatial audio capture in both landscape and portrait device orientations. Multiple microphones can improve spatial resolution of audio capture and increase robustness in different usage environments, e.g., by allowing more effective reduction of disturbing wind noise.

Sufficient distance between microphones and loudspeakers can improve AEC performance by reducing the echo level coupling from loudspeakers to the microphones. More details on loudspeaker placement are provided in the following.

## x.2.3 Microphone integration

Microphone capture is defined by the electroacoustic properties of the microphone transducer and acoustic properties of the microphone integration. Microphone integration on mobile devices is typically product specific and different products have variation between microphone assemblies due to differences in product mechanics and electronics. Smartphones use MEMS microphone technologies that are typically either top port or bottom port type depending on the product mechanics and preferred PWB assembly. The use of MEMS microphone type influences the microphone port geometry. In smartphone design, microphones in the top and bottom ends can be integrated more similarly to each other, see Figure 1, and when microphones are integrated on other surfaces, e.g., next to the main camera, the design needs to use different mechanical structures. In addition to differences in microphone integration, the acoustic diffraction from rigid body of the product will influence the overall frequency response of the microphone. Acoustic diffraction is highly dependent on sound source direction due to the non-uniform shape of the smartphone body.

Figure 1. Integrated microphone frequency responses (incl. port and transducer) for stereo microphone integration in both ends of a commercial smartphone device.

In general, it is unlike for a practical mobile device, especially with more than 2 microphones, to have similar microphone frequency response characteristics for all microphones. Two commercial mobile devices with 4 microphones were analyzed indicating differences between typical smartphone microphone integrations (see Figure 2). Microphone frequency responses illustrated in Figure 2 provide examples where resonances are clearly audible and need to be compensated with microphone signal equalization to enable high-quality audio capture.

Accurate frequency response data for equalization filter design can be created using multiple techniques, such as acoustic measurements in anechoic chamber, use of lumped parameter component models, or application of numerical acoustic simulation tools. The numerical acoustic methods require detailed acoustic port geometry, which can include information about dust and liquid protection filters or membranes. These filter materials have acoustic impedance that is considered in the acoustic design process. Usually, acoustic characterization of microphone integration is done, or at least verified, using acoustic measurements, because equalization relying on inaccurate frequency response data can lead to further issues.

Due to these practical limitations in product design, it would be unrealistic to assume that pressure on the device surface, at microphone port inlet, could be used as an approximation of the pressure on the integrated microphone transducer diaphragm. For example, if this would be the case, the frequency responses, such as presented in Figure 1 and Figure 2, would all be flat.

Furthermore, microphone signal equalization is required to comply with requirements as set forth in [2] for (wideband) sensitivity/frequency characteristics.

Figure 2. Integrated microphone frequency responses (incl. port and transducer) for two commercial smartphone devices with 4 microphones.

In microphone integration it is favorable to aim for simple mechanical structures that implement a uniform acoustic channel from the surface of the device to the microphone sensor without unnecessary bending, cavities, and other variation in the cross-sectional area of the sound channel. This is commonly achieved when the microphone component is integrated close to the device surface, minimizing the length of the sound port relative to its cross-sectional area. As the hole in the device cover is typically desired to be either small or visually hidden, the acoustic targets of the design are generally best achieved when the length of the microphone port is kept short.

Robust microphone operation during the lifetime of the mobile device requires sealing of the acoustic channel. Sealing and IP protection filters are commonly integrated for easier assembly. The acoustic impedance of the different acoustic filter materials should be included in the numeric analysis of the acoustic channel unless the impedance is low enough to be insignificant. Practical design guidelines are available, e.g., in [1].

All microphone integrations include some acoustic filtering effects that typically include resonance. Favorable microphone integration can be characterized by few parameters that should be similar for all microphones: microphone resonances are at high frequencies (>10kHz) and resonance magnitude is small with low Q factor. When all microphones of the device are integrated with similar good design practices, this provides a good basis for multi-microphone capture algorithms.

Acoustic scattering from the rigid body of the device is a significant factor in spatial audio capture algorithms for multi-microphone devices. It can be said that free-field approximation using microphone positions will produce inaccurate results especially at high frequencies.

When a device prototype is available for audio design, it is possible to perform acoustic measurements in an anechoic chamber to measure and analyze frequency responses for each integrated microphone as a function of source direction. This is technically a relatively straightforward measurement task, e.g., based on using a turntable to measure multiple source (azimuth) directions in a 360-degree plane with the same measurement setup. 3D measurements, including azimuth and elevation directions, require a short series of measurements with different device orientations, or turntable measurements with multiple sound sources. Alternatively, frequency responses can be simulated using numerical acoustic methods.

## x.2.4 Loudspeaker number and placement of loudspeakers

In order to produce immersive playback over loudspeakers integrated into a mobile device, at least two loudspeakers are needed. When playback is active, the number and placement of the loudspeakers has an effect also on the spatial audio capture, e.g., due to interaction with AEC. In the following, some relevant observations and guidelines are provided.

Two loudspeakers in a mobile device enable direct stereo content playback and, with suitable rendering processing, can provide an adequate level of immersive experience for the front of the listener. It is straightforward to achieve a wider stereo image extending the physical loudspeaker locations, but creating real feeling of immersion around listener’s head so that sounds are perceived as coming also from behind is very challenging.

Using similar speaker components and placing the speaker outlets symmetrically in the device generally provide the best basis for a high-quality playback. It is beneficial to have the loudspeaker outlets at the device ends so that when listened to in landscape mode, the stereo image will be at its widest. If the loudspeaker ports are facing towards the user, the spatial audio quality can be further improved, and the audio performance is more consistent and independent of device handling, namely interaction effects with user’s hands and grip. More than two loudspeakers can be used especially in larger mobile devices, e.g., in tablets and foldable smartphones to provide favorable stereo loudspeaker pairs at different usage orientations and device configurations.

# x.3 OS interfaces and IVAS applications

Simultaneous access to all raw microphone signals or microphones with known frequency response characteristics that are part of the spatial audio capture need to be supported via the appropriate OS interfaces. Support for immersive voice and audio services in mobile devices requires either low level integration under Hardware Abstraction Layer (HAL) or OS support providing access to multiple microphones for applications. Different OS platforms have their own architectures and technical implementations, and there is no common interface for all platforms.

# x.4 Immersive audio capture with mobile devices

## x.4.1 Overview

Parametric spatial audio capture can be utilized for immersive voice and audio services for a wide range of device form factors with different microphone number and placement. The parametric capture methods can be motivated, e.g., by difficulty of applying linear means for audio format generation for a specific device configuration, concerns over audio quality of alternative approaches (e.g., linear methods), and potential advantages in computational complexity of the spatial audio capture processing. While parametric capture can be combined with a synthesis step to transform the original spatial audio representation into another format, it can be advantageous to maintain the original format to not introduce additional computational complexity, to not increase the number of audio signals (data size) through an up-mixing, to not increase latency, or to not degrade the audio quality unnecessarily. In this case, a suitable parametric format is needed to provide interoperability for different capture implementations and other processing blocks, e.g., audio codecs.

A typical example of a parametric spatial audio format is one or more audio signals provided together with spatial metadata consisting of several spatial parameters. One practical example is the Metadata-assisted spatial audio (MASA) format used with the IVAS codec and defined in 3GPP TS 26.258.

Parametric capture methods utilize algorithms that derive selected spatial information from the captured microphone signals based on knowledge of the device size and shape, and the placement and other properties of the microphones themselves. In general, the algorithms can support, e.g., a varying number of microphones, and they can be easily adapted to different device configurations considering, e.g., the number of microphones and their placement. This makes these methods suitable for practical form factors, e.g., mobile devices.

Figure 3 shows an example implementation of parametric spatial audio capture pipeline on a mobile device.

Figure 3. Example implementation of parametric spatial audio capture.

## x.4.2 Microphone EQ

Microphone equalization smooths over resonance peaks and other microphone-integration related nonidealities from microphone responses. This makes microphone signals comparable to each other. Equalization is typically based on measured microphone responses. In general, equalization can be done independently for each microphone signal.

## x.4.3 AEC

AEC is typically performed separately for each microphone channel, but alternative solutions can also be considered. The target of AEC processing is to remove the loudspeaker signal component from the microphone signals based on a reference signal. In the case of stereo playback both stereo channels are needed as reference input for the AEC.

Traditional AEC solution has been to use linear AEC filter which is followed by residual echo suppression (RES). Nowadays RES is often implemented using DNN. Recently, also solutions in which the whole AEC is managed with a single DNN have been introduced.

## x.4.4 Audio enhancement

Audio enhancement block may consist of multiple different operations which aim at improving audio quality considering the targeted audio source or sources. These operations can include for example wind noise reduction, audio focusing, microphone noise reduction, and background noise reduction.

Wind noise reduction can operate independently for single microphone signal, or it can utilize signal analysis over multiple signals. Either traditional signal processing solutions or DNN-based processing methods can be applied.

Audio focusing can be used to focus spatial audio capture into preferred direction. The focus direction can be defined automatically or manually depending on the use case. With suitably selected microphone locations good focus performance can be achieved using beamforming. Audio focusing can be used for content creation or accessibility purposes.

Microphone noise reduction targets to remove self-noise generated by the microphones. It is generally based on defining the noise floor of the microphones.

Background noise reduction can be used to increase intelligibility of the captured speech. It generally makes audio more pleasant to listen to and can be used to remove irrelevant components from the captured audio. Depending on the use case, background noise reduction may only remove continuous noise such as air-conditioner noise or traffic noise, or it may, e.g., aim at removing everything but speech from the captured audio. Content or context-based classification and processing can be employed if different noise reduction processing is desired, e.g., for speech and music. For immersive voice and audio, background noise reduction settings may be more contextual than for traditional mono voice and audio. Noise reduction can also be required for accessibility purposes. DNN-based solutions are commonly used for noise reduction.

## x.4.5 Downmix and levelling

With parametric spatial audio capture, output spatial audio can be synthesized with one or two audio channels and metadata. Therefore, the number of audio channels can be reduced before synthesis. This can be achieved by selecting one or two representative audio channels from the audio enhancement block output. Leveling, i.e., typically automatic gain control (AGC), is then utilized to adjust the signals to a suitable level for listening. AGC typically also includes limiter which is designed to prevent signal saturation.

One benefit of parametric spatial capture is that the level management does not differentiate from the level management of normal stereo recording. The same capture front-end can therefore typically be used for mono, stereo, and parametric spatial audio capture with activation of additional channels and processing as needed.

## x.4.6 Spatial analysis

The target of spatial analysis in parametric spatial audio capture is to estimate spatial properties of the captured audio signal (audio scene) and generate metadata, which can be later utilized in spatial synthesis or rendering. Spatial analysis uses information of the device shape and the locations of the microphones. It is common to perform spatial analysis for frequency domain sub-bands, and analysis can be based, e.g., on coherence and level analysis between the microphone signals. To make sure spatial analysis works properly, it is important that echo is properly removed from the captured signal (see, AEC in clause x.4.3) and, on the other hand, that audio enhancement processing (see clause x.4.4) has maintained relevant spatial cues in the signals.

In an example spatial analysis, the direction of one or more dominant sound sources are analyzed for every sub-band. In addition, the directional energy and other spatial parameters are estimated. For example, the directional energy can be a ratio relative to the total energy of the corresponding sub-band. Together the analyzed parameters form the metadata for the parametric spatial audio format. The selected audio signals (e.g., mono or stereo) and the analyzed metadata can then be provided as input to spatial synthesis or used, e.g., in audio transmission (as codec input).

As discussed in clause x.2.2, depending on the device orientation and the number and positions of the microphones, as well as depending on the use case, spatial analysis can be done for the full 3D space or for example only for horizontal level (360 degrees in 2D plane).

# x.5 Spatial audio capture quality considerations

## x.5.1 Overview

An ITU-T P.800 Comparison Category Rating (CCR) AB comparison listening test was performed by Nokia Technologies in 2018 to evaluate multi-microphone spatial audio capture quality for mobile devices. The following provides description of the tested conditions, test items and configuration, and results.

The AB comparison test is based on binaural headphone listening with no head-tracking support.

Test content is captured using two mobile device capture setups. The first setup has three microphones and the other has four microphones in typical smartphone microphone positions. The design of these device setups follows the description in clause x.2. While the average size of smartphone devices has slightly increased since 2018, similar listening test results can be expected with current smartphone devices and device sizes.

## x.5.2 Conditions

The main tested conditions relate to two spatial audio captures (formats): a proprietary parametric spatial audio format and first-order Ambisonics (FOA). Two methods for rendering were considered for the FOA audio format: a linear and a proprietary parametric rendering. In addition, the conditions include stereo and mono that act as anchors.

The evaluation is based on uncompressed audio, and in the case of the parametric spatial audio format also uncompressed spatial information (metadata).

The parametric spatial audio format consists of two audio waveform signals (i.e., a stereo downmix) and one spatial information stream. Specifically, the stereo downmix signal in this experiment is produced by selecting a left and a right channel from the three or four raw microphone signals. This channel selection is fixed across the experiments. The bitrate of the uncompressed spatial information stream used here is approximately 25 kbps.

The FOA format in the evaluation consists of the four component signals (WXYZ), which are obtained based on a proprietary FOA up-mix/synthesis from the three or four captured audio input channels.

## x.5.3 Test samples and recording

The recordings carried out for the listening test cover several signal types relating to rich immersive communications and user-generated content streaming. The recordings were performed in a controlled environment in three separate recording sessions using two capture devices (one three-microphone device and one four-microphone device). The recorded sound sources included both loudspeaker playback (well-known immersive audio test samples, background music, nature sound clips, individual instruments, etc.) as well as live sound sources (male and female talkers, a violinist, etc.). The sound scenes thus correspond to, e.g., multi-channel music playback, conference room discussions with or without background sounds, live music performances, outdoor scenes, and so on.

The position of the capture device was different in each of the recording sessions and its relative position to sound sources also varied during some sessions (due to placement and movement of the sources). The capture device was placed on a stand roughly in the middle of the recording space (e.g., middle of a conference room table surrounded by talkers). Sound sources provided either full or partial coverage of the horizontal plane (depending on the sample). The elevation component was generally small (ranging between about 45 degrees above and less than that below the horizontal plane).

In total, 15 samples from these recordings were used in the test.

## x.5.4 Test details and results

The listening test was conducted using Sennheiser HD650 headphones in quiet listening booths. In total 24 listeners participated. Out of the 24 listeners, 14 were audio experts (not necessarily in the field of spatial audio), while 10 listeners were naïve.

Each of the 24 listeners listened to 120 sample pairs, where each individual sample pair was listened to in both directions (AB, BA) by each listener with one of six randomized listening orders. Thus, a grand total of 2880 votes were casted. Mono signal was used as a low reference, and it was compared against the parametric spatial audio format. In addition, a stereo signal (without any spatial processing) was compared to itself as a further anchor (control condition). The main comparisons compare the parametric spatial audio format against FOA (Linear rendering) and FOA (Parametric rendering).

Figure 4 presents the results with all listeners, and Figure 5 and Figure 6 provide the results for expert listeners and naïve listeners, respectively. Positive scores indicate preference for CuT A (CuT A vs. CuT B).

Figure 4. AB listening test results, all listeners (n=24).

Figure 5. AB listening test results, expert listeners (n=14).

Figure 6. AB listening test results, naïve listeners (n=10).

# x.6 References

[1]	Mikko Suvanto, The MEMS microphone book, Mosomic, 2021, ISBN 978-952-94-5660-4

[2]	3GPP TS 26.131

# 3. Conclusion & Proposal

This input document provides an example design description for parametric spatial audio capture for multi-microphone UEs. As part of the discussion, practical factors relating to smartphone form factor and, e.g., microphone and loudspeaker placement are considered. Some of the discussion can be useful also for other capture scenarios. However, the parametric spatial audio capture example can also be generalized for other challenging form factors.

The listening test description in clause x.5 is originally presented in [3] for IVAS codec work. Some clarifications are provided for the current Study Item.

The source proposes to include the above example design description in draft TR 26.933 under clause 8 (Example audio capture processing solutions).

It is noted that there is an earlier proposal [4] in draft TR 26.933 addressing spatial audio capture for multi-microphone UE devices. Since both audio captures relate to multi-microphone UE devices, it can be useful to harmonize or clarify the naming of the examples in a suitable way.

# References

[1]		See clause x.6

[2]		See clause x.6

[3]		S4-180087, “On IVAS audio formats for mobile capture devices”, Nokia Corporation

[4]	S4-240231, “Example design of spatial audio capture for multi-microphone UE devices”, Dolby Sweden AB