Source:	Nokia Corporation

Title:	On ATIAS receiving tests

Document for:	Discussion & Agreement

Agenda Item:	7.5 – ATIAS

# 1. Introduction

In order to enable proposed receiving side tests with Scene-based audio (SBA) and Metadata-assisted spatial audio (MASA) input formats, measurement methods for these formats need to be defined. In addition, to ensure consistent test signal input levels across different input formats, a more evolved calibration procedure than what has been previously utilized is probably required. Especially when the test procedure involves rendering to other format than what was fed into the test system, a careful calibration of input signal levels is necessary.

# 2. Discussion and proposals

# 2.1 Receiving side testing with SBA and MASA input formats

Currently in the ATIAS-1 permanent document [1], input signals of Scene-based audio (SBA) and Metadata-assisted spatial audio (MASA) formats are defined only for receiving tests assessing the inter-channel time difference and source angle dependent band level difference. As the current proposal for selecting the utilized IVAS format for the testing is proposed to follow the below priorities:

1)	Format specified by the manufacturer.

2)	Preference of the UE, as indicated during negotiation in SDP.

3)	Test operator selects format based on form factor and envisioned use case of the UE,

it is necessary to define input signals and possible format specific approaches to all supported formats.

When testing Receiving loudness and Receiving sensitivity/frequency characteristics with SBA and MASA input formats, the same test procedures should be applied as for Object-based audio input format. However, the format specific input signals should be utilized for testing, as currently defined in Receiving with binaural renderer: inter-channel time difference test in ATIAS-1 permanent document sub-clause 5.3.5 and 5.3.6 for SBA and MASA, respectively. I.e., the source position is defined with a suitable component nullification when using SBA input, and with the format specific metadata when using MASA input. The applied test stimulus is the same as defined in the test procedure.

# 2.2 Calibration for receiving loudness testing

To ensure consistent input signal level across different input formats on receiving side testing, a following calibration scheme is proposed:

- Input signal is rendered to a target output format with IVAS reference renderer.

- Level of the rendered signal is measured with ITU-T BS.1770 [2].

- Input signal is adjusted based on the measured level of the rendered signal.

- Adjusted signal is rendered to the target output format with IVAS reference renderer.

- Level of the adjusted, rendered signal is measured with ITU-T BS.1770.

- If obtained level is not correct, re-iterate the steps

Similar calibration procedure was successfully utilized for adjusting the input signal levels in e.g.,  IVAS selection phase [3]. Calibration procedure is illustrated in the Figure 1 below:

Figure 1 Test signal input level adjustment procedure for receiving tests requiring rendering

The same calibration procedure can be applied for all the receiving tests, to mitigate any possible input level dependent issues, and to better align the tests across different input formats. The presented calibration procedure can be implemented in a common clause of ATIAS-1 permanent document.

The proposed textual changes for ATIAS-1 permanent document can be found in Annex A.

# 3. Conclusion

In this document, proposals for missing sub-clauses of SBA and MASA input formats on proposed receiving side test methods in ATIAS-1 permanent document sub-clauses 5.1 – 5.2 are presented. For SBA and MASA inputs, the same test procedures can be applied as for Object-based audio input but with necessary format specific test signal definitions.

In addition, the input signal calibration procedure based on measuring and adjusting input test signal with IVAS reference renderer and ITU-T BS.1770 is proposed. The proposed changes are presented in Annex A.

# References

[1] 	S4-240409: Pdoc ATIAS-1 v0.8.0

[2] 	Recommendation ITU-R BS.1770-5: Algorithms to measure audio

programme loudness and true-peak audio level

[3] 	S4-231306: IVAS-7a: Processing plan for selection phase, v.1.1.0

# Annex A: Proposed changes to ATIAS-1 permanent document

X.X		Test Signals

For receiving test procedures requiring [format conversion / rendering], the test signal input level shall be adjusted according to the Figure x below, by using the IVAS reference renderer [x].

Figure x Test signal input level adjustment procedure for receiving tests requiring rendering

…

5.1		Receiving loudness

…

3)	The source signal is calibrated to a [level/loudness] of [-26 LKFS] as defined in sub-clause x.x.

…

5.1.4           Receiving with binaural rendering: Measurement for scene-based    audio

The method is the same as for object-based audio except that the source position is not set by metadata to the reference client encoder but rather by presenting the encoder with a multi-component Ambisonics signal that represents a source from the particular incidence angle. The First-order ambisonics test signal X for different source positions shall be as in the Table X. The applied signal s is the same signal as defined with object-based audio, and -s is the phase inverted version of the signal s.

Table X: First-order ambisonics signals for different source positions

| Source azimuth [deg] | Source elevation [deg] | FOA signal X = [w, y, z, x] |
| --- | --- | --- |
| 0 | 0 | X = [s, 0, 0, s] |
| 180 | 0 | X = [s, 0, 0, -s] |
| 0 | 90 | X = [s, 0, s, 0] |
| 90 | 0 | X = [s, s, 0, 0] |
| -90 (270) | 0 | X = [s, -s, 0, 0] |

    - 5.1.5	Measurement for metadata-assisted spatial audio

The method is the same as for object-based audio except format specific metadata is set. The applied descriptive metadata for every frame and the applied spatial metadata for every time-frequency tile shall be as in Table X and Table X, respectively. The test signal is the same signal as defined with object-based audio.

Table X: Applied descriptive metadata

| MASA descriptive metadata parameters | Assigned values for every metadata frame |
| --- | --- |
| Format descriptor | Default |
| Number of directions | 1 (bit value 0) |
| Number of channels | 1 or 2 (bit value 0 or 1), depending on the number of applied transport channels |
| Source format | Bit values 00 (Default/unknown) |
| Variable description | 12 bit zero-padding (Default/unknown) |

Table X: Applied spatial metadata

| MASA spatial metadata parameters | Assigned values for every time-frequency tile in all MASA metadata sub-frames |
| --- | --- |
| Spherical index | According to the tested azimuth and elevation |
| Direct-to-total ratio | 1.0 |
| Spread coherence | 0.0 |
| Diffuse-to-total ratio | 0.0 |
| Surround coherence | 0.0 |
| Remainder-to-total ratio | 0.0 |

…

## 5.2 	Receiving frequency characteristics

…

    - 5.2.5	Measurement for scene-based audio

The method is the same as for object-based audio except that the source position is not set by metadata to the reference client encoder but rather by presenting the encoder with a multi-component Ambisonics signal that represents a source from the particular incidence angle (see sub-clause 5.1.4).

### 5.2.6	Measurement for scene-based audio

The method is the same as for object-based audio except format specific metadata is set (see sub-clause 5.1.5).

5.3	Receiving with binaural rendering: inter-channel time difference

…

    - 5.3.5	Measurement for scene-based audio

The method is the same as for object-based audio except that the source position is not set by metadata to the reference client encoder but rather by presenting the encoder with a multi-component Ambisonics signal that represents a source from the particular incidence angle (see sub-clause 5.1.4).

    - 5.3.6	Measurement for metadata-assisted spatial audio

The method is the same as for object-based audio except format specific metadata is set (see sub-clause 5.1.5).

…

5.4	Receiving with binaural rendering: source angle dependent band level difference

…

    - 5.4.5	Measurement for scene-based audio

The method is the same as for object-based audio except that the source position is not set by metadata to the reference client encoder but rather by presenting the encoder with a multi-component Ambisonics signal that represents a source from the particular incidence angle (see sub-clause 5.1.4).

    - 5.4.6	Measurement for metadata-assisted spatial audio

The method is the same as for object-based audio except format specific metadata is set (see sub-clause 5.1.5).