3GPP TSG-S4 Meeting # 127	S4-240259

Sophia-Antipolis, FR,  29th January - 2nd February 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.510 | CR |  | rev |  | Current version: | 1.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5GMS_Pro_Ph2] RTC-related Procedures in 5G Media |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS_Pro_Ph2 |  |  |  |  |  | Date: |  |  | 23rd January 2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Updates to the common procedures to support RTC are not incorporated into 26.510. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 5.2.1	Overview

A Media Application Provider may use the operations in this clause to provision the different features offered by the Media Delivery System in the Media AF. The Provisioning API exposed by the Media AF to the Media Application Provider at reference point M1 offers the following sets of operations:

1.	Provisioning of Provisioning Sessions (see clause 5.2.2) to act as an umbrella for the following provisioning information. Each such Provisioning Session is uniquely identified by a system-dependent Provisioning Session identifier as well as by system-independent service identifier that is subsequently used by a Media-Aware Application to launch media session handling (see clause 10.2) via a 3GPP Service URL (see clause 6).

2.	Discovery of the set of content ingest and/or egest protocols supported by the Media AS for a particular Provisioning Session (see clause 5.2.3):

•	For downlink media delivery, discovery of the content ingest protocols available at reference point M2 and the content distribution protocols available at reference point M4.

•	For uplink media delivery, discovery of the content contribution protocols available at reference point M4 and the content egest protocols available at reference point M2.

3.	Provisioning of Server Certificates within the scope of a Provisioning Session (see clause 5.2.4) to be used by the Media AS to assert its identity to the Media Access Function in Media Clients during media delivery sessions at reference point M4.

4.	Provisioning of Content Preparation Templates within the scope of a Provisioning Session (see clause 5.2.5) that can be used by the Media AS to manipulate media content ingested at reference point M2 or contributed at reference point M4.

5.	Provisioning of Edge Resources within the scope of a Provisioning Session (see clause 5.2.6) to be used to instantiate the Media AS as a set of Edge Application Servers (EAS) in an Edge Data Network (EDN) using the APIs specified in TS 29.558 [29558].

5.	Provisioning of Policy Templates within the scope of a Provisioning Session (see clause 5.2.7) that can be applied to M4 downlink/uplink media delivery sessions in order to realise different Service Operation Points as part of the Dynamic Policies feature (see clause 5.4.3).

7.	Provisioning of media delivery by the Media AS within the scope of a Provisioning Session using the abovementioned building blocks:

-	For downlink media streaming according to TS 26.512 [26512], provisioning of the Content Hosting feature of the Media AS (see clause 5.2.8), which offers functionality equivalent to that of a public Content Delivery Network (CDN): content ingest at reference point M2 for onward distribution by the Media AS to Media Clients via reference point M4 or via other distribution systems such as eMBMS or MBS.

After discovering the set of ingest and distribution content protocols supported by the Media AS (see clause 5.2.2), the Media Application Provider may provision a Server Certificate (see clause 5.2.4), Content Preparation Template (see clause 5.2.5) and/or Edge Resources Configuration (see clause 5.2.6) for each Content Hosting distribution configuration to reference. The Media Application Provider may also provision one or more Policy Templates (see clause 5.2.7) to realise Service Operation Points pertaining to downlink media delivery.

-	For uplink media streaming according to TS 26.512 [26512], provisioning of the Content Publishing feature of the Media AS (see clause 5.2.9), including content contribution by Media Clients at reference point M4 and subsequent content egest of content at reference point M2 after optional manipulation by a Content Preparation Template.

After discovering the set of contribution and egest content protocols supported by the Media AS (see clause 5.2.2), the Media Application Provider may provision a Server Certificate (see clause 5.2.4), Content Preparation Template (see clause 5.2.5) and/or Edge Resources Configuration (see clause 5.2.6) for each Content Publishing contribution configuration to reference. The Media Application Provider may also provision one or more Policy Templates (see clause 5.2.7) to realise Service Operation Points relevant to the parent Provisioning Session.

Editor's Note:	Add some words for RTC, referencing TS 26.114.

8.	Provisioning of QoE metrics reporting within the scope of a Provisioning Session (see clause 5.2.10) to configure how and how often the Media Client should report Quality of Experience metrics to the Media AF during the course of media delivery sessions at reference point M4.

9.	Provisioning of consumption reporting within the scope of a Provisioning Session (see clause 5.2.11) to configure how often the Media Client should report downlink media consumption to the Media AF during the course of media delivery sessions at reference point M4.

10.	Provisioning of rules for processing of UE data (as defined in TS 26.531 [26531]) related to media delivery sessions by the Data Collection AF instantiated in the Media AF (as defined in clause 4.7 of TS 26.501 [26501]), and for restricting its exposure over reference points R5 and R6 by means of Event Data Processing Configurations and Data Access Profiles for a particular Event ID.

11. Provisioning of configuration information that is relevant to RTC sessions (see clause 5.2.12). This information is passed to the MSH via the M5 reference point and is used to configure the RTC session.

NOTE:	The Network Assistance feature is not provisioned by the Media Application Provider at reference point M1. Instead, it is provisioned at the discretion of the Media Delivery System operator using means beyond the scope of the present document.

2nd Change

## 8.13		Configuration Provisioning API

## 8.13.1		General

## The Configuration Provisioning API is used by the Media Application Provider to supply configuration information that will be used by the UE to gain access to the real-time communication service. The API allows for the enablement of trusted RTC functions such as STUN, TURN, and SWAP servers. Additionally, it may provide a set of RTC functions that are maintained by the Application Provider.

## 8.13.2		Resource structure

The data model for the ProvisionedConfiguration resource is specified in Table 10.1-1 below:

Table 10.1-1: Definition of ProvisionedConfiguration resource

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| offerTrustedStunServers | boolean | 0..1 | Indicates if the AF should provide a list of trusted STUN servers to the UE for usage with RTC sessions of this application provider. |
| stunServers | array(URL) | 0..1 | An array of trusted STUN servers that the application can use as ICE candidates. |
| offerTrustedTurnServers | boolean | 0..1 | Indicates if the RTC AF should provide a list of trusted TURN servers to the UE for usage with RTC sessions of this application provider. |
| turnServers | array(URL) | 0..1 | An array of trusted TURN servers that the application can use as ICE candidates. |
| offerTrustedSwapServers | boolean | 0..1 | Indicates if the AF should provide a list of trusted SWAP servers to the UE for usage with RTC sessions of this application provider. |
| swapServers | array(URL) | 0..1 | An array of trusted WebRTC signaling servers that support the SWAP protocol. If provided, the application shall use one of the listed servers for RTC sessions of this application provider. |

2nd Change

#### 8.2.3.1	ProvisioningSession resource

Different properties are present in the ProvisioningSession resource depending on the type of Provisioning Session indicated in the provisioningSessionType property, and this is specified in the Applicability column.

Table 8.2.3.1-1: Definition of ProvisioningSession resource

| Property name | Type | Cardinality | Usage | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| provisioningSessionId | ResourceId | 1..1 | C: RO R: RO U: – | A unique identifier for this Provisioning Session. | All types. |
| provisioningSession‌Type | Provisioning‌Session‌Type | 1..1 | C: RW R: RW U: – | The type of Provisioning Session. | All types. |
| aspId | AspId | 0..1 | C: RW R: RW U: – | The identity of the Application Service Provider responsible for this Provisioning Session, as specified in clause 5.6.2.3 of TS 29.514 [29514]. | All types. |
| appId | ApplicationId | 1..1 | C: RW R: RW U: – | The Application Identifier (see table 5.4.2-1 of TS 29.571 [29571]) to which this Provisioning Session pertains. The same <aspId, ‌appId> duple may be present in several Provisioning Sessions in a given 5GMS System. Used as the AF Application identifier (see clause 5.6.2.3 of TS 29.514 [29514]) for PCF interactions. When a 5GMS AF in the Trusted DN is provisioned from outside the Trusted DN, the NEF is responsible for mapping an external Application Identifier to the corresponding internal AF Application Identifier known to the PCF. | All types. |
| serverCertificateIds | array(ResourceId) | 0..1 | C: RO R: RO U: – | A list of Server Certificate identifiers currently associated with this Provisioning Session. | DOWNLINK, UPLINK |
| contentPreparation‌TemplateIds | array(ResourceId) | 0..1 | C: RO R: RO U: – | A list of Content Preparation Template identifiers currently associated with this Provisioning Session. | DOWNLINK, UPLINK |
| metricsReporting‌ConfigurationIds | array(ResourceId) | 0..1 | C: RO R: RO U: – | A list of Metrics Reporting Configuration identifiers currently associated with this Provisioning Session. | DOWNLINK, UPLINK, RTC |
| policyTemplateIds | array(ResourceId) | 0..1 | C: RO R: RO U: – | A list of Policy Template identifiers currently associated with this Provisioning Session. | DOWNLINK, UPLINK, RTC |
| edgeResources‌ConfigurationIds | array(ResourceId) | 0..1 | C: RO R: RO U: – | A list of Edge Resources Configuration identifiers currently associated with this Provisioning Session. | DOWNLINK, UPLINK, RTC |
| eventDataProcessing‌ConfigurationIds | array(ResourceId) | 0..1 | C: RO R: RO U: – | A list of Event Data Processing Configuration identifiers currently associated with this Provisioning Session. | DOWNLINK, UPLINK |
| ConfigurationProvisioning | array(ResourceId) | 0..1 | C:RO R:RO U: - | A list of the associated Configuration Provisioning identifiers that are currently associated with this Provisioning Session. | RTC |

3rd Change

#### 8.7.3.1	PolicyTemplate resource

Table 8.7.3.1-1: Definition of PolicyTemplate resource

| Property |  |  | Type | Cardinality |  | Usage | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| policyTemplateId |  |  | ResourceId | 1..1 |  | C: RO R: RO U: RO | Resource identifier of this Policy Template assigned by the Media AF that is unique within the scope of the Provisioning Session. |
| state |  |  | string enum | 1..1 |  | C: RO R: RO U: RO | Current state of this Policy Template (see clause 5.2.7.2) exposed to the 5GMS Application Provider by the Media AF. Only a Policy Template in the READY state may be instantiated as a Dynamic Policy Instance and applied to media streaming sessions. |
| stateReason |  |  | Problem‌Details | 1..1 |  | C: RO R: RO U: RO | Additional details about the current state of this Policy Template exposed to the Media Application Provider by the Media AF. The instance sub-property shall be present and shall indicate the URL of this Policy Template resource at reference point M1. The title sub-property shall be present and shall indicate a human-readable representation of the state property specified above, e.g., "Policy Template ready for use" or "Policy Template invalid". The detail sub-property shall be present and shall indicate a human-readable status/error message. All other properties shall be omitted. |
| externalReference |  |  | string | 1..1 |  | C: RW R: RW U: RW | Additional identifier for this Policy Template, unique within the scope of its Provisioning Session, that may be cross-referenced with external metadata about a media delivery session. Example: "HD_Premium". |
| application‌Session‌Contexts |  |  | array(object) | 0..1 |  | C: RW R: RW U: RW | Exactly one application session context at reference point M4 to which this Policy Template may be applied. Each object in the array shall specify at least one property. If more than one property is specified, instantiation of the Policy Template is restricted to the conjunction of all the object's properties. |
|  | sliceInfo |  | Snssai | 0..1 |  | C: RW R: RW U: RW | A Network Slice on which this Policy Template may be instantiated. See clause 5.4.4.2 of TS 29.571 [29571]. |
|  | dnn |  | Dnn | 0..1 |  | C: RW R: RW U: RW | A Data Network on which this Policy Template may be instantiated. (See clause 7.3.2.) |
| qoSSpecification |  |  | M1‌QoS‌Specification | 0..1 |  | C: RW R: RW U: RW | The network Quality of Service policy to be applied to media delivery sessions that instantiate this Policy Template (see NOTE and clause 7.3.3.3). |
| rtcQosSpecification |  | array(RTCQoSSpecification) |  | 0..1 | C: RW R: RO U: RW |  | Specifies the network quality of service to be applied to the different media streams of the RTC session. The RTCQoSSpecification objectis defined in table 8.7.3-2. |
| charging‌Specification |  |  | Charging‌Specification | 0..1 |  | C: RW R: RW U: RW | The charging policy to be applied to media delivery sessions that instantiate this Policy Template is instantiated (see NOTE). |
| NOTE: At least one of qoSSpecification and charging‌Specification shall be present. |  |  |  |  |  |  |  |

Table 8.7.3-2: Definition of RTCQoSSpecification object

| Property name | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| serviceDataFlowDescription | ServiceDataFlowDescription | 1..1 |  | The 5-Tuple that identifies the service data flow for which the QoS dynamic policy is requested. |
| mediaIdentifier | String | 1..1 |  | Provides an identifier for the media stream to associate with the corresponding service component in the QoS Policy. |
| marBwDlBitRate | BitRate | 1..1 |  | Maximum requested bit rate for the Downlink. |
| marBwUlBitRate | BitRate | 1..1 |  | Maximum requested bit rate for the Uplink. |
| minDesBwDlBitRate | BitRate | 0..1 |  | Minimum desired bit rate for the Downlink. |
| minDesBwUlBitRate | BitRate | 0..1 |  | Minimum desired bit rate for the Uplink. |
| mirBwDlBitRate | BitRate | 1..1 |  | Minimum requested bit rate for the Downlink. |
| mirBwUlBitRate | BitRate | 1..1 |  | Minimum requested bandwidth for the Uplink. |
| desLatency | Integer | 0..1 |  | Desire Latency. |
| desLoss | Integer | 0..1 |  | Desired Loss Rate. |
| pduSetMarking | PDUSetMarking | 0..1 |  | A description of the PDU Set and End of Burst marking configuration for the session. In release-18, this shall only be present in the case of an RTP stream. |

The PDUSetMarking object is defined in table 8.7.3-3 below:

Table 8.7.3-3: Definition of PDUSetMarking object

| Property name | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| serviceDataFlowDescription | ServiceDataFlowDescription | 1..1 |  | The 5-Tuple that identifies the service data flow for which the QoS dynamic policy is requested. |
| headerExtensionVersion | Integer | 1..1 |  | The RTP header extension version. |
| localIdentifier | Integer | 1..1 |  | A unique identifier of the RTP header extension in the scope of the media session. |
| format | Boolean | 0..1 |  | Indicates if a short or a long header extension format is used. When set to false, a short 1-byte header extension format is being used. |
| pduSetSizeActive | Boolean | 0..1 |  | A flag to indicate if the PDU Set size in bytes is present in the RTP header extension. |

4th Change

#### 8.10.3.1	MetricsReportingConfiguration resource

Table 8.10.3-1: Definition of MetricsReportingConfiguration resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| metricsReportingConfigurationId | ResourceId | 1..1 | An identifier for this Metrics Reporting Configuration assigned by the Media AF that is unique within the scope of the enclosing Provisioning Session. |
| scheme | Uri | 0..1 | The QoE metrics scheme associated with this Metrics Reporting Configuration. Omitting this property signals to the Media AF that metrics reporting is currently disabled for the Provisioning Session in question. For RTC media streaming, if not specified, the 3GPP metrics scheme urn:‌3GPP:‌ns:‌PSS:‌RTC:‌QM1 defined in clause 5.1.3 shall apply. |
| dataNetworkName | Dnn | 0..1 | Identifies the Data Network which shall be used when sending metrics reports. If not specified, the default Data Network shall be used. |
| reportingInterval | DurationSec | 0..1 | The time interval between successive metrics reports to be sent by the Media Session Handler. The value shall be greater than zero. If not specified, a single final report shall be sent after the media delivery session has ended. |
| samplePercentage | Percentage | 0..1 | The proportion of media delivery sessions for which QoE metrics shall be reported, expressed as a floating-point value between 0.0 and 100.0. If not specified, reports shall be sent for all media delivery sessions. |
| urlFilters | array(string) | 0..1 | If present, a non-empty list of Media Entry Point URL patterns for which QoE metrics shall be reported. If not specified, reporting shall be done for all media delivery sessions initiated within the scope of the parent Provisioning Session. |
| samplingPeriod | DurationSec | 1..1 | The time interval the Media Client should wait between sampling the QoE metrics specified by this Metrics Reporting Configuration. |
| samplingDuration | DurationSec | 0..1 | The time duration specified in the media stream for which the QoE metrics will be reported by the RTC Client. There shall be only one range per measurement specification. If the " samplingDuration" field is not present, the metrics range shall be for the whole session duration. |
| metrics | array(String) | 0..1 | If present, a non-empty list of QoE metrics which shall be collected and reported by the Media Client. A controlled vocabulary of QoE metrics shall be specified by each metrics scheme for use with this property. If omitted, the complete (or default, as applicable) set of metrics associated with the specified metrics scheme shall be collected and reported. In the case of RTC media streaming and for the 3GPP scheme urn:‌3GPP:‌ns:‌PSS:‌RTC:‌QM1 the listed metrics shall correspond to one or more of the metrics as specified in clauses 9.1.2, and the quality reporting scheme and quality metric reporting protocol as defined in clauses 5.1.3 and 9.1.3, respectively, shall be used to produce and send metrics reports. |

5th Change

#### 9.3.3.1	DynamicPolicy resource

Table 9.3.3.1-1: Definition of Dynamic Policy Instance resource

| Property name | Data type |  | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- | --- |
| dynamicPolicyId | ResourceId |  | 1..1 | RO | Unique identifier for this Dynamic Policy assigned by the Media AF. |
| provisioningSessionId | ResourceId |  | 1..1 | C: RW R: RO U: RW | Provisioning Session identifier obtained from Service Access Information (see clause 11.2.3). Uniquely identifies a Provisioning Session, which is linked to the Application Service Provider. |
| policyTemplateId | ResourceId |  | 1..1 | C: RW R: RO U: RW | Identifies the Policy Template to be applied to the application flow(s). |
| serviceDataFlowDescriptions | array(Service‌Data‌Flow‌Description) |  | 1..1 | C: RW R: RO U: RW | Describes the Service Data Flows managed by this Dynamic Policy. |
| mediaType | MediaType |  | 0..1 | C: RW R: RO U: RW | The type of media carried by the application flows listed in service‌DataFlow‌Descriptions. |
| qosSpecification | M5‌QoS‌Specification |  | 0..1 | C: RW R: RO U: RW | The network Quality of Service requirements of this Dynamic Policy (see clause 7.3.3.4). |
| rtcQosSpecification |  | array(RTCQoSSpecification) | 0..1 | C: RW R: RO U: RW | Specifies the dynamic policy that can applied to the streams of the RTC session. The RTCQoSSpecification object is defined in table 8.7.3-2. |
| qosEnforcement | Boolean |  | 1..1 | C: RO R: RO U: RO | Indication that the Quality of Service described in qosSpecification is being enforced by the 5G System. |

6th Change

## 9.7	 Configuration API

### 9.7.1	General

The Configuration API is used by the Media Session Handler to obtain configuration information from the Media AF that is used to configure RTC sessions. The configuration information contains a list of trusted RTC servers that can be used by the RTC session. For STUN and TURN servers, this list shall be passed during the creation of the session. The SWAP servers are servers that can be used for establishment of a connection to a remote endpoint that also supports the SWAP protocol.

### 9.7.2	Resource Structure

The Configuration API is accessible through the following URL base path:

{apiRoot}/3gpp-maf-session-handling/{apiVersion}/configuration-information/

The operations and the corresponding HTTP methods in table 9.7.2-1 are supported. In each case, the sub-resource path specified in the second column shall be appended to the URL base path.

Table 9.7.2-1: Operations supported by the Configuration API

| Operation name | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Retrieve Configuration Information | {provisioningSessionId} | GET | Acquire the Configuration Information resource for the specified Provisioning Session. |

### 9.7.3	Data model

The data model for the ConfigurationInformation resource is specficied in table 9.7.3-1 below.

Table 9.7.3-1: Definition of ConfigurationInformation resource

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| stunServers | array(URL) | 0..1 | An array of trusted STUN servers that the application can use as ICE candidates. |
| turnServers | array(URL) | 0..1 | An array of trusted TURN servers that the application can use as ICE candidates. |
| swapServers | array(URL) | 0..1 | An array of trusted WebRTC signaling servers that support the SWAP protocol. If provided, the application shall use one of the listed servers for RTC sessions of this application provider. |