3GPP TSG-SA WG4 Meeting #129e	S4-241564r03

eMeeting,  19th - 23th  August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.567 | pCR |  | rev | 03 | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ### General procedures for session modifications for Split Rendering over IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_IMS |  |  |  |  |  | Date: |  |  | 12-05-2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 7.2 of the latest draft of TS 26.567 v 0.2.0 is left incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes new high level call flows for general procedures for session modification for split rendering session over IMS and the corresponding text for clause 7.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Clause 7.2 will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7. 2 (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 1st Change

7.2 General procedures for session modifcation from UE1-centric

The following below procedures indicate the session modification procedures during an established SR session over a standalone IMS DC channel, when the UE requires to download a dedicated application via a standalone IMS DC session. The session negotiation for split rendering is indicated as below:

Step 1-10: established IMS session and split rendering session establishment

Step 11: When the UE1 discovers that its media capabilities cannot meet the related media rendering requirements, the UE1 decides to modify the existing split rendering call flow; UE sends a request to modify the split rendering session to the IMS AS.

Step 12: The IMS AS interacts with the DCSF via DC1 to send updated event notifications to modify the split.

Step 13: The DCSF receives event reports from the IMS AS and decides whether the requested data channel service is allowed to be provided during the IMS session.

Step 14: The IMS AS receives the updated data channel control instructions from the DCSF and accordingly interacts with the MF. Note: here it is assumed that there are more than one MF. It is also assumed that the serving MF does not satisfy the new request (due to unavailability).

Step 15: The (new) MF is discovered (if the existing serving MF does not satisfy the UE1 request for session modification).

Step 16: UE-1 sends application request to the newly discovered MF to request an application list.

Step 17: (new) MF response with application list include an identifier that if the application is capable to be split rendered. DCSF received the list and forward to the UE-1.

Step 18: If step 15 is yes, the IMS AS sends a Split Rendering Request to the (new) MF through the established application data channel, the request includes the information of the objects to be rendered in IMS network

Step 19: The (new) MF sends a description of the split rendering output to the IMS AS.

Step 20: The IMS AS sends the media resource allocation request to the MF , to reserve media rendering resource for the UE1.

Step 21: When the resources are allocated successfully, the MF  returns a successful response to the IMS AS.

Step 22: The IMS AS returns a successful response to the UE1.

Step 23: Successful SR session is established between UE1 and MF through the data channel

Step 24: similar session also can be established between UE-1 and UE-2 for remote UE.

## End of change