Source:	Orange

Title:	Clarifications on scope for FS_ULBC

## Document for:	Agreement

## Agenda Item:	7.9

- Introduction

The Rel-20 FS_ULBC study item [1] has been approved in March 2024, and the general objective of this study is to develop recommendations for potential normative work on an ultra-low bit rate codec for voice over GEO.

In this contribution, we request to clarify the actual scope of the FS_ULBC study.

- Discussion

The FS_ULBC study is essentially motivated by the the use case of IMS voice call using GEO access (TR 22.887) and related service requirements and KPIs on IMS voice call using GEO satellite access (TS 22.261) defined by 3GPP SA1.  The key justification to launch the study is that no existing 3GPP codec seems to support all the expected requirements for this use case.

Voice services over IMS are already widely deployed on different access networks (e.g., LTE, NR, Wifi). If a new ultra-low bitrate codec (ULBC) is defined for GEO access, this ULBC codec would be listed as a supported codec by future UEs providing GEO access, and it would then be possible to use this codec for calls even in scenarios where GEO access is not used or not preferred (e.g., when there is a good direct connection over cellular or WLAN access).

It is therefore important to clarify where the foreseen codec design constraints and performance requirements in FS_ULBC will also cover other application scenarios than just GEO access. For instance, on VoLTE or VoNR, an ultra low bit rate operation (around 1 to 3 kbit/s) may not be necessary, or one may even consider operating ULBC at a higher bit rate than 3 kbit/s to enable near-transparent audio quality (in clean channel conditions).

- Proposed text updates to draft TR 26.940 V0.0.1

We suggest the following revisions based on the initial TR skeleton in [2]:

# 1	Scope

The present document develops recommendations for potential normative work on an ultra-low bit rate codec for the use case of IMS voice services over Geostationary Orbit (GEO) access. The studied application scenarios encompass IMS voice services over different access types (e.g,, LTE, NR, WLAN) in addition to GEO satellite access.

## References

- Tdoc SP-250378, “Study on Ultra Low Bitrate Speech Codec,” Source: SA WG4

- Tdoc S4-250451, “[FS_ULBC] TR 26.940 V0.0.1,” Source: China Mobile Com. Corporation