# 3GPP TSG-SA4 Meeting #131-bis-e	S4-250521

# Online, , 11th Apr 2025 - 17th Apr 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR | 0583 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [SR_IMS] Adding metrics threshold values and target values in QoE reporting configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_IMS |  |  |  |  |  | Date: |  |  | 2025-04-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 26.567 (SR_IMS) has defined split rendering profiles and corresponding functions and procedures. The processing delay adaption procedure defined in SR_IMS needs additional parameters in TS 26.114 to configure the target delay range and target delay value for the QoE metrics. Metrics thresholds information was provided in TS 26.510 as part of the clientMetricsReportingConfigurations object in the MetricsReportingConfiguration resource of provisioning service and in the ServiceAccessInformation resource of session handling service. However, the threshold values information is missing in TS 26.114 to trigger reporting of QoE metrics in IMS architecture. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update QoE metric reporting configuration in TS 26.114 for metrics reporting and to enable the processing delay adjustment: - Add positive crossing and negative crossing thresholds information - Add target value for processing delay adjustment |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing features from SR_IMS and TS 26.510 in TS 26.114. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST change

### 16.3.2	QoE metric reporting configuration

The syntax of the text contained in the Metrics leaf is similar to the "3GPP-QoE-Metrics" attribute syntax specified in TS 26.234 [60] and TS 26.346 [74]:

-	QoE-Metrics	= "3GPP-QoE-Metrics:" att-measure-spec *("," att-measure-spec)) CRLF

-	att-measure-spec	= Metrics ";" Sending-rate [";" Measure-Range] 
	[";" Measure-Resolution] *([";" Parameter-Ext])

-	Metrics	= "metrics" "=" "{"Metrics-Name *("|" Metrics-Name) " }"

-	Metrics-Name	= 1*((0x21..0x2b) / (0x2d..0x3a) / (0x3c..0x7a) / 0x7e) 
	 [";" Positive-Threshold ";" Negative-Threshold ";" Target];VCHAR except ";", ",", "{"	or "}"

-	Positive-Threshold	= "positive=" (1*DIGIT ["." 1*DIGIT]) ; positive crossing threshold

-	Negative-Threshold	= "negative=" (1*DIGIT ["." 1*DIGIT]) ; negative crossing threshold

-	Target	= "target=" (1*DIGIT ["." 1*DIGIT]) ; target value

-	Sending-Rate	= "rate" "=" 1*DIGIT / "End"

-	Measure-Resolution	= "resolution" "=" 1*DIGIT ; in seconds

-	Measure-Range	= "range" ":" Ranges-Specifier

-	Parameter-Ext	= (1*DIGIT ["." 1*DIGIT]) / (1*((0x21..0x2b) / (0x2d..0x3a) / (0x3c..0x7a) / 0x7c / 0x7e))

-	Ranges-Specifier	= as defined in RFC 2326 [72].

This attribute is used to indicate which QoE metrics are supported, the reporting interval, the measurement interval and reporting range.

The "Metrics" field contains the list of names that describes the metrics/measurements that are required to be reported in a MTSI call, provided that the MTSI client supports these measurements and the reporting rule conditions are met (see clause 16.3.3). The names that are not included in the "Metrics" field shall not be reported during the session.

The "Sending-Rate" shall be set, and it expresses the maximum time period in seconds between two successive QoE reports. If the "Sending-Rate" value is 0, then the client shall decide the sending time of the reports depending on the events occurred in the client. Values  1 indicate a precise reporting interval. The shortest interval is one second and the longest interval is undefined. The reporting interval can be different for different media, but it is recommended to maintain a degree of synchronization in order to avoid extra traffic in the uplink direction. The value "End" indicates that only one report is sent at the end of the session.

The optional "Measure-Resolution" field, if used, shall define a time over which each metrics value is calculated. The "Measure-Resolution" field splits the session duration into a number of equally sized periods where each period is of the length specified by the "Measure-Resolution" field. The "Measure-Resolution" field is thus defining the time before the calculation of a QoE parameter starts over. If the "Measure-Resolution" field is not present, the metrics resolution shall cover the period specified by the "Measure-Range" field. If the "Measure-Range" field is not present the metrics resolution shall be the whole session duration.

The optional "Measure-Range" field, if used, shall define the time range in the stream for which the QoE metrics will be reported. There shall be only one range per measurement specification. The range format shall be any of the formats allowed by the media. If the "Measure-Range" field is not present, the metrics range shall be the whole call duration.

The optional "Positive-Threshold" field, if used, shall define the positive crossing threshold of a particular QoE metric. When present, the QoE metric shall be reported once when its value exceeds the threshold value indicated in the "Positive-Threshold" property and shall not be reported again until it falls below that threshold and subsequently exceeds it.

The optional "Negative-Threshold" field, if used, shall define the negative crossing threshold of a particular QoE metric. When present, the QoE metric shall be reported once when its value falls below the threshold value indicated in the "Negative-Threshold" property and shall not be reported again until it exceeds that threshold and subsequently falls below it.

The optional "Target" field, if used, shall define the target value of a QoE metric specified in "Metrics" field.

An example for a QoE metric reporting configuration is shown below:

3GPP-QoE-Metrics:metrics={Round_Trip_Time;positive=80;negative=20;target=50};rate=5;resolution=1

End of changes