3GPP TSG-SA WG4 Meeting #131-bis-e					S4-250597

Online, 11 – 17 April 2025

Source:	Nokia

Title:	On testing different capture types in send direction

Document for:	DISCUSSION

Agenda item:	7.7 – ATIAS_Ph2

# 1. Introduction

In the [1], overview on how the IVAS UEs would be expected to perform in the ambient sound fields were presented. In addition, concept of multiple capture types with different noise suppression/speech enhancement characteristics were presented. Furthermore, to assess such different capture types, an overview on different possible test methods for assessing the ambience transmission were discussed, namely loudness, frequency response, temporal characteristics and spatial characteristics. Possible need for tests covering simultaneous speech and ambience was also recognised.

In this contribution, further details on the testing flow with multiple supported capture types are discussed. In addition, necessity of a certain codec improvements is presented and discussed.

# 2. Discussion

If presented concept of different capture types as described in [1] is assumed from the Immersive UEs (IVAS frontends), additional tests and requirements are needed. In the following sections, further discussion on the relation between different capture types with test methods and requirements is presented. In addition, idea on distinguishing between tests targeted to assess only speech transmission or more complex acoustic sound scenes is further described.

## 2.1 Speech testing

To ensure, that the speech quality is sufficient with all the capture types, same test methods and requirements for assessing the speech transmission can be enforced. The test flow with different possible capture types is illustrated in the Figure 1. The idea is that with every UE (despite the supported and applied capture processing) is tested and assessed with the same tests comprising only speech, i.e., what is currently specified in TS 26.260 section 5.6. Furthermore, it is assumed that the assessment could be done against the same requirements for all capture types, as the intention is to ensure that quality of speech transmission meets certain expectations with all types of applied signal processing/noise suppression targets.

Figure 1 Test flow for testing speech transmission

## 2.2 Complex acoustic scene testing

In the [1], certain items were identified to assist on defining necessary tests and requirements for assessing the ambient sound capture. Furthermore, to enable the presented capability differences, three different capture types were defined and presented to illustrate one possible solution for flexible capture performance in varying conditions. Furthermore, to ensure sufficient performance in complex acoustic sound fields comprising also other than speech, initial ideas on possible test methods were presented.

The Figure 2 below illustrates on how the testing of IVAS UEs with different supported capture types could be done. For convenience, the tests should be designed such, that despite the expected UE performance (e.g., in terms of noise suppression), the same test setup and method would be applied. However, it is also recognized that with different capabilities certain tests may become obsolete. Then, based on the UE’s capture type, assessment can be made against correct requirements. Further discussion on the capture type identification is in the next section.

Figure 2 Test flow for assessing transmission in complex acoustic scenes

As an example how such capture type specific requirements could look like, pseudo-requirements for loudness rating of transmitted ambient sound field for different capture types are presented:

The nominal values of SLR for ambient transmission to the POI shall be: SLRTransparent = [X ± 3 dB] SLRNominal = [(X + 6) ± 3 dB] SLRSuppressed > [(X + 9) dB]

The idea is, that the highest loudness would be assumed with Transparent capture type, and a slightly (but significantly) lower loudness would be expected with Nominal transmission. With suppressed transmission, certain limit could be assumed, i.e., captured ambience should be attenuated sufficiently or not transmitted at all.

Other option would be to formulate requirements such that different capture types could be covered with the same requirements. However, this could lead to an extremely relaxed requirements, which is not seen as appropriate.

Furthermore, it is envisioned that requirements may not be needed for a combination of certain send characteristics and capture types. For example, if device can be reliably identified to represent Suppressed audio capture type, assessment on, e.g., send frequency response may not be needed. On the other hand certain suppression quality metrics could be enforced only for Suppressed capture type.

# 3. Capture type identification

## 3.1 Testing aspect

The illustrated test flow in the Figure 2 relays on the capture type identification for deciding correct tests and comparing test results with intended requirements. To further enable this, it should be possible to determine the applied capture type of the UE. In the [2] some considerations were already presented for distinguishing whether the UE suppresses noise or not with a simple “transparency test”. However, it would be highly preferable to enable UEs to indicate explicitly the applied and supported noise suppression mode and further negotiate the applied transmission capture type (in case where multiple types are supported).

For this purpose, the source has recognized a need for a new SDP parameter for negotiating applied capture type when establishing an IVAS session. Furthermore, from testing perspective, definition of such parameter would allow explicit comparison against capture type specific requirements. In addition, the parameter would further ensure sufficient testing coverage when assessing the performance of UEs supporting multiple capture types and varying signal processing capabilities.

In practice, the applied capture type for the assessment would be negotiated with a system simulator. Furthermore, certain test set and/or certain requirements would be used for the assessment based on the negotiated capture type, as discussed in the section 2. In general, the tests should be as similar as possible despite the applied capture type, capture mode, or coded format. However, as in the current TS 26.260 certain degree of the modifications on the testing aspects could be expected for different UE configurations.

In addition, further advantage of such SDP parameter would be that the necessity of different test runs can be evaluated as the parameter could indicate all the supported (and possibly tested) capture types. The tests could be re-run with differently negotiated parameter, and the performance of all supported capture types could be assessed if seen necessary. On the other hand, only “main” capture type (e.g., defined by the manufacturer) could be assessed.

Despite the applied capture type, it is currently anticipated that tests targeted for assessing speech capture should always meet certain common requirements to ensure sufficient voice transmission. However, such common requirement approach is not seen sufficient for assessing the transmission of more complex sound scenes comprising also other audio content than speech.

It should be noted that the considerations here are only targeted for the sending direction and further considerations on receiving and echo performance assessments are out of this document’s scope. However, we recognize that introduction of such concept and new SDP parameter for negotiating the capture types could have some implications on the expected receiving and echo cancellation performance.

## 3.2 Aspects outside testing scope

In addition, it is seen that defining such SDP parameter for negotiating applied capture type would be extremely beneficial also outside testing scope. As the immersive UEs will be used in varying acoustic conditions and for varying purposes, a mechanism to negotiate and establish communication session with desired suppression mode/capture type is seen crucial. For example, a UE may want to choose its capture type based on the received audio content (received capture type) when establishing the immersive call. In addition, the UE’s preference regarding the applied capture type for captured audio may differ based on its applied playback device or received audio content.

Furthermore, it is anticipated that certain usage scenarios may be more difficult for UEs to handle. As an example, a call between two Hand-Held Handsfree devices with loudspeaker playback where both devices are in noisy environments and both devices transmits fully transparent immersive audio could be rather difficult situation to handle by the devices, e.g., in terms of acoustic echo control. Additionally, the experienced quality or speech intelligibility may be compromised due to the combination of fully immersive audio with loudspeakers output in a noisy environment.

To further improve applicability of such SDP parameter, the parameter should be defined for both send and receive directions (similarly as many existing IVAS SDP parameters are defined). Thus, UE could indicate the supported capture type(s) in send direction, and supported/preferred capture type(s) in receive direction. Based on this indication, optimal audio capturing processing types may be negotiated between the parties.

In addition, no sensitive information about the UE would be exploited to the network with the presented SDP parameter approach. For example, by indicating only the transmitted capture type in send direction and preferred capture type in receiving direction, no information about the capture or playback interface is leaked, thus enabling improved quality with secure communication.

Lastly, it is recognized that currently there is a proposal to define Dynamic Audio Supression (DAS) PI data for IVAS RTP extension within the IVAS_Ph2 work item [3]. It is envisioned, that the presented SDP parameter would complement the defined DAS PI data. These two parameters could be tied in a such way, that the proposed SDP parameter would indicate the supported range of DAS requests. For example, if UE 1 indicates that both transparent and suppressed capture types are supported in send direction, UE 2 can expect support for full range of dynamic audio suppression via DAS PI data during the IVAS session. On the other hand, if UE 1 indicates that it supports only suppressed capture type, UE 2 can assume that DAS requests will have no impact on the received audio. Further details on such usage of proposed SDP parameter are open for discussion.

# 4. Conclusion

An overview on the IVAS UE testing flow for sending direction including complex sound fields and multiple different supported capture types by the UE are presented. The overall idea on how the IVAS UEs could have different capture types in terms of noise suppression / speech enhancement are discussed. Based on these views, a new SDP parameter is presented and proposed to be defined.

The new SDP parameter would cover the following missing aspects within an IVAS session:

- Indicate supported and/or preferred capture types in terms of noise suppression, i.e., the expected amount of applied noise suppression

- Mechanism to explicitly indicate and negotiate applied capture type, to further allow testing with a specific test set and assess the performance against capture type specific requirements

- Enable negotiating optimal capture type in both directions for the IVAS session. For example, UEs may have different preferences to different directions due to the acoustic conditions, applied playback interfaces, received audio content, etc.

- Can be used to indicate limits for DAS-requests

- Would avoid any privacy concerns. No need to exploit information about capture/playback interface

More detailed proposal will be made under the IVAS_ph2 work item. In addition, further work towards enabling such SDP parameter in terms of testing aspects and test specification will continue over the upcoming telcos and meetings.

# 5. References

[1]	S4-250205: “On testing UEs in ambience”, Nokia

[2]	S4aA250014: “Ambient noise transparency in send direction”, Head Acoustic

[3]	S4-250203: “Further corrections to Annex A”, Dolby Laboratories Inc., Ericsson LM, Nokia, Orange