Source:	Nokia

Title:	On DaCAS recording scenarios

Document for:	Discussion & Agreement

Agenda Item:	7.6 – DaCAS

# 1. Introduction

For obtaining characterization and development recordings for the DaCAS target devices, necessary recording scenarios are needed. In this contribution we discuss and propose two recording scenarios for purposes of characterizing Signal-to-Noise -Ratio (SNR) of integrated microphones, and recordings for obtaining direction dependent properties of target devices.

# 2. Discussion

## 2.1 Sound source directions

For characterization and development purposes, recordings comprising multiple different sound source directions are required for each target device. Furthermore, to facilitate development of spatial capture algorithm, the applied sound source directions should cover all spatial directions around the UE, within some resolution. Depending on the targeted dimensions of the capture (e.g., 180° frontal spatial capture or 360° 3D capture), the required directions and their resolution may vary.

Naturally, the highest possible number of individual directions would facilitate algorithm development and tuning with the highest accuracy. On the other hand, to achieve feasible recording time and take into account possible limitations on the recording facilities, the number of directions may need to be limited. In addition, different resolutions for horizontal (azimuth) and vertical (elevation) directions may be applicable.

The source considers sufficient resolution of sound source direction in terms of azimuth to be 10°. I.e., sound sources should be recorded from different azimuthal directions with maximum of 10° steps. Similarly, high number of elevated source directions is considered to be beneficial (in case the capture algorithm supports analysis of elevated sources). To cover full 3D space, an appropriate resolution in terms of the elevation is seen to be 30°. Thus, the optimal grid of sound source directions would comprise sound sources for every 10° in azimuth, with every 30° elevation step. However, to reduce number of measurement points, it is seen reasonable to measure elevated sources for every 3rd azimuth step, i.e., with the azimuth step of 30°.

It is recognized that all proponents may not have possibility to produce recordings with exactly the same source directions, e.g., due to the fixed structures in the measurement facilities. Thus, it is seen acceptable to use slightly different source directions, than what is presented here, if necessary. However, same number of measurement points should be achieved, and the applied source directions should be well documented. In addition, source directions at elevation of 0° should be always recorded.

In general, suitable distance between the target device and sound source is between 0.5 – 2 meters.

Thus, we consider following sound source directions appropriate for recording scenario intended to characterize direction dependent properties of the device:

Proposed sound source directions:

- Every azimuth with maximum step of 10°: = [0°, 10°, …,350°] - All azimuth directions at the elevation = 0° shall be always recorded - For every 3rd azimuth step ( = [0°, 30°, …, 330°]), every elevation with a maximum step of 30°: = [-60°, -30°, 0°, 30°, 60°] - Poles: = [-90°, 90°] - In practice, actual elevation direction may differ, e.g., due to recording facility limitations - Distance between sound source and device shall be 0.5 – 2 meters - Applied source directions shall be clearly documented

Furthermore, to obtain comparable recordings for all the target devices, it would be favorable if the proponent proposing target device could provide recordings complying with this recording scenario with the proposed target device. In terms of practicality, azimuthal rotation can be applied for the UE, i.e., keeping the sound source at static direction and turning the UE accordingly. In addition, if no means for lifting the sound source to the target elevation height is available, the UE may be tilted according to the measured elevation. Furthermore, complete recording scenario according to the presented discussion is proposed in section 3.

## 2.2 On applied sources

For characterization and development purposes, the applied test signals should cover the whole targeted frequency range. In addition, typically impulse responses of the measured device are of interest. Thus, exponential sweep is found to be useful, due to its relatively short length and the robust method to obtain impulse responses. Appropriate length of the sweep is typically around 5 seconds, and the signal should cover frequency range from 20 Hz to 20 kHz.

For reproducibility and sufficient playback quality should be expected from the utilized loudspeakers. At least the loudspeaker frequency response should be sufficiently flat, if applied without calibration. In addition, HATS/mouth-simulator according to the [1] may be used for the evaluation.

If loudspeaker is applied without calibrating the frequency response, following requirements are seen sufficient. In addition, the need for loudspeaker response calibration may be mitigated with reference recordings.

| Loudspeaker requirements |  |
| --- | --- |
| Frequency response | ±2.5dB @100 Hz – 20kHz |

Furthermore, to increase reproducibility, common practices on calibrating the sound sources should be used. Depending on the recording scenario, calibration loudspeaker with pink noise to a certain sound pressure level (e.g., to 70 – 80 dB SPL(A)) is seen suitable. In addition, calibrating the applied test signal to a certain sound pressure level may be needed in some scenarios. Furthermore, for subjective evaluation, rough estimates on the SPL levels of the recorded sound field could be documented along the delivered recordings.

# 3. Proposed recording scenarios

Following recording scenarios 1 and 2 are proposed to be included to the Template for recording scenarios.xlsx -database. Proposals can be found in the attached Proposed recording scenarios.xlsx -file.

## 3.1 Recording scenario 1

Following recording scenario in the Table 1 is proposed to be used for characterizing Signal-to-noise ratio (SNR) of integrated microphones. To ensure proper integration as well as sufficient noise floor of the applied components, the SNR of each microphone should be evaluated after the assembly of the target device. Further requirements on the obtained SNR values of integrated microphones are discussed in the [2].

Table 1 Recording scenario for obtaining the integrated microphone SNR

| No: | 1 |
| --- | --- |
| Sound source: | High quality loudspeaker |
| Source signal: | 1 kHz sine |
| Signal characteristics: | Range: 1 kHz Length: 10 s Level: < 0 dBFS (as long as calibrated SPL level is met) Sample Rate: 48 kHz Bit depth: 16 bit |
| Sound source calibration: | Source signal calibrated to 94 dB SPL at the measured microphone inlet position |
| Acoustic environment: | Anechoic chamber or well isolated room without any external noises |
| Detailed Positioning: | Distance: >0.5 m Source direction: Main diaphragm towards the measured microphone inlet Source angles (azi, ele): (0°,0°) relative to the measured microphone inlet |
| Reference recording: | Record 30 seconds of silence with the device under test to obtain the noise floor of the integrated microphones |
| Description/additional info: | Signal-to-noise ratio (SNR) of integrated microphone can be calculated as SNR= SN, where N is the A-weighted level of the recorded silence, and S is the recorded microphone level with 1 kHz sine signal. Both values are in linear scale. In addition, integrated microphone sensitivity and available headroom can be estimated as the S |

## 3.2 Recording scenario 2

To obtain directional dependent properties of the target device for further spatial algorithm development, following recording scenario is proposed. In addition, the recordings can be used for characterizing raw frequency responses of the integrated microphones. However, the presence of acoustical path, as well as impact of the actual device enclosure should be noted, when assessing the raw microphone signals of such recordings.

Table 2 Recording scenario for obtaining direction dependent properties of the device

| No: | 2 |
| --- | --- |
| Sound source: | High quality loudspeaker |
| Source signal: | Exponential sweep |
| Signal characteristics: | Range: 20 Hz – 20 kHz Length: 5 s Level: -3 dBFS Sample Rate: 48 kHz Bit depth: 16 bit |
| Sound source calibration: | Loudspeaker is calibrated with -3 dBFS pink noise to produce 70 dBSPL (A) at the UE position. Same playback settings are then used with the defined test signal |
| Acoustic environment: | Anechoic chamber, free-field properties expected down to 200 Hz, Noise floor should comply with NR15. |
| Detailed Positioning: | Distance: 0.5 m < dUE < 2 m Source direction: Main diaphragm towards the UE Source angles (azi, ele): Required sound source directions: - Every azimuth f with maximum step of 10°: f = [0°, 10°, …,350°] - All azimuth directions at the elevation = 0° shall be always recorded - For every 3rd azimuth step (f = [0°, 30°, …, 330°]), elevations q with maximum step of 30°: q = [-60°, -30°, 0°, 30°, 60°] - Poles: = [-90°, 90°] - In practice, actual elevation direction may differ slightly (e.g., due to recording facility limitations) - Applied source directions shall be clearly documented Space between the walls and sound source/UE should be at least 0.5 m |
| Reference recording: | Free-field measurement microphone towards the loudspeaker at the UE position |
| Description/additional info: | Silence before and after the measured direction shall be at least 1 s |

Furthermore, based on the received comments and discussion, following general requirement for recordings is proposed for agreement:

[

Requirements for recordings:

- Care shall be taken with proper mounting of the device for the measurements. The possible effect of the test fixture (e.g., microphone stand) on the measured device properties, e.g. due to occlusions or reflections, should be as acoustically transparent as possible, even if it cannot fully be avoided. It shall not obstruct any of the input microphones. The mounting arrangement shall be documented.

Note: Possibility to use FEM/BEM simulations as an alternative to real recordings for development purposes should be examined, assuming that the impact of microphone non-idealities is taken into account.

]

# 4. Conclusion

Based on the presented discussion, we propose to include recording scenarios 1 and recording 2 to the Template for recording scenarios.xlsx, for obtaining characterization and development recordings of each target device. Recordings according to the proposed recording scenarios are then proposed to be made, once the target devices are agreed.

Proposals can be found from the attached Proposed recording scenarios.xlsx -file.

# References

[1]	ITU-T Recommendation P.51 (1996): "Artificial mouth".

[2]	S4-250595: “On raw and compensated microphone signal requirements”, Nokia