3GPP TSG-SA WG4 Meeting #131	S4-250241

# Switzerland, Geneva, 17 – 21 February 2025

Source:	InterDigital France

Title:	Pseudo-CR on Haptics media TR conclusion

Spec:	3GPP TS 26.854 v2.0.0

Agenda item:	10.8

Document for:	Agreement

1. Introduction

This contribution provides additional text related to split hapics media operations.

2. Proposal

Agree to the proposed changes to finalize the TR.

* * * First Change * * * *

### 8.1.3	Haptics media integration in XR Split rendering architecture

#### 8.1.3.1	Overview

As expressed in [24], typical use cases for split rendering includes immersive gaming and immersive communication. These use cases, haptic enabled, are also described in section 5 of this document.

Figure 8.1.3.1-1 is based on figure 6.1.4-1 of [24] and illustrates the haptics media functions as part of the SRS, providing support for haptic media coding and delivery, similarly to audio and video media.

Figure 8.1.3.1-1: Haptic media functions in the User Plane Architecture for Split management architecture

On the UE, Figure 8.1.3.1-2, based on Figure 5.1.2-1[24] illustrates the haptics media entities in the XR baseline client.

Figure 8.1.3.1-2 Haptic media entities in the XR Baseline Client architecture

#### 8.1.3.2	Network functions and UE entities

The haptics media related network functions in the XR end to end split rendering architecture are similar to the one described in section 8.1.2

The haptics media entities on the XR baseline Client consist of:

-	the haptic media codec, handling and decompressing a compressed haptics media bitstream is illustrated in the MAF function of the SRC along with AV codecs.

-	the haptic renderer, handling the rendering of haptics effects using the targeted actuators is illustrated in the presentation engine function of the SRC.

#### 8.1.3.3		Split haptics media operations

When a UE intends to offload part of its haptics media processing to the SRS:

- The SRC and the SRS negotiate the desired haptics media capabilities (or profile) on the M4 interface using a SWAP (Simple WebRTC Application Protocol) message or a data channel message.

- The SRS processes and renders the haptics media content and may use pose or interaction information to spatialised the rendered haptics media content in correlation with other rendered media stream (scene, video, objects, audio).

- The SRS transmits to the UE the resulting haptics media streams.

- The UE decodes and renders the haptics media stream.

* * * End of Changes * * * *