Source:	Bytedance

Title:	[ATIAS_ph2] Scene-based audio capturing - Additional Test Methods

Document for:	Discussion and Agreement

Agenda Item:	7.7

# 1 Introduction

In TR 26.933 - Study on Diverse Audio Capturing systems, the necessity of an immersive audio capturing system in UEs has been mentioned. While test methods for scene-based audio format have been addressed in clause 5.6 in TS 26.260, the error metrics are limited to the frequency response and the directional information of a single source. While these test methods are useful for evaluating IVAS, additional test methods are necessary to fully evaluate the performance of a SBA capturing system, with the potential of evaluating the recording scene with more than a single fix-positioned source. Hence, we propose an additional test factor of a scene-based audio capturing solution applied on UEs.

# 2 Test Method

## 2.1 Test condition

The recording environment is an anechoic chamber that meets clause 4.0.3 of 3GPP TS 26.260 [2]

The test equipment should meet clause 4.0.2 of 3GPP TS 26.260 [2].

## 2.2 Test signal

The following test material may be used:

[It’s still TBD on whether using the following material in separate test run or in a single test run.]

- [Uncorrelated pink noise, with length TBD. The loudness of each test signal is calibrated using the method [need clarification, need SPL measurement] in 3GPP TS 26.260 [2].

- the British-English single talk sequence described in clause 7.3.2 of Recommendation ITU-T P.501 [14], which is calibrated as specified in clause 5.4.2 for the corresponding UE type and the tested capture mode (user or spatial).

- Music. Selection of material is TBD]

## 2.3 Test arrangement

Sound source(s) are set to the following directions pointing to the device centre of the DUT to record candidate signals respectively. If the DUT is a headset UE, the DUT should be worn by a HATS. For tests with multiple sources, uncorrelated pink noise is used for each sound source.

- Test 1-7: One single sound source is used, with its direction following Table 5 in clause 5.6.4.1 of 3GPP TS 26.260 [2].

- Test 8: Two sources at source directions No.2, and No.5 in clause 5.6.4.1 of 3GPP TS 26.260 [2].

- Test 9: Two sources at source directions No.3, and No.7 in clause 5.6.4.1 of 3GPP TS 26.260 [2].

- Test 10: Three sources at source directions No.1, No.4, and No.6 in clause 5.6.4.1 of 3GPP TS 26.260 [2].

| Num. Source | Source azimuthφi | Source elevation θi |
| --- | --- | --- |
| 1 | -90 | 0 |
| 2 | -60 | 0 |
| 3 | -30 | 0 |
| 4 | 0 | 0 |
| 5 | 30 | 0 |
| 6 | 60 | 0 |
| 7 | 90 | 0 |

Table 5 in clause 5.6.4.1 of 3GPP TS 26.260

## 2.4 Reference signals

For reference signals, we calculate the test material defined in 2.2 to Ambisonic, with the same order as recorded SBA, and the incident source direction is same as 2.3:

anmt=l=1LSltYn,mθl,ϕl,

where L is the number of active sound sources in the test scene,  Sltis the source signal, and θlφlis the source direction.

## 2.5 Candidate signal capturing

Candidate signal is recorded by placing the center point of DUT at the center of the test environment, facing the No.4 sound source direction. The test signal is calibrated to a loudness of -26 LKFS as defined in clause 5.5.1 in TS 26.260. SBA time domain signals of the DUT an,mt is recorded as the candidate signal.

## 2.6 Error metric

## 2.6.1 Diffuse Field Loudness:

The W channel in the FOA recording is an omni-directional channel, and hence is selected as the representative of the loudness of the diffuse field in a recording.

For each test, the normalized RMS amplitude of W channel WRMSt=aRMS0,0t=t'=tt+δa0,02tδ is calculated,where δis a short period of time. Then the loudness error is calculated by:

ϵWRMSt=WRMSt-WRMSt2WRMSt2

Example output:  Loudness Error: 0.7%

## 2.6.2 Spatial Correlation:

Spatial correlation, as defined as equation 28 in [3], is used to evaluate the similarity between candidate signals and reference signals per Ambisonic order.

Example output:

## 2.6.3 Interaural of binaural signals.

The recorded SBA is rendered into binaural cues for evaluating perceptual error objectively. The same render should be applied to the candidate signal and the reference signal, using the HRTF of HATS. This error metric is only applied to Test 1-7 in 3.3 with a single active sound source.

ITD and ILD of an,mt and an,mt are defined and calculated as described in clause 5.7.4 of 3GPP TS 26.260 [2].

The ITD error is then calculated by:

ϵITD=ITDan,mt-ITDan,mt

and the ILD error of each octave band  f is calculated by:

ϵILDf=ILDan,mtf-ILDan,mtf

Example result:

ITD error:  -41 μs

ILD error:

| Centre Frequency (Hz) | 500 | 1000 | 2000 | 4000 | 8000 |
| --- | --- | --- | --- | --- | --- |
| ILD error (dB) | 2.53 | 9.07 | 12.58 | 7.42 | 6.89 |

# 3 Proposal

We suggest the WG to accept our proposal as candidate test methods for ATIAS SBA sending-direction. If accepted, we will add the testing setup and methods into ATIAS_Ph2 pdoc.

# References

[1] TS 26.933 Study on Diverse Audio Capturing systems

[2] TS 26.260, Objective test methodologies for the evaluation of immersive audio systems.

[3] Politis, A., & Gamper, H. (2017, October). Comparing modeled and measurement-based spherical harmonic encoding filters for spherical microphone arrays. In 2017 IEEE Workshop on Applications of Signal Processing to Audio and Acoustics (WASPAA) (pp. 224-228). IEEE.