Source:	Nokia

Title:	Initial description of DaCAS target device type for draft TS 26.533

Document for:	Discussion & Agreement

Agenda Item:	7.6 – DaCAS

# 1. Introduction

At the recent SA4#131-bis-e meeting, a set of proposals [1] were agreed as potential target devices. The present input provides description of two hardware configurations for TS 26.533 integration. The description is based on [2] and [3]. Initial structure for draft TS 26.533 was discussed in [4].

# 2. Proposed content

## x.2  Device Types

x.2.1	 Device type 1: Four-Microphone Prototype Devices

x.2.1.1  Overview

Prior to launch of the very first immersive voice services, immersive audio capture devices in the market have generally been designed with other use cases in mind. Prototype device designs can in part help to bridge the gap between commercial example devices and new immersive service opportunities, needs, and expectations.

Device type 1 (Four-Microphone Prototype Devices) comprises two separate hardware configurations, denoted Four-Microphone Prototype Device A (clause X.2.1.2) and Four-Microphone Prototype Device B (clause X.2.1.3). Both prototype device configurations utilize four microphones for immersive capture thus implementing four-sided capture geometries.

Device A provides a simplified form factor for a more limited use case, whereas device B corresponds to a practical smartphone design including typical complexities related to more generalized audio design. Both prototype designs aim to optimize certain immersive audio capture principles relevant for their respective form factors, e.g., use of symmetrical microphone array design (Device A) and maximizing the microphone array aperture based on non-planar quadrilateral array shape (Device B). The effects of these choices on performance of the example solutions can provide further guidelines for diverse capture designs relevant for immersive voice services and IVAS encoder format generation.

x.2.1.2  Four-Microphone Prototype Device A

Four-Microphone Prototype Device A, illustrated in Figure x.2.1.2-1, is a prototype immersive audio capture accessory. This is a cylindrical design that could be incorporated, e.g., in a conference room telco equipment or a webcam device. Such device could, e.g., be placed on a table or on top of a display. The device dimensions and the microphone array design are described in more detail in Table x.2.1.2-1.

Table x.2.1.2-1: Device and microphone array information

| Property | Description |
| --- | --- |
| Device category | Accessory device |
| Device shape | Cylindrical |
| Dimensions | Length 162 mm Width 12 mm (cylinder diameter) Connector bump (HDMI-D connector): Width 20 mm Height 6 mm Protrudes by 3.95 mm |
| Microphone array design | Symmetrical array design Planar array shape Two-dimensional, four-sided capture geometry |
| Microphone port designations | Channel 1 – Mic Front Right Channel 2 – Mic Rear Right Channel 3 – Mic Front Left Channel 4 – Mic Rear Left |
| Microphone port positions | Front microphone ports centred with a 140-mm spacing Rear microphone ports centred with a 140-mm spacing |

The Device A form factor enables a simple two-dimensional, four-sided capture geometry by symmetrical placement of the four microphone ports towards the ends of the device’s outer shell. There is generally minimal acoustic shadowing of any of the microphone ports, especially in ideal recording scenarios. The prototype provides connectivity for recording using an HDMI-D connector on the bottom side of the device, where a protruding shape is apparent.

Four-Microphone Prototype Device A is used in landscape orientation. When device is, e.g., placed on a display, front side (Mic Front Left and Mic Front Right) faces the main user direction.

Figure x.2.1.2-1: Four-Microphone Prototype Device A

x.2.1.3  Four-Microphone Prototype Device B

Four-Microphone Prototype Device B, illustrated in Figure x.2.1.3-1, is a prototype immersive audio capture smartphone. It utilizes a typical monoblock shape with a camera bump. The device dimensions and the microphone array design are described in more detail in Table x.2.1.3-1. Specifically, the Device B overall dimensions are derived based on the average dimensions from data collected for 2020-2024 smartphone devices in TR 26.933.

Table x.2.1.3-1: Device and microphone array information

| Property | Description |
| --- | --- |
| Device category | Smartphone |
| Device shape | Monoblock |
| Dimensions | Length 160.6 mm Width 74.4 mm Depth 8.6 mm Camera bump (for position, see Figure x.2.1.3-1): Length 35 mm Width 35 mm Depth 4 mm |
| Microphone array design | Practical array design maximizing microphone array aperture Non-planar quadrilateral array shape Three-dimensional, four-sided capture geometry |
| Microphone port designations | Channel 1 – Mic #1 Channel 2 – Mic #2 Channel 3 – Mic #3 Channel 4 – Mic #4 (Mic #5 and Mic #6 not used in Device B configuration) |
| Capture orientations | Landscape Portrait On table (display side up) |

Targeting high-quality immersive audio capture, this device geometry enables a three-dimensional, four-sided capture geometry that is designed to maximize the aperture of the microphone array. The array has a non-planar quadrilateral shape (or skew quadrilateral).

Four-Microphone Prototype Device B can be used in multiple orientations. Main orientations are portrait orientation (see Figure x.2.1.3-1), landscape orientation (camera bump in top left corner), and flat on table with display face up. The prototype hardware does not include orientation sensors.

Figure x.2.1.3-1: Four-Microphone Prototype Device B

# 3. Summary

The source proposes description for the two hardware configurations “Four-Microphone Prototype Device A” and “Four-Microphone Prototype Device B” to progress work on draft TS 26.533.

Current proposal is targeting the initial structure shown in [4].

# References

[1]	S4-250745: “Proposal of new potential target devices for DaCAS”, Xiaomi Technology

[2]	S4aA250010: “Proposed DaCAS target device type”, Nokia

[3]	S4-250567: “Proposed DaCAS target device type“, Nokia

[4]	S4aA250020: “Target Device Collection for DaCAS”, Xiaomi Technology