3GPP TSG-SA WG4 Meeting #132 	S4-250962

# Fukuoka, Japan, 19th – 23rd May 2025

Source:	Fraunhofer IIS

Title:	Proposal on DaCAS Acoustic Properties Measurements

Document for:	Agreement

Agenda Item:	7.6

- Introduction

The DaCAS Template for recording setups and scenarios-V2.0 [1] already provides two templates on Recording setups and scenarios. To better understand the acoustic properties of the device (e.g., shadowing effects), we are proposing a new method for measuring the acoustic properties.

In particular, we propose to carry out the measurements with a higher spatial resolution in the horizontal plane as compared to [1] to better mimic the spatial resolution of the human hearing. Moreover, we propose to measure with equal spatial resolution along the 3 orientation axes (x-y-z) of the device under test (DUT). This allows us to understand the acoustic properties of the DUT with equal accuracy for the 3 typical device orientations. This is needed since in practice, any of these orientations is equally probable when using the device. We also propose to use a longer measurement signal for better SNR.

- Proposed Measurement Method

The following new method is proposed:

- Place DUT (e.g., mobile phone prototype) on a turntable at a height of around 1,2m

- Play back a sine sweep from a high-quality loudspeaker at 1 – 2m distance from the DUT

- Record the sine sweep with the DUT

- Rotate turntable by 5° and repeat the recording until the entire 360° range is covered

- Repeat the measurements for 3 different orientations of the DUT

- Share the 72 x 3 recordings along with the sweep signal (playback signal)

Additional constraints:

- The measurements should be carried out for 3 different orientations of the device: landscape orientation, portrait orientation, “display-up” orientation

- The recordings should be carried out in a silent room with at most mild reverberation

- The loudspeaker should be located at the same height as the DUT

- A logarithmic sine sweep of at least 5s (ideally 10s) ranging from 20Hz to 20kHz should be used

- The recordings should be carried out at 48kHz sampling rate

- The recordings should be stored as uncompressed PCM/WAV with at least 24 bits

- The raw microphone signals must be recorded, i.e., any noise reduction or other audio pre-processing of the DUT must be disabled or by-passed

- The turntable, stand, and DUT mounting should have only minimal acoustic effects (e.g., no microphones should be blocked, minimal acoustic footprint)

- The microphone channel ordering of the DUT, the DUT orientation, the turntable rotation direction, and the 0° position must be documented (if possible, with photos)

- The recording SNR should be reasonably high (i.e., the playback volume should be reasonably high) but no clipping must occur in the microphone signals

- Discussion

The proposed method is generally similar to method 2 as already contained in [1]. However, the following differences can be pointed out:

- The orientation of the DUT (testing of three different orientations of the DUT instead of changing the elevation of the DUT) - this allows us to understand the acoustic properties of the DUT with equal accuracy for the 3 typical device orientations.

- Maximum step size of azimuth f (5° vs. 10°) - a higher spatial resolution in the horizontal plane as compared to [1] to better mimic the spatial resolution of the human hearing

- The length of the measurement signal (logarithmic sine sweep) should be at least 5s, ideally 10s.

- Conclusion

It is proposed to agree on the inclusion of the proposed method, as introduced in section 2, into the Recording setups and scenarios. An Excel sheet containing the corresponding template is attached.

References:

[1] 3GPP S4aA250015 - Update DaCAS templates to v2.0