3GPP TSG-SA WG4 #132	S4-251004

Fukuoka, Japan, 19-23 May, 2025

Source:	Qualcomm, Incorporated

Title:	Assessing Diffuseness Reproduction of Loudspeaker Layouts for ATIAS

Agenda item:	14.2

Document for:	Discussion

# Introduction

This document follows up on a related presentation [1] at 3GPP SA4-131, which proposes the use of the TS 103 224 reconstruction method in combination with the periphonic speaker array as defined in Clause 4.1.1.3 of 3GPP TS 26.260 [2]. Follow up discussions related to the terms of diffuseness, ambience, and complex sound scene reproduction ensued.

It is the view of the Source that a single array that can achieve a high degree of diffuseness and directionality, and the ability to simultaneously reproduce a multitude of sound sources, is desired for the testing of IVAS audio capture devices. This is both from the perspective of suitable accuracy as well as practical considerations in a laboratory environment (e.g. due to the need to minimize multiple different loudspeaker configurations for various tests).

This contribution focuses on the ability of several different loudspeaker arrays to reproduce the original diffuseness of a recorded sound scene. It concludes that the use of a loudspeaker layout that is uniformly distributed in space, with a high channel count, is most suited for this objective.

# Technical Background

The multipoint reconstruction method described in [3] is based on the principle of regularized inversion in the frequency domain; a comprehensive summary is given in [1] and an abridged set of variable definitions is reiterated below.

## Theoretical Overview

For frequency k, the transform from a vector of loudspeaker signals Xk to a vector of microphone signals Yk is characterized by the equation

Yk=HkXk

where the (i,j)th entry of matrix Hk represents the complex transfer function between the ith microphone element loudspeaker and jth loudspeaker element. A prototype reconstruction signal Xk is then derived by filtering the microphone signal Yk with a compensation filter Ck such that

Xk=CkYk

where Ck serves as a transform between speaker and microphone subspaces. With the inclusion of the final equalization filter Dk, the final reconstruction speaker signal Xk and the corresponding microphone signal Yk are expressed as

Xk=DkXk=DkCkYk

Yk=HkXk=HkDkCkYk

According to TS 103 224, the level difference and complex coherence between reconstructed microphone signal Yk and the original microphone signal Yk are used as metrics of reconstruction quality. In [1], the source defined the Reconstruction System Matrix (RSM), denoted Sk, to inspect the reconstruction quality of a given combination of loudspeaker layout, microphone layout, and playback environment.

For directional plane waves in the presence of isotropic background noise, soundfield diffuseness can be quantified through the eigendecomposition of its spherical harmonic (SH) representation via principal component analysis (PCA) [4].

To obtain the SH representation, the time-domain microphone signals y(t) are multiplied by a SH encoding matrix E,

bt=Eyt

where the lth channel of the resulting representation is indexed to expansion coefficients according to a given channel ordering convention

bt=b0t⋯bl(t)⋯bN+12(t)T

From the SH representation, a covariance matrix is calculated and decomposed into eigenvalues Λ and eigenvector matrix V to be used in analysis

Cbb=EbtbTt

Cbb=VΛVT

where V is the matrix of eigenvectors spanning the SH expansion on a unit sphere and Λ is the diagonal matrix of eigenvalues λ

Λ=diagλ1, λ2,⋯,λN+12

As described in [4], prominent eigenvalues indicate the presence of directional sources, while a collection of lower, similarly-valued eigenvalues indicates a diffuse soundfield. In the scope of the present work item, it is of interest for a reconstructed soundfield to exhibit a similar distribution of eigenvalues, and in turn, diffuseness, as the originally recorded soundfield.

# Experiment

## Objective

The objective of this experiment is to examine the quality of several reconstruction configurations as expressed by the eigenvalue distribution of the SH covariance.

## Measurement Setup

In this work, two reproduction conditions are compared: Condition A consists of eight (8) loudspeakers arranged with low periphony as recommended in [5], and Condition B consists of twenty-five (25) loudspeakers arranged over a spherical surface integration exhibiting high periphony. Conditions A and B are identical to the validation and periphonic conditions, respectively, as described in [1]. Furthermore, two additional subsets of loudspeakers are selected within Condition B; these represent eight-loudspeaker layouts that exhibit lower and higher periphony in a constant room environment.

In order to obtain a spherical harmonic signal of sufficient quality for evaluating devices under test (DUT), it is important to use a microphone array with capsules arranged uniformly on a spherical surface. In addition, the array should either impart minimal disturbance of the surrounding pressure field or allow for compensation in a geometrically simple manner. Two such candidates are the Core Sound OctoMic and mhAcoustics em32. In this work, the Source uses the OctoMic.

The microphone output is converted into second order Ambisonics using an encoding matrix E provided by the manufacturer, the resulting signal utilizes N3D normalization and ACN channel ordering conventions. The transformations used for analysis are depicted in Figure 1.

Figure 1: Block diagram depicting SH eigenanalysis

The soundfield under analysis is an outdoor sports scene comprised of one prominent directional component over a diffuse component. The expected covariance of the HOA signals is approximated via temporal averaging; the Source uses consecutive analysis windows of 512 samples with no overlap between windows, over an interval of 1 s.

## Eigenvalue Distribution

From the measurement setup described in Section 3.2, Figure 2 displays a distribution of 8 eigenvalues for the original soundfield and each reconstruction condition.

Figure 2: Eigenvalue distribution of the spherical harmonic signal.

It is observed that the three most prominent eigenvalues are similar in magnitude across the original soundfield and reconstruction conditions. Eigenvalues 4-8 exhibit more divergence between the reconstruction conditions and original soundfield; the eigenvalues for Condition B are observed to decrease up to 6 dB as compared to those of Condition A.

From the higher decay (less uniformity) of eigenvalues, it is apparent that there is less diffuseness of the reconstructed soundfield using lower speaker counts. This is despite the original metrics of spectra being preserved as required in [3]. This result is not surprising given that it is not an explicit objective of [3] to achieve a diffuse soundfield, but rather to achieve a spectrum equalized reconstruction. Additionally, it should not be surprising that a loudspeaker layout with uniform spatial distribution would result in higher diffuseness than a layout that favors a higher speaker count around the horizontal plane (ear height).

Conclusion

The results above support the use of a loudspeaker layout that is uniformly distributed in space, with a higher channel count to reproduce the diffuseness of a captured soundfield. A separate contribution will focus on the reproduction quality of directional soundfield components.

# References

| [1] | 3GPP S4-250612, “Using a Periphonic Speaker Array for Improved Soundfield Reconstruction”. |
| --- | --- |
| [2] | 3GPP TS 26.260, “Objective test methodologies for the evaluation of immersive audio systems,” 2024. |
| [3] | ETSI TS 103 224, “A sound field reproduction method for terminal testing including a background noise database,” 2022. |
| [4] | C. T. J. Nicolas Epain, “Spherical Harmonic Signal Covariance and Sound Field Diffuseness,” IEEE/ACM Transactions on Audio, Speech, and Language Processing, vol. 24, no. 10, pp. 1796-1807, 2016. |
| [5] | ETSI ES 202 396-1, “Background noise simulation technique and background noise database,” 2022. |
| [6] | 3GPP 26.132, “Speech and video telephony terminal acoustic test,” 2022. |
| [7] | 3GPP SP-241314, ATIAS_Ph2 Tdoc, “WID on Terminal Audio quality performance and Test methods for Immersive Audio Services, Phase 2”. |

The encoding matrix provided from the manufacturer follows a hybrid second-order scheme, which encodes the 8 microphone signals to 8 of the 9 channel coefficients available in second order Ambisonics. Channel l=7 (corresponding with n=3,m=0) contains no signal information and is omitted from eigenanalysis.