3GPP TSG SA WG4 #132	S4-250787

Fukuoka, Japan, May 19-23, 2025	revision of S4aI250080

Agenda item: 	8.3

Source: 	Qualcomm Germany

Title: 	On NTN intended service area in the Service Announcement

Document for	Agreement

# Introduction

During SA4#131-bis-e, 3GPP SA4 received a straightforward LS from RAN2 in S4-250491

RAN2 is working on a solution to provide UEs with geographical information of where within an NTN cell an MBS broadcast service is applicable. This area is called the “intended service area” (ISA) whose geographical definition is provided to the UE via SIB and is associated with an identifier (ISA ID). The attached CR includes the changes introduced by SA2 to adopt the ISA for NTN access.

RAN2 would like to ask whether including the ISA in the Service Announcement would impact in SA4 specifications and would kindly request SA4 to confirm this can be supported within the Rel-19 timeframe.

RAN2 kindly request SA4 to confirm whether the inclusion of the ISA in the Service Announcement can be supported within the Rel-19 timeframe.

Meanwhile, questions to SA2 and RAN2 have been compiled by SA4 in a draft reply in S4aI250077:

1.	Please clarify which Cell Ids/TAIs to include in the MBS User Service Announcement for NR NTN. SA4 assumes that Cell Ids/TAIs are obtained from local configuration within MBSF/NEF.

2.	Please clarify whether an AF (internal or external) can select whether to use NR NTN for the MBS Broadcast Session or not.

3.	Please clarify how to obtain the Intended Service Area, when Cell Ids/TAIs are used by the AF for MBS Service Area description.

4.	Please clarify whether ISA ID is obtainable from RAN so that the local configuration can also contain the ISA IDs for a given MBS Service Area.

# Analysis

After consultation with RAN2 and SA2 experts, the following answers to the above questions are considered.

1.	Please clarify which Cell Ids/TAIs to include in the MBS User Service Announcement for NR NTN. SA4 assumes that Cell Ids/TAIs are obtained from local configuration within MBSF/NEF.

o	Cell IDs and TAIs in NTN are no different to what is there in TN. The MBS Service Area parameter defined in TS 29.571 includes this information and is included in TS 26.517. Clause 5.9.4.4 of TS 29.571 defines the MbsServiceArea

o	Support of ISA and ISA ID is totally optional, and UE may not support it and network has no idea about it, then the cell IDs, TAI must be present for those UEs who do not support ISA as legacy behaviour. ISA is just additional information to assist UE for power saving in searching/scanning. It does not mean we can cut off any existing information. The intendedServAreaList is actually already present in MbsServiceArea type.

2.	Please clarify whether an AF (internal or external) can select whether to use NR NTN for the MBS Broadcast Session or not.

o	There is no change to selection procedure. If the MBS service is needed in a particular area and that area is served by NTN, then AF chooses NTN.

3.	Please clarify how to obtain the Intended Service Area, when Cell Ids/TAIs are used by the AF for MBS Service Area description.

o	The mapped cell ID would be determined by NG-RAN so the AF can keep using existing cell IDs as in TN.

o	The AF can continue to use regular cell ID as AF sends service announcement to UE and UE does not understand Mapped Cell ID. The AMF can convert the regular cell ID to Mapped cell ID when it sends MBS Session Setup message to RAN. How AF knows ISA, is left to AF’s OAM configuration of ISA or Cell ID or TAI.

4.	Please clarify whether ISA ID is obtainable from RAN so that the local configuration can also contain the ISA IDs for a given MBS Service Area.

o	Note that RAN2 has not asked on ISA ID. When NG-RAN broadcasts ISA in system information, it locally gives an ID or INDEX to each area, called ISA ID. AF does not need to know ISA ID. AF needs to provide existing information cell IDs, TA list PLUS additionally if possible, ISA (geographic) as complementary information.

o	If at all, the ISA ID may be used to reduce signaling overhead in service announcement, i.e., adding ISA ID instead of ISA. But this seems to be unnecessary complexity as it would require that the ISA ID is synchronized between NG-RAN (the one who broadcasts SIB) and AF (the one who includes it in USD). It is much simpler to not to use ISA ID and focus on action of the incoming.

It is unclear in which direction we are going.

Please bear in mind that TS 29.571 defines the service area and has added the intendedServAreaList.

| 2025-03 | CT#107 | CP-250046 | 0630 | 1 | B | MBS broadcast support for NTN | 19.2.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

#### 5.9.4.4	Type: MbsServiceArea

Table 5.9.4.4-1: Definition of type MbsServiceArea

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ncgiList | array(NcgiTai) | C | 1..N | List of NR cell ids with their pertaining TAIs (NOTE 1, NOTE 3). |
| taiList | array(Tai) | C | 1..N | List of tracking area Ids (NOTE 1). |
| intendedServAreaList | array(IntendedServiceArea) | O | 1..N | List of NR NTN Intended Service Areas (NOTE 2) |
| NOTE 1: The MBS Service Area consists of the union of the cells in the tracking areas listed in the taiList IE and the cells listed in the ncgiList IE. At least one of the ncgiList IE and taiList IE shall be present. NOTE 2: The NR NTN Intended Service Area may be sent by the NEF/MBSF to the NG-RAN, via the MB-SMF and AMF, to further restrict the MBS service area. It contains information that shall only be used by the NG-RAN, i.e. the MB-SMF and AMF shall transfer this information transparently. See 3GPP TS 23.247 [72]. NOTE 3: In case of NR NTN, the cell IDs contained in the MBS service area may correspond to mapped cell IDs as defined in 3GPP TS 38.300 [73]. |  |  |  |  |

Also note that TS 26.517 already supports the delivery of the MbsServiceArea in the AvailabilityInformation

Table 5.2.9-1: Semantics of AvailabilityInformation data type

| Property name | Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mbsService‌Areas | array(Mbs‌Service‌Area) | O | 1..N | The Target service areas of this MBS Distribution Session, as defined in table 4.5.8-1 of TS 26.502 [6]. |
| mbs‌FSA‌Id | MbsFsaId | C | 0..1 | (Broadcast MBS Distribution Session only.) The MBS Frequency Selection Area (FSA) Identifier of the (broadcast) MBS Distribution Session in the parent service area, as defined in table 4.5.8-1 of TS 26.502 [6]. |
| nrParameters | array(Nr‌ParameterSet) | M | 1..N | (Broadcast MBS Distribution Session only.) The New Radio transmission parameters in the parent service areas, expressed using the data type specified in table 5.2.9-2. |
| nrRedCapUEInfo | NrRedCapUeInfo | O | 0..1 | (Broadcast MBS Distribution Session only.) Indicates whether the MBS session is suitable for consumption by NR RedCap UEs and/or non-RedCap UEs as defined by Target UE classes in TS 26.502 [6]. The data type and its enumerated values are specified in TS 29.571 [30]. If omitted, no information is known and the value NON_REDCAP_UE_ONLY may be assumed. |

And hence the support for delivering the ISA in the Service Announcement is implicitly supported within the Rel-19 timeframe, we just to upgrade the openapi version to the latest release.

# Discussion during and after AHG meeting

A quick review of our Rel-18 specifications:

- TS 26.517 references MbsServiceArea data type from UserServiceDescription.distributionSessionDescriptions[].availabilityInfos (table 5.2.9-1) and cites TS 26.502 table 4.5.8-1 to motivate this, but this motivation is actually missing (see 2 below).

  - This gives us TAI list and Cell Id list.

  - From Rel-19, it also gives us the nasty ASN.1 encoding of Intended Service Area.

    - This encodes geo-locus as circle or polygon.

    - It also encodes the (unwanted) civic address.

- TS 26.502 table 4.5.8-1 makes to mention of Target service areas, and neither does the static information model in figure 4.5.2-1.

  - Conclusion: this stage-3 feature in the User Service Announcement is not currently motivated at stage-2!

  - (Agrees with Thorsten's analysis.)

- The static information model for the MBS Distribution Session does support the concept of Target service areas in figure 4.5.2-1 and clause 4.5.6 and table 4.5.6-1.

  - I'm not sure why that isn't exposed in the announcement mentioned in 2 above, but that's how it ended up.

At stage-3, TS 29.580 allows the either of the following to be provisioned in the NEF/MBSF:

- Full MbsServiceArea, as above, including the new Rel-19 ASN.1 representation of Intended Service Area).

- Cut-down ExternalMbsServiceArea, which excludes internally-significant identifiers Cell Id and TAI and doesn't include the new Intended Service Area.

As for the stage-2 design of Target service areas, TS 26.502 is currently silent, so the definition of TS 23.247 currently prevails. We could be more explicit in TS 26.502 clause 4.5.6.

In TS 26.517, I would be fine to deprecate the AvailabilityInformation.mbsServiceAreas property and to define a more appropriate TargetServiceArea data type in OpenAPI YAML to represent this high-level concept. Based on what is specified in TS 29.571, we can then cherry pick what is sensible to include in the data type for use in the User service Announcement. For example:

- Target service area: MbsServiceArea:

  - Cell Id list: array of NcgiTai. Yes? Can reuse existing data type from TS 29.571.

  - Tracking Area Identifier (TAI) list: array of Tai. Yes. Can reuse existing data type from TS 29.571.

  - Intended Service Area (ISA) list: array of IntendedServiceArea (each encoded as an ASN.1 blob):

    - Geographic service area: GeoServiceArea:

      - Geographical area list (circles and/or polygons): array of GeographicArea. Yes. Can reuse existing data type from TS 29.571.

      - Civic address list: array of CivicAddress. No, thanks.

Yes, I would correct both Rel-17 and Rel-18 of TS 26.517.

Finally, I would say there isn't a rush to complete this at SA4#132. We could agree a way forward next week and work on Category F alignment CRs to TS 26.502 and TS 26.517 Rel-17 (plus Category A mirror CRs to Rel-18) for agreement at SA4#132-bis-e in July. If we can at least agree a way forward, we could respond to the RAN2 LS next week confirming this way forward. Or we could postpone replying until we have agreed CRs to attach to the LS response.

After further discussion

If I understand the logic correctly, you're saying that TS 26.517 Rel-17/Rel-18 is fine to leave as is because the MbsServiceArea data type we currently used in our User Service Announcement provides the geographical targeting property to satisfy the stage-2 requirement in TS 23.247. I can accept that argument. At minimum, though, I think we will still need Rel-17/Rel-18 Category F corrections to our stage-2 TS 26.502 (and probably Rel-19 too now) to fill in the small gap we have identified between the SA2 stage-2 and our stage-3 in figure 4.5.2-1, clause 4.5.6 and table 4.5.6-1. These three CRs to TS 26.502 would all be under Work Item code 5MBUSA+TEI17, I think.

And, if we do agree to make a change to TS 26.517 Rel-19, I think that deprecating the mbsServiceArea property in favour of a new property with a more generic name, such as targetServiceArea sounds like a good way forward. This CR could be tagged with RAN2's Work Item code, NR_NTN_Ph3-Core.

# Proposal

It is proposed that

- Generate Rel-17/Rel-18/Rel-19 Category F corrections to TS 26.502 to fill in the small gap between the SA2 stage-2 and stage-3 in figure 4.5.2-1, clause 4.5.6 and table 4.5.6-1 under Work Item code 5MBUSA+TEI17.

- Make a change to TS 26.517 Rel-19, deprecating the mbsServiceArea property in favour of a new property with a more generic name, such as targetServiceArea tagged with RAN2's Work Item code, NR_NTN_Ph3-Core.

response to the LS in S4-250491 significantly thanking RAN2 for the request, and responding that we confirm that the inclusion of the ISA in the Service Announcement based on the above approach.

This work may be completed during SA4#132 or in a follow-up telco.