# 3GPP TSG-SA4 Meeting #132	S4-250955

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025 						 revision of S4-250523

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.565 | CR | 0003 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [SR_MSE] Enhancements to SR_MSE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_MSE |  |  |  |  |  | Date: |  |  | 2025-05-19ss |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SR_IMS has defined split rendering profiles that complement those defined in SR_MSE and corresponding functions and procedures. These SEAMLESS_ADAPTIVE, DELAY_ADAPTIVE, ASSET REQUEST and EYE STATUS features are also relevant in SR_MSE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the processing delay adpatation feature: - Add call flow - Add information message format - Add extra configuration - Add metadata capability in the SR profiles - Add enabler to use the processing delay adpatation feature with Adaptive Split rendering profile. Add Seamless adaptive split Add Asset request Add eye status |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing features from SR_IMS in SR_MSE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 8.3.2, 8.4.2, C.1.2.2.3, C.1.2.3.6, C.1.3.2.3, C.1.3.3.5, C.2.1, C.2.2, C.2.3, C.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST change

## 5.2	Procedures and Call Flows

### 5.2.1	Call flow for Split Rendering instance discovery

#### 5.2.1.1	Call flow for edge server and split rendering session setup

Figure 5.2.1.1-1 demonstrates a general call flow for split-rendering.

Figure 5.2.1-1: High-level call flow for split-rendering

Steps:

1.	In this optional step, the Media Application Provider requests and sets up the edge server(s) used for the split-rendering as described in TS 26.506 [5] clauses 6.1 or 6.2. The Media Application Provider may use any other method to allocation edge servers, or leave it to the MNO to set up appropriate edge servers to run the split-rendering process.

2.	The Media Application Provider provisions the split-rendering session using M1 and M3, as defined in call flow of clauses 5.2.1.1. If the edge servers were provisioned in step 1, the edge servers ids are provided in this session to employ them for split-rendering.

NOTE: In the case of the client-driven edge management (TS 26.501 [18], clause 8.1), only the client-driven split-rendering (5.2.1.1) is applicable.

3.	The split-rendering session is set up according to clause 5.2.2.

#### 5.2.1.2	Client-driven procedures and call flows

Figure 5.2.1.2-1 demonstrates a call flow for setting up the split rendering by the client.

Figure 5.2.1.2-1: High-level call flow for initiating a split

Steps:

1.		The Media Application Provider requests from the Media AF, the creation of a Provisioning session for split rendering.

2.		The SR configuration is announced to the MSH as part of the Service Access Information.

3.		The Application requests the deployment of split rendering from the SRC.

4.		The SRC requests the discovery of a suitable SRS from the MSH. It may provide the client’s media capabilities as input parameters.

5.		The SRC and SRS negotiate the configuration of the split rendering session.

6.		The SRS starts the split rendering process.

7.		The SRC provides the session information via the M11 interface and requests the application of dynamic policy and subscription to network assistance from the Media AF, via the MSH.

8.		The SRC establishes the WebRTC session.

9.		The SRC informs the application that the split-rendering on edge is running via M7.

10.		The SRC sends uplink metadata, such as pose and action information.

11.		0The SRS sends the rendered media to the SRC.

### 5.2.2	Call flow for Split Rendering session setup

The split rendering operation can be described by the call flow in Figure 5.2.2-1.

Figure 5.2.2- 1 High-level call flow for split rendering session setup and operation

The steps are:

1.		The Presentation Engine discovers the split rendering server and sets up a connection to it. It provides information about its rendering capabilities and the XR runtime configuration, e.g the OpenXR configuration may be used for this purpose.

2.		In response, the split rendering server creates a description of the split rendering output and the input it expects to receive from the UE.

3.		The Presentation Engine requests the buffer streams from the MAF, which in turn establishes a connection to the split rendering server to stream pose and retrieve split rendering buffers.

4.		The Source Manager retrieves pose and user input from the XR runtime.

5.		The Source Manager shares the pose predictions and user input actions with the split rendering server.

6.		The split rendering server uses that information to render the frame.

7.		The rendered frame is encoded and streamed down to the MAF.

8.		The MAF decodes and processes the received frame.

9.		The MAF passes the decoded frame to the Scene Manager which passes it to the XR Runtime.

10.		The XR runtime composes and renders the frame onto the display.

### 5.2.3	Call flow for Processing Delay Adaptation

For processing delay adaptation, the general procedures and call flows in clause 5.2.2 applies with the following additions and modifications.

-	The SRS and SRC agree on the QoE metrics considered for delay adaptation. That may contain all or a subset of the QoE latency metrics (e.g., poseToRenderToPhoton, roundtripInteractionDelay) negotiated in the configuration message in clause 8.4.2.3.

-	In the rendering loop, The SRC measures and collects the QoE metrics considered for delay adaptation. The SRC may send periodically the delay adaptation information to the SRS when the measured QoE metrics goes out of the target delay range.

Figure 5.2.3-1 illustrates a high-level call flow set up and operation for a split rendering session which supports the processing delay adaptation profile.

Figure 5.2.3-1: High level call flows for Processing Delay Adaptation Profile

The steps are:

Steps 0 to 4 are as described in clause 5.2.2.

5.	The Source Manager shares the pose predictions and user input actions. In addition, the metadata may include a delay adaptation information message to the SRS. The trigger to send the delay adaptation information message is based on the configured periodicity to inform the SRS that the measured delay of a QoE latency metric is out of the target delay range. For example, the measured poseToRenderToPhoton QoE latency metric goes out of the target delay range due to new network conditions.

6a	The SRS may adjust the processing delay based on the delay adaptation information message.

NOTE: 	For example, the SRS may change the LoD of the objects that are part of the scene for the delay adaptation.

6b	The SRS renders the frame.

7 to 10:	As described in clause 5.2.2.

Next change

### 8.3.2	Metadata Formats

#### 8.3.2.1	General

Both SRC and SRS shall support the usage of the WebRTC data channel for the exchange of split rendering metadata. The WebRTC data channel shall declare “3gpp-sr” as the data channel sub-protocol. The message content format depends on the type of the message. The data channel sub-protocol is defined in clause 8.3.3.

Message types shall be unique identifiers in the URN format. This clause defines a set of message types and their formats. The messages are derived from the OpenXR API to ensure smooth operation with AR devices that support OpenXR. In case other XR APIs are used, mapping the message payload to the appropriate XR API structures shall be performed by the split rendering client.

#### 8.3.2.2	Pose Format

The pose format that is used by all split rendering profiles defined by this specification shall comply with the format defined in TS 26.119 [4] clause 12.2. The pose information shall be carried as part of the data channel messaging mechanism defined in clause 8.3.3 and shall be provided in JSON format. The message type shall be “urn:3gpp:split-rendering:v1:pose”.

#### 8.3.2.3	Action Format

The action information format that is used by all split rendering profiles defined by this specification shall comply with the format defined in TS 26.119 [4] clause 12.3. The action information shall be carried as part of the data channel messaging mechanism defined in clause 8.3.3 and shall be provided in JSON format. The message type shall be “urn:3gpp:split-rendering:v1:action”.

#### 8.3.2.4	Processing Delay Adaptation Format

During a split rendering session, the operating environment of the split rendering server, the split rendering client or the network conditions may change. Consequently, the roundtrip delay may need to be adjusted to deliver a consistent QoE.

When processing delay adaptation procedure is enabled, the SRC checks for the QoE metrics being monitored whether the measured delays are within the target delay range or not. When a measured delay is outside the target delay range for a QoE metric, the SRC may report the measured delay based on the configured periodicity.

The delay adaptation information message format shall comply with the format defined in Table 8.3.2.4-1. The delay adaptation information shall be carried as part of the data channel messaging mechanism defined in clause 8.3.3 and shall be provided in JSON format. The message type shall be “urn:3gpp:split-rendering:v1:daqoe:information”.

Table 8.3.2.4-1 Message format for Delay Adaptation information

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the IMS-based split rendering session. |
| type | string | 1..1 | urn:3gpp:split-rendering:v1:daqoe:information |
| message | object | 1..1 | Message content |
| qoeMetrics | array | 1..1 | An array of the QoE metrics for which delay adaptation is needed. This qoeMetrics array may contain all or a subset of the QoE metrics negotiated in the configuration message in clause 8.4.2.3. |
| qoeMetricId | string | 1..1 | A unique identifier of the QoE metric within the scope of the split rendering session. |
| delayValue | number | 1..1 | The measured delay value of that QoE metric. |

Next change

### 8.4.2	Split Rendering Configuration Format

#### 8.4.2.1	Introduction

The Split Rendering client establishes an XR session locally based on the device configuration and user selection. The SR client defines the view configuration (e.g. mono or stereo views), the projection format (such as projection, equirectangular, quad, or cubemap), the swap chain image configuration, etc.

In addition, XR space and action configurations are negotiated between the SR client and server. This includes defining common XR spaces and defining and selecting actions and action sets.

The format is extensible to support the exchange of additional/future configuration information.

#### 8.4.2.2	Split Rendering Configuration Format

The session configuration information shall be in JSON format. It shall have the following format:

Table 8.4.2.2-1 Split Rendering Configuration Format

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| renderingFlags | Array(SR_CONFIG_FLAGS) | 0..1 | Provides a set of flags to activate/deactivate selected rendering functions. The defined SR_CONFIG_FLAGS are: - FLAG_ALPHA_BLENDING - FLAG_DEPTH_COMPOSITION - FLAG_EYE_GAZE_TRACKING |
| splitRenderingProfile | array(URI) | 0..1 | A list of supported split-rendering profile identifiers on the UE. The profile identifiers are listed in Annex C for each profile. |
| deviceCapabilities | Object | 0..1 | Device capabilities as defined in TS 26.119 [4], clause 6.1. |
| spaceConfiguration | Object | 0..1 | The space configuration is typically sent by the split rendering server to the split rendering client. Upon reception of this information, the SR client uses this information to create the reference and action spaces as well as to agree on common identifiers for the XR spaces. |
| referenceSpaces | Array | 0..1 | An array of reference spaces and their identifiers. |
| id | number | 1..1 | A unique identifier of the XR space in the context of the split rendering session. |
| refSpace | enum | 1..1 | One of the defined reference spaces in OpenXR. These may be: XR_REFERENCE_SPACE_TYPE_VIEW, XR_REFERENCE_SPACE_TYPE_LOCAL, or XR_REFERENCE_SPACE_TYPE_STAGE. |
| actionSpaces | Array | 0..1 | An array of action spaces that need to be defined by the split rendering client in the XR session. |
| id | number | 1..1 | A unique identifier of the XR space in the context of the split rendering session. |
| actionId | number | 1..1 | Provides the unique identifier of the action. |
| subactionPath | string | 1..1 | The subaction path identifies the action, which can then be mapped by the XR runtime to user input modalities. |
| initialPose | Pose | 0..1 | Provides the initial pose of the new XR space’s origin. |
| viewConfiguration | Object | 0..1 | Conveys the view configuration that is configured for the XR session. |
| type | Enum | 1..1 | The type indicates the view configuration. Defined values are MONO and STEREO. Other values may be added. |
| width | number | 1..1 | The recommended width of the swapchain image. |
| height | number | 1..1 | The recommended height of the swapchain image. |
| compositionLayer | string | 1..1 | An identifier of the selected composition layer. |
| minPoseInterval | number | 0..1 | The minimum time interval between two consecutive pose information instances sent to the network, in milliseconds. |
| fovs | Array | 0..1 | An array that provides a list of the field of views (FoV) associated with each view. |
| fov | Object | 1..n | Indicates the four sides of the field of view used for the projection of the corresponding XR view. The number of views n is determined by the type enum of the viewConfiguration. Both the viewPoses in the Pose Format and the fovs arrays shall be ordered in a consistent way (i.e., a same index can be used to retrieve the view pose and the related FoV information). |
| angleLeft | number | 1..1 | The angle of the left side of the field of view. For a symmetric field of view this value is negative. |
| angleRight | number | 1..1 | The angle of the right side of the field of view. |
| angleUp | number | 1..1 | The angle of the top part of the field of view. |
| angleDown | number | 1..1 | The angle of the bottom part of the field of view. For a symmetric field of view this value is negative. |
| environmentBlendMode | enum | 1..1 | The type indicates the environment blend mode configuration. Defined values are OPAQUE, ADDITIVE and ALPHA_BLEND. Other values may be added. |
| actionConfiguration | Array | 0..1 | This contains a list of the actions that are to be defined by the SR client. |
| action | Object | 1..n | A definition of a single action object. |
| id | number | 1..1 | A unique identifier of the action. |
| actionType | enum | 1..1 | The type of the action state. This can be a Boolean, float, vector2, pose, vibration output, etc. |
| subactionPaths | string | 1..n | An array of subaction paths associated with this action. The split rendering client will provide the state of all defined sub-action paths. |
| extraConfigurations | Object | 0..1 | A placeholder for addition configuration information. |

#### 8.4.2.3	Processing Delay Adaptation Configuration Format

The configuration format defined in clause 8.4.2.2 with the additional fields defined in Table 8.4.2.3-1 shall be used for split rendering configuration exchange in processing delay adaptation procedure. This delayAdaptation object shall be present as part of the extraConfigurations Object as defined in clause 8.4.2.2 for extensibility of split rendering configuration format.

Table 8.4.2.3-1 - Configuration message format for Processing Delay Adaptation based on QoE metrics

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| delayAdaptation | Object | 1..1 | An object containing the configuration parameters for the processing delay adaptation based QoE metrics. |
| qoeMetrics | array | 1..1 | An array of the QoE metrics for which delay adaptation is considered. This qoeMetrics array may contain all or a subset of the QoE latency metrics negotiated in the Metrics reporting configuration defined in clause 8.11.3 of TS 26.510 [9]. |
| qoeMetricId | string | 1..1 | A unique identifier of the QoE metric within the scope of the split rendering session. The name of that QoE metric is chosen as unique ID, this name should be consistent with the name provided in the metrics reporting configuration as defined in clause C.1 of TS 26.113 [6]. |
| periodicity | string | 1..1 | The periodicity of the delay adaptation information for that QoE metric. This periodicity value shall be lesser than or equal to the samplingPeriod value of the MetricsReportingConfiguration defined in clause 8.11.3 of TS 26.510 [9]. Whenever a delay adaptation information message is sent, the SRC shall reset its timer to the value of the periodicity property, and it shall begin countdown of the timer again. |

NOTE: 	The target delay range for a QoE metric is delimited by the positiveCrossingThresholds and the negativeCrossingThresholds properties provided to the SRC in the clientMetricsReporting‌Configurations resource of ServiceAccessInformation as defined in clause 9.2.3.1 of TS 26.510 [9].

Next change

### C.1.2.2	SRC Capabilities

#### C.1.2.2.1	Overview

Requirements for UE-based SRC functionalities for following functions are defined in this clause:

-	Media Decoding

-	Media Encoding

-	Metadata Formats

The capabilities of the receiving UE are shared with the split rendering server prior to the start of the split rendering session.

#### C.1.2.2.2	Media Capabilities

The SRC shall support the media capabilities of a device type 3 as defined in TS 26.119 [4], clause 10.4.

#### C.1.2.2.3	Metadata Formats

XR-Pose-Cap 1: the SRC shall be able to retrieve one or more pose predictions for each view and for every frame to be rendered. The pose prediction shall be formatted according to clause 8.3.2.2.

XR-Pose-Cap 2: the SRC shall be able to retrieve and collect the user actions that occurred during an identified time interval. The action information shall be formatted according to clause 8.3.2.3.

XR-DelayAdapt-Cap 1: the SRC shall be able to handle the processing delay adaptation of QoE metrics in a split rendering session during an identified time interval. The processing delay adaptation information shall be formatted according to clause 8.3.2.4.

### C.1.2.3	SRS Capabilities

#### C.1.2.3.1	Overview

Requirements for network-based SRS functionalities for following functions are defined in this clause:

-	Media Encoding

-	Media Decoding

-	Metadata Formats

The capabilities of the SRC are shared with the SRS prior to the start of the split rendering session.

#### C.1.2.3.2	Video encoding

The SRS shall at least be able to support the encoding of video that complies to the capabilities in clause 10.4.3 of TS 26.119 [4].

#### C.1.2.3.3	Audio and Speech encoding

The SRS shall at least be able to support the encoding of audio that complies to the capabilities in clause 10.4.4 of TS 26.119 [4].

#### C.1.2.3.4	Video decoding

The SRS has no requirements for the decoding of video streams.

#### C.1.2.3.5	Audio and Speech decoding

The SRS has no requirements for the decoding of audio or speech streams.

#### C.1.2.3.6	Metadata Formats

The SRS shall support the exchange of Pose and action information as defined in clause 8.3.2.

The SRS shall support the exchange of processing delay adaptation information as defined in clause 8.3.2.4.

Next change

### C.1.3.2	SRC Capabilities

#### C.1.3.2.1	Overview

Requirements for UE-based SRC functionalities for following functions are defined in this clause:

-	Media Decoding

-	Media Encoding

-	Metadata Formats

The capabilities of the receiving UE are shared with the split rendering server prior to the start of the split rendering session.

#### C.1.3.2.2	Media Capabilities

The SRC shall support the media capabilities of a device type 1 as defined in TS 26.119 [4], clause 10.2.

If the device is a device type 2 as defined in TS 26.119 [4], clause 10.4, it shall also support the media capabilities of a device type 2 as defined in TS 26.119 [4], clause 10.3.

If the device is a device type 4 as defined in TS 26.119 [4], clause 10.5, it shall also support the media capabilities of a device type 2 as defined in TS 26.119 [4], clause 10.5.

#### C.1.3.2.3	Metadata Formats

XR-Pose-Cap 1: the SRC shall be able to retrieve one or more pose predictions for each view and for every frame to be rendered. The pose prediction shall be formatted according to clause 8.3.2.2.

XR-Pose-Cap 2: the SRC shall be able to retrieve and collect the user actions that occurred during an identified time interval. The action information shall be formatted according to clause 8.3.2.3.

XR-DelayAdapt-Cap 1: the SRC shall be able to handle the processing delay adaptation of QoE metrics in a split rendering session during an identified time interval. The processing delay adaptation information shall be formatted according to clause 8.3.2.4.

### C.1.3.3	SRS Capabilities

#### C.1.3.3.1	Overview

Requirements for network-based SRS functionalities for following functions are defined in this clause:

-	Media Encoding

-	Media Decoding

-	Metadata Formats

The capabilities of the SRC are shared with the SRS prior to the start of the split rendering session.

#### C.1.3.3.2	Video encoding

The SRS shall at least be able to support the encoding of video that complies to the capabilities in clause 10.4.3 of TS26.119.

#### C.1.3.3.3	Audio and Speech encoding

The SRS shall at least be able to support the encoding of audio that complies to the capabilities in clause 10.4.4 of TS26.119.

#### C.1.3.3.4	Video decoding

The SRS has no requirements for the decoding of video streams.

#### C.1.3.3.5	Audio and Speech decoding

The SRS has no requirements for the decoding of audio or speech streams.

The SRS shall support the exchange of processing delay adaptation information as defined in clause 8.3.2.4.

#### C.1.3.3.6	Metadata Formats

The SRS shall support the exchange of Pose and action information as defined in clause 8.3.2.

Next change

# C.2	Adaptive Split Rendering Profile

## C.2.1	Introduction

This profile defines procedures and requirements for SRS and SRC to support split rendering features beyond a remote rendering paradigm.

Adaptive split rendering profile allows the SRC to render some objects of a scene locally and the rendering split can be adapted between the SRS and SRC during a session. The adaptation of the rendering split may be triggered either by the SRS or the SRC to maintain a consistent QoE of the SR session or to accommodate changes in operating conditions. The triggers may be, for example, channel conditions, SRC or SRS conditions or defined by the application provider.

To successfully render two parts of a scene separately in a split fashion, additional aspects of the rendering process need to be considered. Two basic requirements are maintaining a coherent state of the scene between the SRS and SRC and awareness of rendering split. Another requirement is seamless composition and display of the media rendered by the SRS and SRC into a frame to be displayed.

The processing delay adaptation procedure can be used with the Adaptive split rendering profile. The processing delay adaptation procedure may include adjusting various round-trip delays between the SRC and the SRS during a split rendering session. The delay adaptation allows the SRS to adjust the rendering task delay to maintain a consistent round-trip delay (e.g., pose-to-render-to-photon or roundtrip interaction delay).

Next change

## C.2.2	Procedures and Call Flows

For adaptive split rendering, the general procedures and call flows in clause 5.2 are followed with the following additions and modifications.

-	The SRS and SRC should share a scene description.  The implementation details may vary. The application provider may decide whether to provide identical scene descriptions to the SRS and the SRC or whether to provide a truncated version of the scene description to the SRC.

Note: The Application Service Provider may provide the scene description resource to the SRS and SRC, for example, via M8 to SRC and via M2 to SRS.

-	The SRS and SRC agree on an initial rendering split during session negotiation and the states to be synchronized, for example, in Step 5 of the procedure in clause 5.2.1.2.

-	The initial rendering split and states to be synchronized are indicated in the SR configuration.

-	In the rendering loop, exchange of split adaptation messages and state synchronization messages between the SRS and SRC is supported.

When the processing delay adaptation procedure is used, the SRC shall send delay adaptation information to the SRS.

Figure C.2.2-1 illustrates a high level call flow set up and operation for a split rendering session which supports the adaptive split rendering profile.

Figure C.2.2.-1: High level call flows for Adaptive Split Rendering Profile

The steps are:

0.		In this optional step the SRC and the SRS acquire scene description of the scene to be rendered during the split rendering session. The actual implementation of delivery of the scene description by to the SRC and SRS is up to the application provider.

1.		The Presentation Engine discovers the split rendering server and sets up a connection to it. It provides information about its rendering capabilities and the XR runtime configuration, e.g the OpenXR configuration may be used for this purpose. States to be synchronized and the initial rendering split is negotiated during this step.

2.		In response, the split rendering server creates a description of the split rendering output and the input it expects to receive from the UE.

3.		The Presentation Engine requests the buffer streams from the MAF, which in turn establishes a connection to the split rendering server to stream pose and retrieve split rendering buffers.

4.		The Source Manager retrieves pose and user input from the XR runtime and state changes in negotiated states and possible requests from the Scene Manager.

5.		The Source Manager shares the pose predictions and user input actions , state changes and possible split adaptation messages with the split rendering server.

6.		a. The split rendering server uses that information to, update states, render the frame and possibly update the split.

b. The Scene Manager update states, renders a frame and possibly updates the split.

7.		a. The rendered frame is encoded and streamed to the MAF.

b. Possible split adaptation and state change messages are shared with the presentation engine,

8.		The received media frames decoded and processed,

9.		The raw buffer frames are passed to the Scene Manager, this includes the frames received from the SRS and the frames rendered locally by the PE,

10.		The scene manager prepares composition layers and their corresponding swapchain images.

11.		The swapchain images are forwarded to the XR runtime for composition and rendering,12.		The frames are composed and displayed.

The final composition of a frame from media received from the SRS and locally rendered objects depends on the application logic. Implementation guidelines in C.2.7 provide a simple example.

Next change

## C.2.3	Metadata Formats

### C.2.3.1	Split Rendering Configuration Format

The configuration format defined in clause 8.4.2.2 with the additional fields defined below shall be used for split rendering configuration exchange in adaptive split rendering profile.

| renderingSplit | Object | 1..1 | An object identifying objects to be rendered and where they are to be rendered (SRS or SRC), for example, as a dictionary with keys “SRS” and “SRC” and lists of object indices from a scene description or a scene graph |
| --- | --- | --- | --- |
| splitRenderingFeatures | array( SR_FEATURE_FLAGS) | 1..1 | A list of split-rendering features supported by the SRS and the SRC. The supported features flags are: ADAPTIVE SEAMLESS_ADAPTIVE DELAY_ADAPTIVE |
| synchronizedStatesInit | Object | 1..1 | An object identifying states to be synchronized between the SRS and SRC and their initial state |
| states | Object | 1..1 | A list of state identifiers, their current values |
| state | String/number | 1..n | Identifier of a state |
| initVal | String | 1..n | Initial value of the state |
| stateVals | Array | 1..1 | An array of values possible for the state |

These renderingSplit object shall be present as part of the extraConfigurations Object as defined in clause 8.4.2.2 for extensibility of split rendering configuration format.

### C.2.3.2	Split Adaptation Message Format

During a split rendering session, the operating environment of the split rendering server, the split rendering client or the network conditions may change. Consequently, the rendering split may need to be adapted to deliver a consistent QoE. When adaptive split rendering is enabled, the SRS or SRC shall request a new rendering split by sending a message of the type “urn:3gpp:split-rendering:v1:asrp:sr-split”. The message shall be conformant to the metadata message format specified in clause 8.3.3. The same message type shall be used to acknowledge, accept or reject the request by the receiver, with the message subtype identifying whether it is a request, acceptance, acknowledgement or rejection. The message shall follow the format in Table C.2.3.1-1.

Table C.2.3.2-1 Message format for split adaptation messages

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 | urn:3gpp:split-rendering:v1:asrp:sr-split |
| message | Object | 1..1 | Message content |
| subtype | string | 1..1 | An identifier of the subtype of the message, it may be a request (REQ) for new split or acknowledgement (ACK), acceptance (OK) or rejection of a request (NOK). |
| renderingSplitId | string | 1..1 | An identifier of the rendering split unique within the scope of the SR session |
| renderingSplit | Object | 0..1 | A object identifying objects to be rendered and where they are to be rendered (SRS or SRC). The message shall be a dictionary object . with keys “SRS” and “SRC”, and values corresponding to a key shall be a list of named nodes from the scene description being rendered in the SR session. The keys shall indicate where the objects named in the corresponding value list are rendered. |

Split adaptation messages indicating acceptance, acknowledgment or rejection of a split adaptation request may not include the renderingSplit Object.

### C.2.3.3	State Synchronization Message Format

During a split rendering session, various states associated with the scene being rendered may transition. Depending on the nature of the application being executed, a transition may occur at the SRS, at the SRC or at both the SRS and SRC. For the application execution to be consistent, some state transitions need to be synchronized between the SRS and SRC. The SRC and SRS may agree on which states to synchronize during session setup.  To synchronize state transitions during a split rendering session the SRS and SRC shall exchange messages of the type  “urn:3gpp:split-rendering:v1:asrp:sr-state” . The same message type shall be used to send a state synchronization update, acknowledge a state synchronization update or simultaneously send and acknowledge a state synchronization update. The state synchronization update messages shall be conformant with the meta-data message format defined in clause 8.3.3 and the message content shall be formatted as shown in Table C.2.3.2-1.

Table C.2.3.2-1 Message format for state synchronization messages

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 | urn:3gpp:split-rendering:v1:sr-state |
| message | Object | 1..1 | Message content |
| subtype | string | 1..n | An identifier of the subtype of the message, it may be a state synchronization update (SYNC), acknowledgment (ACK) or both (SYNC_ACK) |
| syncUpdateId | string | 1..1 | An identifier of the synchronization update unique within the scope of the SR session |
| synchronizedStates | Object | 1..1 | An object identifying states that are synchronized between the SRS and SRC and their current state. Only states that have transitioned may be exchanged |
| states | Object | 1..1 | A list of state identifiers, their current values and last change time |
| identifier | String/number | 1..n | Identifier of a state |
| val | Object/String/number | 1..n | Value of the state |
| lastChangeTime | number | 1..1 | The timestamp of the last change in state |

Split adaptation messages indicating an acknowledgment of a state update may not include the synchronizedStates Object.

### C.2.3.4	Seamless Adaptive Split Message Format

An SRC that supports the adaptive split rendering with seamless adaptation shall support the seamless adaptive split message format defined in Table C.2.3.4-1.

The message shall be conformant to the metadata message format specified in clause 8.3.3. The same message type shall be used to acknowledge, accept or reject the request by the receiver, with the message subtype identifying whether it is a request, acceptance, acknowledgement or rejection.

Table C.2.3.4-1 Message format for seamless adaptive split messages

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| Id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 | urn:3gpp: split-rendering: v1: asrp:sr-split-seamless |
| message | Object | 1..1 | Message content |
| subtype | string | 1..1 | An identifier of the subtype of the message, it may be a request (REQ) for new split or acknowledgement (ACK), acceptance (OK) or rejection of a request (NOK). |
| renderingSplitId | string | 1..1 | An identifier of the rendering split unique within the scope of the SR session |
| renderingSplit | Object | 0..1 | An object identifying objects to be rendered and where they are to be rendered (SRS or SRC). The message shall be a dictionary object. with keys “SRS” and “SRC”, and values corresponding to a key shall be a list of named nodes from the scene description being rendered in the SR session. The key ‘SRC is used for objects that are to be rendered by the SRC and key ‘SRS is used for objects that are to be rendered by the SRS, when seamlessSplit conditions are not met. |
| seamlessSplit | object | 0..1 | An object that if present indicates a seamless adaptation of the rendering process when possible. |
| radius | number | 1..1 | A distance in meters that defines a sphere centered at the SRC, such that preferential rendering is used for objects that lie within this sphere. An object lies within the preferential rendering sphere if for all or some points defining the collider associated with the object the distance from the SRC is less than the radius. |
| type | string | 1..1 | A string that indicates the type of preferential rendering to be used for the objects defined by the key ‘SRC’ in renderingSplit. The following values are supported: “Local”: The objects are rendered at the SRC when they are within the sphere define by R and rendered by the SRS when they are outside of it. “LOD”: The SRS renders the objects when they are within the radius R at a high fidelity. For example, 3D models with a higher Level-of-detail may be rendered. |

NOTE: 	Deterministic calculation of objects to be rendered by the SRC, by the SRS, and by the MAP needs to be ensured. The floating point representation used by the SRC, SRS and the MAP for physics calculation and latency between the SRC, SRS and the MAP may impact determinism of such calculations.

### C.2.3.5	Adaptive split rendering with eye status information

If an SRC that supports the adaptive split rendering with eyes status information, it shall support the message format defined in Table C.2.3.5-1. The eye status information shared by SRC to SRS during the adaptation procedure shall be in JSON format according to the Metadata Data Channel Message Format defined in clause 8.3.3. The message type shall be “urn:3gpp:split-rendering:v1:sr-split-eyeinfo”.

Table C.2.3.5-1 Message format for eyes status information

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| eyesInfo | Object | 1..1 | An array of eye information objects corresponding to past and current eyes status for the viewer. |
| eyesStatus | number | 1..1 | The current eyes status, e.g.: 0: Eyes are open 1: Eyes are closing 2: Eyes are closed 3: Eyes are opening |
| eyesStatistics | Object | 1..1 | The current viewer eyes statistics |
| averageDuration | number | 1…3 | Array of average eyes duration for the viewer, e.g.: [1] represents average closing time, [2] represents average closed time [3] represents average opening time |
| averageInterval | number | 1..1 | Average interval between two eye blinking. |
| elapsedTime | Number | 0…n | Time expressed in milliseconds since the last eyes open status. |

### C.2.3.6	Asset Request Message Format

An SRC that supports the split rendering shall support the asset request messages below.

Table C.2.3.6-1 Message format for asset requests

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 | urn:3gpp:split-rendering:v1:asrp:sr-asset |
| message | Object | 1..1 | Message content |
| request | string | 1..1 | A request for assets, identifying the assets requested, for example as a list of nodes of a scene graph or a list of URIs which reference assets for nodes in the scene graph. |

Next change

### C.2.4.2	Metadata Formats

XR-Pose-Cap 1: the SRC shall be able to retrieve one or more pose predictions for each view and for every frame to be rendered. The pose prediction shall be formatted according to clause 8.3.2.2.

XR-Pose-Cap 2: the SRC shall be able to retrieve and collect the user actions that occurred during an identified time interval. The action information shall be formatted according to clause 8.3.2.3.

XR-ObjId-Cap 1: the SRC shall be able to receive, retrieve and collect identifiers of objects in a scene being rendered by the SRC in a split rendering session during an identified time interval. The rendering split and seamless rendering split information shall be formatted according to clause C.2.3.2 and C.2.3.4 respectively.

XR-ObjState-Cap 1: the SRC shall be able to receive, retrieve and collect state changes in identified objects in a scene being rendered in a split rendering session during an identified time interval. The state information shall be formatted according to clause C.2.3.3

XR-EyeStatus-Cap 1: the SRC shall be able to retrieve and collect the eye status that occurred during an identified time interval. The eye status information shall be formatted according to clause C.2.3.5.

XR-Asset-Cap 1: the SRC shall be able to receive identified objects in a scene being rendered in a split rendering session by sending the asset request message to a SRS. The asset request message shall be formatted according to clause C.2.3.6.

End of changes